VersaWare Custom Audio Digitizing

Adrienne Cousins

Littleton, MA

508-486-3083 voice

The files in this SHORTCI3 zip are sound clips from the movie

SHORT CIRCUIT.  All files were recorded at a rate of 180 or 15.7Khz.

stupid.v8       Number 5 "Stupid, foolish, gullable, doltish, dumbell,

lamebrain, shutup, silent, hush, sitonit!"

snowbl.v8       Number 5 "Hey LaserLips, your mama was a snowblower!"

fiddle.v8       Number 5 "No way Jose, I'm fit as a fiddle!"

------------------------------------------------------------

All sound files are industry standard PCM (Pulse Code Modulation) 8-bit

format with a 16-byte header record that includes the playback rate.

These files can be played with any of the following players depending on

the device used for output.

When using the PC's internal speaker, the best utility is SAYINT.  The

default playback rate for SAYINT is 180 or 15.7Khz.  Type SAYINT with no

arguments for help text.

SAYINT <filename>       (output to internal speaker)

If you are using a SpeechThing, SoundMaster or VoiceMaster I or II then

use SAY.  SAY will automatically determine playback speed from the header

record, and default to 132 (9.6Khz) if no header is found.  Type SAY with

no arguments for help text.

SAY <filename> /Plpt1   (output to SpeechThing)

or      SAY <filename> /Pisp    (output to internal speaker)

Or you may optionally use REPLAY (not included in this archive).  The

default playback speed for REPLAY is 3, which is equivalent to a rate of 181

or 16Khz.

REPLAY <filename>       (output to internal speaker)

------------------------------------------------------------------------------

Player Program rate comparisons  (* indicates default)

Freq    SAY     SAYINT  REPLAY

Khz    rates   rates   speeds

--------------------------------

4.7      1

6.3     66      66

9.6    132*    132

11.5    152     152     0

12.5    161     161     1

14.0    171     171     2

15.7    180     180*

16.0    181     181     3*

17.8    189     189

18.5    192             4

22.0    202             5

27.0    212             6

1193    255

rate formula  FREQ = 1193180 / (256-RATE)

SAY    range 1-255 default 132 (9600hz)

SAYINT range 66-189 default 180 (15700hz)

REPLAY range 0-6 default 3 (16000hz)