C A S C A D E   D R A W

Operations Manual

Information  in this manual is subject to change  without  notice

and  does  not  represent a commitment on  the  part  of  Cascade

Graphics Systems or its vendors.  The software described in  this

manual,  as  well  as the manual text, is  protected  by  Federal

Copyright  Laws and may not be copied or reproduced  without  the

express written permission of Cascade Graphics Systems, Inc.

Last Revision:  4/4/90

Copyright 1990, Cascade Graphics Systems, Inc.

All rights reserved.

Cascade Graphics Systems, Inc.

18001 Cowan

Suite C

Irvine, California  92714

(714) 757-2972

C A S C A D E   D R A W

Cascade DRAW is a functional program which demonstrates

the wide range of Computer-aided Drafting and Design products

available from Cascade Graphics Systems.

Cascade DRAW contains the basic features found in all Cascade

products.  Cascade DRAW also contains advanced features like

"Flip Screen" user interaction, and a two-digit command "On-

The-Fly" expert mode.  These are a few of the productivity

features not found in other CADD products.

Use Cascade DRAW for all your light drafting needs.

If your drafting needs require more functionality, or larger

drawings, consider Cascade's other products.  From architecture,

to manufacturing, to electronics, or most other fields, Cascade

has the productivity solution to fit your needs and your

budget.

OTHER PRODUCTS AVAILABLE FROM CASCADE

CasCAD I: Is To The Computer What Pencil Is to Paper...$19.95

Contains all of the features found in DRAW, plus mouse

drivers and the ability to create larger drawings.  CasCAD I

comes complete with a in-depth 65 page manual, instruction

guide, and tutorial.

CasCAD II: For Home or Office..........................$65.00

Includes hatching, automatic dimensioning, plotting, and

other features for home/office and desktop publishing needs.

Also supports most popular mouse and tablet input devices.

CasCAD III: CADD For The Drafter & Designer...........$350.00

Includes advanced CAD features when productivity and speed

are important.

ATLAS: Advanced CADD Solutions For The Engineer.......$995.00

Special software applications tools for all drafting and

engineering disciplines make Atlas the total 2D CAD solution.

New tasks can be created using graphics programming routines.

Atlas includes all specialized software application tools and

symbol libraries.

SOFTWARE APPLICATION TOOLS AND OTHER PRODUCTS

All Cascade products can be enhanced through the addition of

specialized programs, symbol libraries, and application

programs called Tools.

GO (Graphics Output) for CasCAD I.  GO supports the majority

of popular plotters.  Line width, color, drawing size, and

other functions can be manipulated by GO.

DXF In/Out for CasCAD II.  DXF is a data exchange format

program which converts CasCAD II drawings to AutoCAD,

VersaCAD, and other popular CAD programs.

FONT EDITOR for CasCAD II.  Unlimited font styles can be

created with the Font Editor.

ARCHITECTURAL TOOLS for CasCAD III.  Specialized

architectural applications such as fractional dimensioning,

vanishing points, sketch modes, and other linesstyles are but

a few of the Architectural Tools.

MECHANICAL TOOLS for CasCAD III.  The most frequently used

manufacturing symbols are automatically drawn.  Flanges are

also automatically drawn.  Bezier and B-Spline curves can be

created by selecting points on the drawing.

ELECTRONIC TOOLS for CasCAD III.  Drawing Bus lines, ICs, and

electronic symbols are easy to accomplish.  ICs and symbols

are automatically inserted when lines are drawn.

SYMBOL LIBRARIES for CasCAD II and CasCAD III.  Predrawn

architectural, mechanical, electrical, and petrochemical

symbols increase productivity.  All symbols are created in

accordance with industry specifications.

For a free brochure call or write us at...

Cascade Graphics Systems             To order call toll-free

18001 Cowan,Suite C                     (800) 523-7188

Irvine, CA  92714

(714) 757-2972

Table of Contents

1.0  USING THIS MANUAL ...........................1

The CASCADE DRAW Program ....................1

Tutorial Sections ...........................1

Menu Screens ................................1

Typographical/Style Conventions .............1

Default/Highlighted Options .................2

Other Terms You'll Need to Know .............2

2.0  INSTALLATION ................................3

Installing on Floppy Disk ...................3

Installing on the Hard Disk .................3

The Logon: Screen ...........................4

The Configuration Menu ...... ...............4

Video Device ...........................4

Input Device ...........................5

Input Port .............................5

Printer Port ...........................5

Esc/Exit ...............................5

Configure System .......................5

The CONFIG.SYS File .........................5

The DRAW.BAT File ...........................6

Tablet Initialization .......................6

Set Up Tablet ..........................7

3.0  STARTING A DRAWING SESSION ..................7

Logging On ..................................7

QUIT ........................................8

INIT.........................................8

The Opening Menu ............................8

Active Drawings .............................8

11 CREATE Drawing ...........................8

16 LogOff ...................................8

10 EDIT Drawing .............................8

33 Next Page ................................8

50 Drive: ...................................9

51 Path: ....................................9

4.0  LAYING OUT A DRAWING ........................9

Naming A Drawing ............................9

Units of Measure ............................10

27 Scale: 1.00 ..............................10

Drawing Size ................................10

Text/Graphics Screen--<Esc> Key .............10

ii

5.0  THE MASTER/AUXILIARY MENU SCREENS ...........11

Status Line .................................11

Cursor and Prompt Lines .....................11

Drawing Name ................................11

Auxiliary Menu ..............................11

Selecting Menu Options ......................11

6.0  OBJECT PLACEMENT ............................12

Typing Coordinates from the Keyboard ........12

Absolute Coordinates (x,y) ..................12

Relative Coordinates [x,y] ..................12

Polar Coordinates <x,y> .....................13

Saving Coordinates to Function Keys .........13

Locating Coordinates with an Input Device ...14

Graphics Tablet and Stylus ..................14

Mouse .......................................14

Using the Keypad as an Input Device .........14

7.0  DRAWING AIDS ................................16

Grid Display - /G ...........................16

Isometric Grid - /X .........................16

Changing Grid Size - /N .....................16

Working with the Input Device ...............16

Graphics Cursor .............................16

Picking Points and Objects ..................16

Hit Window ..................................17

GRID - 00 ...................................17

FREE - 01 ...................................17

SNAP - 02 ...................................17

NON-AXIS and AXIS - 03 / 04 .................17

Assigning Pen Numbers - /P ..................18

Assigning Intensity - /I ....................18

Assigning Color - /C ........................18

Assigning Drawing Levels - /L ...............19

Important Keys and Features .................19

Backspace Key ..........................19

Enter Key ..............................20

Rejecting an Operation .................20

Caret / Plus Keys ......................20

Error Messages .........................20

Zoom ...................................21

Pan ....................................21

Full Window ............................21

Redraw .................................21

Percent Full (0%) ......................21

Saving/Exiting Your Drawing .................21

Backup Drawings .............................22

iii

8.0  DRAW Options ................................23

27 Line .....................................23

Linestyle (11, 12, 13, 14) ..............23

20 +NO Fill / +Solid ....................23

22 MoveToPnt ............................24

28 Rectangle ................................24

16 Ang= 0.000 ..........................24

29 Arcs .....................................25

15 +DIR = CW / CCW .....................25

16 ARC - 3 Point .......................25

17 Arc - S/C/End .......................25

18 Arc - S/E/Rad .......................25

30 Circle ...................................25

15 Circle - Pick .......................26

16 Circle - 3 Pnt ......................26

17 Concentric ..........................26

31 Ellipse ..................................26

13 Maj/Min .............................26

19 +Full Ellipse / +Half Ellipse .......26

32 Arrows ...................................26

11 +Flow (Arrowhead Styles) ............27

12 Reverse Dir. ........................27

13 +Solid / +Open ......................27

16 +Place At End .......................27

18 Angle= 0.00 .........................27

19 ArwLEN = 0.200 ......................27

20 ArwLTW = 1.800 ......................28

33  Points ..................................28

Point Styles ...........................28

34 Move .....................................28

16 Drag Object .........................28

11 Origin ..............................28

12 Rectangle ...........................30

35 Copy .....................................30

16 Drag Object .........................30

11 By Origin ...........................30

12 By Rectangle ........................30

36 Delete ...................................30

11 Object ..............................30

12 Rectangle ...........................30

37 Rotate ...................................30

11 Ang= 90.000 deg .....................31

12 Repeat=0 ............................31

38 Scale ....................................31

11 Xscale= / 12 Yscale= ................31

15 Calc. SCALE .........................31

39 Mirror ...................................32

11 +Mirror X / +Mirror Y ...............32

12 +Copy OFF / +Copy ON ................32

iv

41 Group ....................................32

11 Pick Objects ........................32

12 Rectangle ...........................33

11 Unnest TOP / Unnest ALL .............33

43 Move Origin ..............................33

9.0  51 Text .....................................34

11 Xsize= 0.125 .............................34

12 Ysize= 0.125 .............................34

13 Xstep= 0.125 .............................34

14 Ystep= 0.000 .............................35

15 Angle = 0.00 deg .........................35

16 Slnt= 0.000 ..............................35

17 Font = 0 .................................36

10.0  64 ATTR Edit ................................36

11 +Edit / +Display .........................36

12 +Member / +Group / +All ..................36

14 All Defaults .............................37

15 Edit KIND ................................37

16 Edit PEN .................................37

17 Edit LEVEL ...............................37

18 Edit INTENSITY ...........................37

19 Edit COLOR ...............................38

20 Edit STYLE ...............................38

11.0  65 Line Edit ................................39

12 Gap Segment ..............................39

13 Trim/Extend ..............................39

14 Erase Segment / 15 UnErase Seg ...........39

16 Move Vertex ..............................39

12.0  Graphics Screen Utilities ...................40

88 Zoom .....................................40

Zooming with Function Keys .............40

Re-programming the Zoom Function Keys ..40

89 Pan ......................................41

Panning with the Numeric Keypad ........41

90 FullWindow ...............................41

99 Redraw ...................................41

13.0  94 Deactivate ...............................42

10 PACK DRAWING .............................42

11 Name: ....................................42

12 Path: ....................................42

15 Save Drawing .............................43

16 Exit .....................................43

v

14.0  97 PrintPlot ................................44

10 Standard / 11 Rotated ....................44

15.0  98 DrawDefs .................................45

11 Ref Grid = 10 ............................45

12 DOT Grids / LINE Grids ...................45

13 Ref Color-Blue ...........................45

14 Wrk Color-Aqua ...........................45

16 Set Range On / 17 Set Range Off ..........45

18 Hitwin= 0.150 in .........................46

28 X-Grid= 1.00 in / 29 Y-Grid= 1.00 in .....46

vi

1.0  USING THIS MANUAL

This manual covers the installation and operation of the  CASCADE

DRAW  program. To use this program, follow the instructions  pro-

vided in the "Installation" section for copying the programs from

floppy diskette to your hard disk drive; or running the  software

on floppy diskette.

1.1  The CASCADE DRAW Program

Your DRAW program works using a combination of hardware and soft-

ware  to create drawings. Hardware is the actual computer  equip-

ment  (the  disk  drives,  monitors,  plotter,  input   devices).

Software is the computer programs contained on the SYSTEM DISK.

The software is designed to operate on IBM compatible PC, XT,  AT

and  386  computers.  A minimum of 512k  of  RAM  (Random  Access

Memory) is required for the system to function properly. You must

also  have two floppy disk drives, or a hard disk and one  floppy

disk drive to run the program.

1.2  Tutorial Sections

The fastest way to learn how to use the program is to try it.  We

suggest  that you create a practice drawing for  experimentation.

The  sections  "Starting  a Drawing Session" and  "Laying  Out  a

Drawing" are step-by-step tutorials that will help you to lay out

a drawing and get you going.

1.3  Menu Screens

The  DRAW program is a "menu driven" program.  This means that  a

list  of  options will be displayed on the screen.  To  make  the

program perform a certain operation (task) you select one of  the

options on the menu.

Each  numbered option is documented in the order they  appear  on

the  menu screen.  Don't worry if you can't find all the  options

that  are documented on your computer screen; some  menu  options

are not displayed until after another menu option is selected.

1.4  Typographical/Style Conventions

Throughout this manual you will see words in bracketed type, such

as:

<Enter>

<Space Bar>

page 1

This  indicates  that you should press a particular  key  on  the

keyboard. When the text tells you to select an option by pressing

a number or symbol, such as:

Press <20>

you should press the keys numbered 2 and 0.

As  a  matter  of convenience and clarity, we  refer  to  picking

points with your "stylus."  Unless otherwise indicated, this also

refers to any input device.

1.5  Default/Highlighted Options

Throughout your drawing session certain options (and values)  are

pre-selected for you.  These are called "default" options and are

the most commonly used options and settings. Default options  are

highlighted on you menu.  A highlighted option indicates that  it

has been selected for use.

1.6  Other Terms You'll Need To Know

A  "cursor"  is a symbol displayed on the  computer  screen  that

shows you where the next character will be typed or where a point

will be picked using your mouse or stylus.  There are two differ-

ent types of cursors found in the DRAW program: a blinking under-

line (text cursor); and an x-shaped symbol used for drawing  (the

graphics cursor).  The graphics cursor is described in detail  in

the section "Drawing Aids."

A "prompt" is a message that the DRAW program displays  instruct-

ing  you to do something.  Prompts usually appear at the  top  of

your computer screen.

An "error message" is a prompt that tells you that you have  made

a mistake or selected an option that isn't available.  The  error

message usually tells you what is wrong and what you should do.

Some  options  change their menu description when you  press  the

number.   These  are called "toggle" options. They allow  you  to

select  one  of several choices.  When the manual  tells  you  to

"toggle"  an option, it means to press the menu number until  the

option  displays  the  choice you  want.   Most  toggled  options

display a + (plus) sign in front of the description.

page 2

2.0  INSTALLATION

The programs (software) which operate CASCADE DRAW may be run  on

two  floppy disk drives (A: and B:) or installed on a  hard  disk

drive  (C:  or D:).  Follow the instructions in  your  computer's

operations  manual for starting (booting) the computer  with  the

"Disk Operating System" (DOS) disk.  Wait until the DOS prompt is

displayed.

Before  installing your software, you should make a back-up  copy

of the disk labeled SYSTEM DISK and store the original in a  safe

place.  Follow the instructions in the DOS manual for  using  the

DISKCOPY command.

2.1  Installing on Floppy Disk

If  you  are going to use the DRAW program on floppy  drives  you

should  make a copy of the original SYSTEM DISK (see above).  Use

the copied diskette as the program disk for running CASCADE DRAW.

You should also have a blank, formatted diskette ready to use for

storing  your drawings.  Use the DOS "FORMAT" command to  prepare

the blank diskette.

To  begin  the program and install it for your  computer  system,

insert the SYSTEM DISK in drive A; type the word

DRAW

at the A> prompt and press <Enter> to start the program.   Follow

the  instructions  in the "Logon: Screen" section for configuring

(CONF) and initializing (INIT) the program diskette.

2.2  Installing on the Hard Disk

To install the program on your hard disk drive, make a new direc-

tory by typing this command at your DOS prompt:

MD \DRAW

Press the <Enter> key to execute the command.  Insert the  SYSTEM

DISK  into the A: floppy drive and close the door/latch.  At  the

DOS prompt, type the command:

COPY A:*.* \DRAW

and press the <Enter> key. All the files on the floppy drive will

be copied into the directory. Now type:

CD \DRAW

page 3

and press <Enter> to change directories;  then type:

DRAW

and press <Enter> to start the program.  Follow the  instructions

in the "Logon: Screen" section for configuring the system for your

hardware.

2.3  The Logon: Screen

After you start the DRAW program, the "Logon: Screen" appears and

a  blinking  cursor  is displayed next to  the  "Logon:"  prompt.

Before  you may begin using CASCADE DRAW, you must configure  the

software so that it operates properly with your computer's  hard-

ware. Use the down-arrow key to move to:

Change Hardware Configuration

and press <Enter> to display the Configuration Menu.

2.4  The Configuration Menu

There  are four different options (fields) that  must  configured:

the "Video Device",the "Input Device", the "Input Port" and

the "Printer Port."

2.4.1  Video Device

The  Video Device (monitor) field has six  different  single

screen options from which to select:

CGA 320x200 4-Color

CGA 640x200 1-Color

EGA 640x350 8-Color

EGA 640x480 (AutoSwitch 480)

VGA 640x480 8-Color

Hercules 720x348

Note: Cascade's "Phoenix" and "Atlas" CAD Programs

offer dual screen video options with higher

resolutions.

The  single  screen CGA four color option has  already  been

pre-selected  (the default setting).  To display one of  the

other options listed above, press the space bar. Refer to the

original materials that  came with your computer to determine

the setting appropriate for your system.

page 4

2.4.2  Input Device

To select the Input Device field press the down-arrow key on

the  numeric  keypad.   (Press the up-arrow to return to  the

Video Device field.)  The Input Device field will be highlighted

and  you  may select one of three different input devices:

SummaGraphics BIT Pad I or Compatible

Keyboard  (Using the numeric keypad)

Microsoft Serial Mouse, or Compatible

If  you  select  "Keyboard",  the  numeric  keypad  on  your

computer's  keyboard will be used as your input device.  See

the chapter "Object Placement" for instructions on operating

the keypad as an input device.

2.4.3  Input Port

The COM ports are used for commnunication with input

devices, plotters, and for file transfers.

The standard COM port configuration is:

COM1: - tablet or mouse.

COM2: - plotter and file transfer.

2.4.4  Printer Port

The printer ports are LPT1:, LPT2:, and LPT3:.

LPT1: is most often used for printer output.

LPT3: is usually a printer attached to a network.

In most cases the printer should be assigned to LPT1:.

2.4.5  Esc/Exit

You may terminate and exit the configuration process at  any

time by pressing the <Esc> key on the keyboard.

2.4.6  Configure System

Now check all selections to be sure that you have made  the

correct choices; then, press the down-arrow key to highlight

the Configure System field.  Press <Enter> to configure  the

system.

2.5  The CONFIG.SYS File

For optimum performance the following lines should appear in the

CONFIG.SYS File in the root directory.

To display the contents of the CONFIG.SYS file change to the root

directory (CD \), type the command:

TYPE CONFIG.SYS

page 5

and press the <Enter> key. The file must be edited (if necessary)

to read:

BUFFERS=20

FILES=15

You  may create and edit this file using the COPY CON command  in

DOS.  At the DOS prompt type:

COPY CON CONFIG.SYS

and  press <Enter>. The cursor will move to the next line  below.

Now, type the three lines as shown above, pressing <Enter>  after

each line.

Once  the lines have been typed,  hold  down  the  <Ctrl>

(control)  key  and  press  <Z>.  The  symbol  ^Z  will   appear,

indicating the end of the file.  Press <Enter> and a message will

be displayed confirming "1 file(s) copied". (If you made a typing

error, just repeat the process again.)

If you change the CONFIG.SYS file you must re-boot to read the new

file into memory.

Note:

The  files MODE.COM, MOUSE.COM and  MOUSE.SYS  are

not supplied in this software package. MODE.COM is

a  DOS command. MOUSE.COM and MOUSE.SYS are  files

necessary  to  operate the  Microsoft  Mouse  (and

compatibles).  They  are  supplied  by  the  mouse

manufacturer.

2.6  The DRAW.BAT File

Located in your DRAW directory (or on floppy disk if you are  not

using  the hard drive) will be a file named DRAW.BAT.  This is  a

DOS  "batch" file that starts the DRAW program.  In  some  cases,

this  file may need to be edited to correct path names to  direc-

tories  you have created  Generally, though, it  should  function

properly  as  is. (Refer to your DOS manual for  instructions  on

editing and using .BAT [batch] files.)

2.7  Tablet Initialization

After  editing the CONFIG.SYS file, change to the DRAW  directory

and  re-start the program by typing DRAW and pressing <Enter>  at

the DOS prompt.

The LogOn: Menu will be displayed again. This time type:

YOUR INITIALS

and   press  <Enter>.

page 6

2.7.1  Set Up Tablet

If you have specified a tablet for your input device, you

must set up the tablet for the CAD System.  The system will

ask you to define the "active drawing  area"  on your

graphics tablet or digitizer as well as the "Group" and

"Command" menu areas.  The Command menu area is a module  of

small  boxes  that  allow you pick menu  options  with  your

stylus  or  puck (instead of typing menu  numbers  from  the

keyboard).  The Group menu is another module of small  boxes

that allows you to pick frequently used objects and  symbols

from a "library."  (Although you will be prompted to  create

a  Group  menu  area, the DRAW program does  not  offer  the

LIBRARY  option.  However,  it may  be  found  in  Cascade's

"Phoenix" and "Atlas" CAD/Drawing programs.)

Follow the prompt instructions which will ask

you  to pick the lower left and upper right corners  of  the

drawing,  Group and Command areas.  Some tablets  will  have

marks indicating these points. Use your stylus, puck or joy-

stick to select the points.

If  your table does not accommodate an overlay for the  Com-

mand  and Group Menus, pick any points outside  the  drawing

area when the prompts ask you to mark these areas.

The mouse and the numeric keypad (keyboard), do not  require

picking the corners as described above.

3.0  STARTING A DRAWING SESSION

Change  to the DRAW directory. At the DOS system prompt type  the

word DRAW on your keyboard and press the <Enter> key. You may use

either  upper or lower case letters. You have now  activated  the

DRAW program and the "Logon Menu" will be displayed.

3.1  Logging On

The  cursor  will be blinking next to the "Logon:"  prompt.   The

DRAW program is telling you that it's ready to accept information

which  can be entered from the keyboard.  Type your name or  ini-

tials  at  the Logon: prompt and press <Enter>.  This  is  called

Logging On. All the drawings you create during this session  will

have your name automatically recorded into the computer with  the

rest  of  the drawing's information. Once you've logged  on,  the

Opening Menu will appear.

Note:

If  you haven't configured your tablet the tablet

setup prompts will be displayed.  See the section

on Tablet Initialization for instructions on setting

up your tablet.

page 7

3.2  QUIT

ESC - exits the Logon menu and returnes you to DOS.

16 Logoff - exits the drawing management menu.

94 Deactivate, 16 Exit - exits the current drawing.

3.3  The Opening Menu

Every  time you begin or end a drawing session you will  see  the

"Opening  Menu."   From this menu you may  activate  the  primary

tasks of The DRAW program.

The  blinking cursor at the top of the Opening Menu  screen  lets

you  know  that the DRAW program is ready for you  to  select  an

operation  by typing a two-digit number on your  keyboard.  Don't

press  the <Enter> key after typing the number. You only need  to

press the corresponding digits. The option will be highlighted.

3.4  Active Drawings

Drawings  that  have already been created are  listed  under  the

heading  **** Active Drawings ****.  To select an active  drawing

to  be  edited, press the corresponding two-digit number  of  the

drawing  you want to work with; the number is listed to the  left

of the drawing's name.

3.5  11 CREATE Drawing

Press  <11> to lay out a new drawing.  This option is  fully  ex-

plained in the section, "Laying Out A Drawing."

3.6  16 LogOff

Press  <16> to end your drawing session and return to the  Logon:

prompt.

3.7  10 EDIT Drawing

This option allows you to work with an existing drawing by typing

the  drawing's name from the keyboard. It is  generally  quicker,

however, to retrieve a drawing from the Active Drawings menu.

To use this option, press <10>, type the drawing's name and press

<Enter>.

3.8  33 Next Page

If  you have more than 16 drawings in the Active Drawings  direc-

tory,  33 Next Page is displayed.  It allows you to see the  next

page  of drawings. Press <33> to "page" through drawings  in  the

directory.

page 8

3.9   50 Drive:

This option determines which disk drive the DRAW program will use

to  save and search for drawings.  The default setting  for  this

option is the A: drive.  To change the drive press <50> and  type

a new drive letter (A, B, C, etc.); then, press <Enter>.

3.10  51 Path:

The Path option allows you to specify a DOS directory search path

for your drawings. For example, if you wanted to search for draw-

ings and save them in a sub directory to \DRAW called "Drawings",

you would press <51> and type the path name:

\DRAW\DRAWINGS

then, press the <Enter> key.  The new path name will be displayed

next to the option.

Note:

You  must  first create a directory with  the  DOS

Make Directory command (MD) before you can specify

its search path.

4.0  LAYING OUT A DRAWING

Imagine  that  your monitor and graphics tablet (or  other  input

device) are your art board and paper. The stylus attached to your

tablet will act as your art tools, pens, pencils, triangle,  etc.

The keyboard will be used to help select the tools you need,  and

enable you to move around the board.

To lay out a new drawing, select 11 CREATE Drawing from the  menu

by  pressing  <11>. (Remember: Don't press the <Enter>  key;  the

command will be understood.)

4.1  Naming A Drawing

The prompt at the top of the screen--below the flashing  cursor--

is  now  asking you to "Enter New Drawing Name."  Select  a  name

with a maximum of eight letters or numbers (you can use a combin-

ation  of both).  Type the name from your keyboard and press  the

<Enter> key.

Note:

Do  not use " ^ , * P : ; @ # - . $ % { } [ ]  and

other  symbols in drawing names. If you  type  any

spaces in the drawing name they will be automatic-

ally removed, so it is suggested that blank spaces

be filled with an underscore (__).

page 9

4.2  Units of Measure

Once  you  have  properly named your drawing a  new  screen  will

appear.  On  this screen you may choose the scale  and  units  of

measurement (inches, centimeters, feet, etc.) for your drawing.

Option "17 IN" is highlighted; this is the default option select-

ed. If you wish your drawing to be in a different unit of measure

select one of the other options (11 through 20) displayed.  Press

the corresponding number to the left and the option will be high-

lighted, indicating your choice.

4.3  27 Scale: 1.00

This  option lets you set the scale factor of your  drawing.  The

default  scale  is  1:1.  If you wish to set  a  different  scale

factor,  press <27>; type a new scale factor and  press  <Enter>.

The new scale will be displayed next to the menu option.

For example, if you wanted a half-scale drawing you would enter a

scale  factor  of <.5>.  If you wanted a 2 to 1 (2:1)  scale  you

would type <2.0>.

4.4  Drawing Size

Now  that the drawing's scale and unit of measurement  have  been

chosen, you may change the drawing's size with options 29 and 30.

The default setting is for an 11x17 inch drawing.  To change  the

width  or height press <29> (Width) or <30> (Height); type a  new

value and press <Enter>.  The new value will be displayed next to

the  menu  option.  The size will be in the unit of  measure  you

have selected.  Select <42> to continue.

4.5  Text/Graphics Screen--<Esc> Key

At  this  point your drawing has been laid out and  you  will  be

working  with  two different screens: the "text screen"  and  the

"graphics screen."  Press the <Esc> key to switch back and  forth

(toggle) between the two screens.

The  graphics  screen will display your drawing which  is  now  a

blank  sheet.  It is surrounded by a dashed line.  This  is  your

drawing  border and represents the edge of the paper.  Your  text

screen will display the "Master Menu."  Using the Master Menu and

drawing  with  your  input device is  covered  in  the  following

chapters.

page 10

5.0  THE MASTER/AUXILIARY MENU SCREENS

The "Master" and "Auxiliary" menu screens let you select  drawing

"tasks"  (options) and move around the drawing.  They  also  tell

you the current "status" of your drawing, the drawing's name, and

other  important information you need to know.  Each  task/option

on the menu is explained in separate sections in this manual. The

following  information is provided as an overview to help  orient

you to the screen layout.

5.1  Status Line

Across the top of the screen is the status line. It tells you the

current  status  of several of the DRAW program  functions  which

make  drawing  easier.  We will explain these  functions  in  the

chapter "Drawing Aids".

5.2  Cursor and Prompt Lines

Below the status line you will see the flashing text cursor;  and

below that, the "prompt line". The prompt line tells you what  to

do next, and anything you type from the keyboard is displayed  on

the cursor line.

5.3  Drawing Name

For reference, the name of the drawing you are currently  working

with is highlighted at the bottom of the menu.

5.4  Auxiliary Menu

To  the left of the Master Menu is an area with  the  highlighted

headline:  *** AUXILIARY MENU ***.  When you select  a  two-digit

option from the Master Menu, another menu of numbered sub-options

(auxiliary options) will be displayed in this area.

The  Auxiliary  Menu allows you to choose  specific  "operations"

such  as  drawing  solid  or dashed  lines;  moving  or  deleting

objects;  and  typing  text on your drawing.  These  options  may

change depending upon the items you select from the Master and/or

Auxiliary menus.

5.5  Selecting Menu Options

As with the previous menus you've encountered, you need only type

the corresponding two-digit number from the keyboard to select an

option (pressing <Enter> is not necessary). If you are changing

from text you must enter the two-digit number followed by a <^>

to indicate a menu selection.

page 11

6.0  OBJECT PLACEMENT

Throughout your entire drawing session, the prompt at the top  of

the  Master  Menu Screen will continually ask you  to  locate  or

"pick" certain points on your drawing such as: the start and  end

points of lines; objects to be moved, copied, rotated and scaled;

or the center and radii of arcs and circles.  Each point you pick

refers  to an exact place on your drawing.  The  computer  inter-

prets  and  stores these points in its memory  as  a  "Cartesian"

coordinate. (Remember high school geometry?)

6.1  Typing Coordinates from the Keyboard

The  DRAW program works with a graphics tablet and a  stylus  (or

other  input  device).  When you select a point  using  an  input

device,  you are actually selecting a Cartesian  coordinate.  For

accuracy,  coordinates may be typed from the keyboard instead  of

using the tablet and stylus.  This is helpful when you know  spe-

cific distances and/or angles.

When  typing or "keying-in" a coordinate (x,y), the first  number

corresponds to a positive or negative point on the "x-axis",  the

second number corresponds to a positive or negative point on  the

"y-axis".  The resulting coordinate is the intersection of  those

two points.

6.2  Absolute Coordinates (x,y)

An absolute coordinate is an exact point measured from the  draw-

ing's  origin (0,0).  Assuming that your drawing was  full  scale

and  measured  in  inches, the absolute  coordinate  (3,2)  would

indicate  a point 3 inches across and 2 inches up from the  (0,0)

coordinate.

To  type  the absolute coordinate (3,2) from  the  keyboard,  you

would press the left parenthesis "(" key, then the number 3  fol-

lowed  by a comma and the number 2.  To complete  the  coordinate

entry,  you could press the right parenthesis ); or simply  press

the <Enter> key.

6.3  Relative Coordinates [x,y]

A  relative coordinate is a point located a specific "x" and  "y"

distance  from  a  previous point  selected.   Again,  using  our

original example, the relative coordinate [6,2] would be a  point

six inches across and two inches up from the absolute  coordinate

(3,2).  Relative coordinates are entered in the same manner as an

absolute coordinate, however, brackets "[ ]" are used instead  of

parenthesis.

page 12

6.4  Polar Coordinates <x,y>

A polar coordinate is a specific distance and angle from a previ-

ous point selected.  The angle is measured counterclockwise  from

the  positive "x-axis", and can be entered in either  degrees  or

radians.

For  example,  if you wanted to draw a five-inch line  at  a  35-

degree  angle from the absolute coordinate (3,2), you would  type

the  polar coordinate <6,35> using the left and right arrow  keys

"< >".

You  may express an angle in degrees, minutes and  seconds  using

any unit individually, or using varied combinations of the three.

To indicate degrees, type the the letter "D" following the number

of  degrees; to indicate minutes, type the letter "M"  after  the

number  of minutes; to indicate seconds type the letter "S"  fol-

lowing the number of seconds.

For example, if you wanted to enter the polar coordinate 5 inches

at  an angle of 35 degrees, 22 minutes, and 30 seconds you  would

type:

<5,35D22M30S

and press the <Enter> key.

If  the angle is expressed in radians, type the letter "R"  after

the number of radians.

6.5  Saving Coordinates to Function Keys

Relative and polar coordinates are normally located from the last

point picked.  However, you can bypass this restriction by saving

a picked point immediately after it is selected with your stylus;

then,  restore the point later.  Up to four different points  may

be saved to the function keys numbered F5 through F8.

To  save  a coordinate, hold the <Alt> key down after  picking  a

point  and press one of the four function keys (F5, F6, F7,  F8).

The  point being saved will flash on the screen.  To restore  the

point, press the function key you used to save the coordinate.

For example, let's draw a line with three vertices (points):  The

first  vertex will one inch to the right of the start point;  and

the  second,  three inches at a 30-degree angle  from  the  start

point.   Pick  the start point with your stylus and  press  <Alt>

<F5> to save the point.

Then, type:            [1,]

which draws a line to the first vertex.

page 13

Now, press <F5> to restore the start point as the relative origin

and type:

<3,30>

to draw the next segment.

6.6  Locating Coordinates with an Input Device

You could do an entire drawing by typing in coordinates from  the

keyboard; however, it would be very time consuming.  It's  faster

and more efficient to use an input device.

The  following sections contain a brief description of the  types

of input devices that can be used with the DRAW program.

When the prompt or manual asks you to "pick" a point on the  gra-

phics screen, you can select that location using an input  device

or  by typing a coordinate.  You can also select points  using  a

combination of the two methods.  For instance, the start point of

a  line  might be selected with the stylus, while the  end  point

might be a typed coordinate.

6.7  Graphics Tablet and Stylus

This  is  an electronic pad and pencil that is connected  to  the

computer.  As your stylus moves across the magnetized tablet, the

graphics  screen  displays an x-shaped cursor  which  tracks  the

stylus movement.

To  select  or "pick" a point with your stylus, you  must  gently

press the stylus against the tablet.  A small "x" will appear  at

the point you have selected.  The stylus can also be used to  se-

lect objects to be moved, copied or deleted in the same fashion.

6.8  Mouse

The  mouse  is a small hand-held device that works on  any  flat,

hard surface (such as a desk top).  With a mouse, you don't  need

a tablet.  A bearing-like roller on the bottom of the mouse moves

across  the surface area and allows you to position the  graphics

screen  cursor.  A button on the mouse is used to  select  points

and objects.

6.9  Using the Keypad as an Input Device

When installing the software you may choose the numeric keypad on

your keyboard as the input device (see "Installation").  The keys

numb	ered 1 through 9 control cursor movement; keys 8, 2, 4 and 6

move  the  cursor up, down, left and right as  indicated  by  the

arrows; keys 7, 9, 1 and 3 move the cursor diagonally across  the

screen.

page 14

By  pressing the minus (-) key, you can select either  "fine"  or

"coarse"  cursor  control.  Fine moves the cursor in  very  small

increments every time one of the cursor control keys is  pressed;

coarse  moves the cursor in larger increments around the  screen.

Press  the  minus key to toggle between coarse  and  fine  cursor

control.

When  using  the "fine" mode, the <Ins> and <Del> keys  will  in-

crease or decrease the cursor movement for more precision.  Press

<Ins>  to  move  in larger increments; <Del>  for  smaller.   The

increment  becomes  slightly smaller or larger each time  one  of

these keys is pressed.

To pick a point, press the plus (+) key.

page 15

7.0  DRAWING AIDS

There  are several "built-in commands" that help make  using  the

menu and input devices fast and easy.

7.1  Grid Display - /G

To make drawing more precise, you can display or overlay a  "grid

pattern" on your drawing.  The grid is not plotted on your  final

drawing,  they are used only as drawing aids.  To turn  the  grid

display on or off, press the </> and <G> keys on your keyboard.

7.2  Isometric Grid - /X

You may change from a "standard" grid to an "isometric" grid when

drawing isometric views.  To switch from the standard grid to the

isometric  grid display, press the </> and <X> keys on your  key-

board.

7.3  Changing Grid Size - /N

Unless  you change it, you will have a one-unit grid pattern  (in

the  units of measurement chosen when you set up  your  drawing).

Changing  grid size (spacing) can make it easier to place  points

and  objects exactly where you want them to appear.  The size  of

your  grid is determined by the units of measurement chosen  when

you laid out the drawing.

You  may  change the grid spacing at any time by  pressing  </N>,

typing a new grid size, and pressing the <Enter> key.  Grid  pat-

terns can also be changed, as well as checked, using  98 DrawDefs

on the Master Menu.

7.5  Working with the Input Device

There are several functions of the DRAW program which make  using

your input device easier and faster.

7.6  Graphics Cursor

When working with your input device, an x-shaped "cursor" appears

on your graphics screen.  The cursor will travel over the drawing

area  when  you position the stylus on the tablet  or  slide  the

mouse across the table.

7.7  Picking Points and Objects

Throughout  this manual we will ask you to "pick" a point on  the

drawing  or select an object with your input device.  To  pick  a

coordinate  or object, position the cursor at the point you  wish

to  select  and press down lightly with stylus; if your  using  a

mouse,  press the button.  If you have selected the  keyboard  as

your input device, press the <+> key on the numeric keypad.

page 16

7.8  Hit Window

The  "hit window" is a small, invisible, rectangular area  inside

the graphics screen cursor.  The window acts as a detect area for

all  objects which fall within its boundaries.  This area can  be

made larger or smaller using 98 DrawDefs on the Master Menu.

Note:

When  the size of the hit window is  made  larger,

the cursor size changes on the graphics screen  to

show the area in which picks can be made.

7.9  GRID - 00

When  the word GRID appears on the status line at the top of  the

Master  Menu screen, the DRAW program places your stylus pick  at

the nearest grid point.

To select the GRID mode, press <00>.

7.10  FREE - 01

When  the word FREE (free pick) is shown on the status line,  the

DRAW  program locates the point at the exact location  picked  by

the stylus, regardless of any grid point within the hit window.

To select FREE pick, press <01> on the keyboard.

7.11  SNAP - 02

When the word SNAP is shown on the status line, the DRAW  program

places  or "snaps" the stylus pick to an existing  object  within

the  cursor's hit window.  If there are several lines or  objects

within the hit window, the SNAP mode will pick points or  objects

using the following priorities:

1) At the intersection of two objects;

2) At the endpoint of a line, arc or quadrant point of a

circle; then

3) The closest point on a single object.

If  none of the above are found within the hit window,  the  DRAW

program places the point as it would in FREE pick.  To select the

SNAP mode, press <02> on your keyboard.

7.12  NON-AXIS and AXIS - 03 / 04

While drawing, it may be necessary to place a point along the same

page 17

axis as a previous point.  Using the AXIS mode aligns your stylus

picks to the vertical or horizontal axis based on your last pick.

To select the AXIS mode, press <04> on your keyboard.  To  select

NON-AXIS, press <03>.

7.13  Assigning Pen Numbers - /P

Individual  pen numbers can be assigned to lines and  objects  to

produce  a  different colors or line weights when  a  drawing  is

plotted.   Depending on the plotter, you can draw with up  to  16

different pens.

The pen number you are currently using is displayed on the status

line at the top of the Master Menu screen.  Lines or objects will

be  assigned to pen number 1 unless you specify a  different  pen

number.  To  select a new pen number, press </P>,  and  type  pen

number  between  0 and 15; then press the <Enter>  key.  You  can

alter  the pen number of an object after it is drawn with  option

64 ATTR Edit

The  "PLOT" option used to output drawings on conventional  plot-

ters is available in Cascade's "Phoenix" and "Atlas"  CAD/Drawing

programs.

7.14  Assigning Intensity - /I

Intensity  (brightness) changes the way a line or object  appears

on  your  screen,  but  does  not  affect  the  actual   drawing.

Depending on your monitor, you may choose from three intensities:

1  is  the  dimmest  intensity, while 3  is  the  brightest.  The

intensity  level  is displayed on the status  line;  the  default

setting is "3".

To assign a different intensity level to a line or object  before

you draw it, press </I>, type a number between "1" and "3"; then,

press  the <Enter> key.  You may change the intensity level of  a

line or object after it is drawn with 64 ATTR Edit on the  Master

Menu.

7.15  Assigning Color - /C

On systems using color graphics monitors, different colors can be

assigned to objects on the graphics screen.  You may choose  from

several  different  colors: (W)hite,  (A)qua,  (M)agenta,  (R)ed,

(G)reen, (B)lue and (Y)ellow.  The color will affect the  monitor

display only and not the completed drawing.  When you change  in-

tensity, you will also change the hue of a color.

page 18

The  current color is displayed on the status line; white is  the

default color selected for each drawing.  To change the color  of

a  line or object before you draw it, press </C> and  type  first

letter of the color; the press the <Enter> key.  After a line  or

object has been drawn, you may alter its color using 64 ATTR Edit

on the Master Menu.

7.16  Assigning Drawing Levels - /L

The  DRAW program allows you to produce drawings with up  to  256

"levels"--which  is the equivalent of a  conventional  mechanical

drawing with 255 overlays.

This is particularly helpful when creating a master drawing  with

individual  overlays.  For example, architects can generate  sep-

arate  plumbing,  air conditioning,  electrical  and  landscaping

drawings  from  the same master plan.  What's  more,  complicated

mechanical drawings can have different elements--like dimensions,

text  or various sub-assemblies--on separate levels.  This  makes

manipulating and editing objects easier.

You may draw on only one level at a time. The level on which  you

are currently drawing is displayed on the status line.  Any lines

or  objects  you  draw  will be  assigned  to  that  "active"  or

"working"  level,  which  will  be  level  1,  unless   otherwise

specified.

To  change the level on which a line or object will be drawn  and

displayed you must select the level prior to drawing the  object.

Press </L> and type a level number from "0" to "255"; then, press

the <Enter> key.

To turn a specific level off, you enter </L> and type the level

number from "0" to "255" followed by a "-".

To  display all levels of a drawing, press /L*+ and  <Enter>.  To

turn off all levels, press /L*- and <Enter>.

After  drawing a line or object, you can edit the level on  which

it  appears  using the option 64 ATTR Edit.  You may  also  view,

plot   or  print  any  possible  combination  of  levels   on   a

conventional   plotter  with  Cascade's  "Phoenix"  and   "Atlas"

CAD/Drawing programs.

7.17  Important Keys and Features

There  are several important keys and features you should  become

familiar with before starting your drawing session.

7.17.1  Backspace Key

If  you  make a mistake while typing,  erase  characters  by

using the backspace key (<-); then, type the correct charac-

ter, text line or value before pressing the <Enter> key.

page 19

7.17.2  Enter Key

The  <Enter>  key must be pressed after typing any  line  of

text (for title blocks, material lists, descriptions, etc.);

or  after  typing  any numeric value (such as  an  angle  or

radian).  It is not necessary to press <Enter> when  select-

ing options from the menu.

7.17.3  Rejecting an Operation

If you make a mistake in selecting a point or object on your

graphics screen, use the <Enter> key to "reject" the  opera-

tion.  By pressing <Enter>, you delete the last point,  line

or object entered; or have a line or object returned to  the

graphics screen in its original position after moving, copy-

ing,  or  deleting it.  Selecting another menu  option  will

complete an operation and you will be unable to reject  with

the <Enter> key.

Note:

Some  operations can't be rejected once  com-

pleted.  A  message may  appear  beneath  the

prompt warning you that the "operation is not

rejectable" before you begin.

7.17.4  Caret / Plus Keys

The ^ (caret) and the numeric keypad's + (plus) key are used

to select a two-digit menu option whenever the DRAW  program

is  expecting  you  to  enter a value  or  text  line.   For

example,  if the prompt is asking you to type in a  line  of

text  from the keyboard, but you wish to choose a  two-digit

menu  command instead, you must press the ^ or +  key  after

the two-digit number; an <Enter> is not needed.

Note:

Both the <Shift> and <^> keys must be pressed

at the same time to type the ^ symbol.

7.17.5  Error Messages

If  you enter a command or value that the DRAW program  does

not expect or cannot do, it will signal you with a beep  and

display  an  error message below the prompt line.  Read  the

message  to determine the cause of the error, then  continue

according to the prompt instructions.

page 20

7.17.6  Zoom

When  a drawing area becomes filled with details, it may  be

necessary to "zoom" in on a section of the drawing.  88 Zoom

works like the zoom lens of a camera, allowing you to magni-

fy  a rectangular section of your drawing  without  changing

the actual drawing.  You may also use your function keys  to

zoom.  Refer to the section on "Graphics  Screen  Utilities"

for further information.

7.17.7  Pan

You can move the drawing image up, down, left or right using

89 Pan  or the arrow keys on your numeric keypad.  Refer  to

the chapter "Graphics Screen Utilities" for details on  pan-

ning.

7.17.8  Full Window

After   working   with  the  zoom  and  pan   options,   use

90 Full Window to center your entire drawing on the graphics

screen.  The  chapter "Graphics Screen  Utilities"  explains

this option.

7.17.9  Redraw

As  you work with a drawing it may be necessary  to  refresh

the  screen image every so often using 99 Redraw.  Refer  to

the  chapter "Graphics Screen Utilities" for an  explanation

of this option.

7.17.8 Percent Full (0%)

On the far right of the status line a percentage is display-

ed.   The percentage value indicates how full  your  drawing

is.  Each  drawing starts out at 0%.  As  more  objects  are

drawn,  the  drawing storage are fills  and  the  percentage

increases.

Once  this figure reaches 95% full, it is  recommended  that

you "pack" your drawing.  This removes computer  information

on all objects you've deleted and frees more drawing  space.

No major drawing operation should be undertaken if the draw-

ing memory exceeds 95%.  The pack operation can be performed

using 94 Deactivate.

7.18 Saving/Exiting Your Drawing

As  you  draw, it is extremely important to  periodically  "save"

your  work to the hard or floppy disk.  You must do this to  pre-

vent accidental loss of your drawing due to power or system fail-

ures.  Use 94 Deactivate to save and/or exit your drawing.

page 21

7.19  Backup Drawings

It's  always a good idea to backup (copy) your work to  a  floppy

disk after every drawing session, or after several hours of work.

If  for  some reason your original drawing is  deleted,  you  can

restore  your  work from the backup copy, saving  hours  or  days

worth of work.

Backup  copies  can  be made using the DOS "COPY"  command  to  a

floppy disk.  Drawings use the DOS file extension .DWG.  To  copy

all  the  drawings from the C: to the floppy A: drive  you  would

type from the DOS prompt:

COPY C:*.DWG A:

The DRAW program also makes its own backup drawings as you  work.

Backup drawings are identified with the DOS file extension  .BDG.

Should  you accidentally erase an original drawing  that  doesn't

have a backup on floppy disk you could rename the .BDG using  the

DOS "RENAME" command.

For  example, if you wanted to make the file EXAMPLE.BDG  into  a

usable drawing you would type:

RENAME EXAMPLE.BDG EXAMPLE.DWG

This  would  restore  most,  if not all of  the  drawing  to  its

original form.

page 22

8.0  DRAW Options

The DRAW options are used to draw and edit objects: lines,  arcs,

circles,  rectangles, and ellipses.  Objects may also be  copied,

deleted,  rotated, scaled and mirrored with the DRAW options.  In

addition,  you  may place points on a drawing; group  and  unnest

objects; and move the origin of an object or group.

When a drawing is loaded, the DRAW program automatically displays

the  DRAW  options  and selects the line drawing  option  on  the

Auxiliary Menu (27 Line) as the default option.

8.1  27 Line

To  draw  a line, select a linestyle from options 11  through  14

(solid, dashed, center or phantom).  Pick the start point of your

line  with the stylus (or type in a coordinate); then,  pick  the

next  vertex point of the line.  Continue picking points in  this

fashion to create a "multi-segment" line.

You may erase (reject) one or more of the line segments while you

are drawing by pressing the <Enter> key.

Press  27  or select another menu option to end  the  line.   The

reject option will not work once a new menu option is selected.

8.2  Linestyle Options (11, 12, 13, 14)

The DRAW program has four different linestyle options listed

below  the  heading ** Linestyle ** on the  Auxiliary  Menu:

11 Solid; 12 Dash; 13 Center; and 14 Phantom.  The linestyle

options  may  be  used when drawing any  object,  except  an

ellipse.

To  change  a linestyle, press the  corresponding  two-digit

menu number and the selected linestyle will be  highlighted.

11 Solid is the default option.

8.3  20 +NO Fill / +Solid

This is a toggled option that allows you to "fill" a  closed

rectangle  or polygon with a solid pattern of  lines.   This

option  is limited to an object with a maximum of  50  sides

that has a common vertex as the start/end point.

If +NO Fill is displayed on menu option 20, the fill  option

is disabled.  To turn it on, press <20> so that "+Solid"  is

displayed on the option.

page 23

To fill an object drawn with 27 Line, first toggle 20 to read

20 +Solid, then select 19 Close  after the end point has been

picked; the DRAW program will fill the object once 19 is

pressed.  The option 19 Close  appears only  when 27 Line is

highlighted. When  drawing  rectangles and polygons (options 28

and 32) the object is automatically filled if 20 +Solid is

displayed. Only the first 50 segments of a polygon are considered

by the fill software.

8.4  22 MoveToPnt

The  "MoveToPnt" (Move To Point) option makes the next  line

segment  drawn invisible.  It can be used, for  example,  to

draw a continuous line through an existing object; the  line

you  are drawing still exists, however, the segment(s)  pas-

sing through the object are hidden from view.

Before picking the endpoint of the segment you don't want to

appear, press <22> and highlight the MoveToPnt option; then,

pick  the  next vertex point on the line.  22  MoveToPnt  is

disabled  after each use, so press <22> before each Move  To

Point operation.

Note:

A   segment   that  has  been   hidden   with

"MoveToPnt"   can   be   re-displayed   using

65 Line Edit.

8.5  28 Rectangle

To create a rectangle press <28>.  Change the linestyle option if

necessary  (11 through 14); then, pick two opposite corners  with

your  stylus.  Once the rectangle is drawn, you can remove it  by

pressing the <Enter> key once.  An "x" remains at the location of

the first corner picked.  You may select another corner with your

stylus, or remove the "x" by pressing <Enter> a second time.

8.5.1  16 Ang= 0.000

This option appears with 28 Rectangle. It changes the  angle

at which the rectangle is placed on the drawing.

To  change  the  angle,  press 16,  type  a  new  angle  (in

degrees),  then  press <Enter>.  The new angle  will  appear

next  to the menu option.  If a positive angle  is  entered,

the  rectangle  will be rotated  counterclockwise  from  the

positive x-axis; clockwise when a negative value is entered.

page 24

8.6  29 Arcs

Press  <29> and several new Auxiliary Menu options will  be  dis-

played to help you draw arcs.

To  make an arc, select an "arc-type" from options 16 through  18

and specify clockwise (CW) or counterclockwise (CCW) with  option

15.  You may also change the linestyle options if desired.  Then,

pick  points  with your stylus as directed by the  prompts.   (In

some  instances  you  will be directed by prompts  to  provide  a

numeric value such as an angle, radius or chord.)

8.6.1  15 +DIR = CW / CCW

This  toggled option lets you change the direction in  which

an  arc  is drawn.  Press <15> to toggle  between  clockwise

(CW) and counterclockwise (CCW)

8.6.2  16 ARC - 3 Point

Use  this  option to draw a "three-point arc".  Press  <16>,

then  select a start point with your stylus; followed  by  a

second point on the arc; then, the end point.

8.6.3  17 Arc - S/C/End

Use  this option to draw an arc with a known start point,  a

center point (not on the arc) and an end point.  Press <17>,

select the start point, the center point, and the end  point

with your stylus.

8.6.4  18 Arc - S/E/Rad

Use this option to draw an arc with a known start point, end

point  and "radius" (Rad).  Press <18> and select the  start

and  end points of the arc with your stylus.  Then,  type  a

value for the radius and press <Enter>.

8.7  30 Circle

Press <30> and several new menu options will be displayed to help

you draw circles.

To  draw a circle, select a "circle-type" from options 15, 16  or

17.  You may also change the linestyle option, if  desired.  Then

pick points on your drawing as directed by the prompts.  (In some

instances you will be asked to provide a numeric value such as  a

diameter or radius.)

page 25

8.7.1  15 Circle - Pick

This is the default option when you select the Circles task.

It  allows you to draw a circle by picking its center  point

and  a  point  on its radius.  Press <15>;  then,  select  a

center point and a point on the radius with your stylus.

8.7.2  16 Circle - 3 Pnt

This  option  draws a circle through "three  points"  picked

with  the stylus.  Press <24>; then, select a point  on  the

circle and two radius points.

8.7.3  17 Concentric

This  option draws concentric circles which share  a  common

center  point.   Press <17> and select a center  point  with

your  stylus.  Now pick a point on the radius. Continue picking

radius points until you have drawn the number of  concentric

circles desired.

To  choose  a new center point or  complete  the  operation,

press  <17>  or select another menu  option.

8.8  31 Ellipse

Press <31> and several new menu options will be displayed to help

you draw ellipses.  To draw an ellipse, choose whether to draw  a

full  or half ellipse with option 19.  13 Pick Maj/Min  is  high-

lighted when you select the "Ellipse" task and cannot be changed.

Now,  pick  points  (or  type coordinates)  as  directed  by  the

prompts.

8.8.1

The "Maj/Min" option draws an ellipse when two end points on

the major axis are selected, followed by an end point on the

minor axis.

8.8.2  19 +Full Ellipse / +Half Ellipse

Press  <19>  to toggle this option between a  full  or  half

ellipse.

8.9  32 Arrows

Press  <32> and several new Auxiliary Menu options  appear  which

allow  you  to  create arrowheads.  The arrowheads  may  then  be

placed  in the at the end(s) of a line or at arbitrary  locations

on the drawing.

page 26

To  place an arrowhead select the style with options 11  and  13;

choose  where you want to place the arrowhead with option 16  (at

the end of a line; at both ends; or FREE placement); then  deter-

mine  the angle, the length, and the length-to-width  ratio  with

options 18, 19 and 20.

Once these options have been specified, use your stylus to select

a location.  Use option 12 after an arrowhead has been placed  to

reverse its direction.

8.9.1  11 +Flow (Arrowhead Styles)

This option allows you select one of seven different  arrow-

head  styles.  Press <11> until the style of  arrowhead  you

wish to create is displayed.

8.9.2  12 Reverse Dir.

Press  <12> to change the direction of the  arrowhead.  This

option  works only immediately after the arrowhead has  been

placed on the drawing.

8.9.3  13 +Solid / +Open

Press <13> to toggle between solid and open arrowheads.

8.9.4  16 +Place At End / +Both Ends / +Free

Press  <16> to select one of three different  placement  op-

tions: At the end of a line (+Place At End); at both ends of

a  line  (+Both Ends); or at an arbitrary  location  on  the

drawing (+Free).

8.9.4  18 Angle= 0.00

This  option specifies a rotation angle for  arrowheads  not

placed on a line.  Angles are measured counterclockwise from

the  horizontal x-axis, unless a negative value is  entered.

To  change the angle, press <18> before placing  the  arrow-

head;  type  a new value and press <Enter>.  The  new  angle

will be displayed on the menu.

8.9.5  19 ArwLEN = 0.200

This  option  specifies the arrowhead  length.  The  default

value is 0.200 inches or centimeters.  To change the  length

press  <19>;  type a new value from 0.05 to 3.00  and  press

<Enter>.  The new value will be displayed on the menu.

page 27

8.9.6  20 ArwLTW = 1.800

This  option sets the arrowhead's  "length-to-width"  ratio,

which determines the shape.  High values will produce  long,

narrow arrows; low values produce wider arrows.

The  default ratio is 1.8.  To change the ratio press  <20>;

type a new value from 0.25 to 10.00 and press <Enter>.   The

new value will be displayed on the menu.

8.10  33  Points

Press <33> and several new Auxiliary Menu options appear allowing

you to mark points on your drawing using a +, X , * or .  symbol.

Select  a  point style from options 11 through 14 ;  then,  touch

your stylus to the tablet to place the point on the drawing.

You may place as many points as needed.  Press <Enter> to  remove

the last point placed on the drawing.

8.10.1  Point Styles

Options 11 +, 12 X, 13 *, and 14 . are used to select  point

styles.   Press  the corresponding number  to  indicate  the

point  style you wish to use; your selection will  be  high-

lighted, indicating choice.

8.11  34 Move

Press <34> and new menu options will be displayed that allow  you

to move an object or a group to a new location.

You  can move an object by its origin point using  11 Origin;  or

move objects within a specified area with 12 Rectangle.

8.11.1  16 Drag Object

When 16 Drag Object is highlighted, you may drag the  object

or group you have moved across the drawing by pressing  your

stylus to the graphics tablet (or by holding down the  mouse

button).  Press <16> to toggle this option on or off.

8.11.2  11 Origin

Moves  the  object or group using the origin  point  as  the

reference point for relocation.

To move by origin, press <11> and select the object or group

you  wish to move.  The origin point will be marked with  an

"x".   Now, select a new point for the origin.   The  object

will appear with its origin point at the new location picked

with your stylus.

page 28

Press  <Enter>  once when the move is complete to  lock  the

object  in  place. If you press <Enter> a  second  time  the

object will be returned to its original location.

8.11.3  12 Rectangle

Moves an object or group within a rectangular area that  you

pick with your stylus.

Press  <12>, then select the lower left corner of  the  rec-

tangle.  A mark will appear to indicate that point.

Next,  select the upper right corner and a dotted  rectangle

will  be  superimposed around the area to  be  moved.  Then,

select  a point on the drawing which will be the  object  or

group's  new lower left corner.  All objects  having  origin

points within the rectangle will be moved.

A  rectangle move can also be accomplished  with  coordinate

input.

8.12  35 Copy

Press <35> and new menu options will be displayed that allow  you

to copy an object or a group to a new location.  You can copy  an

object by its origin point using 11 Origin; or copy objects with-

in a specified area with 12 Rectangle.

8.12.1  16 Drag Object

When 16 Drag Object is highlighted, you may drag the  object

or group you have copied across the drawing by pressing your

stylus to the graphics tablet (or by holding down the  mouse

button). Press <16> to toggle this option on or off.

8.12.2  11 By Origin

Moves an object or group using the origin point as the  ref-

erence point for relocation.

To copy by origin, press <11> and select the object or group

you  wish to copy.  The origin point will be marked with  an

"x".  Now,  select a new point for the origin.   The  object

will appear with its origin point at the new location picked

with your stylus.

If Drag is on, press  <Enter>  once when the copy is complete

to  lock  the object  in  place.  Press <Enter> again and the

object will be returned to its original location.

If Drag is off, <Enter> will return the object to its

original location.

page 29

8.12.3  12 By Rectangle

Moves an object or group within a rectangular area that  you

pick with your stylus.

Press  <12>, then select the lower left corner of  the  rec-

tangle.  A  mark will appear to indicate that  point.  Next,

select the upper right corner and a dotted rectangle will be

superimposed  around the area to be copied.  Then, select  a

point on the drawing which will be the object or group's new

lower left corner.  All objects having origin points  within

the rectangle will be copied.

A  rectangle copy can also be accomplished  with  coordinate

input.

8.13  36 Delete

This option deletes an object or group from a drawing. Press <36>

and choose one of the two delete options listed under the heading

** Aux. Options **.

8.13.1  11 Object

This  option allows you to remove a single object  or  group

from the drawing.  Press <11> and select an object or  group

to be deleted with your stylus.

8.13.2  12 Rectangle

To delete objects within a rectangular area, press <12>  and

select  the the lower left corner of the area.  A mark  will

appear to indicate the point.  Next, select the upper  right

corner  and a dotted rectangle will be  superimposed  around

the area to be deleted.  Now, touch your stylus on the  gra-

phics  pad  or  press  <42> to confirm  that  you  want  the

object(s) deleted.

Only objects whose origins fall within the rectangle will be

deleted.

8.14  37 Rotate

Press <37> and two new options will be displayed allowing you  to

rotate and copy an object around its origin.

Use 11 Ang= to determine the angle of rotation. 12 Repeat= copies

the object a specified number of times while rotating the  object

around its origin in increments specified by option 11. Once  the

options  have been properly set, select the object to be  rotated

using your stylus.

page 30

8.14.1  11 Ang= 90.000 deg

Specifies the angle of rotation.  The object will be rotated

counterclockwise  unless  a negative angle is  entered.  The

default angle is set at 90 degrees.

To change the angle, press <11>; type new angle from .10  to

360 and press <Enter>.  (You may enter degrees expressed  as

decimal  fractions, e.g. 31.5).  The new value will be  dis-

played on the menu.

8.14.2  12 Repeat=0

Determines  the  number of copies to be made  of  a  rotated

object.  Each copy is rotated at the specified angle  around

the object's point of origin.  When Repeat=0, you will  only

rotate the object--no copies are made.

To  make multiple rotated copies, press 12; then,  type  the

number  of copies you want to make and press  <Enter>.   The

new value will be displayed on the menu.

8.15  38 Scale

Press  <38> and several new Auxiliary Menu options will  be  dis-

played allowing you to scale (re-size) groups or objects.

8.15.1  11 Xscale= / 12 Yscale=

"Xscale"  sets  the horizontal (x-axis)  scale  factor,  and

"Yscale"  sets the vertical (y-axis) scale factor.  The  de-

fault factor values are "0.500".

To  change  the scale press <11>; type a  new  factor,  then

press  the  <Enter> key. Both Xscale= and  Yscale=  will  be

changed to the new factor and displayed on the menu. To  set

a separate "Yscale" value press <12>, type a new factor  and

press <Enter>.

8.15.2  15 Calc. SCALE

This option helps you to calculate the proper scale  factor.

This  is helpful when you know what the length of  a  scaled

line  will be but can't calculate the exact values  for  op-

tions 11 and 12.

Press <15> and select a line with your stylus.  The  current

length  of  the line will be displayed above  the  Auxiliary

Menu.   Now  type the a new line length--the size  that  you

want  it to be after it is scaled--and press  <Enter>.   The

DRAW program will automatically enter the proper Xscale  and

Yscale in options 11 and 12.

page 31

Note:

You  may only calculate an object that has  a

line; arcs and circles may not be selected.

8.16  39 Mirror

This option "mirrors" a group or object through its origin across

the "x" or "y" axes.

Press <39> and choose to mirror an object across the x or  y-axis

by  toggling option 11. Decide whether to turn the copy  function

on  or off with option 12.  Then, pick the object to be  mirrored

with your stylus.

8.16.1  11 +Mirror X / +Mirror Y

Press <11> to toggle between the "x-axis" (+Mirror X) or the

"y-axis" (+Mirror Y) when using 39 Mirror.

With  +Mirror  Y,  the object mirrors from  left  to  right;

+Mirror X, the object mirrors from top to bottom.

8.16.2  12 +Copy OFF / +Copy ON

Press  <12> to choose to only mirror an object (+Copy  OFF),

or to mirror and copy (+Copy ON). With +Copy ON the original

object remains on the drawing with the mirrored copy.

8.17  41 Group

Use  41 Group to take an object, or several objects, and  combine

them so the DRAW program interprets the group as a single entity.

Some  of the DRAW tasks require that an object or  objects,  also

known  as primitives (i.e. rectangles, circles, arcs) be  grouped

or "nested" before you can manipulate them.

8.17.1  11 Pick Objects

To  create  a  group  by picking  objects,  press  <11>  (if

necessary)  and select the object(s) you want to include  in

the group. As you select each object, it will disappear from

the graphics screen.

When  you are finished selecting all the objects to  be  in-

cluded  in the group, press <41> and they will  reappear  on

the screen.  Now, whenever you work with any objects in that

group,  all  the others will be affected (such as  with  the

Move, Copy, Delete and Scale options).

page 32

8.17.2  12 Rectangle

You  may  also group objects within a  rectangular  area  by

using 12 Rectangle. It works the same as the moving, copying

and deleting by rectangle.

To  group all objects within a rectangular area, press  <12>

and  select  the the lower left corner of the area.  A  mark

will  appear to indicated the point picked.  Now select  the

upper  right corner and a dotted rectangle will be  superim-

posed  around the area to be grouped.  Touch your stylus  on

the  graphics tablet or press <41> to confirm that you  want

the object(s) grouped.

Note:

Only  objects whose origins fall  within  the

rectangle will be grouped.

8.18  42 Unnest

Use  the  option  to unnest (un-group) a  collection  of  grouped

objects.

It  is possible to create one large group out of several  smaller

groups.  This  process  is called "nesting"  groups.  When  using

42 Unnest you can choose to "unnest" only the large group (called

the  top  level), or you can unnest all levels (all  the  smaller

groups  contained  within the top level). Unnested  objects  will

revert back to their their original primitive (or group) form.

To  unnest a group, press <42> and choose whether to  unnest  the

top level of the group, or all levels with option 11. Then select

the group to be unnested with your stylus.

8.18.1  11 Unnest TOP / Unnest ALL

This  Auxiliary  Menu option appears when using  42  Unnest.

Press  <11> to toggle between the Unnest TOP and the  Unnest

ALL options.

8.19  43 Move Origin

This  option moves the "origin point" of a group to a  new  loca-

tion.  Moving the origin point can help make working with  groups

easier  when  performing  tasks such as Move,  Copy,  Rotate  and

Mirror.

To  move  the origin point of a group, press 43  and  select  the

group with your stylus.  A small "x" will appear at the  location

of the current origin point. Select a new origin point with  your

stylus  and  the "x" will be moved to the point picked.  You  may

continue  to  pick  points  until  you  are  satisfied  with  the

position.

Press <Enter> to reject the operation.

page 33

9.0  51 TEXT

51 Text allows you to place lines of text (also called "strings")

on  your drawing. Press <51> and new Auxiliary Menu options  will

be displayed.

To  place text on a drawing, change the options (11  through  17)

listed  under  the heading ** Text Attr. ** if  necessary.  Then,

type a line of text and press the <Enter> key. The line you  just

typed  will  be  displayed at the top of the  screen,  below  the

prompt  line.  You may place the displayed line  on  the  drawing

using  your  stylus,  or replace it with  another  text  line  as

described above.

Once  a line of text has been placed, you may drag it across  the

screen by pressing your stylus to the graphics tablet (or holding

down the mouse button). A line of text is considered an object by

the  DRAW program and can be moved, copied, grouped  and  deleted

with the DRAW options detailed in the previous sections.

Note:

Remember  to  use the ^ or the +  keys  to  select

different menu options while using 51 Text; other-

wise,  the DRAW program will interpret  your  menu

selections as text strings.

9.1  11 Xsize= 0.125

This option determines the width of a text character. The default

values  are  .300  cm  (metric drawings)  or  .125  in.  (English

drawings).  To change the Xsize press <11^> or <11+>; type a  new

width  between .02 and 10.0, then press <Enter>.  The  new  value

will be displayed on the menu.

9.2  12 Ysize= 0.125

This  option determines the height of a text character.  The  de-

fault  values are .300 cm (metric drawings) or .125 in.  (English

drawings).

To  change the Ysize press <12^> or <12+>; type a new height  be-

tween  .02 and 10.0, then press <Enter>.  The new value  will  be

displayed on the menu.

9.3  13 Xstep= 0.125

This option lets you adjust the spacing between text  characters.

It  defines the distance between the start point of  a  character

and the start point of the next character which follows on a line

(x-axis). The default Xstep values are .300 cm (metric  drawings)

or .125 in. (English drawings).

page 34

To  change the Xstep, press <13^> or <13+>; type a  new  distance

between  .02  and 10.0 and press <Enter>. The new value  will  be

displayed on the menu.

9.4  14 Ystep= 0.000

This option creates a "stairstep" effect with your text line.  It

defines  the  vertical y-axis distance  between  each  successive

character in the text line.

To  change the Ystep, press <14^> or <14+>; type a  new  distance

between  .02  and  10.0 (positive or negative  value)  and  press

<Enter>.  The new value will be displayed on the menu.

The   default  value  of  Ystep  is  0.000,  which  places   text

horizontally, like this:

TEXT

If  you changed Ystep to 0.5, each successive character would  be

moved up .5 on the y-axis, similar to this:

T

X

E

T

9.5  15 Angle = 0.00 deg

Use this option to place a line of text on the drawing at a  spe-

cified  angle. To rotate text, press <15^> or <15+>; type  a  new

angle and press <Enter>.  The new value will appear on the menu.

If the value specified is positive, the text line will be rotated

couterclockwise  above the x-axis; a negative angle  rotates  the

line below.

9.6  16 Slnt= 0.000

Use  this  option slant each character,  creating  italicized  or

backslanted text. You may slant characters +/- 60 degrees.  Posi-

tive angles slant text to the right; negative angles to the left.

To  slant  text, press <16^> or <16+>; type an  angle  and  press

<Enter>.  The new value will appear on the menu.

9.7  17 Font = 0

This  option  allows you to specify one of  two  different  fonts

(type styles) for your text. To change a font, press 17^ or  17+;

type a new font number (0 or 2), then press <Enter>.

page 35

In  Cascade's "Phoenix" and "Atlas" CAD/Drawing programs you  may

choose from five different fonts available.  The "Atlas"  program

allows you to create your own fonts.

10.0  64 ATTR Edit

64 Edit Attribute lets you examine and edit any of six  different

object attributes: Kind, Pen, Level, Intensity, Color, and Style.

Press <64> and a new Auxiliary Menu will be displayed.

10.1  11 +Edit / +Display

This  option  allows you to edit (+Edit)  or  display  (+Display)

attributes of a group or object. Press <11> to toggle between the

two choices.

To Display the attributes of an object, choose to look at  either

+Member  (a single object) or +Group (the entire group) with  op-

tion  12.  Then, pick the object with your  stylus.  The  current

attributes of the object will be displayed on the Auxiliary  Menu

screen.

To Edit an object, select the attribute to be edited from options

14 through 20. If you select 14 All Defaults, then all attributes

will be edited, except Style.

Now  change the attribute(s) listed on the Status Line using  the

built-in  / (slash) commands described at the beginning  of  this

manual (/K = Kind, /I = Intensity, /C = Color, /P = Pen).

Once  you have changed the attribute(s) to be edited, select  the

object  with your stylus. If +Member was displayed on option  12,

only  the primitive object selected will be edited; if +Group  is

displayed,  only the top level group will be edited; if  +All  is

displayed, all objects (including nested groups) will be edited.

Toggle  the  option to +Display and select the  object  again  to

confirm your edit(s).

10.2  12 +Member / +Group / +All

Press <12> to select one of the following options when editing or

displaying attributes:

+Member:

Displays/edits  only primitive objects in a  group

(line, arc, circle).

+Group:

Displays/edits  the  top level of a  nested  group

(see Group/Unnest in the DRAW options section).

page 36

+All:

Edits  all  objects in a group,  including  nested

groups.

10.3  14 All Defaults

When  this  option is highlighted you will  edit  all  attributes

(except  style) of the object selected with your  stylus.  Change

the  attributes displayed on the Status Line using the /  (slash)

commands described at the beginning of this manual; then,  select

the  object  to be edited with your stylus.  The object  will  be

edited  with  the attributes currently displayed  on  the  Status

Line.

10.4  15 Edit KIND

Kind  is  a special attribute that refers to  non-graphical  data

that  is assigned to different objects or groups.  It  tells  the

DRAW  program what an object is and how it is used.  Kind  values

are assigned automatically by the program and are used  primarily

for technical/diagnostic purposes.

This  option will rarely be used; however, you can "fill" an  ob-

ject  (rectangle  or ellipse) by changing its kind to  the  value

131.  To do this, press <15> to highlight the option; then,  type

the  number <131> and press <Enter>.  Now select the object  with

your stylus and it will be filled with a solid line pattern.

10.5  16 Edit PEN

Press  <16> to edit the pen number of a group or  object.  Change

the pen number displayed on the status line using the /P command.

For  example: to change the pen number to three, type  </P3>  and

press <Enter>; then, select the object to be edited.

10.6  17 Edit LEVEL

Press  <17>  to edit the level of a group or object.  Change  the

level  displayed  on the status line using the  /L  command.  For

example: to change the level number to ten, type </L10> and press

<Enter>; then, select the object to be edited with your stylus.

10.7  18 Edit INTENSITY

Press <18> to edit the intensity of a group or object. Change the

intensity  value  displayed  on  the status  line  using  the  /I

command. For example: to change the intensity to two, type  </I2>

and press <Enter>; then, select the object to be edited with your

stylus.

page 37

10.8  19 Edit COLOR

Press  <19>  to edit the color of a group or object.  Change  the

color  displayed  on the status line using the  /C  command.  For

example:  to  change the color attribute to red  type  </CR>  and

press  <Enter>;  then, select the object to be edited  with  your

stylus.

10.9  20 Edit STYLE

Press <20> to edit the linestyle of a group or object. When  this

option  is highlighted, the four linestyles will be displayed  as

options 11 through 14. Choose a linestyle by pressing the corres-

ponding  two-digit  menu option; then, select the  object  to  be

edited with your stylus.

Note:

Remember to change your drawing attributes  before

leaving this option and creating new objects.

page 38

11.0  65 Line Edit

Press <65> and several new menu options that will be displayed to

help you edit line segments. You cannot use this option on group-

ed or nested objects. An error message will prompt you to un-nest

groups   before  using  65 Line Edit  (see  42 Unnest  in   "DRAW

Options").

11.1  12 Gap Segment

Use  this  option to gap a line. Press <12> and select  the  line

with your stylus; then, pick the two end points of the gap.

11.2  13 Trim/Extend

Use  this  option to trim or extend a line.  Press <13>  (if  not

already highlighted) and select the line to be trimmed or extend-

ed with your stylus; then, pick a new end point for the line.

11.3  14 Erase Segment / 15 UnErase Seg.

Use  these options to erase a line segment; or to restore a  seg-

ment that has been erased. Press <14> and select the line segment

to be erased with your stylus.

To  restore an erased segment press <15>; then, select  the  line

near where the segment was erased.

Note:

You may use this option to restore a segment  that

was  erased with the "Move to Point" operation  in

"DRAW Options".

11.4  16 Move Vertex

This option allows you to move a vertex in a multisegment line.

Press  <16> and select a line segment at a point near the  vertex

you  wish to move; the end points will be marked on your  screen.

Pick a new vertex location and the line is redrawn to include the

new point.

page 39

12.0  Graphics Screen Utilities

The  DRAW program provides four utility options for  viewing  and

displaying your drawing on the graphics screen: 88 Zoom,  89 Pan,

90 FullWindow,  and 99 Redraw.  Each individual option  is  docu-

mented in this section.

12.1 88 Zoom

The  Zoom option works like a combination  magnifying  glass/zoom

lens.  Use this task to enlarge the image on the graphics  screen

without affecting the drawing.

Zoom  can  be  used at any time  during  your  drawing  session--

regardless of what menu is displayed--to help you pick points and

objects with your stylus in detailed areas. Like a telephoto lens

on  a camera, you can zoom in and enlarge small sections  of  the

drawing image.

To zoom in on a section of your drawing, press <88>, and pick two

opposite  corners  of the area to be enlarged with  your  stylus.

This rectangular area is your zoom "window." The graphics  screen

image  will disappear, and be replaced by the  enlarged  section.

Continue drawing as before.

You may also zoom in and out of your drawing by using the F9  and

F10 function keys. Each time you press <F9>, the screen image  is

enlarged from the center about one and a half times. Press  <F10>

to zoom out in approximately the same increments.

To re-display and center the entire drawing, use 90 FullWindow.

12.1.1  Zooming with Function Keys

Function  keys  F1, F2, F3 and F4 have  been  programmed  to

allow  you  to zoom in on four different  quadrants  of  the

graphics screen.

Press <F1> to zoom in on the upper left quadrant

Press <F2> to zoom in on the upper right quadrant

Press <F3> to zoom in on the lower left quadrant

Press <F4> to zoom in on the lower right quadrant

Use 90 Full Window to re-display the full graphics screen.

12.1.2  Re-programming the Zoom Function Keys

You may re-program the four function keys to zoom in on  any

area.  To do this use 88 Zoom and mark the window with  your

stylus.  Then hold down the <Alt> key and press one  of  the

four function keys (F1, F2, F3 or F4) at the same time.

page 40

Now, every time you press the re-programmed function key  it

will  zoom in on the area you selected. The programmed  keys

will be restored to their normal operation once you exit the

DRAW program.

12.2  89 Pan

The  Pan  option moves the image on the graphics  screen  to  the

left, right, top or bottom. This option only changes the  viewing

area  and  does not affect the drawing. Pan  around  the  drawing

image at any time during your drawing session, regardless of  the

menu displayed.

To pan, press <89>; then, using your stylus, pick a point on  the

drawing  to  be the new center of the screen. The  drawing  image

will  disappear  for a few seconds and be re-displayed  with  the

point picked as the center of the graphics screen.

12.2.1  Panning with the Numeric Keypad

If you are not using the Keyboard as your input device

you may also pan using the keys on the numeric keypad.  This

can  be done only when the <Num Lock> key is not  activated.

The arrows on the number <4>, <6>, <8> and <2> keys indicate

the  direction of the pan when you press the  button:  left,

right, top and bottom (respectively). In addition, keys <1>,

<7>, <9> and <3> pan to each of the four corners.

Use  90  FullWindow  to  center  and  redisplay  the  entire

drawing.

12.3  90 FullWindow

The FullWindow option is usually used after working with  88 Zoom

or 89 Pan. Press <90> to center and re-display the entire drawing

image on the graphics screen.

You  may use this option at anytime during your  drawing  session

regardless  of the menu displayed.  After using  FullWindow,  the

program returns you to the task you were using.

12.4  99 Redraw

As you work, portions of your drawing may become obscured through

manipulation  and editing of lines and objects. Redraw  refreshes

your  screen  image by redrawing the graphic  screen.    You  may

redraw  at  any time during the drawing session.  To  redraw  the

graphics screen image, press <99>.

page 41

13.0  94 Deactivate

Use Deactivate when a drawing session has ended or when you  need

to save your drawing and continue work. You may save your drawing

to  the  hard disk or floppy disk. The drawing name can  also  be

changed  using this operation and you may "pack" the  drawing  to

eliminate unwanted data and free-up your drawing's memory.

Press <94> and a new Auxiliary Menu is displayed.

It is recommended that you save your drawing after every  fifteen

minutes  of work, or when you have finished a major  DRAW  opera-

tion.  You  should  also save your work on a  floppy  disk  using

12 Path:  at  the conclusion of every drawing session.  (You  may

also copy drawings to floppy disk with the DOS "COPY" command.)

13.1  10 PACK DRAWING

A "PACK" eliminates items in you drawing's memory such as deleted

objects  and  unnecessary information, to  free-up  more  drawing

space.

Press <10> and the DRAW program will pack the drawing.  If  there

are  a  lot of objects on the drawing, the process could  take  a

minute  or  two.  Wait  until  the  prompt  says  "DRAWING   PACK

COMPLETED" before selecting another Master Menu option.

The  percentage  on the upper right of the menu  screen  will  be

lower after a drawing is packed.

13.2  11 Name:

The drawing name selected when you laid out your drawing will  be

displayed with this option.  You may change the name before  sav-

ing  or exiting the drawing.  Press <11> and type a  new  drawing

name (maximum of eight characters); then, press <Enter>.  The new

name will be displayed on the menu.

13.3  12 Path:

Change the path to save your drawing to another disk: either  the

hard  disk (referred to as the C: or D: drive); or a floppy  disk

(A: or B: drive).

The path is automatically set to the A: drive.  Other directories

can be created to hold drawings.  Refer to your DOS manual on the

procedures for creating directories.

page 42

To change the path to the hard disk, press 12 and type <C:>  fol-

lowed by the specific path name.  For example, to specify a \DRAW

sub-directory named "Drawings" you would type:

\DRAW\DRAWINGS;

then, press <Enter>.

To change the path to the floppy disk press <12> and type A:  (or

B:  if  you are specifying the second floppy disk  drive);  then,

press the <Enter> key.

Note:

A  floppy disk must be "formatted" before you  may

save a drawing on it. Refer to the DOS manual that

came with your computer for instructions on  using

the "FORMAT" command.  NEVER FORMAT THE HARD  DISK

DRIVE.

13.4  15 Save Drawing

This  option  saves your drawing to the hard or floppy  disk,  as

specified by 12 Path: and 11 Name:.

If  you have chosen a new name, all changes will be saved to  the

new  drawing and the old one will remain the same.  To save  your

drawing  press  <15>.  You may continue working  by  selecting  a

Master Menu task or exit the drawing with 16 Exit.

13.5  16 Exit

Press  <16> to end the drawing session and return to the  Opening

Menu  where you may: 1) create a new drawing; 2)  select  another

drawing  from  the "Active Drawing" section; or 3)  "Logoff"  and

exit from the DRAW program.

If  you have not saved the drawing, a warning message will  flash

below  the  prompt telling you the drawing has  not  been  saved.

Press <Enter> to reject the operation and select 15 Save.  Other-

wise, press <16> again to exit without saving the drawing.

page 43

14.0  97 PrintPlot

97 PrintPlot  quickly  makes a hard copy of the  graphics  screen

using your printer. This is not a high-quality plot; it's a rough

printout  (called a "dump") of your graphics screen.   It  allows

you to see on paper how a drawing--or a portion of it--is shaping

up.

Cascade's  "Phoenix" and "Atlas" programs provide a  Plot  option

which  allows  you  to  output your  drawing  on  a  conventional

plotter.  You may also output the drawing to an HPGL file for use

with most DeskTop Publishing programs such as "Ventura Publisher"

and "PageMaker."  The "Atlas" program also has a conversion util-

ity that allows you use drawings from other CAD/Drawing programs.

Press <97> and a new Auxiliary Menu screen is displayed

The PrintPlot option works with CGA, EGA and HERCULES  compatible

monitors, and prints out on most "Epson"## or Epson-compatible  dot

matrix printers.  Some of the printers supported by  97 PrintPlot

are listed under the heading **** Printers **** on the left side

of the Auxiliary Menu.

Since  PrintPlot prints what's on your graphics screen,  you  can

use 88 Zoom and/or 89 Pan to display only a portion of the  draw-

ing  for printing; if you want to PrintPlot the  entire  drawing,

use 90 FullWindow.

Select a printer from the list displayed on the Auxiliary Menu by

pressing  the two-digit menu number.  If you don't see your  par-

ticular  brand or model, choose one that is compatible (refer  to

your printer's operations manual).

When  the suitable printer is highlighted, select 10 Standard  or

11 Rotated to begin the PrintPlot. After the printer begins,  you

may press the Esc key to interrupt the operation.

14.1  10 Standard / 11 Rotated

Press <10> to print the graphics screen horizontally on the page;

press  <11> to print (rotate) the screen vertically. Press  <Esc>

to interrupt the PrintPlot.

page 44

15.0  98 DrawDefs

Press  <98>  and new Auxiliary Menu options are  displayed  which

allow you to change certain drawing definitions.

15.1  11 Ref Grid = 10

This option determines the interval at which a colored  reference

line appears when line grids are displayed using the /G  command.

It does not affect dot grids.

The default interval is 10, which displays a reference line every

tenth  grid line. To change the "reference grid", press <11>  and

type a new reference grid interval; then, press <Enter>.

A color monitor is required for this option.

15.2  12 +DOT Grids / +LINE Grids

This  option determines whether dot or line grids  are  displayed

when  the /G command is used. Dot grids are the default  setting.

Press <12> to toggle between "DOT" and "LINE" grids.

15.3  13 Ref Color-Blue

This  option determines the color of the "reference  grid"  lines

specified in 11 Ref Grid = ("Blue" is the default setting).

To  change  color, press <13> and type the first  letter  of  the

reference   grid  color:  (W)hite,  (A)qua,  (M)agenta,   (B)lue,

(Y)ellow, (G)reen or (R)ed; then, press <Enter>.  The new  refer-

ence grid color will be displayed on the menu.

A color monitor is required for this option.

15.4  14 Wrk Color-Aqua

Determines  the color of the "work grid" lines or dots  (Aqua  is

the default setting).

To  change  color, press <14> and type the first  letter  of  the

working grid color: (W)hite, (A)qua, (M)agenta, (B)lue, (Y)ellow,

(G)reen  or (R)ed; then, press <Enter>. The new work  grid  color

will be displayed on the menu.

A color monitor is required for this option.

15.5  16 Set Range On / 17 Set Range Off

When  a drawing session begins, all the levels of a  drawing  are

page 45

displayed on the graphics screen.  You may choose to turn some of

these  levels  off so they won't be displayed.   (Turning  off  a

level  does not permanently remove parts of the drawing;  objects

on turned-off levels won't be displayed.  They will reappear when

the level is turned back on.)

Use  17  Set Range Off to turn some or all of the levels  off  by

indicating a range of consecutively-numbered levels.  Press  <17>

and  type the "beginning level number" of the range to be  turned

off (lowest level); then, press <Enter>.

Next,  type the "ending level number" (highest level)  and  press

<Enter>.   All  levels between the two numbers  entered  will  be

turned off.

Turn  a  range  of levels on with  16 Set Range On  in  the  same

fashion.  Press <16> and type the "beginning level number" of the

range to be turned off (lowest level); then press <Enter>.  Next,

type  the ending level number (highest level) and press  <Enter>.

All  levels between the two numbers entered will be  turned  back

on.

Note:

To  display all levels, you would choose  "0"  and

"255".  You  can also turn all levels  off  or  on

during  your drawing session by using the /L*-  or

/L*+  commands which are explained in the  section

"Drawing Aids".

15.6  18 Hitwin= 0.150 in

This  option  determines  the size of your  input  device's  "hit

window" (the area inside the cursor that can pick objects on  the

drawing).

The  default settings are 0.150 inches for English drawings,  and

0.300  centimeters for metric drawings. To change the hit  window

size  press  <18> and type a new value from .015 to  1.00;  then,

press <Enter>.

15.7  28 X-Grid= 1.00 in / 29 Y-Grid= 1.00 in

Press  <28> to change the horizontal (X-Grid) grid size.  Type  a

new value and press <Enter>. Both 28 X-Grid= and 29 Y-Grid=  will

display the new grid size.

To  make  the "Y-Grid" a different size, press <29>  and  type  a

different value; then, press <Enter>.  The new grid display  will

appear on your screen when activated with the /G command.

Note:

You may also change both grid sizes while drawing using the /N

command, which is also explained in the section "Drawing Aids".

page 46

This manual was written and prepared by J. Matthew Fabris

for Cascade Graphics Systems, Inc.

Copyright 1988,  Cascade Graphics Systems, Inc.

This manual is protected by Federal Copyright Laws and may not be

copied, revised, translated or reproduced in any form without the

express written permission of Cascade Graphics Systems, Inc.

For  information  on software documentation  services,  including

desktop publishing and disk/on-line manuals,  contact:

J. Matthew Fabris

c/o Cascade Graphics Systems, Inc.

2698 White Road

Suite 200

Irvine, California  92714

(714) 474-6200

page 47

##################################################################################