���������������������������������������������������������Ŀ

�       ASCII Documentation File for ... CAPQUANT         �

�                       Version 9311                      �

���������������������������������������������������������Ĵ

�             A MS-DOS Software System for...             �

�                                                         �

�      ***  Quantification of Process Capability  ***     �

�                                                         �

�     Via the Cumulative Distribution of Regret Indices   �

���������������������������������������������������������Ĵ

�            A Quality Assurance Training Tool:           �

�        Statistics Committee of the QA Section of        �

�      the Pharmaceutical Manufacturers Association       �

�                                                         �

�       Bob Obenchain, CompuServe User [72007,467]        �

�����������������������������������������������������������

WARNING: Running CapQuant.EXE requires at least 375kBytes of RAM.

Table-of-Contents

Introduction to CapQuant's Capability Quantification Methodology . . 1

Usage of the CapQuant Pop-Up Menus . . . . . . . . . . . . . . . . . 3

Preserving Time-Series Order and Forming Moving Statistics . . . . .18

REGRET FUNCTIONS: Quantify Impact of Deviations from Target. . . . .22

Equivalent Nonconformance and Expectancy . . . . . . . . . . . . . .27

REFERENCES . . . . . . . . . . . . . . . . . . . . . . . . . . . . .30

Appendix: Technical Descriptions of the Poissonization Process . . .33

CapQuant Software Update History . . . . . . . . . . . . . . . . . .38

�����������������Ŀ

�   INTRODUCTION  �

�������������������

CapQuant.EXE is a software system for IBM-compatible (MS-DOS) Personal

Computers that implements the statistical methodology described in my

1991 and 1993 manuscripts "Regret Indices and the Quantification of

Process Capability" and "Cumulative Capability Curves," respectively.

Here, "regret" can be almost any surrogate measure of how cost-of-

poor-quality (from a customer and/or regulatory viewpoint) increases

as product deviation from an intended "target" value increases.

CapQuant.EXE constructs Cumulative Capability (CC) curves and displays

them in an "interactive" graphics mode that allows you to read off

numerical values by moving an on-screen cursor.  CC curves quantify

process "yields" (conformance fractions) for a entire class of

intervals.  When regret is a "2-to-1" transformation of the original

process characteristic (so that the only information loss is the

numerical sign of the deviation from target), CC yields correspond to

ALL intervals with equal, maximum regret at their two end-points.

Specifically, when the regret function is symmetric-about-the-target

CapQuant [Version 9311] . . . . . . . . . . . . . . . Page 2 of 39

as well as 2-to-1, CC yields are those for ALL symmetric-about-the-

target intervals.

CC curves provide a highly-intuitive and widely-applicable capability

quantification methodology that characterizes performance on a cost-

unitless "index" scale.  One ends up with a CC curve for each product

or process studied.  In summary,

�������������������������������������������������������������������Ŀ

� CC curves not only... serve as benchmarks for current capability  �

�                       as well as for quality improvement efforts  �

�                                                                   �

�           but also... enable and, in fact, encourage comparisons  �

�                       across diverse processes.                   �

���������������������������������������������������������������������

Any type of quantitative (variables) data measurement can be used as a

basis for CC quantification via a three-stage analysis process:

(i) Convert each observed numeric value into a REGRET, which is a

measure of long-range economic impact or customer/regulatory

dissatisfaction that results from product deviations from an

intended TARGET value (or values.)

CapQuant allows the user to pick a regret function from

any of 10 general families and to specify its target value.

(ii) Smooth and Standardize the observed distribution of historical

regret values (via POISSONIZATION) to form a REGRET INDEX.

CapQuant determines an EQUIVALENT NONCONFORMANCE value for

each regret and the corresponding EQUIVALENT EXPECTANCY,

the expected regret within historical capability levels.

The corresponding INDEX is simply the ratio of observed

to expected regret.  An index value of 0 is ideal, while

an index value of 1 is the historical quality standard

(or capability threshold.)  Customer and/or regulatory

dissatisfaction increases as the index value increases.

Deciding on exactly how much of available historical data

to use in defining current process capability can be

troublesome, especially if you are using a regret function

(like quadratic) which is highly sensitive to outliers.

Anyway, I recommend that you "start out" by examining

all available history that you feel are still relevant to

current operations.

(iii) Display the CC curve, which is the Empirical Distribution

Function (EDF) of observed Regret Indices.  CapQuant draws

CC curves in interactive, medium-resolution (CGA, 320x200

pixel) mode.

CapQuant [Version 9311] . . . . . . . . . . . . . . . Page 3 of 39

CapQuant can plot the cumulative distribution function for

process history consisting of as many as 1200 observations.

Both the observed (raw) and Poisson smoothed forms are

displayed.

CapQuant also allows direct graphical comparisons between

the EDF of at most 400 of your "newest" sample results

(most recent production) with your historical CC curve.

CapQuant provides all of the computational/graphical tools you needed

to implement specific analyses of the above type.  CapQuant creates an

ASCII (text) output file of extremely detailed results that, following

minor manual editing, provides an ideal data input file to statistical

and/or presentation graphics systems, such as SAS.  But only a

fraction of CapQuant's outputs (those which are displayed on your

computer screen as well as written to the ASCII output file) are

needed to characterize the (historical) capability of a process.

�����������������������������Ŀ

�  *** USAGE of CAPQUANT ***  �

�������������������������������

The CapQuant software system is invoked from MS-DOS by entering its

8-character name, CAPQUANT, at your DOS prompt...

Prompt> capquant

CapQuant does not expect or accept any "command-line" arguments.

Instead, CapQuant will prompt you to provide all information it needs

to perform analyses.  And each such prompt usually displays a [default

value] in square brackets that you may accept by simply pressing the

ENTER key.

WARNING ONE: Aberrant behavior WILL result if you attempt to execute

CapQuant at any time that your system has less than 375kBytes of

available random access memory (RAM).  CapQuant is actually a much

larger program than it might appear to be.  Yes, its .EXE module

occupies only 85kBytes.  But, before compression with LZEXE 0.91 (c)

Fabrice BELLARD, CapQuant.EXE was 251kBytes in size, and this does NOT

include the space CapQuant needs to read data and store intermediate

results.

WARNING TWO: CapQuant assumes that your personal computer has a COLOR

monitor that is capable of making (at least) CGA graphical displays.

CapQuant uses color in its text-mode windows/menus.  Screens with text

written in black on a green or cyan background will appear to be BLANK

if you do not have a color monitor.  And if your system is incapable

of graphics, your system may crash when CapQuant attempts to draw P-P

probability plots and Cumulative Capability Curves.

CapQuant [Version 9311] . . . . . . . . . . . . . . . Page 4 of 39

����������������������������������������������������Ŀ

�  CapQuant Input/Output File Naming Conventions...  �

������������������������������������������������������

The ASCII (text) files used by CapQuant for data input and output

consist of a filename (of at most eight characters) specified by the

user followed by a period (.) and one of four specific 3-character

filename EXTensions.

CONTAMIN.HST  ...Batch input of Historical Data (at most 1200 obs.)

CONTAMIN.NEW  ...Batch input of New Data (at most 400 observations)

CONTAMIN.OUT  ...CapQuant Output Save File.

CONTAMIN.XXX  ...CapQuant Output of Trial REGret Parameter Settings.

CONTAMIN.REG  ...Edited .XXX File for Automatic REGret Calculations.

��������������������������������������Ŀ

�   Example CapQuant Walk-Through...   �

����������������������������������������

The CapQuant start-up screen will appear as follows...

����������������������  Capability Quantification �����������������ͻ

�                                                                   �

�                      CAPQUANT.EXE...Version 9311                  �

�                                                                   �

�                   A Quality Assurance Training Tool:              �

�           Statistics Committee of the QA Section of the PMA       �

�                                                                   �

� ���������������������������������������������������������������ͻ �

� �  Will Process HISTORY be Input via K = Keyboard ? ...or       � �

� �                                    B = Batch File ?           � �

� �                                                               � �

� �  Press the   K   or   B   key now -->                         � �

� ���������������������������������������������������������������ͼ �

�������������������������������������������������������������������ͼ

If you select Keyboard input, you will be prompted to type in all

historical data values using your keyboard.  But, in this case,

CapQuant will also be creating a "Batch Input" file that you could use

in future CapQuant runs to repeat and/or modify your initial analyses.

On the other hand, if you are familiar with a "screen oriented"

personal computer (ASCII) text editor, you might prefer to start by

creating your own "Batch Input" file.  This strategy can give you more

flexibility in data entry and validation than is possible inside of

CapQuant.  Simply mimic the format of an example CapQuant Batch Input

file.  For example, the Batch Input file could consist for the

following 7 records (rows), each starting in column 1:

1        ...Number of Data Values per Observation

THICKNES ...Name of Variable ONE

1.09

2

13

CapQuant [Version 9311] . . . . . . . . . . . . . . . Page 5 of 39

4.7

3

q

where the letter Q at the start of the last record signals the "end of

data entry."

Once you respond to the initial CapQuant prompt by pressing either the

K or the B key, the "Window" will clear itself, and you will either

(i) be prompted to specify a MS-DOS filename, consisting of at most 8

characters, for capture of Keyboard input or (ii) select an existing

Batch Input (.HST) file from the bounce-bar menu...

���������������������� Capability Quantification ��������������������Ŀ

�                                                                     �

�    Batch Input of Process History Selected...                       �

�                                                                     �

�    At colon Prompts : ...simply press ENTER to get the [default].   �

�                                                                     �

�   �File Names�                                                      �

�   � CONTAMIN �      Arrow Keys: Move Highlight Bar Up / Down.       �

�   ��NORMAL��ۺ                                                      �

�   � DISSOLUT �      Return Key: Selects the Highlighted File.       �

�   ����������ͼ                                                      �

�                     Escape Key: Abandon BATCH CapQuant Input.       �

�                                                                     �

�����������������������������������������������������������������������

Whether input comes from your Keyboard or from a Batch file, the

CapQuant Output File can have the same filename (of at most 8

characters) as does the Input File.  But the 3-character MS-DOS

filename EXTension for CapQuant Output Files will always be ".OUT".

On the other hand, although the [default] output filename will always

be to use the same filename as the input file, there are circumstances

when you will probably want to us a DIFFERENT filename for your

output.  After all, if you always use the same output filename, that

file will get over-written and all previously saved results will be

lost.  Below, we illustrate changing the output filename to

NORMDIFF...

�������������������  Capability Quantification ������������������ͻ

�                                                                 �

�  Batch File Input Selected...                                   �

�                                                                 �

�  At colon Prompts : ...simply press ENTER to get the [default]. �

�  -------------------------------------------------------------- �

�                                                                 �

�  The Batch Input file is to be: normal.hst                      �

�                                                                 �

�  Specify filename for Detailed Output [normal.out] : normdiff <<<<<<

�  The CAPQUANT Output Save file is to be: normdiff.out           �

�                                                                 �

�          Press Q to QUIT now...Other Key to Continue...         �

�����������������������������������������������������������������ͼ

CapQuant [Version 9311] . . . . . . . . . . . . . . . Page 6 of 39

A [Black on Green] window for Parameter Specification appears next...

��������������������    CapQuant  Parameters    �����������������ͻ

�                                                                 �

�  Does data consist of Single Values or Pairs (1 or 2) :         �

�  Number of Data Values per Observation =  1.                    �

�                                                                 �

�  Should "Moving" statistics be calculated ?                     �

�          0 => No moving statistics...                           �

�          1 => Form first differences...                         �

�          2 => Exp.Wgt.Mov.Average...                            �

�          3 => Form Deviations from EWMA...                      �

�                                                                 �

�  WARNING: Options 1, 2, & 3 can only be used when               �

�           data are input in time-series order.                  �

�        ...And the first observation will be lost                �

�           unless a Start-Up Value is specified.                 �

�                                                                 �

�  What is your choice ? [0] :      <<<< Here we accept the DEFAULT by

�  Moving Statistics Option =  0.        simply pressing the ENTER key.

�                                                                 �

�  What is the NAME of Variable ONE ? [variable1] :               �

�  First  Variable Name: THICKNES                                 �

�                                                                 �

�  Historical Data, in any sequence, are to be entered            �

�  in the Next Window...  Press Q to QUIT now...                  �

�����������������������������������������������������������������ͼ

We will discuss the Moving Statistics options (1,2,3 above) in the

section of this documentation that starts on page 11.  Anyway, another

good reason to change the name of the Output File on the previous

screen would be because you want to explore several "smoothing" and/or

"differencing" transformations of the data from one Input File.

A [Blue on Cyan] Data Entry window now appears.  If you are using

Keyboard input, CapQuant will continue prompting for either single

values or data pairs until you ENTER the letter Q to signal an end to

data entry.  The number of observations entered will then be

displayed...

����������������������   Data Entry Window    �������������������ͻ

�                                                                 �

�        List Historical Data in Output File? [y|n] : n           �

�                                                                 �

�        Number of Historical Observations =  500                 �

�                                                                 �

�        Data Entry Complete...                                   �

�        Press Q to QUIT now...Other Key to Continue...           �

�����������������������������������������������������������������ͼ

Small windows will now Pop-Up within the [Blue on Cyan] Large Window.

You first selection from the left-hand "Process Capability MENU"

should always be "R".  (...Unless you wish to "eXit" without making

any calculations!)

CapQuant [Version 9311] . . . . . . . . . . . . . . . Page 7 of 39

You first selection from the middle "REGRET Analysis MENU" should

always be "S" or "A"...unless you wish to Return without making any

calculations!

After "S", you choose a Regret-Function-Family from the right-hand

"REGRET Function MENU"...

�����������������������������ͻ

�  Process Capability MENU    �

�                             �           �������������������������ͻ

�     R = Regret Analysis     �           � REGRET Function MENU    �

�             ���������������������������ͺ                         �

�     D = Disp�     REGRET Analysis MENU  � 1...GOAL-POSTS          �

�             �                           � 2...QUADRATIC           �

�     C = CC C� S = Specify Regret Functio� 3...ABSVALUE            �

�             �                           � 4...1-SIDED QUADRATIC   �

�     M = Moni� I = Regret INDEX Computati� 5...BILINEAR            �

�             �                           � 6...END-POINT QUADRATIC �

�     X = eXit� L = Lack-of-Fit to a Poiss� 7...LOGISTIC            �

�             �                           � 8...INVERTED NORMAL     �

� Choice --> R� C = Composite Lack-of-Fit � 9...RANGEMAX            �

�             �                           � 0...NONCONFORM/EXPTANCY �

�������������ͺ A = Automatic Regret Calcs� A...Abandon/Quit        �

�                           �                         �

� R = Return to Main Menu   �                         �

�                           � Choice -->              �

� Choice --> S              �                         �

�                           �������������������������ͼ

������������������������������ͼ

The regret functions which can be defined in CapQuant belong to one of

ten general families.

1) attributes regret, which is zero inside an interval and one

outside that same interval,

2) quadratic regret, where the regret is the square of the

deviation of the measurement from its target value,

3) linear regret, in which the economic impact of a value is

directly proportional to that value,

4) one-sided regret, which is quadratic on one side of a

target and zero on the other side of that target value,

5) opportunity regret, in which the economic impact of a

demand for a perishable product is measured relative to

the inventory on hand,

6) end-point quadratic regret, in which the minimum and

maximum observed values in samples of fixed size are

monitored, and regret is the sum of the quadratic regret

at the two end-points,

CapQuant [Version 9311] . . . . . . . . . . . . . . . Page 8 of 39

7) bounded max regret, in which regret follows a double-

logit pattern that is V-shaped in its center (at the

target) but flat like attribute regret in its tails,

8) inverse gaussian regret, in which regret looks like an

upside-down bell-shaped curve (quadratic at the target

but flat like attributes regret in its tails),

9) range (linear) regret, in which the minimum and maximum

observed values in samples of fixed size are monitored,

and regret is the difference, maximum minus minimum,

and

0) an unspecified pattern called "nonconformance-expectancy."

In this case, the data are assumed to have been

"Poisson"ized outside of CapQuant, but you can still use

CapQuant to plot Cumulative Capability Curves and/or to

test Lack-of-Fit.

See the section of this CapQuant documentation, starting on page 10,

that discusses REGRET FUNCTIONS for detailed information on these ten

families.

The numerical example used here in the documentation is based upon...

Regret Type:    QUADRATC

TARGET = Value corresponding to ZERO REGRET = 20.00

Once you have specified a functional form for your Regret Function and

have selected values for its Parameters (TARGET, PARM1, PARM2), the

"REGRET Analysis MENU" will re-appear.  One's second selection from

this menu is usually the second item, "I"...

CapQuant [Version 9311] . . . . . . . . . . . . . . . Page 9 of 39

������������������������������ͻ

�     REGRET Analysis MENU     �

� S = Specify Regret Function  �

� I = Regret INDEX Poisson Fit �

� C = Composite Poisson Fit    �

� Q = Q-Q Plot for Gamma Fit   �

� A = Automatic Regret Calcs   �

� R = Return to Main Menu      �

� Choice --> I                 �

������������������������������ͼ

������������������   Regret Calculations  ���������������ͻ

�                                                         �

� Observations (Periods) = 500                            �

� Regret Type   =  2                                      �

�                                                         �

� Observed Average (Expected) Regret =   34.962           �

� Observed Variance of Regret =  2354.166                 �

� Corresponding Equivalent Expectancy = 0.5192            �

�                                                         �

� Enter Objective Expected Regret [   1.000] : 35         �

�         using...   35.000                               �

�                                                         �

� Enter Objective Variance [2354.321] :  2500             �

�         using...   2500.000                             �

�                                                         �

� Equivalent Expectancy per Observation =   0.4900        �

���������������������������������������������������������ͼ

As in the above example, the "Objective Expected Regret" value is

usually taken to be a convenient, "rounded" value (35, here) that is

fairly close to the observed, sample value (34.9).  The initial

default value for Expected Regret will always be [ 1.000].  But, if

you select item "I" on the "REGRET Analysis MENU" more than once, the

subsequent default value will be the value you entered in the previous

iteration.

The default value [2354.321] for the "Objective Variance," is the

numerical value that (given an objective expected regret of 35 rather

than the observed 34.9) would equate the objective "Equivalent

Expectancy per Observation" to the observed value of 0.5192.  This

default value will not, necessarily, also equate the mean and variance

of the rescaled regret distribution...unless the objective and

observed expected regret values are exactly equal.

In the above example, a convenient, "rounded" value (2500) is also

entered for the "Objective Variance,"  and the resulting "Equivalent

Expectancy per Observation" is computed by CapQuant to be 0.49.

Here... EXPREGRET objective =    35.0 ...observed =   34.9621

VARREGRET objective =  2500.0 ...observed = 2354.3206

You should try out several choices for ER = Objective Expected Regret

and VR = Objective Variance (35 and 50, above.)  For example, you

CapQuant [Version 9311] . . . . . . . . . . . . . . . Page 10 of 39

should change ER and/or VR when you are unhappy with the resulting

"Equivalent Expectancy per Observation" (0.49 above ) or when the

resulting Kolmogorov-Smirnov Lack-of-Fit statistic is significant...as

it is here!

�����������������Poisson Lack-of-Fit Test����������������ͻ

�                                                         �

� Kolmogorov-Smirnov Statistic =   0.56263                �

� 5% Critical Value =  0.06                               �

� 1% Critical Value =  0.07                               �

� Lack-of-Fit is Highly Significant,                      �

� 1% Critical Level.                                      �

�                                                         �

� Press a Key to View the P-P Plot...                     �

�                                                         �

� Index, ObsFreq, CumFreq, and Poisson Fit...             �

�  0.00   0.05000   0.05000   0.61263                     �

�  0.01   0.00000   0.05000   0.61263                     �

�  0.02   0.12000   0.17000   0.61263                     �

���������������������������������������������������������ͼ

See the section of this documentation, starting on page 17, that

discusses Equivalent Nonconformances and Expectancy for detailed

information on this CRITICAL phase of the analysis.  Statistically

significant Poisson Lack-of-Fit can even suggest the need to change

the very form of your regret function ( item "S" ).

NOTE1:  Always re-run item "I" after each change you make to the

regret function via item "S".  And never select items "C" or "Q" except

after item "I".

NOTE2:  All "Equivalent Nonconformance" values were rounded to the

nearest full integer value for the purpose of the above Lack-of-Fit

test because Poisson random variables take on only integer values.

But this rounding really represents an unnecessary loss-of-precision

in most quality monitoring applications.  Thus CapQuant does NOT round

nonconformance numbers to integers in most of its calculations.  In

fact, CapQuant lists nonconformance/expectancy results to 2 decimal

places in the ASCII output file.

The highly significant lack-of-fit in the above example is probably

not "practically important."  It resulted, primarily, because the

equivalent expectancy was "small" (0.49 => a step-size larger than 2

units on the index scale) and our sample of data is "large" (500.) To

illustrate this, choose item "C" and examine composites of 6

consecutive regret values...

CapQuant [Version 9311] . . . . . . . . . . . . . . . Page 11 of 39

������������������������������ͻ

�     REGRET Analysis MENU     �

� S = Specify Regret Function  �

� I = Regret INDEX Poisson Fit �

� C = Composite Poisson Fit    �

� Q = Q-Q Plot for Gamma Fit   �

� A = Automatic Regret Calcs   �

� R = Return to Main Menu      �

� Choice --> C                 �

������������������������������ͼ

��������������Composite Poisson Lack-of-Fit��������������ͻ

�                                                         �

� Composite Regret Calculations...                        �

� Number of Original Observations = 500                   �

� Regret Type = 2                                         �

� Equivalent Expectancy (Single Obs.) =   0.4900          �

�                                                         �

� How many consecutive Regret values should be Added      �

� to form Composite Regrets ? (2 to 10) [4] : 6           �

�                                                         �

� Number of Composite Observations = 83                   �

� Composite Equivalent Expectancy =   2.9400              �

� Composite Poisson Lack-of-Fit...                        �

�                                                         �

� Kolmogorov-Smirnov Statistic =   0.13451                �

� 5% Critical Value =  0.15                               �

� 1% Critical Value =  0.18                               �

� Composite distribution is not significantly             �

� different from the fitted Poisson.                      �

�                                                         �

� Press a Key to View the Composite P-P Plot...           �

���������������������������������������������������������ͼ

In other words, lack-of-fit to a Poisson distribution declines

steadily for this dataset as 2, 3, 4, or 5 regrets are "composited";

all lack-of-fit becomes statistically insignificant when 6 or more are

"composited."

The hallmark of continuous improvement is then this: the EE=ER*ER/VR

resulting from individual measurements made on the process will

continually drop.  And yet, statistically significant lack-of-fit in

discrete Poisson approximations is common in cases where EE is small

(0.5 or less) and data are plentiful.  After all, the Poisson integer

"step-size" then balloons to 2 units or greater when re-expressed on

the regret index scale.  The rounded and un-rounded regret index

values (corresponding to integer or non-integer ENs) can thus be quite

different numerically.

CapQuant [Version 9311] . . . . . . . . . . . . . . . Page 12 of 39

The fourth item ("Q") on the "REGRET Analysis MENU" provides a way to

smooth a sample of observed regret indices with a continuous gamma

distribution fit only to a specified number of the smallest regret

order statistics...                                ===============

================

������������������������������ͻ

�     REGRET Analysis MENU     �

� S = Specify Regret Function  �

� I = Regret INDEX Poisson Fit �

� C = Composite Poisson Fit    �

� Q = Q-Q Plot for Gamma Fit   �

� A = Automatic Regret Calcs   �

� R = Return to Main Menu      �

� Choice --> Q                 �

������������������������������ͼ

������������������ Gamma Q-Q Probability Plots ���������������Ŀ

�                                                              �

� Gamma Lack-of-Fit...                                         �

� Initial Equivalent Expectancy =   0.4900                     �

� Sorting Observed Regret Values...                            �

�                                                              �

� How many of the smallest Regret Order Statistics should used �

� to estimate Gamma Parameters ? [125 to 500] : 200            �

�                                                              �

� Number of Iterations = 5                                     �

� Final Slope =  1.0081                                        �

� Final Equivalent Expectancy =   0.5426                       �

�                                                              �

� Press a Key to View the Q-Q Probability Plot...              �

�                                                              �

����������������������������������������������������������������

Poisson or gamma lack-of-fit that occurs only in the extreme

right-hand tail of an EN sample is easily tolerated.  The essential

feature of a "successful" smoothing for a regret index distribution is

that it provides a good representation over the range that represents

relatively good performance, I < or = 1.  In my experience at least,

processes in doubtful states of statistical control tend to be

unstable primarily in their right-hand regret tail.

Q-Q probability plotting methods yield an estimate of scale as well as

shape of the best fitting gamma distribution.  Thus, while preliminary

EN rescaling of regret is not really necessary, EN rescaling is still

recommended because the estimated gamma scale parameter will then be

approximately 1, at least when all of the available data are used in

the analysis.

For gamma Q-Q probability plotting, Chambers, Cleveland, Kleiner and

Tukey(1983), Chapter 6, recommend use of the Wilson-Hilferty(1931)

normal approximation for the cube root of gamma variables and the

Hastings(1955) rational function approximation for normal quantiles.

Then it is straight-forward to estimate the slope, �, of a

zero-intercept regression of the k smallest observed EN regrets onto

CapQuant [Version 9311] . . . . . . . . . . . . . . . Page 13 of 39

their corresponding approximate gamma EE quantiles.  At iteration i+1,

the trial value for EE(i+1) would be, say, one third of the way from

EE(i) to [EE(i) times the i-th � estimate.]  Iterative fitting

would then halt when either [1] the i+1-th zero-intercept regression

fails to show increased multiple correlation between the k smallest

observed EN(i+1) regrets and their fitted gamma EE(i+1) quantiles, or

[2] � converges to 1.0.

Let us suppose that you now select item "R" on the REGRET MENU

followed by item "D" on the Main MENU...

�����������������������������ͻ

�  Process Capability MENU    �

�     R = Regret analysis     �

�     D = Display CC curves   �  <--- 2 nd Selection = D

�     C = CC Confidence Limits�

�     M = Monitor capability  �

�     X = eXit CAPQUANT       ����������������ͻ

� Choice --> C                �ysis MENU       �

�����������������������������ͼoss Function    �

� . . . . . . . . . . . . . . �

R = 1 st Selection --->  � R = Return to Main Menu     �

� Choice --> R                �

�����������������������������ͼ

�������������������� Process Capability Plots ������������������ͻ

� Number of Observations = 500                                   �

� Show CC Curve as Stair-Steps ? [Y|n] :n       <---Alternative  �

� Show Poisson or Gamma Fitted CDF ? [P|g] :g   <---Alternative  �

����������������������������������������������������������������ͼ

While viewing the CGA graphics display of the Observed and Fitted

Cumulative Capability Functions, you may always press the SpaceBar or

Function Key F1 to view the following HELP Screen...

�����������������������������������������Ŀ

�  This HELP screen .... Space Bar or F1. �

�                                         �

�  ERASE PLOT......press ENTER or ESCape. �

�    Note: 2nd KeyPress sometimes needed. �

�                                         �

�  On-Screen Cursor & VALUES ... press V. �

�  Move Right/Left with ArrowKeys by 0.01 �

�             or... Tab/BackSpace by 0.25 �

�                                         �

�  Screen PRINT/SAVE Keys:  First Press S �

�  and then press a Second Special Key... �

�   L or F10 dumps to HP LaserJet/DeskJet �

�   I or F9  dumps to IBM Graphics Matrix �

�   E or F8  dumps to Epson FX, JX, or LQ �

�   O or F7  dumps to Okidata Microline   �

�   S saves screen to disk in PCX format. �

�                                         �

�  CapQuant, ver9311.......Press any key. �

�������������������������������������������

CapQuant [Version 9311] . . . . . . . . . . . . . . . Page 14 of 39

One iteration through the three-stages of quality monitoring

calculations is now complete.  You now have the options of either

pressing "R" to return to the REGRET Menu for further calculations,

"C" to calculate and display Confidence Limits on the CC curve, "M" to

Monitor Capability using NEW observed process results, or "X" to exit

CapQuant and return to DOS.  (Except possibly in "training" exercises,

there will usually be no need to simply repeat Cumulative Capability

display, item "D", without first changing either the regret function

or the objective regret mean or variance.)

Let's choose item "C" next...

�����������������������������ͻ

�  Process Capability MENU    �

�     R = Regret analysis     �

�     D = Display CC curves   �

�     C = CC Confidence Limits�

�     M = Monitor capability  �

�     X = eXit CAPQUANT       �

� Choice --> C                �

�����������������������������ͼ

���������������������� CC Confidence Limits ��������������������ͻ

�                                                                �

� Number of Observations = 500                                   �

�                                                                �

� On the graphics screen, press F1 or Space Bar for HELP.        �

� Press the V key to interactively View numerical Values.        �

� Press a Key to View the CC curve and Confidence Limits...      �

�                                                                �

����������������������������������������������������������������ͼ

Nair and Freeny(1993) point out that asymptotic confidence bands,

constructed from (nonparametric) empirical distribution functions, are

of the form

CC(I)   +/-   K * sqrt[ CC(I) * ( 1 - CC(I) ) / N ]

where K is a constant that depends upon the desired confidence level

(one minus alpha) and N is the total number of regret indices defining

the CC curve.

Exact confidence intervals that are valid pointwise (for a given

index, I) can be constructed from the fact that N * CC(I) follows a

Binomial distribution.  When a Normal approximation to this Binomial

is used, the constant K above can be taken to be the (one minus

alpha/two) quantile of the standard Normal distribution.

Simultaneous (rather than pointwise) confidence bands of the above

form can be constructed from the variance-weighted Kolmogorov-Smirnov

statistic, but the constant K will then be sensitive to the index

range covered, L <= CC(I) <= U.  In fact, K approaches infinity as L

approaches 0 and U approaches 1.  Stephens(1986) provides tables of

asymptotic K values (large N), a few of which are:

CapQuant [Version 9311] . . . . . . . . . . . . . . . Page 15 of 39

��������������������������Ŀ

� alpha significance level �

���������������������������������������������������Ĵ

�  index range covered   �  0.01     0.05     0.10  �

���������������������������������������������������Ĵ

�     L=0.01, U=0.99     �  3.81     3.31     3.08  �

�     L=0.05, U=0.95     �  3.68     3.16     2.91  �

�     L=0.10, U=0.90     �  3.59     3.06     2.79  �

�����������������������������������������������������

Nair and Freeny(1993) recommend the choice L=0.05 and U=0.95 as

yielding a relatively wide index range without producing excessively

wide confidence limits.  Thus CapQuant uses K = 2.91 to display

asymptotic 95% confidence lower and upper simultaneous limits on CC(),

so that the region between these lower and upper limits provides an

central 90% confidence band about CC().

When N is small (less that 25, say), this asymptotic band tends to be

too narrow.  The exact critical values of Niederhausen(1981) could be

used in these cases.

Remember that, while viewing the CGA graphics display of the CC curve

confidence limits, you may simply press the SpaceBar or Function Key

F1 to get HELP on the KEYs used for on-screen annotation.

CapQuant [Version 9311] . . . . . . . . . . . . . . . Page 16 of 39

Next, try item "M" from the main menu...

�����������������������������ͻ

�  Process Capability MENU    �

�     R = Regret analysis     �

�     D = Display CC curves   �

�     C = CC Confidence Limits�

�     M = Monitor capability  �

�     X = eXit CAPQUANT       �

� Choice --> M                �

�����������������������������ͼ

���������������������� Capability Monitoring �������������������ͻ

�                                                                �

� Display Historical Capability in R = Raw     ...or             �

�                                  S = Smoothed Form ?           �

�                                                                �

� Press the   R   or   S   key now --> R                         �

�                                                                �

� Will NEW Process Data be Input via K = Keyboard ? ...or        �

�                                    B = Batch File ?            �

�                                                                �

� Press the   K   or   B   key now --> B                         �

�                                                                �

� Batch Input of New Data Selected...                            �

�                                                                �

� Specify filename for New Data Batch Input [NORMAL] :           �

�                                                                �

� The New Data Batch Input file is to be: NORMAL.new             �

� Pause at each New Data Value [Y|n] :                           �

����������������������������������������������������������������ͼ

NOTE: Because the NORMAL.NEW file contains 200 observations, you

should probably either respond 'N' to the final prompt above or else

press-and-hold-down the ENTER key when the Capability Monitoring

graphics screen appears...

CapQuant [Version 9311] . . . . . . . . . . . . . . . Page 17 of 39

�������������������� CapQuant Regret Function ������������������ͻ

�                                                                �

� Regret Type:      QUADRATC                                     �

�                                                                �

�                            Variables Measurement -->           �

�                  *                   |                   *     �

�                   *                  |                  *      �

�                    *                 |                 *       �

�                     **               |               **        �

�                       **             |             **          �

�                         **           |          ***            �

�                            ***       |       ***               �

�                               ****   |   ****                  �

�     Regret = 0  ------------------*******--------------------- �

�                                      |                         �

�                                   Target                       �

�                                                                �

�                    REGRET = ( Value - Target )^2               �

�                                                                �

� TARGET = Value corresponding to ZERO REGRET =   20.0000        �

� Press a Key to Continue...                                     �

����������������������������������������������������������������ͼ

���������������������� Capability Monitoring �������������������ͻ

�                                                                �

� Show Cumulative Distributions as Stair-Steps ? [Y|n] :n        �

�                                                                �

� Between data-entry prompts, press F1 or Space Bar for HELP.    �

� Press the V key to interactively View numerical Values.        �

�                                                                �

� Press a Key to Start Capability Monitoring...                  �

����������������������������������������������������������������ͼ

The 300 "new" process observations stored in the NORMAL.NEW datafile

start out looking good.  Unfortunately, they end up showing that

performance is actually deteriorating somewhat!  Try it and SEE!

CapQuant [Version 9311] . . . . . . . . . . . . . . . Page 18 of 39

When you do eXit from CapQuant, you screen will display...

�����������������������������ͻ

�  Process Capability MENU    �

�     R = Regret Analysis     �

�     D = Display CC Curves   �

�     C = CC Confidence Limits�

�     M = Monitor Capability  �

�     X = eXit CAPQUANT       �

� Choice --> X                �

�����������������������������ͼ

REMINDER(S):

CAPQUANT created a Regret Calcs file... NORMAL.xxx

You may edit this file and change its filename extension

to .REG to use Automatic Regret Calculations in future

CAPQUANT runs with Batch History Input from MORMAL.hst.

CAPQUANT created an Output File named... NORMAL.out

Use the DOS invocation... TYPE NORMAL.out | MORE

to review detailed computational results from

�������������������������������Ŀ

�  Preserving Time-Series Order �

� & Forming "Moving" Statistics �

���������������������������������

The primary statistical methodology implemented in CapQuant involves

study of the "marginal distribution" of regret indices...i.e. the

distribution which results from discarding all information about the

time-series order in which values were actually observed.  Therefore,

CapQuant displays screen messages such as...

�����������������������������������������������������������������Ŀ

�  Historical Data, in any sequence, are to be entered            �

�  in the Next Window...  Press Q to QUIT now...                  �

�������������������������������������������������������������������

On the other hand, there usually are very good reasons for keeping

your data stored in their proper time-series order in History (.HST)

and MostRecent (.NEW) Batch Input files.

For example, CapQuant can be used to convert almost any kind of

variables data into Equivalent Expectancies and Equivalent Defects

(EEs and EDs).  These statistics will be recorded in the same order in

CapQuant .OUT files that they were input from .HST and .NEW files.  So

their proper time-series order is maintained if they are input in the

desired order.  In fact, it is quite simple and straight-forward for

you (using your favorite personal computer editor or word processor)

CapQuant [Version 9311] . . . . . . . . . . . . . . . Page 19 of 39

to read-in a CapQuant .OUT file and create a file suitable for input

to my Quality Trend Monitoring software system, QMPT.EXE (from the

QMPTC.EXE archive.)

Consider the following two segments of the NORMAL.OUT file, each

of which contains 6 lines of data...

�����������������������������������������������������Ŀ

�         Process Capability Calculations...          �

�         exptc  noncf  var1   var2   index           �

�          0.49   1.69   9.00   0.00   3.46           �

�          0.49   0.13  23.00   0.00   0.26           �

�          0.49   2.37  33.00   0.00   4.83           �

�          0.49   0.06  22.00   0.00   0.11           �

�          0.49   0.22  16.00   0.00   0.46           �

�          0.49   0.90  28.00   0.00   1.83           �

�����������������������������������������������������Ĵ

�        Capability Monitoring Calculations...        �

�         exptc  noncf  var1   var2   index           �

�          0.49   2.02   8.00   0.00   4.11           �

�          0.49   2.02   8.00   0.00   4.11           �

�          0.49   1.13  11.00   0.00   2.31           �

�          0.49   0.00  20.00   0.00   0.00           �

�          0.49   0.00  20.00   0.00   0.00           �

�          0.49   0.35  25.00   0.00   0.71           �

�������������������������������������������������������

CapQuant [Version 9311] . . . . . . . . . . . . . . . Page 20 of 39

Using these segments to create input files for QMPT.EXE is a

simple matter of adding 11 appropriate lines at the top of each

file...

�����������������������������������������������������Ŀ

�                                                     �

�          I                       <-- I-plots        �

�          1989                    <-- Year           �

�          6                       <-- 6 Periods to Plot

�          1 2 3 4 5 6             <-- Labels for Periods

�          Quadratic Regret Example     <-- Header 1  �

�          Historical Normal Data       <-- Header 2  �

�          Target at Thickness = 20     <-- Footer 1  �

�          Exp.Regret=35, Variance=2500 <-- Footer 2  �

�          6                       <-- QMP Window Width

�          1                       <-- E-factor       �

�          0                       <-- No Previous History

�          0.49   1.69   9.00   0.00   3.46           �

�          0.49   0.13  23.00   0.00   0.26           �

�          0.49   2.37  33.00   0.00   4.83           �

�          0.49   0.06  22.00   0.00   0.11           �

�          0.49   0.22  16.00   0.00   0.46           �

�          0.49   0.90  28.00   0.00   1.83           �

�                                                     �

�����������������������������������������������������Ĵ

�                                                     �

�          F                       <-- Forecast Plots �

�          1991                    <-- Year           �

�          6                       <-- 6 Periods to Plot

�          1 2 3 4 5 6             <-- Labels for Periods

�          Quadratic Regret Example     <-- Header 1  �

�          Most Recent Data             <-- Header 2  �

�          Target at Thickness = 20     <-- Footer 1  �

�          Exp.Regret=35, Variance=2500 <-- Footer 2  �

�          6                       <-- QMP Window Width

�          1                       <-- E-factor       �

�          0                       <-- No Previous History

�          0.49   2.02   8.00   0.00   4.11           �

�          0.49   2.02   8.00   0.00   4.11           �

�          0.49   1.13  11.00   0.00   2.31           �

�          0.49   0.00  20.00   0.00   0.00           �

�          0.49   0.00  20.00   0.00   0.00           �

�          0.49   0.35  25.00   0.00   0.71           �

�                                                     �

�������������������������������������������������������

NOTE: QMPT.EXE actually reads only the FIRST 2 VALUES off of each

of the lines transferred from the CapQuant Output file to the QMPT

Input file.  You could delete the last three numbers from each of

these lines using your Editor/WordProcessor, but that is not

really necessary.

CapQuant [Version 9311] . . . . . . . . . . . . . . . Page 21 of 39

The "Moving" Statistics options offered by CapQuant can only be used

when you enter data in their proper time-series sequence...

����������������������  CapQuant  Parameters  �������������������Ŀ

�                                                                 �

�  Should "Moving" statistics be calculated ?                     �

�          0 => No moving statistics...                           �

�          1 => Form first differences...                         �

�          2 => Exp.Wgt.Mov.Average...                            �

�          3 => Form Deviations from EWMA...                      �

�                                                                 �

�������������������������������������������������������������������

These options are NOT available when pairs of data-values are entered.

But, if single values are entered (again, in correct time-series

order!), the values actually input can be (optionally) shifted into

storage space reserved for Variable2, and a TRANSFORMATION of the

original data can be retained as Variable1.  The transformations

offered by CapQuant involve first computing some sort of "Moving

Average" for Variable1 over time and, then, either replacing Variable1

with this Moving Average or else replacing Variable1 by its deviation

from that Moving Average.

Option 1: First Differences...

=========

Moving Average[T] = Previous Observation in the Series

= var1[T-1]

Transformation = Current Deviation from Moving Average

= var1[T] - var1[T-1]

Option 2: Exponentially Weighted Moving Average...

=========

Short-Term-Memory Weight = Proportion between 0.1 and 0.9

EWMA[T] =       Weight    *  var1[T-1]  +

(1.0-Weight) *  EWMA[T-1]

Transformation = Moving Average = EWMA[T]

Option 3: EWMA Deviations...

=========

Short-Term Memory Weight and EWMA[T] same as in Option 2.

Transformation = Current Deviation from Moving Average

= var1[T] - EWMA[T]

NOTE ONE: Option 3 with Weight=1.0 would be the same as Option 1.

NOTE TWO: Regret functions with Target Value at ZERO are frequently

appropriate with Options 1 and 3 because these Options re-express

Variable1 as a Deviation from its Moving Average.

CapQuant [Version 9311] . . . . . . . . . . . . . . . Page 22 of 39

����������������������  CapQuant  Parameters  �������������������Ŀ

�                                                                 �

�  How much Short-Term-Memory should the EWMA have ?              �

�          0.1 => MINimum Weight on Most Recent Observation...    �

�          0.9 => MAXimum Weight on Most Recent Observation...    �

�                                                                 �

�  NOTE:   1.0 => EWMA is the Most Recent Observation...          �

�                                                                 �

�  How much weight ? [0.20] :                                     �

�  Short-Term-Memory Weight = 0.20.                               �

�������������������������������������������������������������������

As the Short-Term-Memory Weight decreases (down from its maximum

allowed value of 0.9) toward its minimum at 0.1, considerable

SMOOTHING of the Variable1 data series can result.  In other words,

the EWMA series can have much smaller variance that the original

series.

����������������������  CapQuant  Parameters  �������������������Ŀ

�                                                                 �

�  Is there a Start-Up Value for the First History Observation ?  �

�     -999.0 => No Start-Up Available; Start at 2nd Observation.  �

�      OTHER => Start at 1st Observation with this Start-Up Value.�

�                                                                 �

�  What Start-Up Value ? [-999.0] :                               �

�  Start-Up Value =    -999.0000.                                 �

�                                                                 �

�������������������������������������������������������������������

NOTE: The final value of the Moving Average for the Historical data

from the NORMAL.HST file is automatically used as the Start-Up Value

for Capability Monitoring with the data from the NORMAL.NEW file.

�������������������������Ŀ

�     REGRET FUNCTIONS    �

���������������������������

The methodology employed in CapQuant requires a general "rule" for

transforming every possible value for a variable (or a pair) into a

corresponding REGRET value, a surrogate measure of Cost-of-Poor-

Quality.  In other words, we need to specify a "functional

relationship."  The variable may assume negative values, but the

corresponding regret must be zero or positive.  This usually means

that the regret function is "non-linear" and that more than one value

for the variable can yield the same value for the regret...i.e. regret

may be a non-invertible function of its input(s).  All that is really

necessary is that the regret function provide some sort of

common-sense "proxy" for the economic impact of deviations from

intended target value(s).

In CapQuant, you may select a Regret Function from any of ten general

families:

GOALPOST     QUADRATC     ABSVALUE     ONESIDED     BILINEAR

ENDPOINT     LOGISTIC     INVTNORM     RANGEMAX     NONCEXPT

CapQuant [Version 9311] . . . . . . . . . . . . . . . Page 23 of 39

Each of these Regret Functions can have as many as three Parameters:

TARGET (the value of the variable resulting to Zero-Regret), PARM1,

and PARM2.

REGRET Type:    GOALPOST

Description:    Economic Impact is either 0 (negligible) or 1

(complete).  A variable measurement outside of an

acceptable range indicates a nonconformance (defect.)

Variables Measurement -->

|                   |

Regret = 1  ************|                   |*************

|                   |

|                   |

|                   |

Regret = 0  ------------*********************------------

|                   |

Lower               Upper

Bound               Bound

Parameters:     No single TARGET value is specified.  Regret is zero

over the interval from  PARM1 = Lower Bound

to    PARM2 = Upper Bound.

REGRET Type:    QUADRATC         (Taguchi's "Loss-to-Society")

Description:    Economic Impact is assumed to be proportional

to the SQUARE of the deviation of the measured

value from its (zero nonconformance) TARGET value.

*      Variables Measurement -->  *

|

*              |              *

*             |             *

**           |           **

**         |         **

***      |      ***

***   |   ***

Regret = 0  ------------------*******---------------------

|

TARGET

REGRET = ( Value - TARGET )^2

Parameters:     TARGET = Value corresponding to ZERO REGRET.

No PARM1 or PARM2 values need be specified.

-------------------------------------------------------------------

CapQuant [Version 9311] . . . . . . . . . . . . . . . Page 24 of 39

REGRET Type:    ABSVALUE    (...symmetric BILINEAR )

Description:    Economic Impact is assumed to be directly

proportional to the measured value.

|    Variables Measurement -->

***                         |                         ***

***                      |                      ***

***                   |                   ***

***                |                ***

***             |             ***

***          |          ***

***       |       ***

***    |    ***

*** | ***

Regret = 0 -------------------***------------------------------

|

TARGET = 0

Parameters:     No TARGET, PARM1 or PARM2 values need be specified.

Restrictions:   Variable measurements must be non-negative.

TARGET is implicitly ZERO.

REGRET Type:    ONESIDED

Description:    Economic Impact is ZERO on one side of the TARGET

and proportional to the SQUARE of the deviation

from TARGET on the other side.

Variables Measurement -->

*               |

|

*              |       PARM1 = +1

*             |

**           |       Zero Regret on

**         |       Right Side.

***      |

***   |

Regret = 0  ------------------**********************************

|

TARGET

Parameters:     PARM1 is a LEFT(-1) or RIGHT(+1) indicator.

PARM1 = -1 indicates zero regret left of TARGET.

No PARM2 value need be specified.

-----------------------------------------------------------------

CapQuant [Version 9311] . . . . . . . . . . . . . . . Page 25 of 39

REGRET Type:    BILINEAR  (...asymmetric ABSVALUE)

Description:    Two variables measurements are required, a DEMAND

level & an INVENTORY level for a perishable product.

Economic Impact is proportional to production cost per

unit when inventory > demand, but there is a Profit

(Opportunity) Loss per unit when demand > inventory.

****            Demand -->

****                                        **

****                                  **

****                            **

****               |      **

****           |    **

****       |  **

****   |**

Regret = 0  ------------------------****-----------------------

|

Inventory

Level

Parameters:     PARM1 = Regret per Unit of Excess Inventory.

PARM2 = Regret per Unit of Inventory Shortage.

No TARGET value need be specified.

REGRET Type:    ENDPOINT (quadratic)

Description:    Two variables measurements are required, the MINIMUM

and MAXIMUM observed values in a random sample of

fixed size (say, 10 measurements.)  Economic Impact

is assumed to be the SUM of the QUADRATC regrets at

the min. and max. values.

*---------------|               *

|*              |              *

| **            |            **

|   **          |          **

|     ***       |-------***

|        ***    |    ***|

Regret = 0  -----+-----------*********---+----------------

MIN              |      MAX

|            TARGET     |

REGRET = ( MIN - TARGET )^2 + ( MAX - TARGET )^2

Parameters:     TARGET = Value corresponding to ZERO REGRET.

No PARM1 or PARM2 values need be specified.

---------------------------------------------------------------------

CapQuant [Version 9311] . . . . . . . . . . . . . . . Page 26 of 39

REGRET Type:    LOGISTIC

Description:    Economic Impact follows a double-logit pattern that

is V-shaped at the target but flat like GOALPOST

in the tails.

Measurement-->

Regret = 1  ------------|---------|---------|------------

*        |         |         |        *

*    |         |         |    *

* |         |         | *

|*        |        *|

|  *      |      *  |

|    *    |    *    |

|      *  |  *      |

|       * | *       |

|        *|*        |

Regret = 0  ----------------------*----------------------

|      Target       |

Target-PARM1        Target+PARM1

REGRET = ( DEVIATION ) / ( PARM1 + DEVIATION )

Parameters:     TARGET = Value corresponding to ZERO REGRET.

PARM1  = DEVIATION from TARGET with regret = half MAX.

REGRET Type:    INVTNORM

Description:    Economic Impact follows an inverted bell-shaped

pattern that is quadratic at the target but flat like

GOALPOST in the tails.

Measurement-->

Regret = 1  ------------|---------|---------|------------

*            |         |         |            *

*      |         |         |      *

*  |         |         |  *

*|         |         |*

|*        |        *|

| *       |       * |

|  *      |      *  |

|    *    |    *    |

|      *  |  *      |

Regret = 0  ---------------------***---------------------

|      Target       |

Target-PARM1        Target+PARM1

REGRET = 1.0 - 0.5 to the power [square(DEVIATION/PARM1)]

Parameters:     TARGET = Value corresponding to ZERO REGRET.

PARM1  = DEVIATION from TARGET with regret = half MAX.

---------------------------------------------------------------------

CapQuant [Version 9311] . . . . . . . . . . . . . . . Page 27 of 39

REGRET Type:    RANGEMAX   (...linear in Max - Min )

Description:    Two variables measurements are required, the

MINIMUM and MAXIMUM observed values in a random

sample of fixed size (say, 10 measurements.)

Economic Impact is assumed to be proportional to the

RANGE from the min. to the max. values.

|    Range = ( Max. - Min. ) -->

|                          ***

|                       ***

|                    ***

|                 ***

|              ***

|           ***

|        ***

|     ***

|  ***

Regret = 0  ---------***------------------------------

|

TARGET = 0

Parameters:     No TARGET, PARM1 or PARM2 values need be specified.

Restrictions:   Range must be non-negative; TARGET is implicitly ZERO.

REGRET Type:    NONCEXPT

Description:    Two variables measurements are required, the

EQUIVALENT NONCONFORMANCES and the EQUIVALENT

EXPECTANCY for each observation (production lot,

period, etc.)

With input data of this type, no Regret Function is

actually specified.  Similarly, no Regret Mean or

Variance are needed to Poissonize the regret.

Parameters:     No TARGET, PARM1 or PARM2 values need be specified.

---------------------------------------------------------------------

Equivalent Nonconformance & Expectancy

The process of converting Regret into "Equivalent Nonconformances" and

"Equivalent Expectancy" is called "Poissonization"...

REGRET = Long Range Economic Impact and/or Customer/Regulatory

Dissatisfaction associated with Deviations from intended

Target Value(s).

EXPREGRET = Expected Value of REGRET within the Historical

Distribution of Process Capability.

CapQuant [Version 9311] . . . . . . . . . . . . . . . Page 28 of 39

VARREGRET = Variance of REGRET within the Historical Distribution of

Process Capability.

EN = Equivalent Nonconformances = REGRET * EXPREGRET / VARREGRET.

EE = Equivalent Expectancy = EXPREGRET * EXPREGRET / VARREGRET.

Note, in particular, that this conversion process involves nothing

more than a simple "Change-of-Scale."  REGRET is multiplied by the

EXPREGRET / VARREGRET ratio to compute Equivalent Nonconformances, EN.

Multiplication by this constant value assures that the mean and

variance of EN are equal (to EE = Equivalent Expectancy.)  This

equality of mean and variance is an essential characteristic of

Poisson distributions.

The two key parameters needed to establish a successful application of

variables data to regret quantification are thus the expected value

(EXPREGRET) and the variance of the regret (VARREGRET) at standard

quality.  Many different shapes for regret functions might be

appropriate for any particular variable in any particular production

process.  The exact form of the regret function chosen is sometimes

not terribly important.  As a result, even after the regret function

and all of its describing parameters (TARGET, PARM1, and PARM2) have

been specified, the critical final phase of customizing a CPQ

application still remains...choice of numerical values for EXPREGRET

and VARREGRET.

While it might be possible to specify the distribution of regret at

standard quality using only technical assumptions and theoretical

calculations, the more standard approach is an empirical one in which

actual data on the operation of the process are converted to regrets,

and the statistical distribution of those sample regrets is then

studied.  If one has data on only a dozen or so lots, it could be

misleading to use the observed sample values of the mean and variance

of regret as EXPREGRET and VARREGRET, respectively, because small

sample estimates are imprecise.

On the other hand, even if one had data for hundreds of lots, it could

still be misleading to use the observed sample mean and variance of

regret to establish CapQuant parameter values because even large

sample results can be biased.  There could be several different causes

of bias.  The data at hand may not represent the results of the

process operating at (or even near) an industry-wide "standard"

quality level.  The history being used might, in fact, reflect an

operation in which quality is either well above or well below

standard.  Modern, new manufacturing equipment (using, say, robotics)

or a change in ingredient formulations may have been introduced

without changing the quality standard that was barely within the

capability of the old technology.

If you aren't an expert on the operation of the manufacturing process

in question, consult with an expert.  Develop opinions about the

quality level of the process as reflected in its history.  Ask

CapQuant [Version 9311] . . . . . . . . . . . . . . . Page 29 of 39

yourself, "How did I react to these quality monitoring results when I

first saw them?" (Or, "How would I have reacted if I had been the

manager in charge at that time?")  Were you ever concerned that the

process might be out of "statistical control?"  Did you ever become

alarmed because rejection rates jumped at final inspection or because

in-process yield dropped?  Might you have reacted quite differently if

the data had been only slightly different?  Or, would your reactions

have been basically the same even if the data had shifted even more

than it did in level and/or were even more highly variable from

lot-to-lot?

A common situation is that all the observed history reflects data

taken from the process when it was operating well within its current

process capability.  In other words, the variable could have deviated

much more from its target without management having been alarmed at

all!  This means that the values for EXPREGRET and VARREGRET that

should be specified to CapQuant will indeed differ from what is

observed in the history.  General rules-of-thumb for this well-

within-capability situation are...

* the EXPREGRET parameter should usually be chosen to be rather close

to the observed value of the sample mean regret if the history that

is available consists of 50 or more observations.

* the VARREGRET parameter, on the other hand, might be selected to be

10 or even 100 times larger than the observed regret variance of

the available data. (Note: This corresponds to a "Standard Error"

value between 3 and 10 times larger than that actually observed.)

There are two primary reasons for using a rather large VARREGRET

value.  Again, the history data may be very well behaved, and

management would not have been alarmed even if the deviations of

measurements from the target had been much larger than observed.  The

second reason for using a large variance is that variables data are

very rich in information. (Variables data are usually much more

informative than nonconformance data.)

��������������������������������������������������������������Ŀ

� NOTE:  CapQuant illustrates accumulation of results across   �

� variables, across products or over time to form either       �

� COMPOSITE statistics or reporting-period summaries.  Due to  �

� "Poissonization", composite Equivalent Nonconformances and   �

� Expectancies result simply by SUMMING their components over  �

� all relevant variables, periods, or processes.               �

����������������������������������������������������������������

CapQuant [Version 9311] . . . . . . . . . . . . . . . Page 30 of 39

The CapQuant philosophy is to fine-tune parameter values by making

repeated "iterations" as follows:

(i)   The general form and specific parameter values of the

Regret Function are set using item "S" on the Regret Menu.

(ii)  Using item "I" on the Regret Menu, the sample mean and

variance of regret are displayed and you are prompted to

enter values for EXPREGRET and VARREGRET.  As explained

above, you may wish to consider using EXPREGRET and

VARREGRET values that are not very close, numerically, to

the values in your sample.

(iii) Goodness-of-Fit of sample results to a Poisson distribution

(with "intensity" EXPREGRET*EXPREGRET/VARREGRET) can then be

checked using a Kolmogorov-Smirnov test (item "L" on the

Regret Menu.) Statistically significant Lack-of-Fit is NOT

fatal.  On the other hand, insignificance of distributional

differences is always a welcome indication that your choices

are supported by the available data.  [Of course, you could

return to step (ii) now to see if different values of

EXPREGRET and VARREGRET reduce Lack-of-Fit.]

(iv)  Plot a detailed Cumulative Capability Curve using item "C"

on the Main MENU.  Study these results, using the on-screen

cursor activated by the "V" key, and determine whether your

results are consistent with the opinions of local "experts."

(v)   If you are unhappy with the outcome of step (iv), you should

either

(1st choice): return to step (ii) and try out

different values of EXPREGRET and VARREGRET

or

(2nd choice): return to step (i) and try different

regret function parameters or even a different regret

function family.

Once you are happy with the outcome of step (iv), the

modeling process is complete.

REFERENCES

AN INTRODUCTION TO THE QUALITY MEASUREMENT PLAN. (1980.)  Western

Electric Company, Inc., Quality Assurance, September 1980,

16 page booklet.

CHAMBERS, J. M., CLEVELAND, W. S., KLEINER, B., and TUKEY, P. A.

(1983).  Graphical Methods for Data Analysis, Monterey, CA:

Wadsworth.

CONOVER, W. J. (1980). Practical Nonparametric Statistics, 2nd. Ed.,

Wiley [Kolmogorov-Smirnov table and approximation, page 462.]

CapQuant [Version 9311] . . . . . . . . . . . . . . . Page 31 of 39

GUNTER, Bert. (1991a,b,c,d,e). "Process Capability Studies:"...parts 1

through 5.  Statistics Corner, Quality Progress, Volume 24

(February, April, June, August, and October Issues, 1991.)

HASTINGS, C., Jr.  (1955).  Approximations for Digital Computers.

Princeton, NJ: Princeton University Press.

HOADLEY, Bruce. (1978). "Equivalent Defects: A Unified Approach to

Attributes Quality Rating."  Bell Telephone Laboartories,

Quality Assurance Center, Technical Memorandum, November 4.

HOADLEY, Bruce. (1981). "The Quality Measurement Plan, QMP."  Bell

System Technical Journal, 60, 215-273.

HOADLEY, Bruce. (1986). "QUALITY MEASUREMENT PLAN(QMP)."  Encyclopedia

of Statistical Sciences. (Kotz, Johnson and Read, Editors)

Volume 7, pages 393-398.  New York, John Wiley.

INTRODUCTION TO QUALITY TREND CHARTS.  Bell Communications Research,

Inc. (Bellcore), January 1986, 18 page booklet.

JOHNSON, N. L. and KOTZ, S.  (1969).   Distributions in Statistics:

Discrete Distributions. [Chapter 4:  Poisson Distribution]

New York, NY: John Wiley.

JOHNSON, N. L. and KOTZ, S.  (1970).   Distributions in Statistics:

Continuous Univariate Distributions - 1. [Chapter 17:  Gamma

Distribution.]  New York, NY:  John Wiley.

LINK, P. A., and RIPACKI, B. R., "Quality Assurance Approach to Fiber

Optic Cable," International Wire and Cable Symposium

Proceedings, 1985.

MONTGOMERY, D. (1985).  Introduction to Statistical Quality Control.

New York, NY: John Wiley.

NAIR, V. N. and FREENY, A. E.  (1993).  "Methods for Assessing

Distributional Assumptions in One and Two Sample Problems."

Probabilistic and Statistical Methods in the Physical Sciences.

(J. Stanford and S. Vardeman, editors.)  San Diego, CA:

Academic Press.

NIEDERHAUSEN, H. (1981).  "Tables and Significant Points for the

Variance Weighted Kolmogorov-Smirnov Statistic."  Technical

Report 298, Department of Statistics, Stanford University.

OBENCHAIN, Robert. (1991). "Regret Indices and the Quantification of

Process Capability."  Indianapolis, IN: Lilly Research

Laboratories. (29 pages.)

OBENCHAIN, Robert. (1993). "Cumulative Capability Curves."

Indianapolis, IN: Lilly Research Laboratories. (49 pages.)

CapQuant [Version 9311] . . . . . . . . . . . . . . . Page 32 of 39

QUALITY MEASUREMENT PLAN (QMP).  Bell Communications Research

Technical Reference, TR-TSY-000438.  Issue 1, April 1987.

SPIRING, Fred. (1991). "The Inverse Normal Loss Function."

Unpublished manuscript, University of Manitoba.

STEPHENS, M. A. (1986).  "Tests based upon EDF Statistics."  Goodness-

of-Fit Techniques, Chapter 4.  (R. B. D'Agostino and M. A.

Stephens, editors.)  New York, NY: Marcel Dekker.

WILSON, E. B. and HILFERTY, M. M.  (1931).  "The Distribution of Chi-

Square."  Proceedings of the National Academy of Sciences 17,

684-688.

---------------------------------------------------------------------

CapQuant [Version 9311] . . . . . . . . . . . . . . . Page 33 of 39

APPENDIX ONE

Poissonization:  Conversion of Economic Impact Measures into

Equivalent Nonconformances and Equivalent Expectancy.

Section A.1    Nonconformance Expectancy

Attributes Data

Definition:     Nonconformance Expectancy is the product of the

Sample Size times the Quality Standard (expressed in

expected nonconformances per unit.)

Example:        When the quality standard is 0.004 nonconformances

per unit, one needs to inspect 250 units in order to

expect to see 1 nonconformance when quality is at the

standard level.

Information

Content:        Under a Binomial model, the "moments" of the sampling

distribution would be given by the familiar

expressions: mean = n*p and variance = n*p*(1-p).  QMP

(the Quality Measurement Plan) is based upon the

Poisson (maximum variance) model with mean = variance

= n*p, which is the nonconformance expectancy.

Expectancy is the primary measure of "information

content" used in QMP and CPQ for sampling inspection

data. [See Section A.3, below, for details on

information content.]

Variables Data

Definition:     Equivalent nonconformances and equivalent expectancy

can be defined by "Poissonizing" any continuous

measure of regret, R.  [See Section A.2, below, for

details on poissonization.]

Reporting       Experience with use of QMP at both Bell Communications

Period:         Research (Bellcore) and at AT&T Bell Laboratories

suggests the following "Rule-of-Thumb" for aggregating

sampling inspection data over "reporting periods"

(usually either the 12 calendar months or 8-per-year):

�������������������������������������������������������������Ŀ

� An effective quality monitoring program requires sufficient �

� sampling inspection to produce a nonconformance expectancy  �

� of at least 2 nonconformances per reporting period.         �

���������������������������������������������������������������

CapQuant [Version 9311] . . . . . . . . . . . . . . . Page 34 of 39

If production is so low or nonconformances are so rare

that one would have to inspect more than 20% of total

production to achieve a nonconformance expectancy of

at least 2 nonconformances per reporting period, then

"quantitative" quality monitoring is probably

impractical.  (After all, QMP is the "most powerful"

statistical method currently available; if QMP can't

get the monitoring job done, what method can?)

Section A.2    Equivalent Nonconformances & Expectancy

Definition:     Equivalent nonconformances and expectancy can be

defined by "Poissonizing" a continuous measure of

regret, R.  For example, if E is the expected value of

R and V is the variance of R, then the number of

"equivalent nonconformances" is said to be R*E/V,

which is usually not an integer value.  But the mean

and variance of R*E/V are both equal to the

"equivalent expectancy" of E*E/V...and equality of

mean and variance is an intrinsic property of all

Poisson distributions.

Example:        Suppose a measurement produces a continuous variable,

Z, that has (approximately) a "normal" distribution

with mean = target value = T and variance = D, say,

when quality is "at standard."  Consider the "Taguchi-

like" measure of regret given by the sum-of-squares of

N deviations of independent Z-values from the target,

T, all divided by D.  Then normal distribution theory

implies that E(R) = N and V(R) = 2N.  It follows that

the number of "equivalent nonconformances" is R/2, and

the "equivalent expectancy" is N/2.  (Note that the

sample index value = nonconformances/expectancy is

R/N; the average squared deviation from target, scaled

by dividing by D.)

"Power" of

Variables

Data:           Variables data can be incredibly "rich" in sample

information about quality compared with attributes

data.  The above "normal theory" example shows that a

single continuous observation can yield a

nonconformance expectancy of one-half!  And the

"uniform" distribution would yield an even higher

nonconformance expectancy of .66 per observation with

the above "Taguchi-like" R measure!

Do-It-Right

from the

START:          Quality engineers should consult a professional

statistician if there is any doubt in their minds

about how to implement the above basic concepts.  One

CapQuant [Version 9311] . . . . . . . . . . . . . . . Page 35 of 39

needs to utilize a realistic "Regret Function," which

might need to be one-sided (rather than symmetric

about the target) or linear or saturating (rather than

always quadratically increasing.)  The calculation of

the mean and variance of the regret depends upon the

statistical distribution of test results when the

manufacturing process is within its historical

"process capability."  Theoretical calculations can be

complicated or require numerical integration;

empirical studies of historical results are generally

more straight-forward and more satisfactory.

Section A.3    Expectancy and Information

Contents of This Section:

Two fundamental concepts are established by the highly

technical arguments of this section.

...The information content in a set of regret measurements

is directly proportional to the Poissonized

"nonconformance expectancy" of the regrets at standard

process capability.

...The sample regret index (ratio of observed to

expected nonconformances) is the minimum variance,

unbiased estimate of the true index.

Minimum Variance Bound:

If a sample statistic, tau, is an unbiased estimate of a

parameter, theta, of the statistical distribution of the

observed data vector, x, then the so-called "Cramer-Rao-

Aitken-Silverstone Inequality" states that:

��������������������������������������������������Ŀ

�  the product of the variance of tau times the    �

�                     ---------------              �

�  Fisherian Information in the data must be >= 1, �

�  --------- -----------                           �

����������������������������������������������������

where this "information" is defined to be the expected

value of the square of the derivative of the log-

likelihood function of the data with respect to the

parameter theta.  Furthermore, equality holds if and

only if the derivative of the log-likelihood with

respect to theta is of the form:

CapQuant [Version 9311] . . . . . . . . . . . . . . . Page 36 of 39

A(theta) * (tau-theta),

in which case, tau is the minimum variance estimate of

theta, and A(theta) is the Fisherian Information.

Poisson Example:

If the observed numbers of nonconformances, { x(1),...,

x(n) }, are stochastically independent and each follows a

Poisson distribution with intensity parameter lambda(i),

then the total number of observed nonconformances is

also Poisson distributed with intensity parameter equal

to the sum of the individual lambdas.  This total

intensity can be re-expressed as:

lambda(1)+...+lambda(n) = (nonconformance expectancy) * theta

where theta is the true regret index (total lambda

divided by the "standard" nonconformance intensity) and

expectancy is n times the standard intensity.  As a

result the log-likelihood function is

[ x(1)+...+x(n) ] * log(theta) - expectancy * theta

+ terms that do not depend on theta.

The theta-derivative of the log-likelihood is thus:

���������������������������������������������Ŀ

�  expectancy     x(1)+...+x(n)               �

�  ----------  (  -------------  -  theta )   �

�    theta         expectancy                 �

�����������������������������������������������

Thus two basic facts are established by the C-R-A-S

inequality...

...the sample index (observed / expected nonconformance

ratio) is the minimum variance estimate of the true

regret index, theta, and

...the (Fisherian) information content of the data

is A(theta) = expectancy / theta, which is directly

proportional to the nonconformance expectancy at

standard process capability.

Section A.4    Scale Invariance

Definition: Equivalent nonconformances and expectancy are "scale

invariant" quantities in the following sense:

multiplying the regret by any strictly positive, constant

value does not change the implied equivalent

CapQuant [Version 9311] . . . . . . . . . . . . . . . Page 37 of 39

nonconformances and expectancy.  One implication of this

is that it doesn't make any difference whether regret is

expressed in terms of dollars or pennies or pounds

sterling.

Proof:      Suppose that a regret measure, R, is multiplied by any

constant value, K.  Then the expected value of K times R

would be K times E, the original expected value of R.

And the variance of K times R would be K-squared times

V, the original variance of R.  As a result,

multiplicative factors of K-squared cancel from both the

numerators and the denominators of the ratios, R*E/V and

E*E/V, that define equivalent nonconformances and

expectancy.

Specific Implications for

the Ten Regret Families:

GOALPOST.....Max Impact is unitless; call it ONE or

LOGISTIC     use any convenient value.

INVTNORM

LINEREGRET...The slope of the regret line is unimportant.

RANGREGRET

QUADRATC.....Regrets that are squared deviations from

ENDPOINT     target values can be measured in any

ONESIDED     convenient units.

BILINEAR.....Either the Inventory Shortage or the Excess

Inventory Regret Slope parameter can be set

equal to 1 if the second Slope is expressed

as a MULTIPLE of the first.

CapQuant [Version 9311] . . . . . . . . . . . . . . . Page 38 of 39

Section A.5    Summary

�������������������������������������������������������������������Ŀ

�                                                                   �

� Knowledge of the statistical distribution of Regret when Quality  �

� is at a STANDARD LEVEL (that is within the Present Capability     �

� of the Manufacturing Process) is needed for Poissonization.       �

�                                                                   �

�   The two statistical parameters which must be specified are:     �

�           EXPREGRET = Expected Regret at Standard Quality,        �

�           VARREGRET = Regret Variance at Standard Quality.        �

�                                                                   �

�   Poissonization is accomplished by multiplying Regret by         �

�   EXPREGRET and then dividing by VARREGRET...                     �

�                                                                   �

�           EQUIVALENT DEFECTS = REGRET * EXPREGRET / VARREGRET.    �

�                                                                   �

�   Equivalent nonconformances have a statistical distribution that �

�   is like the Poisson in the sense that the mean and the variance �

�   of this distribution are EQUAL.  This common value is...        �

�                                                                   �

�           EQUIVALENT EXPECTANCY = EXPREGRET^2 / VARREGRET.        �

�                                                                   �

���������������������������������������������������������������������

CapQuant Software Update History:

=================================

Version 9009 ...Beta Test Version

Version 9010 ...First Upload to CompuServe

...Fix "bugs" in User-Interface of Beta Test

Version 9101 ...Update for Submission to Technometrics

...Pop-Up Batch-Input-File Selection Menu

...StreamLine of User-Interface / Default-Settings

...Second Upload to CompuServe

Version 9107 ...Update & Submission to Journal of Quality Technology

...Add Inverse-Gaussian Regret Function Option

...Automatic Regret Calcs: Add Screen Pause & Rewinds

Version 9110 ...Change titles on plots to "Cumulative Capability..."

Version 9201 ...Improve staircase "smoothing" algorithm.  Previous

versions connected the points at the top/front edge

of each step; present version reduces perception bias

by connecting the mid-points of the vertical-rise

portions.  In the diagram below, the points numbered

(2) are thus connected instead of those labeled (1):

CapQuant [Version 9311] . . . . . . . . . . . . . . . Page 39 of 39

(1)���

�

(2)

�

(1)����������

�

(2)

�

(1)�����������

�

When a Poisson distribution is fitted to an empirical

cumulative capability curve, the horizontal spacing

of the fitted steps will be exactly uniform.  When

equivalent expectancy is small, the fitted steps can

be quite LARGE.  With the old algorithm, one might

get the false impression that the fitted capability

is always "higher" than the observed capability.

Version 9202 ...List/Save only those numerical values from cumulative

distributions at which observed/fitted values CHANGE.

...Reduce CapQuant maximum capacity from 2,500 history

values to 1,200; this makes CapQuant will much less of

a "memory hog."

...Add capability to form COMPOSITES of 2 or more

observed regret values.  CapQuant now shows how

accumulation of expectancy reduces Poisson step-size

& can greatly reduce Kolmogorov-Smirnov lack-of-fit.

...Add capability to form 1st Differences, Exp.Wgt.Mov.

Averages (EWMAs), or Deviations from an EWMA when

data are input in time-series order.

Version 9302 ...Add calculation and display of CC curve confidence

limits (lower 95% & upper 95% limits => central 90%

interval.)

...Add calculation and display of Gamma Q-Q plots for

estimation of Equivalent Expectancy.

...Combine items "I" and "L" on the REGRET Analysis MENU

so that Kolmogorov-Smirnov lack-of-fit can be computed

for several different ER and VR combinations before

making the P-P plot for the Poisson approximation.

...Display variances rather than standard deviations on

"Poissonization" screens.

...The ATTRIBUT regret function was renamed to GOALPOST.

The BOUNDMAX regret function was renamed to LOGISTIC.

The INVGAUSS regret function was renamed to INVTNORM.

Version 9310 ...Add display of Summary Statistics (mean, std.dev.,

variance, min & max) and a 20-line Histogram of

historical data (Var1 and, if present, Var2.)

Version 9311 ...Provide option to display standardized Gamma fit

(instead of Poisson fit) to the empirical CC curve.

#