Warning

=======

1: Design Filters:

------------------

The filter design section is based upon theoretical mathematical

models.  Attenuation levels beyond 90 db for Bandpass and Lowpass,

and beyond 70 db for Highpass and Notch are difficult to obtain

with real world hardware.  The passband noise for any filter is

directly proportional to the highest Q value used to implement

that filter. Therefore, the best possible dynamic range in the

filter passband can be obtained by avoiding Q values higher than 10.

3: Implement Filter:

--------------------

********************************************************************

* This filter program works with theoretical calculations and      *

* assumes perfect SCF biquads.  Unrealistic, unrealizable resistor *

* values may be generated for extreme filter cases.  Since LTC     *

* devices are real world ICs, designs must be verified first by    *

* breadboarding to ensure they are realistic.                      *

* LTC will support all filter designs with its devices.            *

* For help call Applications : 408-432-1900                        *

********************************************************************

1. Dynamics are optimized in this version for all sections

implemented in mode 3A.

2. Modes can be mixed, ie 1, 2, and/or 3 when applicable, and R1

values have been adjusted to maintain less than or equal to 0 db

at the filter's output.  Extreme filter cases (i.e. decade wide

bandpass filters) may cause nodes other than the output node to

exceed 0 db.  Be careful, breadboard and/or call LTC for advice.

More Information about filter design is contained below:

A filter Mode defines a specific filter topology.  LTC

data sheets define many such modes for a variety of reasons.  In

the design of FCAD, it was decided to limit the mode choices to

1, 2, 3, 3A, 6A and 6B because these modes will cover all

practical designs.  However, in some specific cases a few

additional modes may be required for optimum performance.  In

these cases assistance is available from the LTC Applications

group to help optimize your design.

LTC fully supports all filter hardware design efforts that

involve LTC parts.  LTC is a linear integrated circuit manu-

facturer, not a software vendor, and as such cannot promise to

fully correct all FCAD software errors or omissions (although

we will try). It is hoped that the text that follows explains the

mode selection criteria.

Feedback from users is always appreciated and solicited.

LOWPASS FILTERS:

Case 1:  All Fn are infinite  such as Butterworth,

Bessel, Chebyshev and custom filters with infinite Fn.  This case

allows only Modes 1, 2, 3, 6A or 6B to be used.  Modes 6A and 6B

are 1st order circuits and can be used only in the last stage

of an odd order filter where there is a finite fo and Q=0.

However, instead of Modes 6A or 6B you may use a 1st order low-

pass RC network whose resistor and capacitor are calculated

using the formula fo (last stage)= 1/(2piRC).  Of course, this

network is not clock tuneable.

Case 2:  All Fn are finite.  Only Mode 3A is allowed in

this case where each stage has a finite Fn or each stage has a

finite Fn except the last stage which has either Fn=Infinity

(2nd order section) or Q=0 (1st order section).  Although Mode 3A

is a second order circuit, FCAD requires entering Mode 3A in the

last stage of an odd order even though a 1st order section is

indicated.  The program will then calculate the proper resistor

and capacitor values.

HIGHPASS FILTERS:

Case 1:	 All Fn equal 0 such as Butterworth, Chebyshev

and custom filter with all Fn equal 0.  Only Mode 3 and 6A

are allowed in this case.  If the last stage is 1st order (Q=0),

only Mode 6A is allowed by FCAD.  Mode 6A can be replaced by

a 1st order RC highpass network whose resistor and capacitor

values can be calculated by the formula fo (last stage) = 1/

(2piRC).

Case 2:  All Fn are finite and not 0.  Only Mode 3A is

allowed in designs with a finite Fn (not equal to zero) in each

stage, or with a finite Fn (not equal to zero) in each stage except

the last which has Fn=0 (2nd order section) or Q=0 (1st order

section).  When the last section is 1st order, FCAD requires

Mode 3A be selected (See Lowpass).  The resistor and capacitor

values are calculated by the program.  Of course, this network

is not tuneable.

BANDPASS FILTERS:

Case 1:  All Fn equal 0 or infinity (Butterworth, Bessel

and Chebyshev bandpass designs).  Only Modes 1, 2 and 3 are allowed

for this case.

Case 2:  All Fn are finite and not 0.  Only Mode 3A is

allowed in this case where each stage has a finite Fn (and not 0)

or each stage has a finite Fn (and not 0) except the last stage

has Fn equal to zero.

NOTCH FILTERS:

Only Mode 3A is allowed for all Notch filter cases.

CUSTOM FILTERS:

Custom filter design is a non-trivial pursuit.  Experts in

the topic are few and far between.  Design of a custom or non-

traditional filter presumes a fair amount of knowledge about filter

design.  A good example occurs in the design of Mode 3A sections

useful in the design of Elliptic filters or custom filters with

"notches" (also known as Fn's).  The calculations of FCAD do not

allow Mode 3A to be used with an infinite Fn value except if this

value occurs in the last filter section.  But, what can be done

is to make Fn large to the extent that it does not affect the filter

response while not making it infinite. This allows the program to

not deal with divide by zero and other related pure mathematical

problems.  Choosing an Fn value that is 100 times the cutoff

frequency of the filter is often a good method to "drop the

notch" (move it toward infinity) for lowpass filters.

Likewise, in a highpass filter, a Fn can be moved towards zero by

choosing a Fn value that is fo divided by 100.  This effectively

places this notch at zero.

Please contact LTC for more information.