�����������������������������������������������������������������Ŀ

�      *** One, Two, and Three Stage TRINOMIAL Sampling ***       �

�                                                                 �

�               A Quality Assurance Training Tool:                �

�        Statistics Committee of the QA Section of the PMA.       �

�                                                                 �

�             Bob Obenchain, CompuServe User [72007,467]          �

�������������������������������������������������������������������

Note:  PMA = Pharmaceutical Manufacturers Association

NoMajors.EXE designs one-, two-, and three-stage "trinomial" acceptance

sampling plans for detecting MINOR defects, assuming that observance of

a single MAJOR defect also results in lot rejection.  For example, the

USP Capsule sampling plan calls deviations from target of between 25%

and 35% minor nonconformances, while deviations greater than 35% are

major.  Highly portable C-language source code is included; no graphics.

TABLE-OF-CONTENTS

=================

NoMajors.EXE.................................Executable Module

NoMajors.IN............................ASCII Batch Input Files

NoMajors.CSV................................ASCII Output Files

PROMPTS and DEFAULTS..................Minimum & Maximum Values

NUMERICAL EXAMPLES..............Schilling, Acceptance Sampling

NoMajors.C..............................C-language Source Code

*********************************************************************

NoMajors.EXE

NoMajors calculates the expected percentage of acceptable lots

in single, double, or triple stage TRINOMIAL sampling in which

acceptance implies that NO MAJOR Nonconformances were observed.

Testing of each unit sampled is assumed to yield one of three

possible results...

Observance of a Conformance to Specification

(say, within 25% of a target value), or

Observance of a Minor Nonconformance

(say, deviation from target by 25% to 35%), or

Observance of a Major Nonconformance

(say, deviation from target by more than 35%).

Furthermore, NoMajors assumes that...

the observance of a single MAJOR Nonconformance implies

rejection of the lot, and

NoMajors.EXE, ver 9402 . . . . . . . . . . . . . . . . Page 2 of 5

a one, two, or three stage acceptance sampling plan for MINOR

Nonconformances will be specified.

NoMajors.EXE is an executable module for IBM-compatible personal

computers.  It was compiled using Microsoft C, version 6.00AX

NoMajors.EXE thus provides automatic detection (and use or

emulation) of an 8087/80287/80387 numeric math coprocessor.  If

your PC has a math chip, NoMajors should run much faster.  If

your PC does NOT have a math chip, NoMajors will still run and

should yield identical numerical results.

For each Total (Major+Minor) Nonconformance Percentage input by

the user, NoMajor calculates the 5 expectated First-Pass Yield

Percentages that would be expected in random sampling from a large

population (trinomial model) when the split between Major and

Minor Nonconformances is...

0% Major vs. 100% Minor,

10% Major vs.  90% Minor,

25% Major vs.  75% Minor,

50% Major vs.  50% Minor and

100% Major vs.   0% Minor.

NoMajors.IN

NoMajors prompts the user to choose between Keyboard (K) and ACSII

Batch file (B) input of sample sizes and accept/reject numbers as

well as input of alternative true values for the underlying

percentage nonconforming in the process.

The format for a NoMajors batch input file is similar to that of

its output (.CSV) files.  Each line of the file (except for the

first line giving the output filename) must start with a number,

and only the first number on each line of the batch input file is

actually read.  This number may be either an integer or a floating

point number - but must be separated from other information on the

same line by at least one comma or "white" space (blank or tab.)

The example input file distributed with NoMajors is NOMAJORS.IN.

NoMajors.CSV

This is the default filename for the ASCII summary file of inputs

and outputs from NoMajors.EXE.  You may specify an alternative

filename for this file when prompted.  The filename extension of

.CSV stands for "Comma Separated Values," which puts the output

in a format convenient for importing into a spreadsheet system,

such as Excel, to graph the calculated Operating Characteristic

curve.  Each row of the .CSV file is separated into 6 columns by

5 commas.  In the expected yields table at the bottom of the file,

NoMajors.EXE, ver 9402 . . . . . . . . . . . . . . . . Page 3 of 5

Column 1 contains the Total Percentage (Major+Minor) of

nonconforming units in lots incoming for inspection,

Column 2 contains the Expected Yield when those incoming

nonconformances are   0% Majors and 100% Minors.

Column 3 contains the Expected Yield when those incoming

nonconformances are  10% Majors and  90% Minors.

Column 4 contains the Expected Yield when those incoming

nonconformances are  25% Majors and  75% Minors.

Column 5 contains the Expected Yield when those incoming

nonconformances are  50% Majors and  50% Minors.

Column 6 contains the Expected Yield when those incoming

nonconformances are 100% Majors and   0% Minors.

You can read/print this file after the program terminates, so you

do not need to make hand-written notes during NoMajors execution.

And the easiest way to create a NoMajors .IN file is to edit an

existing .CSV or .IN file to use whatever new sampling parameters

and/or process characteristics you desire.

PROMPTS and DEFAULTS

...Type responses to prompts using the number keys along the

top row of the keyboard, and then press the ENTER key.

...If you press the ENTER key without typing a value, your

response will be assigned to the DEFAULT value.

...The prompts are listed below with minimum, default, and

maximum allowed values.

Enter your First Stage Sample Size: n1

Minimum Sample Size =     2

Default Sample Size =    50

Maximum Sample Size = 5,000

Enter your First Stage MINOR Nonconformance Accept Number: a1

Minimum Acceptance Number =     -1

(i.e. lot acceptance impossible at stage 1

even if zero nonconformances are observed.)

Default Acceptance Number =      0

Maximum Acceptance Number =      min(n1,999)

Enter your First Stage MINOR Nonconformance Reject Number: r1

Minimum Reject Number =      max(a1+1,1)

(i.e. single stage sampling)

Default Reject Number =      max(a1+1,1)

Maximum Reject Number =      n1

NoMajors.EXE, ver 9402 . . . . . . . . . . . . . . . . Page 4 of 5

Enter your Second Stage Sample Size: n2

Minimum Sample Size =     1

Default Sample Size =    50

Maximum Sample Size = 5,000

Enter your Second Stage MINOR Nonconformance Accept Number: a2

Minimum Acceptance Number =      a1

Default Acceptance Number =      a1

Maximum Acceptance Number =      min(n1+n2,999)

Enter your Second Stage MINOR Nonconformance Reject Number: r2

Minimum Reject Number =      a2+1

(i.e. two stage sampling)

Default Reject Number =      a2+1

Maximum Reject Number =      n1+n2

Enter your Third Stage Sample Size: n3

Minimum Sample Size =     1

Default Sample Size =    50

Maximum Sample Size = 5,000

Enter your Final MINOR Nonconformance Accept Number:  a3

Minimum Acceptance Number =      r2 - 1

Default Acceptance Number =      r2 - 1

Maximum Acceptance Number =      min(n1+n2+n3,999)

Specify Testing Efficiency (Percentage):

Minimum Testing Efficiency =    10%

Default Testing Efficiency =   100%

Maximum Testing Efficiency =   100%

YIELD Calculations...

...The user is repeatedly prompted to specify total (major+

minor) nonconformance percentages of units within lots.

These are the values of quality  Total % Nonconforming

incoming to the acceptance sampling inspection.

...Enter values between 0.0% and 100.0%.

...Enter any negative value (say, -1) for % Total

Nonconforming to halt execution of NoMajors.EXE.

...NoMajors.EXE will calculate the "Yield Percentage"

corresponding to 5 splits of this total incoming %

percentage between major and minor nonconformances.

These Yield %s are 100 times the corresponding Probability

of Lot Acceptance, which defines the Operating

Characteristic Curve for the Trinomial Sampling Plan.

NoMajors.EXE, ver 9402 . . . . . . . . . . . . . . . . Page 5 of 5

NoMajors.C

This is an ASCII file of C-language source code for NoMajors.  The

only external functions called by NoMajors are the logarithm and

exponential functions, so the code should be highly portable.  For

example, to compile and run the program on a VAX under VMS, upload

NoMajors.C to the VAX in ASCII mode and enter the commands...

VAX Prompt> cc nomajors

VAX Prompt> link nomajors

VAX Prompt> run nomajors

*********************** End of NoMajors.DOC *************************