�����������������������������Ŀ

�   SeqDir.EXE, Version 9109  �

�������������������������������

SeqDir is an IBM-compatible Personal Computer Software System for...

���������������������������������������������������������������������Ŀ

�                                                                     �

�      ***   Design and Evaluation of Sequential Test Plans   ***     �

�                                                                     �

�            based upon Gamma or Exponential Distributions            �

�         using Aroian's Direct Method of Sequential Analysis         �

�                                                                     �

���������������������������������������������������������������������Ĵ

�                  A Quality Assurance Training Tool:                 �

�              Statistics Committee of the QA Section of              �

�            the Pharmaceutical Manufacturers Association             �

�                                                                     �

�             Bob Obenchain, CompuServe User [72007,467]              �

�����������������������������������������������������������������������

TABLE of CONTENTS

Section 1 - Introduction..............................................1

Section 2 - PC System Requirements....................................8

Section 3 - SeqDir Walk-Through.......................................9

Section 4 - Effects of Changing SeqDir Parameters....................19

References...........................................................22

����������������������������Ŀ

�  Section 1 - Introduction  �

������������������������������

SeqDir.EXE applies the Aroian(1968) "Direct Method" of Sequential

Analysis to evaluate Operating Characteristic (OC) and Average Sample

Number (ASN) curves corresponding to any specified shape of Accept /

Continue / Reject boundary.  And, besides these numerical approximations

to "exact" results, SeqDir also performs Monte-Carlo simulations and

displays Sample Paths graphically!  But only applications in which

sequential observations have independent, identical gamma (or

exponential) distributions with an integer valued "shape" parameter are

treated by SeqDir.

NOTE: The best known forms of sequential tests that use exponential

distributions are the ones that treat "survival/lifetime" data.

Specifically, that type of data consists of numbers and failure

times of certain units, plus "truncated" data (time so-far

on-test) for units that have not yet failed.  The applications

treated by SeqDir cannot use this sort of truncated lifetime data.

SeqDir designs sequential tests to detect shifts in non-negative

measurements of "cost", "loss" or "regret"; i.e. situations where large

or increasing numerical values represent undesirable performance.  These

sorts of measures can frequently be viewed as "surrogates" for the

long-term economic impact of deviations from an intended target value,

i.e. measures of the Cost-of-Poor-Quality.  Almost any sort of numerical

data can be CONVERTED into cost/loss form via a so-called "regret"

function.  (Four forms of regret function are illustrated below.)  Once

this "conversion to regret" process is applied to historical data, the

resulting regret distribution is frequently well approximated by a gamma

or exponential.  In particular, regret distributions are confined to the

range from 0 to infinity and frequently are positively skewed (i.e. they

have a [long] tail "trailing off" to the right...)

����������������������������������������������������������������Ŀ

�                                                                �

�       �                        Gamma Distribution              �

�       �                        ( with Shape Parameter > 1 )    �

�       �         *   *                                          �

�       �     *             *                                    �

�       �                        *                               �

�       �  *                         *                           �

�       �                                   *                    �

�       �*                                           *           �

�       ������������������������������������������������         �

�      0                  Cost -->                               �

�                                                                �

����������������������������������������������������������������Ĵ

�                                                                �

�       �          SPECIAL CASE: Exponential Distribution        �

�       �*                       ( Shape Parameter = 1 )         �

�       �  *                                                     �

�       �     *                  NOTE: The Gamma MODE cost is    �

�       �         *              ZERO when the Shape parameter   �

�       �              *         is less than or equal to 1.     �

�       �                    *                                   �

�       �                           *                            �

�       �                                   *                    �

�       �                                              *         �

�       ������������������������������������������������         �

�      0                  Cost -->                               �

�                                                                �

������������������������������������������������������������������

There are two minimal requirements a function must satisfy before it can

be used to convert an arbitrary quantitative measurement, Y, into a

measure of "regret."  These are...

(i) the function may assume only non-negative values, r(Y) >= 0,

and

(ii) the function must assume the value ZERO at the (or all) target

value(s) for Y,  r(T) = 0.

Although not required, a third property is frequently present...

(iii) regret functions usually are monotone non-decreasing as the

deviation from target increases (above or below target.)

Three types of regret function likely to yield gamma or exponentially

distributed cost/loss results are:

���������������������������������������������������������������������Ŀ

� Regret Type:      Absolute Value (Possibly Asymmetric)              �

���������������������������������������������������������������������Ĵ

�                      Percent of Label Claim -->                     �

�                                 |                                   �

�               ***               |                                   �

�                  ***            |                   ****            �

�                     ***         |               ****                �

�                     +--***------|-----------****                    �

�                     |     ***   |       ****|                       �

�                     |        ***|   ****    |                       �

�    Regret = 0  -----+-----------****--------+-----------            �

�                    95%         100%        105%                     �

�                                                                     �

�    Regret = Left/Right Factor * Absolute Deviation from Target.     �

�����������������������������������������������������������������������

���������������������������������������������������������������������Ŀ

� Regret Type:      Quadratic (Taguchi's "Loss to Society")           �

���������������������������������������������������������������������Ĵ

�                             Measurement -->                         �

�                                                                     �

�                *                  |                  *              �

�                 *                 |                 *               �

�                  **               |               **                �

�                    **             |             **                  �

�                      **           |          ***                    �

�                         ***       |       ***                       �

�                            ****   |   ****                          �

�    Loss = 0  ------------------*******---------------------         �

�                                   |                                 �

�                                Target                               �

�                                                                     �

�                   Regret = ( Value - Target )^2                     �

�����������������������������������������������������������������������

���������������������������������������������������������������������Ŀ

� Regret Type:              OneSided Quadratic                        �

���������������������������������������������������������������������Ĵ

�              *              Measurement -->                         �

�               *                                                     �

�                *                  |                                 �

�                 *                 |        NOTE: There is no        �

�                  **               |        penalty, whatsoever,     �

�                    **             |        for deviations on        �

�                      **           |        ONE SIDE of the target.  �

�                         ***       |                                 �

�                            ****   |                                 �

�    Regret = 0  ------------------****************************       �

�                                   |                                 �

�                                Target                               �

�����������������������������������������������������������������������

A fourth type of regret which can yield cost/loss measures which,

although actually bounded, are somewhat like a (truncated) gamma or

exponential is:

���������������������������������������������������������������������Ŀ

� Regret Type:      Double Logit (Bounded Maximum)                    �

���������������������������������������������������������������������Ĵ

�                          |  Measurement-->   |                      �

�          ******          |         |         |          *******     �

�                ******    |         |         |    ******            �

�                      *****         |         *****    <-- Half      �

�                          |****     |     ****|            Maximum   �

�                          |    ***  |  ***    |                      �

�                          |       **|**       |                      �

�  Regret = 0  ----------------------*----------------------          �

�                          |         |         |                      �

�                        minus     Target     plus                    �

�                      HalfWidth            HalfWidth                 �

�                                                                     �

�         Regret = AbsDeviation / ( HalfWidth + AbsDeviation )        �

�����������������������������������������������������������������������

************************************************************

EXAMPLE: Suppose that historical costs derived from potency assay values

using "absolute value" regret have an exponential distribution

(gamma with shape parameter 1) with expected value of 1% of

Label Claim.  Food and Drug Administration regulations require

this regret be no more than 5% of Label Claim.  For each new

batch of product, the manufacturer wishes to perform as many

assays as necessary to classify the new batch is either "good"

(within established process capability) or "bad".

************************************************************

Sequential test plans detect situations where the expected value of a

cost/loss distribution has shifted (from lower-cost) toward higher-cost

values...i.e. situations where costs become markedly higher than their

historical and/or current process capability levels.

Sequential Probability Ratio Test (SPRT) procedures, Wald(1947), for

gamma distributed regret can utilize any test statistic that is a

monotonic function of the cumulative sum of regret values over

sequential steps.  At step N, this regret cumulative sum is

Rsum(N) = r(1) + r(2) + ... + r(N),

where r(s) is the regret observed at sequential step "s" of the test.

The argument that Rsum(N) is the pivotal statistic for all values of the

gamma Shape parameter "p" and the gamma Scale parameter "R" goes as

follows.  The general form of gamma probability density function is

f( r ) = r^(p-1) * exp( - r / R ) / [ R^p * Gamma(p) ]

over the range from r = 0 to infinity, where R^p means "R raised to the

p-th power," Gamma(p) denotes the gamma function, and the p and R

parameters are both strictly greater than zero.  Although only cases

where the Shape is a positive integer ( p = 1, 2, 3, ... ) are allowed

in SeqDir, Rsum(N) is the pivotal quantity for all positive Shapes.

|  ASIDE:  Narrowing attention to integer valued Shapes is not      |

|  really very restrictive, but it greatly simplifies generation    |

|  of pseudo-random gamma variates.  The gamma function reduces to  |

|  Gamma(p) = [p-1] factorial for positive, integer p.  Chi-Squared |

|  distributions are special cases of gamma distributions where the |

|  degrees-of-freedom parameter is 2*p.  Thus Chi-Squared distri-   |

|  butions with an odd number of degrees-of-freedom are not treated |

|  in SeqDir.                                                       |

The expected value of gamma distributed regret is E( r ) = R*p, even

when p is not an integer.  Similarly, the standard deviation of regret

is R*sqrt(p), and the mode density falls at the maximum of 0 and

R*(p-1).

The objective of all sequential test procedures treated here is to

distinguish between the following two hypotheses...

������������������������������������������������Ŀ

�        NULL HYPOTHESIS: Mean Regret <= p * R0  �

������������������������������������������������Ĵ

� ALTERNATIVE HYPOTHESIS: Mean Regret >= p * R1  �

��������������������������������������������������

where the gamma distribution has the SAME, known Shape (p) under BOTH

hypotheses, and R1 > R0 are known Scale parameter values.

*************************************************************

EXAMPLE: R0 = 1 and R1 = 5 are appropriate for the above example where

regret is assumed to have an exponential distribution (Shape

p=1) with expected value of 1% of Label Claim under the null

hypothesis and 5% at the regulatory specification limit.

*************************************************************

Continuing our argument that Rsum(N) is pivotal, the log-likelihood

RATIO statistic is of the form:

log(L1/L0) = - N * p * log(R1/R0) + Rsum(N) * [(1/R0)-(1/R1)].

where log => logarithm to the base e = 2.71828...  In particular, note

that all terms in (p-1)*log[r(j)] cancel out when forming the RATIO

because the null and alternative gamma distributions both have the SAME

Shape parameter, p.  Again, remember we are assuming that the null and

alternative gamma distributions differ only in their Scale parameters,

R0 and R1.

The Wald(1947) formula for approximate accept / reject boundaries for

Rsum(N) are then of the form...

����������������������������������������������������������������������Ŀ

� Reject Value: {log[(1-Beta)/Alpha]+N*p*log[R1/R0]} / [(1/R0)-(1/R1)] �

����������������������������������������������������������������������Ĵ

� Accept Value: {log[Beta/(1-Alpha)]+N*p*log[R1/R0]} / [(1/R0)-(1/R1)] �

������������������������������������������������������������������������

where Alpha is the objective for Producer's risk (probability of

rejecting a batch when quality is at or better than its "good" level,

R0), and Beta is the objective for Consumer's risk (probability of

accepting a batch when quality is at or worse than the "bad" level, R1.)

***************************************************************

EXAMPLE: R0=1 & R1=5 implies that [R1/R0] = 5 and [(1/R0)-(1/R1)] = 0.8.

The approximate Wald(1947) limits for p=1 & Alpha=Beta=0.05 are

then

Reject: ( log(19) + N*log(5) )/0.8 =  3.68 + N * 2.012,

and

Accept: (-log(19) + N*log(5) )/0.8 = -3.68 + N * 2.012.

Step:    1      2     3      4      5      6      7      8

Accept: -1.67   0.34  2.35   4.37   6.38   8.39  10.40  12.44

Reject:  5.69   7.70  9.72  11.73  13.74  15.75  17.76  19.77

***************************************************************

The two main troubles in practical application of Wald's approximate

method are:

...the Alpha and Beta levels actually achieved my not be very close

to their intended values, and

...accept and reject boundaries formed by a pair of parallel lines

can lead, in some situations, to large ASNs.  "Truncated"

tests (in which the limits converge after a specified number of

sequential steps) frequently have much greater practical appeal.

SeqDir automates a 4-stage process that allows quality practitioners to

design sequential gamma test plans having KNOWN (exact) OC and ASN

curves that correspond to boundaries of ARBITRARY form.  SeqDir

achieves this by implementing the Aroian(1968) "Direct Method" of

sequential analysis.  From the definitions:

C(N) = Probability of Continuing Past Sequential Stage N,

= Probability that Rsum(N) is between the accept(N) and

reject(N) limits at sequential step N of the plan,

A(N) = Probability of Accepting the Null Hypothesis at Stage N,

= Probability that Rsum(N) is less than accept(N), and

R(N) = Probability of Rejecting the Null Hypothesis at Stage N,

= Probability that Rsum(N) is greater than reject(N),

it follows that

A(N) + C(N) + R(N) = C(N-1),

where, by convention, C(0) = 1 and A(0) = R(0) = 0.

It is straight-forward to compute C(1); this is simply a definite

integral over the Stage 1 continue region for a gamma distribution with

Shape p and any given Scale, R.  The conditional distribution of Rsum(2)

[given that it is necessary to continue past stage 1] is then the

convolution of {i} the probability density of Rsum(1)=r(1) truncated to

the continue region of Stage 1 with {ii} the original gamma distribution

[of r(2)].  C(2) is then the definite integral of this convoluted

distribution over the Stage 2 continue region.

By induction, the conditional distribution of Rsum(N) [given that it is

necessary to continue past stage N-1] is the convolution of {i} the

probability density of Rsum(N-1) truncated to the continue region of

Stage N-1 with {ii} the original gamma distribution [of r(N)].  And C(N)

is the definite integral of that convoluted distribution over the Stage

N continue region.  etc., etc.

Rather than use numerical methods that actually integrate and convolute

continuous distributions, SeqDir approximates all distributions (both

the original gamma or exponential and all singly/doubly truncated

"continue" distributions) by discrete distributions...with relatively

large numbers of relatively narrow cells.  Definite integrals then

become sums of cell-probabilities, while convolutions become sums of

cell-probability products.

See McWilliams(1989) for a flow-chart of Aroian's method and an example

application to pass/fail (binomial distribution) testing.

SeqDir: Four Stages of "Design of Sequential Gamma Test Plans"

�����������������������������������������������������������������������Ŀ

� STAGE ONE: Invoke SeqDir and Specify Accept/Continue/Reject Limits    �

�����������������������������������������������������������������������Ĵ

� STAGE TWO: Calculate OC & ASN Values over a Range of Expected Regrets �

�            ...and possibly Confirm Calculations via Simulation        �

�����������������������������������������������������������������������Ĵ

� STAGE THREE: Interactively View Plots of Boundary, OC & ASN Curves    �

�����������������������������������������������������������������������Ĵ

� STAGE FOUR: Exit SeqDir to examine the Summary and/or Detailed OUTPUT �

�             Files...and Formulate Boundaries for the Next SeqDir Run. �

�������������������������������������������������������������������������

Section 3 of this documentation ("SeqDir Walk-Through") illustrates use

of SeqDir.EXE in Stages 1, 2, and 3 of the above process for Design of

Sequential Gamma Test Plans.

��������������������������������������Ŀ

�  Section 2 - PC System Requirements  �

����������������������������������������

The hardware and software requirements for the personal computer system

that will use SeqDir are as follows:

...First of all, your machine should be IBM compatible with at least

300K (kilobytes) of available RAM memory.  A hard disk is

optional...but greatly speeds up SeqDir execution, especially when

writing an optional (potentially very large) "details" file.

...Secondly, to use item "P" on the main menu, your monitor should be

capable of displaying medium resolution (IBM/CGA) screen graphics.

Most PC monitors used in business applications support graphics

at this resolution or better (EGA, VGA, etc.)  Color is optional.

...Thirdly, a laser or dot-matrix printer can be used to produce

quick screen prints.  Boundary, OC, and ASN curves displayed on

your screen can be converted into hard copy by pressing the

letter S followed by a second "special" key:

L or F10 ...Hewlett-Packard LaserJet/DeskJet

G or F9  ...IBM-compatible Graphics Dot Matrix Printer

E or F8  ...Epson FX, JX, or LQ Printers

To make efficient use of SeqDir, DOS must be informed of SeqDir's

requirements to read and write multiple files.  This is accomplished by

having a CONFIG.SYS file (on your boot disk or in your C:\ root

directory) that contains lines (records) stating...

FILES=20

BUFFERS=8

*** SeqDir Input/Output FileName Conventions ***

The ASCII (text) files used by SeqDir for data input and output consist

of a filename (of at most eight characters) specified by the user

followed by a period (.) and one of three specific 3-character filename

EXTensions.

12345678.LIM  ...SeqDir Batch Input of Limit Parameters

12345678.OUT  ...SeqDir Summary  Output File.

12345678.DTL  ...SeqDir Detailed Output File.

���������������������������������Ŀ

� Section 3 - SeqDir Walk-Through �

�����������������������������������

Let us take a brisk walk through usage of SeqDir menus/prompts to

familiarize first time users.  The SeqDir software system is invoked

from MS-DOS by entering its 8-character name, SEQDIR, at your DOS

prompt...

Prompt> seqdir

SeqDir does not expect or accept any "command-line" arguments.  Instead,

SeqDir will prompt you to use your keyboard and/or a bounce-bar menu to

set parameters and provide all information SeqDir needs to perform

analyses.  And each SeqDir prompt has an implied or [displayed] DEFAULT

VALUE that you may accept by simply pressing the ENTER key.  [Displayed

default values are usually shown in square brackets.]

�������������������� Direct Sequential Methods ���������������������Ŀ

�                                                                    �

�                    SEQDIR.EXE...Version 9109                       �

�                A Quality Assurance Training Tool:                  �

�        Statistics Committee of the QA Section of the PMA           �

�           Bob Obenchain, CompuServe User [72007,467]               �

�                                                                    �

�  ��������������������������������������������������������������Ŀ  �

�  �  Will Accept/Reject LIMITS be Input via K = Keyboard ? ...or �  �

�  �                                         B = Batch File ?     �  �

�  �                                                              �  �

�  �  Press the   K   or   B   key now -->                        �  �

�  ����������������������������������������������������������������  �

����������������������������������������������������������������������

If you press the B key to select input from a "batch" file, the

following screen [with a pop-up, bounce-bar filename selection menu]

will appear...

��������������������� Direct Sequential Methods ��������������������Ŀ

�                                                                    �

�   Batch Input of A/R Limits Selected...                            �

�                                                                    �

�   At colon Prompts : ...simply press ENTER to get the [default].   �

�                                                                    �

�  �FileNames�      Arrow Keys: Move Highlight Bar Up / Down.        �

�  �5STEP1   �                                                       �

�  �5STEP5��ۺ      Return Key: Selects the Highlighted File. >>>------+

�  �9WALD1   �                                                       � |

�  �9WALD5   �      Escape Key: Abandon BATCH SeqDir Input.          � |

�  ���������ͼ                                                       � |

�                                                                    � |

�   The Limits/Parms Input file is to be: 5STEP5.lim <-----------------+

�                                                                    �

�   Specify filename for Detailed Output [5STEP5.out] :              �

�                                                                    �

�   The SEQDIR Output Save file is to be: 5STEP5.out <--Default Accepted

�                                                                    �

�           Press Q to QUIT now...Other Key to Continue...           �

����������������������������������������������������������������������

If you initially press the K key to select KEYBOARD input, SeqDir will

prompt you for each parameter value it needs.  And SeqDir will also save

your keystrokes in a file (that you may modify with any [ ASCII ] text

editor)...to rerun analyses in "batch" mode!

The Screens for entering parameter values from your Keyboard look like

this...

�������������������� Direct Sequential Methods ���������������������Ŀ

�                                                                    �

�  Keyboard Input of A/R Limits Selected...                          �

�                                                                    �

�  At colon Prompts : ...simply press ENTER to get the [default].    �

�                                                                entered

�  Specify filename to save Keyboard Input [seqdir] : 5step5 <----------

�  The filename for saving Keyboard input is to be : 5step5.lim      �

�                                                                    �

�  Specify filename for Summary Output [5step5.out] : <---Default    �

�  The SeqDir Output Save file is to be: 5step5.out       Accepted   �

�                                                                    �

�  Save Detailed Output in filename = 5step5 ? [N|y] : Y <--This may �

�  The SeqDir Details Save file is to be: 5step5.dtl        create a �

�                                                          VERY LARGE�

�      Press Q to QUIT now...Other Key to Continue...       File!!!  �

����������������������������������������������������������������������

������������������������ SeqDir Parameters �������������������������Ŀ

�                                                                    �

�  What will be the Gamma SHAPE Parameter ? [    1] : 5       <--Entered

�  SHAPE Parameter:     5.                                           �

�                                                                    �

�  What is the Null (low) Exp. REGRET ? [    1.00] : <--Default Accepted

�  NULL Exp.Value:     1.00.                                         �

�                                                                    �

�  What is the Alt. (hi) Exp. REGRET ? [    5.00] :  <--Default Accepted

�  ALT. Exp.Value:     5.00.                                         �

�                                                                    �

�  What is the Producers RISK ? [    0.05] :         <--Default Accepted

�  Producers RISK :     0.05.                                        �

�                                                                    �

�  What is the Consumers RISK ? [    0.05] :         <--Default Accepted

�  Consumers RISK :     0.05.                                        �

����������������������������������������������������������������������

Wald's approximate limits (lying on a pair of parallel lines) are

computed and displayed by SeqDir; these are the default SeqDir limits.

But, instead of accepting these default limits by simply pressing the

ENTER (or Carriage Return) key in response to each prompt, let us enter

custom boundaries...

������������������������� SeqDir Parameters ������������������������Ŀ

�   Respond to the following prompts for Limits...                   �

�   To quit data entry, enter the letter...  Q                       �

�                                                                    �

�           Sequential Step Number:   1                              �

�           Wald Lower Limit =     1.28                              �

�           Enter <CR>, Lower Limit, NA or Q : na <------ Not Applicable

�                                                                    �

�          ����������������������������������������������������Ŀ    �

�          � NOTE: An NA Lower Bound is equivalent to a -1.     �    �

�          �       An NA Upper Bound is equivalent to a +9,999. �    �

�          ������������������������������������������������������    �

�                                                                    �

�           Sequential Step Number:   1                              �

�           Wald Upper Limit =     2.75                              �

�           Enter <CR>, Upper Limit, NA or Q : 6  <--------------Entered

�                                                                    �

�           Sequential Step Number:   2                              �

�           Wald Lower Limit =     3.29                              �

�           Enter <CR>, Lower Limit, NA or Q : 0  <--------------Entered

�                                                                    �

�           Sequential Step Number:   2                              �

�           Wald Upper Limit =     4.76                              �

�           Enter <CR>, Upper Limit, NA or Q : 8  <--------------Entered

����������������������������������������������������������������������

etc.

������������������������� SeqDir Parameters ������������������������Ŀ

�                                                                    �

�           Sequential Step Number:   5                              �

�           Wald Lower Limit =     9.32                              �

�           Enter <CR>, Lower Limit, NA or Q : 12.99 <-----------Entered

�                                                                    �

�           Sequential Step Number:   5                              �

�           Wald Upper Limit =    10.80                              �

�           Enter <CR>, Upper Limit, NA or Q : 13 <--------------Entered

�                                                                    �

�           Sequential Step Number:   6                              �

�           Wald Lower Limit =    11.33                              �

�           Enter <CR>, Lower Limit, NA or Q : q <-----------Quit Signal

�                                                                    �

�   Total Number of Sequential Steps =  5                            �

�                                                                    �

�   Data Entry Complete...                                           �

�   Press Q to QUIT now...Other Key to Continue...                   �

����������������������������������������������������������������������

The corresponding "batch" input file created by SeqDir is formatted as

follows...

�����������������������������������������������������������������������Ŀ

�               Format for Batch Input: "FileName.LIM"                  �

�����������������������������������������������������������������������Ĵ

� assayreg ...Name of Variable  �  Line  1: Name String and Comment     �

�     5    ...Shape Parameter   �  Line  2: Gamma Dist. Peakedness.     �

�     1.00 ...Null Parameter    �  Line  3: Exp. Regret of Null Dist.   �

�     5.00 ...Alt. Parameter    �  Line  4: Exp. Regret of Alternative  �

�     0.05 ...Producers Risk    �  Line  5: Wald Approx. Alpha Level    �

�     0.05 ...Consumers Risk    �  Line  6: Wald Approx. Beta Level     �

�    -1.00     6.00             �  Line  7: Step 1 Lower & Upper Limits �

�     0.00     8.00             �  Line  8: Step 2 Lower & Upper Limits �

�     4.00    10.00             �  - - -                                �

�     8.00    12.00             �  - - -                                �

�    12.99    13.00             �  Next to Last Line: Last Limit Pair   �

� q                             �  Last Line: Quit (End-of-Data) Signal �

�������������������������������������������������������������������������

NOTES: Limit pairs for Accept / Continue / Reject Boundaries start on

line 6; the Upper Limit (second value) must equal or exceed the

Lower Limit (first value.)

The last Lower / Upper Limit pair may, optionally, "Close Off"

the Continue Region...as illustrated above.  If the Continue

Region is left "Open" at the last step specified to SeqDir, then

the value of the Operating Characteristic (Accept Probability)

plus the Producer's Risk (Rejection Probability) my sum to much

less than one for some (or all) values of True Expected Regret.

The SeqDir MAIN MENU now appears.  You may proceed directly to item "D"

if you are willing to accept SeqDir's default values for Number of Cells

and Range of Integration (with implied Cell Width)...

������������������������������� SeqDir Main Menu �������������������Ŀ

�  ������������������������������Ŀ                                  �

�  �  Direct Sequential MENU      �                                  �

�  �      M = set Maximum cells   �                                  �

�  �      D = Direct Analyses     �                                  �

�  �      S = Simulations         �                                  �

�  �      P = Plots: Lims/OC/ASN  �                                  �

�  �      Q = Quit SeqDir         �                                  �

�  �  Choice --> M                �                                  �

�  ��������������������������������                                  �

����������������������������������������������������������������������

However, if you press the M key (as illustrated above), the following

screen will appear...

������������������������ set Maximum cells �������������������������Ŀ

�                                                                    �

�  SeqDir uses probabilities within cell intervals to approximate    �

�  continuous Gamma distributions (and their convolutions) by        �

�  discrete distributions.  This approximation is fast but           �

�  inaccurate when only a few hundred cell intervals are used.       �

�  You will need to use one or two thousand cells to explore         �

�  numerical sensitivities and confirm simulation results...         �

�                                                                    �

�  The Maximum number of cells that can be defined is 2,500.         �

�                                                                    �

�  The Maximum Rejection Upper Limit is  13.00.                      �

�                                                                    �

�  The Total Regret Range presently covered is [ 0.00, 25.00].       �

�  Specify Maximum Regret Limit [ 25.00] :  <-----------Default Accepted

�  Maximum Regret :    25.00.                                        �

�                                                                    �

�  Specify Total Number of Cells [   200] : 250   <--------------Entered

�  Number of Cells :    250.                                         �

�                                                                    �

�  The implied Regret Cell WIDTH is :   0.1000                       �

�                                                                    �

�  Press any Key to Return to the Main Menu...                       �

����������������������������������������������������������������������

Once you return to the MAIN MENU, you should press the "D" key to

perform SeqDir Direct Calculations.  Each time you change cell widths

or integration range, you should re-do calculations by pressing "D."

������������������������� SeqDir Analyses ��������������������������Ŀ

�  Direct Sequential Calculations...                                 �

�  NULL Exp.Value = 1.00                                             �

�  ALT. Exp.Value = 5.00                                             �

�                                                                    �

�  At most 50 Expected Regret Values can be evaluated.               �

�  How many Expected Regret Values ? [5] :   <----------Default Accepted

�  Number of Expected Regret Values = 5.                             �

�                                                                    �

�   Calculations for Expected Regret =     1.00.                     �

�   ================   Accept   LowLim   Contin   UprLim   Reject    �

�   At Seq. Step  1:  0.00000    -1.00  1.00000     6.00  0.00000    �

�   At Seq. Step  2:  0.00000     0.00  1.00000     8.00  0.00000    �

�   At Seq. Step  3:  0.92822     4.00  0.07178    10.00  0.00000    �

�   At Seq. Step  4:  0.99876     8.00  0.00124    12.00  0.00000    �

�   At Seq. Step  5:  0.99998    12.99  0.00000    13.00  0.00002    �

�   Performance Statistics...                                        �

�           Operating Characteristic =  1.00000                      �

�           Producers Rejection Risk =  0.00000                      �

�           Average Sample Number    =     3.07                      �

�           ASN Standard Error       =     0.26                      �

�                                                                    �

����������������������������������������������������������������������

NOTE: In SeqDir screen displays, the Accept, Continue, and Reject

probabilities are normalized so that

A(N) + C(N) + R(N) = 1 .

Internal calculations in SeqDir use this normalization to preserve

numerical precision, i.e. to prevent ALL of these probabilities from

being small.  However, before results are written to the SeqDir output

file, the correct normalization is imposed:

A(N) + C(N) + R(N) = C(N-1).

For example, the analysis from the above screen for Exp.Regret=1 is

written to the output file as:

Step ===   Accept   LowLim   Contin   UprLim   Reject

Step  1:  0.00000    -1.00  1.00000     6.00  0.00000

Step  2:  0.00000     0.00  1.00000     8.00  0.00000

Step  3:  0.92822     4.00  0.07178    10.00  0.00000

Step  4:  0.07169     8.00  0.00009    12.00  0.00000

Step  5:  0.00009    12.99  0.00000    13.00  0.00000

1.00000 ...Operating Characteristic

0.00000 ...Producers Rejection Risk

3.07 ...Average Sample Number

0.26 ...ASN Standard Error

Similar calculations for Exp.Regret= 2, 3, 4, and 5 will be displayed as

the screen scrolls upward within the window...

�������������������������� Direct Analyses �������������������������Ŀ

�                                                                    �

�   Calculations for Expected Regret =     5.00.                     �

�   ================   Accept   LowLim   Contin   UprLim   Reject    �

�   At Seq. Step  1:  0.00000    -1.00  0.71494     6.00  0.28506    �

�   At Seq. Step  2:  0.00000     0.00  0.40090     8.00  0.59910    �

�   At Seq. Step  3:  0.00012     4.00  0.29734    10.00  0.70254    �

�   At Seq. Step  4:  0.00419     8.00  0.25264    12.00  0.74318    �

�   At Seq. Step  5:  0.09905    12.99  0.00000    13.00  0.90095    �

�   Performance Statistics...                                        �

�           Operating Characteristic =  0.00252                      �

�           Producers Rejection Risk =  0.99748                      �

�           Average Sample Number    =     2.11                      �

�           ASN Standard Error       =     0.96                      �

�                                                                    �

�   Direct Sequential Calculations Complete...                       �

�                                                                    �

�   Press a KEY to return to the SeqDir Main MENU.                   �

����������������������������������������������������������������������

Once you return to the MAIN MENU, you could press the "P" key to view

medium-resolution (CGA) graphical display of the MOST RECENT results you

calculated using item "D" (or items "M" and "P").  Item "P" will NOT

OPERATE until you have selected "D" at least once.

However, let us choose option "S" next...

������������������������������� SeqDir Main Menu �������������������Ŀ

�  ������������������������������Ŀ                                  �

�  �  Direct Sequential MENU      �                                  �

�  �      M = set Maximum cells   �                                  �

�  �      D = Direct Analyses     �                                  �

�  �      S = Simulations         �                                  �

�  �      P = Plots: Lims/OC/ASN  �                                  �

�  �      Q = Quit SeqDir         �                                  �

�  �  Choice --> S                �                                  �

�  ��������������������������������                                  �

����������������������������������������������������������������������

SeqDir can confirm the results it calculated using Aroian's direct

method using Monte-Carlo simulation.  Furthermore, the graphical

simulation display provided by SeqDir can give us a very good "visual

impression" of what Sample Paths can be expected to "look like..."

����������������������������� Simulation ���������������������������Ŀ

�                                                                    �

�   Sequential Simulations...                                        �

�   NULL Exp.Value =     1.00                                        �

�   ALT. Exp.Value =     5.00                                        �

�                                                                    �

�   Number of Expected Regret Values = 5                             �

�                                                                    �

�   Maximum Number of Simulation Replications = 25,000.              �

�   Desired Number of Replications ? [500] :   <----------Accept Default

�   Number of Replications = 500.                                    �

�                                                                    �

�   Input Start-up Seed ( 0 means use your Clock ): 123456  <------Enter

�   Startup Seed  = -7616                                            �

�                                                                    �

�   Press a key to Start Simulations...                              �

����������������������������������������������������������������������

If you enter a non-zero seed value, as illustrated above, this exact

same "starting point" in SeqDir's pseudo-random sequence will be used

for each distinct value of Expected Regret that is simulated.  The

(linear) correlation between OC and ASN estimates at different values of

Expected Regret will then be plus one; as highly correlated as possible!

On the other hand, zero is the default seed value (i.e. what you would

get by simply pressing the ENTER key in response to the above prompt)

and using that seed value has two implications.

First of all, SeqDir will get a seed value from your system clock in

this case (and write that value to the output file for simulation

documentation.)

Furthermore, SeqDir will NOT now reset its pseudo-random sequence to

the same starting point for each different Expected Regret value it

simulates.  In other words, the OC and ASN estimates for different

Expected Regrets will then be statistically independent.

Usage of COLOR on SeqDir Simulation Displays:

Cyan Box  => An observed Rsum(N) value in the Continue Region.

Cyan Line => Connects pairs of Cyan Boxes in the Continue Region.

Magenta Box  => An observed Rsum(N) value in the Reject Region.

Magenta Line => Connects Cyan Continue Box to a Magenta Reject Box.

White Box  => An observed Rsum(N) value in the Accept Region.

White Line => Connects Cyan Continue Box to a White   Accept Box.

************************************************************************

WARNING:  Do not press ANY KEY while Monte-Carlo Simulation is underway

unless you desire EARLY TERMINATION for the Expected Regret value

currently being simulated.  If you do press key(s), SeqDir will clear

out the keyboard buffer and go on to the next Expected Regret value as

soon as at least 10 replications have been completed.

************************************************************************

Once Direct Calculations and Simulations have both been performed, it is

definitely time to "plot" results using item "P" on the Main Menu...

������������������������������� SeqDir Main Menu �������������������Ŀ

�  ������������������������������Ŀ                                  �

�  �  Direct Sequential MENU      �                                  �

�  �      M = set Maximum cells   �                                  �

�  �      D = Direct Analyses     �                                  �

�  �      S = Simulations         �                                  �

�  �      P = Plots: Lims/OC/ASN  �                                  �

�  �      Q = Quit SeqDir         �                                  �

�  �  Choice --> P                �                                  �

�  ��������������������������������                                  �

����������������������������������������������������������������������

Note that item "P" is separate from items "D" and "S" on the Main Menu

because you may wish to cycle through the graphical displays (the

Accept/Continue/Reject limits and the OC and ASN Curves) several times

for a single set of Calculated or Simulated Results.  Simply select item

"P" several times.  If no simulations have been performed, SeqDir plots

will start appearing as soon as you choose item P in the Main Menu.

If both "D"irect Calculations and "S"imulations have been performed, the

following sub-menu will appear...

������������������������������� SeqDir Plotting ��������������������Ŀ

�                                                                    �

�        Critical Region, Operating Characteristic,                  �

�        and Average Sample Number Plots...                          �

�                                                                    �

�        From the NULL Exp.Value =     1.00 on the Left,             �

�        To  ALTERNATE Exp.Value =     5.00 on the Right.            �

�        5 = No. of Expected Regret Values per Plot.                 �

�                     ������������������������������Ŀ               �

�                     �Which Type of Plot ?          �               �

�                     �                              �               �

�                     �      R = Crit. Region        �               �

�                     �      D = Direct Calc.        �               �

�                     �      S = Simulation          �               �

�                     �      Q = Quit                �               �

�                     �                              �               �

�                     �  Choice -->                  �               �

�                     ��������������������������������               �

����������������������������������������������������������������������

At this point, choose the type of plot you wish to view (R, D, or S) or

else return to the Main Menu by pressing Q.

Once a graphics screen is displayed by SeqDir, you may interactively

annotate it.  For example, you may move an on-screen cursor to read

numerical values off of the plot.  Simply, press the Space Bar or

Function Key F1 for on-line HELP on interacting with graphics screens!

���������������������������������������������Ŀ

�   This HELP screen .... Space Bar or F1.    �

�                                             �

�   ERASE PLOT......press ENTER or ESCape.    �

�     Note: 2nd KeyPress sometimes needed.    �

�                                             �

�   On-Screen Cursor & VALUES ... press V.    �

�     Move Cursor Right with Right Arrow.     �

�     Move Cursor Left  with Left  Arrow.     �

�                                             �

�   Screen PRINT/SAVE Keys:  First Press S    �

�   and then press a Second Special Key...    �

�    L or F10 dumps to HP LaserJet/DeskJet    �

�    I or F9  dumps to IBM Graphics Matrix    �

�    E or F8  dumps to Epson FX, JX, or LQ    �

�    S saves screen to disk in PCX format.    �

�                                             �

�   SeqDir, ver9109.........Press any key.    �

�����������������������������������������������

Note in particular that OPTIONAL screen dumps can be obtained by

pressing first "S" then a second "special" key while the Limits, OC, or

ASN screen is being displayed.  If you have annotated the screen, that

annotation will be part of the screen dump.

��������������������������������������������������Ŀ

�          HP LaserJet, Plus & Series II           �

�  (press either the L key or function key F10.)   �

��������������������������������������������������Ĵ

�    IBM-compatible Graphics Dot Matrix Printer    �

�   (press either the G key or function key F9)    �

��������������������������������������������������Ĵ

�          Epson FX, JX, or LQ Printers            �

�   (press either the E key or function key F8)    �

����������������������������������������������������

If you press any key other than one of the six "special" keys listed

above after pressing "S", the Boundary, OC or ASN screen will be erased.

After viewing the ASN curve, you will be returned to the Main Menu.

Press the "Q" key to terminate SeqDir execution and return to DOS...

������������������������������� SeqDir Main Menu �������������������Ŀ

�  ������������������������������Ŀ                                  �

�  �  Direct Sequential MENU      �                                  �

�  �      M = set Maximum cells   �                                  �

�  �      D = Direct Analyses     �                                  �

�  �      S = Simulations         �                                  �

�  �      P = Plots: Lims/OC/ASN  �                                  �

�  �      Q = Quit SeqDir         �                                  �

�  �  Choice --> Q                �                                  �

�  ��������������������������������                                  �

����������������������������������������������������������������������

SeqDir will write "reminders" on your screen as it terminates...

REMINDER(S) :

SeqDir created a Batch History Input file... 5step5.lim

You can invoke SeqDir and specify 5step5 [.lim]

as Batch History Input file to replay the present session.

SeqDir created an Output File named... 5step5.out

Use the DOS invocation... LIST 5step5.out

to review summary statistics from this run.

SeqDir created a Details File named... 5step5.dtl

Use the DOS invocation... LIST 5step5.dtl

to review detailed computational results.

����������������������������������������������������Ŀ

�  Section 4 - Effects of Changing SeqDir Parameters �

������������������������������������������������������

This section of the SeqDir documentation discusses the effects of

changing the gamma Shape parameter and the (numerical integration) cell

width.

*** gamma Shape parameter ***

You must specify a numerical value of the gamma distribution Shape

parameter, p, in response to the following SeqDir prompt...

What will be the Gamma SHAPE Parameter ? [  1] :

Shape p=1 (an Exponential distribution) is the SeqDir default because

calculations are very simple in this case.  For example, the right-hand

tail probability of regret exceeding a value X when the Scale parameter

is R would then be Pr[ r > X ] = exp( - X / R ), i.e. e = 2.71828...

raised to the power - X / R.

Consider what would happen if the gamma distribution Shape parameter, p,

were to increase as the gamma Scale parameter, R, decreases...leaving

the expected regret value, E=p*R, constant.  The standard deviation of

regret, R*sqrt(p)=E/sprt(p), would then decrease and the mode regret,

max[ 0, E*(p-1)/p], will increase from 0 [for p<=1] toward its limiting

value of E.

The slope of the Wald(1947) approximate accept / reject boundaries for

Rsum(N) would be unchanged as p increases with E=p*R fixed, but the

spacing between these boundaries would decrease (assuming that Alpha and

Beta remain unchanged.)  In other words, it becomes easier and easier to

distinguish between Scaling differences, R0 and R1, as the Shape

parameter increases (i.e. small regrets become relatively less likely in

the R1 scaling, and large regrets become relatively less likely at R0.)

Using those same sorts of arguments, if accept / reject boundaries are

held fixed as the Shape increases, then the Operating Characteristic

curve will increase near R0 (reduced producers risk) and decrease near

R1 (reduced consumers risk.)  This point is illustrated in the following

tabulation for the 5STEP1.LIM and 5STEP5.LIM boundary examples that use

2,500 cells over regret range [0,25]...

�����������������������������������������������������Ŀ

� ExpR= 1    ExpR= 2    ExpR= 3    ExpR= 4    ExpR= 5 �

� (NULL)                                      (ALT.)  �

����������������������������������������������������������������������Ĵ

� Shape = 1 (Exponential)                                              �

� Op. Char.      � 0.99227    0.75959    0.44273    0.25156    0.15122 �

� ASN            � 3.26       3.58       3.26       2.85       2.52    �

� ASN Std. Error � 0.52       1.04       1.30       1.37       1.34    �

����������������������������������������������������������������������Ĵ

� Shape = 5      �                                                     �

� Op. Char.      � 1.00000    0.92428    0.26564    0.02677    0.00210 �

� ASN            � 3.10       4.33       3.96       2.79       2.10    �

� ASN Std. Error � 0.30       0.67       1.17       1.19       0.95    �

������������������������������������������������������������������������

*** Range and Number of Cells ***

The following tabulation shows (again, for the 5STEP5.LIM boundary

example) that OC and ASN calculations are somewhat MORE sensitive to the

"cell width" used to represent definite integrals with finite sums of

cell probabilities than they are to the number of replications used in

Monte-Carlo simulation!

������������������������������������������Ŀ

�    Direct Method   ��    Monte-Carlo     �

�    Calculations    ��    Simulations     �

������������������������������������������Ĵ

Range:  0.0 to 25.00      40.00 ��       0            Startup

No. Intervals    250       2000 ��  -25012      -7616 Seed

Interval Width  0.10       0.02 ��     500      10000 Replics

�������������������������������������������������������������Ĵ

�  Expected Regret �    1.00       1.00 ��    1.00       1.00 �

�  Operating Char. � 1.00000    1.00000 �� 0.99800    0.99990 �

�  Producers Risk  � 0.00000    0.00000 �� 0.00000    0.00000 �

�  Aver.Samp.No.   �    3.07       3.10 ��    3.11       3.10 �

�  ASN Std. Error  �    0.26       0.30 ��    0.31       0.31 �

�������������������������������������������������������������Ĵ

�  Expected Regret �    2.00       2.00 ��    2.00       2.00 �

�  Operating Char. � 0.93415    0.92544 �� 0.90619    0.92341 �

�  Producers Risk  � 0.06585    0.07456 �� 0.09182    0.07609 �

�  Aver.Samp.No.   �    4.26       4.32 ��    4.31       4.34 �

�  ASN Std. Error  �    0.68       0.67 ��    0.69       0.67 �

�������������������������������������������������������������Ĵ

�  Expected Regret �    3.00       3.00 ��    3.00       3.00 �

�  Operating Char. � 0.28768    0.26805 �� 0.25549    0.26307 �

�  Producers Risk  � 0.71232    0.73195 �� 0.74251    0.73603 �

�  Aver.Samp.No.   �    3.98       3.96 ��    3.96       3.95 �

�  ASN Std. Error  �    1.16       1.17 ��    1.20       1.18 �

�������������������������������������������������������������Ĵ

�  Expected Regret �    4.00       4.00 ��    4.00       4.00 �

�  Operating Char. � 0.03081    0.02719 �� 0.02794    0.02770 �

�  Producers Risk  � 0.96919    0.97281 �� 0.97006    0.97210 �

�  Aver.Samp.No.   �    2.81       2.79 ��    2.81       2.78 �

�  ASN Std. Error  �    1.21       1.19 ��    1.23       1.20 �

�������������������������������������������������������������Ĵ

�  Expected Regret �    5.00       5.00 ��    5.00       5.00 �

�  Operating Char. � 0.00252    0.00214 �� 0.00000    0.00280 �

�  Producers Risk  � 0.99748    0.99786 �� 0.99800    0.99710 �

�  Aver.Samp.No.   �    2.11       2.10 ��    2.10       2.09 �

�  ASN Std. Error  �    0.96       0.95 ��    0.92       0.95 �

���������������������������������������������������������������

A somewhat surprising observation from the above table is the closeness

of agreement between the simulation results with 500 and 10,000

replications, respectively.  And direct calculations with 2,000

intervals over the [0,40] range also agree closely with the simulation

results.  In fact, direct calculations with only 250 cells over [0,25]

appear to be relatively less accurate than simulations with only 500

replications!

Of course, SeqDir direct calculations can be performed much more

quickly with fewer cells (200 to 500) than with many (say, 2000 to

2500.)  But relatively "wide" cell widths (say, 0.1 to 0.25 regret

units) are recommended only for preliminary, exploratory SeqDir

runs.

As your sequential test design process approaches its final

stages, you will need to make SeqDir runs with (much) smaller cell

widths to investigate numerical sensitivity in your computed OC

and ASN curves.

McWilliams(1991) has done some independent numerical calculations

and simulations that seem to agree well not only with the SeqDir

direct calculations (at least, those using a couple thousand cells)

but also with SeqDir simulations.

REFERENCES

AROIAN, L. A. (1968).  "Sequential Analysis, Direct Method."

Technometrics 10, 125-132.

AROIAN, L. A. (1976).  "Applications of the Direct Method in Sequential

Analysis."  Technometrics 18, 301-306.

McWILLIAMS, T. P. (1989).  How To Use Sequential Statistical Methods.

Volume 13 of "The ASQC Basic References in Quality Control:

Statistical Techniques."  American Society for Quality Control,

Milwaukee, WI 53203.

McWILLIAMS, T. P. (1991).  Computer programs DRCONT and GAMSIM.

Personal Communication.

PRESS, et.al. (1988). "Numerical Recipes in C: The Art of Scientific

Computing."  Cambridge University Press.  C-source code Copr.

(C) 1985, 1987 by Numerical Recipes Software, P.O. Box 243,

Cambridge, MA 02238. (Gamma & pseudo-random number functions.)

WALD, A. (1947).  Sequential Analysis.  John Wiley & Sons, New York.

���������������������������������������������������������������������Ŀ

� SeqDir was compiled using Microsoft C, ver6.00AX, using the default �

� mode that will automatically detect and utilize a 8087 numeric math �

� coprocessor if your PC has this type of chip.                       �

�����������������������������������������������������������������������

SeqDir Software Update History:

=================================

Version 9107 ...Beta Test Version, Exponential Distributions Only

Version 9108 ...Handle Gamma & Exponential Distributions

Version 9109 ...Add graphical Simulation Module