*******************************************************************************

ASYNC FUNCTIONS FOR MICROSOFT C

Mike Dumdei,  6 Holly Lane,  Texarkana TX  75503

*******************************************************************************

These functions provide a library of interrupt driven communications routines

for users of MicroSoft C versions 3 and 4.  They make calls to the MSC library

functions 'malloc' and 'free' so if you want to modify them to work with a

compiler other than MicroSoft's you will need to use your compiler's version

of these functions.  The ASYNC.H header file sets up most of the segment names

which should make conversion easier if you want to attempt it.

The routines support all memory models (small, compact, medium, large, and

huge) and a library for any size model can be created using the included

MAKE file 'A_LIB.NEW' and defining the model size desired. Ex:

make size=LARGE a_lib.new

If a memory size is not given a SMALL lib is the default.  If you are not

using make, you can assemble for different sizes using MASM by defining the

memory model on the command line.  Ex:

masm async /dLARGE /mx;

COMM.H is an include file that contains the function decalarations and various

definitions that apply to the ASYNC routines.

DESCRIPTION OF FUNCTIONS:

-------------------------------------------------------------------------------

ASYNC_CARRIER -- fast check for carrier detect present.

-------------------------------------------------------------------------------

Example:

int         async_carrier(int);

rcode = async_carrier(port);

Returns:

TRUE  (1)   - carrier was present

FALSE (0)   - no carrier detected

R_NOPORT    - the port hasn't been opened

To monitor carrier detect while transmitting, use this function to get the

fastest executing code.  When receiving, for the fastest code, check the CD

bit in the STAT1 byte which is returned with the character.  Async_stat also

returns carrier detect status but is slightly slower than either of the above

since it also gets some other information.  TRUE and FALSE are not defined

in COMM.H.

-------------------------------------------------------------------------------

ASYNC_CLOSE -- close a serial communications port.

-------------------------------------------------------------------------------

Example:

int         async_close(int);

rcode = async_close(port);

Returns:

R_OK        - close was successful

R_NOPORT    - the port hasn't been opened

This function closes an opened port.  All data in the buffers is lost.  The

modem registers and the interrupt controller are returned to the state they

were in when the port was opened.  This function makes a call to the MSC

library function 'FREE'.

-------------------------------------------------------------------------------

ASYNC_DTR -- toggle DTR modem signal.

-------------------------------------------------------------------------------

Example:

int         async_dtr(int, int);

rcode = async_dtr(port, flag);

Returns:

R_OK        - operation was successful

R_NOPORT    - the port hasn't been opened

A zero flag will cause the modem signal 'Data Terminal Ready' to be brought

to its FALSE state.  Often used to hang up the modem.  A non-zero flag

re-enables the DTR signal.  The DTR signal is set to the TRUE condition when

a port is first opened.

-------------------------------------------------------------------------------

ASYNC_MSR -- get the value of the modem status register.

-------------------------------------------------------------------------------

Example:

int         async_msr(int);

msr_val = async_msr(port);

Returns:

High byte = 0, low byte equals current contents of the MSR.

R_NOPORT    - the port hasn't been opened

This function does not actually read the modem status register.  That is done

using the modem status interrupt function of the comm chip.  When a modem

status interrrupt occurs the MSR is read and the value placed in a storage

area.  This function returns the value from the storage area.

-------------------------------------------------------------------------------

ASYNC_MSRFLOW -- set the mask for modem signals to be monitored while tx'ing.

-------------------------------------------------------------------------------

Example:

int         async_msrflow(int, int);

rcode = async_msrflow(port, mask);

Returns:

R_OK        - operation was successful

R_BADARG    - attempted to set bits in mask other than CTS, DSR, or CD

R_NOPORT    - the port hasn't been opened

Use this function to set the bit mask for the modem status signals that must be

in the TRUE state for transmit interrupts to be active.  Signals that can be

monitored are Clear to Send, Data Set Ready, and Carrier Detect.  To set the

mask for CTS handshaking you would -     async_msrflow(COM1, B_CTS);

where COM1 == 1 and B_CTS == 0x0010.  These are defined in COMM.H.

Be careful when monitoring the CD bit. If you set the bit to monitor this

signal and a carrier is not present all bytes transmitted will go to the tx

buffer and no further (this includes modem dial commands, etc).  If you want to

stop transmitting if CD is lost set the bit after CD is in its TRUE state.

-------------------------------------------------------------------------------

ASYNC_OPEN -- open a serial communications port.

-------------------------------------------------------------------------------

Example:

int         async_open(int, char *, int, int);

rcode = async_open(port, "2400N81", txbuf_size, rxbuf_size);

Returns:

R_OK        - the operation was successful

R_NOPORT    - the port is shown as non-existent in the BIOS data area

R_BAUDERR   - attempted to set the baud rate higher than 19.2K

R_PARITYERR - parity indicator in BPDS string wasn't E, O, or N

R_DTABITERR - data bits were not either 7 or 8

R_STPBITERR - stop bits were not either 1 or 2

R_PORTINUSE - attempted to open an already opened port

R_BADARG    - requested bufr size outside limits or invalid BPDS string

R_NOMEM     - internal call to MSC's 'malloc' function failed

Currently async_open supports COM1 - COM2 so in the example port must be either

1 or 2.  The string, Baud Parity Data Stop (BPDS string), is used to set the

line parameters.  The size of the transmit and receive buffers is dynamically

allocated by a call to the MSC library's MALLOC function. The maximum size for

either buffer is 32000 bytes and the minimum size is 100 for the tx buffer and

750 for the rx buffer.

-------------------------------------------------------------------------------

ASYNC_RESET -- resets rx error bits in STAT1 byte.

-------------------------------------------------------------------------------

Example:

int         async_reset(int);

rcode = async_reset(port);

Returns:

R_OK        - operation was successful

R_NOPORT    - the port hasn't been opened

This function resets any rx error conditions that are signaled in the STAT1

byte.  It is the only way to clear the error bits once they are set.  See

async_rx for more information on STAT1.

-------------------------------------------------------------------------------

ASYNC_RTS -- toggle RTS modem signal.

-------------------------------------------------------------------------------

Example:

int         async_rts(int, int);

rcode = async_rts(port, flag);

Returns:

R_OK        - operation was successful

R_NOPORT    - the port hasn't been opened

A zero flag will cause the modem signal 'Request to Send' to be brought

to its FALSE state.  This signal is used for handshaking.  A non-zero flag

re-enables the RTS signal.  When a port is first opened the RTS signal will

be set TRUE unless RTSFALSE was defined on the command line when assembling

the ASYNC.ASM module.  The MAKE files included in the ARC do not make that

definition so if you use it RTS will initially be TRUE.

-------------------------------------------------------------------------------

ASYNC_RX -- get a character from the receive buffer.

-------------------------------------------------------------------------------

Example:

int         async_rx(int);

stat_and_char = async_rx(port);

Returns:

Low byte is character read and high byte is STAT1.

STAT1: 0=rx bufr ovrfl   1=char overrun     2=parity err    3=framing err

4=break intrpt    5=invalid port     6=rx bufr empty 7=no carrier

STAT1 was designed so that it would be equal to 0 if no line type errors have

been detected, the buffer had something in it, and a carrier is present.  If

a call is made to async_rx when the buffer has nothing in it a '\0' will be

returned as the character and bit 6 will be set in STAT1.  Bit 0 of STAT1

will be set if the rx buffer has overflowed.  Bits 1 thru 4 indicate a line

error or a break signal was received on the comm port.  Bit 5 indicates the

comm port hadn't been opened and bit 7 that the carrier has been lost.

The conditions reflected in bits 0-4 indicate that an error of that type has

occurred since the port was opened or since async_reset was called.  Since

the receive is buffered the char read when STAT1 first indicates the condition

is not necessarily the char that triggered the condition.  Once bits 0-4 are

set they remain set until async_reset is called.

The STAT1 byte can be ignored by casting the returned value to the char type.

-------------------------------------------------------------------------------

ASYNC_RXCNT -- get the number of bytes of unprocessed data in the rx buffer.

-------------------------------------------------------------------------------

Example:

int         async_rxcnt(int);

chars_to_read = async_rxcnt(port);

Returns:

Number of bytes in the rx buffer that have not been read if successful.

R_NOPORT    - the port hasn't been opened

The R_NOPORT return value is a negative number and a successful return is

always positive so you can't mistakenly interpret an error return as the number

of bytes in the buffer.  Also as long as the port has been opened the error

return will never occur anyway.

-------------------------------------------------------------------------------

ASYNC_RXFLUSH -- flush the receive buffer.

-------------------------------------------------------------------------------

Example:

int         async_rxflush(int);

rcode = async_rxflush(port);

Returns:

R_OK        - operation was successful

R_NOPORT    - the port hasn't been opened

-------------------------------------------------------------------------------

ASYNC_SETBPDS -- set baud, parity, # data bits, # stop bits.

-------------------------------------------------------------------------------

Example:

int         async_setbpds(int, char *);

rcode = async_setbpds(port, bpds_string);

Returns:

R_OK        - operation was successful

R_NOPORT    - the port hasn't been opened

R_BAUDERR   - attempted to set the baud rate higher than 19.2K

R_PARITYERR - parity indicator in BPDS string wasn't E, O, or N

R_DTABITERR - data bits were not either 7 or 8

R_STPBITERR - stop bits were not either 1 or 2

R_BADARG    - extra characters in BPDS string

This function changes the line parameters of an already opened comm port. This

function also changes the STRIP_MASK to no high bit stripping if the number

of data bits is set to 8 (see async_strip for more on the STRIP_MASK).  Mark

parity, and less than 7 data bits are not supported.  The baud rate may be

set to any value between 1 and 19200 baud.

The BPDS string has the form "2400N81", "1200E71", "19200N81", "733N72", etc.

First is the baud rate desired, then E O or N for type of parity, followed by

the number of data bits, and last the number of stop bits.

-------------------------------------------------------------------------------

ASYNC_STAT -- get port status information.

-------------------------------------------------------------------------------

Example:

int         async_stat(int, int);

status = async_stat(port, status_mask);

Returns:

STAT1 - high byte, STAT2 - low byte  if successful

R_NOPORT    - the port hasn't been opened

STAT1: 0=rx bufr ovrfl   1=char overrun     2=parity err    3=framing err

4=break intrpt    5=not used         6=rx bufr empty 7=no carrier

STAT2: 0=XON/OFF in use  1=XOFF received    2=XOFF sent     3=tx buf empty

4=montr'g CTS     5=montr'g DSR      6=flw hlt actv  7=montr'g CD

Information contained in the STAT1 byte is described in async_rx section.

There is one difference which is bit 5 is not used by this function to indi-

cate an invalid port as it is in async_rx.  Bits 0 - 2 of STAT2 return infor-

mation on XON/OFF protocol status.  Bit 3 - the transmit buffer is empty,

bits 4, 5, & 7 - tell which signals the transmit interrupt routine is watching

and will shut down if they go false (set by async_msrflow), bit 6 - indicates

that either XON/OFF protocol is being used and an XOFF was received or one

of the monitored modem signals has gone to the low state.

The status mask passed in the function is anded with STAT1;STAT2 information

before it is returned.  Ex:  To check if XON/OFF protocol is being used you

would use --      if (async_stat(COM1, B_XUSE);      where COM1 = 1 and

B_XUSE is defined in the COMM.H file as 0x0001.

-------------------------------------------------------------------------------

ASYNC_STRIP -- set the strip mask for incoming characters.

-------------------------------------------------------------------------------

Example:

int         async_strip(int, char);

rcode = async_strip(port, mask);

Returns:

R_OK        - operation was successful

R_NOPORT    - the port hasn't been opened

The character passed as the mask to this function is ANDed with all incoming

data. It is automatically preset to 0x7F when using 7 data bits and 0xFF when

using 8 data bits by the async_open and async_setbpds functions.

-------------------------------------------------------------------------------

ASYNC_TX -- move a character to the transmit buffer.

-------------------------------------------------------------------------------

Example:

int         async_tx(int, char);

free_space = async_tx(port, 'A');

Returns:

Number of bytes left available in the tx buffer if successful.

R_TXERR     - the tx buffer was full or the port hadn't been opened

-------------------------------------------------------------------------------

ASYNC_TXFLUSH -- flush the transmit buffer.

-------------------------------------------------------------------------------

Example:

int         async_txflush(int);

rcode = async_txflush(port);

Returns:

R_OK        - operation was successful

R_NOPORT    - the port hasn't been opened

-------------------------------------------------------------------------------

ASYNC_TXFREE -- get number of bytes of free space left in the tx buffer.

-------------------------------------------------------------------------------

Example:

int         async_txfree(int);

nmbr_chars_that_can_be_txd = async_txfree(port);

Returns:

Number of bytes of free space left in the transmit buffer.

R_NOPORT    - the port hasn't been opened

This value is also the return value of a call to async_tx.

-------------------------------------------------------------------------------

ASYNC_TXIMD -- put a block of data at the head of the transmit buffer.

-------------------------------------------------------------------------------

Example:

int         async_tximd(int, char *, int);

tx_bytes_free = async_tximd(port, data_ptr, count);

Returns:

Number bytes left available in transmit buffer if successful.

R_TXERR     - the port hasn't been opened or not enough room in tx buffer

This function moves 'count' bytes of data starting at 'data_ptr' to the head

of the transmit buffer.  These bytes will be transmitted ahead of any bytes

already in the buffer and then the bytes already in the buffer will continue

to be transmitted.

-------------------------------------------------------------------------------

ASYNC_XOFFCLR -- manually clear an XOFF received condition.

-------------------------------------------------------------------------------

Example:

int         async_xoffclr(int);

rcode = async_xoffclr(port);

Returns:

R_OK        - operation was successful

R_BADARG    - not using XON/XOFF protocol

R_NOPORT    - the port hasn't been opened

Provides a means to clear an XOFF recieved condition if the sending system

fails to send an XON.

-------------------------------------------------------------------------------

ASYNC_XONANY -- allow any char received to treated as an XON if XOFF active

-------------------------------------------------------------------------------

Example:

int         async_xonany(int, int);

rcode = async_xonany(port, flag);

Returns:

R_OK        - operation was successful

R_BADARG    - not using XON/XOFF protocol

R_NOPORT    - the port hasn't been opened

Passing a non-zero value as the flag in this function causes any character

received to de-activate an XOFF received condition.  A zero value as the flag

will make the protocol treat only the XON character as the XOFF clearing

character.  This is included for the ^S to pause, any key to restart type

setup.

-------------------------------------------------------------------------------

ASYNC_XONOFF -- enable/disable use of XON/OFF flow control.

-------------------------------------------------------------------------------

Example:

int         async_xonoff(int, int);

rcode = async_xonoff(port, flag);

Returns:

R_OK        - operation was successful

R_NOPORT    - the port hasn't been opened

Turns use of XOFF protocol on if flag is non-zero else disables use of XOFF.

If an XOFF is active when disabling protocol use it will be automatically

cleared and transmission will resume.  If an XOFF has been sent when disabling

XOFF protocol use, an XON character is sent and then the protocol is disabled.

The XOFF protocol as implemented by these routines sends an XOFF when the

buffer is within so many bytes of being full rather than a percentage.  The

number of bytes is defined as the variable XOFFCOUNT in ASYNC.H.  The XON is

sent when the number of free bytes in the rx buffer is at least XONCOUNT.

These values are currently about 50 bytes different to prevent repeated sending

of XON's and XOFF's when the buffer is at the threshhold level.  Another

variable defined in ASYNC.H, REPTXOFFVAL, determines how many bytes can be

received after an XOFF is sent before another XOFF is sent. Some systems

allow any character received after getting an XOFF to be interpreted as the

XON signal including another XOFF so setting REPTXOFFVAL too low will effect-

ively disable the protocol.  Also some systems, RBBS, seem to be picky about

when they will accept an XOFF while other systems, PROCOMM, respond instantly.

The values chosen seem to be the most likely to work in all situations.