EN.DOC - PROTECTED MODE - EXIT TO REAL MODE EDMOD.OBJ - RETURN PROTECTED MODE

-----------------------------------------------------------------------------

EN.EXE is a fun and games program that illustrates how to enter protected

mode, type 19 lines in protected mode using a KISS mini-keyboard decoder,

offers 12 each HELP choices via the F1 through F12 keys, and most importantly

allows the user to near instantly go to REAL MODE's EDMOD.OBJ program via the

Ctrl key.

�����������������������������������������ͻ

� F1 key:     PROTECTED MODE       HELP 1 �

�����������������������������������������͹

� This modified program uses a KISS mini- �

� keyboard decode routine.                �

�                                         �

� Enter       beginning next line down    �

� backspace   erases & moves back a byte  �

� arrow keys  PgUp PgDn Home are active   �

� Alt+C or c  cls all inside rectangle    �

� Alt+keypad  3 digits for 0-255 ASCII    �

� F2          displays all pages EN.DOC   �

� F3          segments size and addresses �

� F4          EDMOD.OBJ typical page      �

� F5          speed up/slow down keyboard �

� F6          Print Screen subroutine     �

� F7          Pentium protected exceptions�

� F8          display all CS DS ES FS GS  �

� F9          set FS/GS seg to 0000h-F000h�

� F10         diz control-debug-flag regs �

� F11         shift F1 thru shift F12 menu�

� F12         Interrupt Descriptor Table  �

� Ctrl        go Edmod - Del ret protected�

� Please      read EN.DOC in C:\PROTECT   �

�����������������������������������������ͼ

EDMOD.OBJ is an `olde` (circa 1977 though frequently updated) program that

allows the user to edit/modify any/all ram memory in real mode, plus displays

the disassembled instruction on the lower left bottom of the video display (you MUST know where the instruction begins to disassemble it correctly).

You may switch back to PROTECTED MODE near instantly while in EDMOD.OBJ by

pressing the Del key.  Please write down these directions.

You may switch to the DOS> prompt any time in either program by pressing the

Esc key.

EN.ASM, the source code file for EN.EXE is rather well commented and should

explain the fundamentals of what is going on after the STRUCT and MACRO files

are executed.

Use the enclosed EDLIN.COM to read all of them.  It is an updated version that

will read and write 33% larger files in DOS 4.0, DOS 5.0 and DOS 6.0 than the

standard versions.

The STRUCT and MACRO1 through MACRO4 explanations follow:

---------------------------------------------------------

STRUCT: defines the structure of the descriptions (descp) + page tables + old

IP and CS values.

MACRO1: defines the stack in the Task State Segment (TSS), + CR3 in TSS +

general registers in TSS + segment registers in TSS + plus CALL FAR & JMP FAR.

(note: EN.EXE does NOT switch tasks so the TSS is not used, but is included

for general info).

MACRO2: steps 0, 1, 2, 3, 4 which defines the General Descriptor Table (GDT),

Interrupt Descriptor Table (IDT), limits, + initializes the segment base

address for each segment decriptor.

MACRO3: step 5 defines all of the DS segment including messages, mini-keyboard

decoder and Help pages.

MACRO4: steps 6, 7, 8 defines pivilege level 0, 1, 2 stack segments, + set up

task state segment, + define a dummy segment to define old real mode segment

registers, + CR0 register value.

Code Segment - Step 8A:

-----------------------

Most all Intel 80386, 80486 and Pentium microprocessor programs are

initialized in real mode.  Since this demo program illustrates how to switch

back and forth between real mode and protected mode, it must tell the real

mode program, EDMOD.OBJ that is loaded high to segment 9000h:8000h its code

segment so that EDMOD.OBJ knows where to return.  The st_art lines do just

that by stuffing its code segment down into low memory at 0000h:0300h plus

telling EDMOD.OBJ it is a .EXE program rather than .COM program by loading an

'E' into 0000:0382h.

Step 8B - save or restore `typed.dat` from C:\PROTECT:

------------------------------------------------------

This is a simple routine that allows the user to save or restore the second

welcome page that is displayed only ONCE on program initialization.  If you

save it as a blank page via Alt + C then the shift F2 key, it will display a

blank page after the first welcome page is displayed the next time you run

EN.EXE.  Alternately, you may type in anything you wish and save it via the

shift F2 key and it will be displayed after the first welcome page when you

run EN.EXE the next time.

Steps 9, 10 and 11:

-------------------

Initialize the interrupt descriptor table, get the general descriptor table

(GDT) and interrupt descriptor table (IDT) limits & linear address, and set

the linear base address for each corressponding descriptor.

Step 12 - switch to protected mode:

-----------------------------------

Clear interrupts, load global descriptor table (GDT) and limit into global

descriptor table register (GDTR), load interrupt descriptor table (IDT) and

limit into interrupt descriptor table register (IDTR), and set register CR0 to

protected mode enable.  Then, a far jump to enter_prot flushes the instruction

queue and the program is in protected mode.

Step 13 - set LDTR, SS, SP, DS, ES, FS, GS:

-------------------------------------------

The local descriptor table register (LDTR) is not used in this program so it

is set to a null value and segment register SS, stack pointer, segment

registers DS, ES, FS and GS set to new protected mode values.

Steps 14 - 14A display message and keyboard input decode in protected mode:

---------------------------------------------------------------------------

Since protected mode does not allow the use of real mode interrupts, every

familiar function/interrupt that you are accustomed to in real mode that you

wish to use must be created.  Step 14A after label bb0 calls kybd1 in

approximately line 510 where all the protected mode keyboard input functions

are executed (approximately lines 510 through line 2027).

Rather than reinvent the wheel by rewriting every interrupt function desired,

EN.EXE does some fancy footwork when a number of functions are desired.  What

it does is to near instantly exit protected mode, perform the function in real

mode, and then near instantly return to protected mode doing exactly what is

was doing when it departed.

Shift F11 will display HELP 11, the SHIFT + F1 thru F5 functions.  Shift + F5

is an interesting option.  When pressed, it displays a VGA graphics page we

use in one of our IBM touchscreen information system programs.  In this case

it is one of the 4 Welcome pages used in our Ford Scientific Research Center

program. Any key pressed after displaying the page will near instantly return

you to protected mode.

Exit protected mode - go to EDMOD.OBJ in real mode - return to protected mod:

-----------------------------------------------------------------------------

In protected mode the Ctrl (control) key is used to switch to EDMOD.OBJ in

real mode.  Here is the sequence after the Ctrl key is pressed (line numbers

are approximate).

line  608: senses the Ctrl key (al,29) and jumps go_edmod in line 1909.

line 1909: sets bp,3 - calls stor_video - and jumps to ky9b in line 701.

line  701: displays 'Now in real mode' and jumps to bb4 in line 261.

line  261: is part of Step 15.  Steps 15 to 22 are exit protected mode.

line  365: senses bp,3 so jumps to ed_mod in line 389.

line  389: on up first checks 9000h:8000h for e9h = jmp byte.

if NOT there it loads EDMOD.OBJ to 9000h:8000h and jmps 9000h:8001h

if there it immediately jmps 9000h;8011h.

Now we are in EDMOD in real mode.  Press F7 to read EDMOD.DOC.  The F1 key

displays an index of the help options.

�����������������������������������������ͻ

� F1 key:     EDMOD.OBJ - HELP            �

�����������������������������������������͹

� EDIT MODE (green)                       �

� Esc key     DOS> prompt or Windows      �

� Shift F1    ret ur pgm if 0000:0300 set �

� F6 key      display any directory       �

� D key       decimal to hex converter    �

� H key       hex to decimal converter    �

� S key       global search ASCII/hex     �

� F8 key      using the KISS disassembler �

� F9 key      zeros out entire segment    �

� F10 key     zeros out displayed page    �

� note:       will not zero BIOS or EDMOD �

� MODIFY MODE (yellow)                    �

� ASCII key   modifies mem beneath cursor �

� F2 key      toggles back to EDIT MODE   �

� F3 key      input hex beneath cursor    �

� keypad      directions keys active      �

� MODIFY MODE (red) if in BIOS or EDMOD   �

� press F7    to display EDMOD.DOC        �

�����������������������������������������ͼ

Since we came from protected mode, NOT all of EDMOD's 'save the newcomer from

fouling up the program' functions are available.  Yes, you can edit/modify

most all of memory including EN.EXE's DS and CS segments which will prevent

you returning to it, plus the BIOS areas in segment 0000h and maybe segment

1000h which will lockup the program.  If you do, about the worst that can

happen is that you will have to restart with a cold boot.  To the best of our

knowledge it will not harm any variety of computer.

EDMOD is also a fun & games program.  It is NOT a replacement for debug or a

first class disassembler, though if you know where an instruction begins, the

keypad + key will probably disassemble it correctly.  The Pentium disassembler

function is about 66% complete and should be finished by Christmas 1993.

A typical EDMOD page is illustrated below.

Edit/Modify 1024K Mem and Kiss Disassembler  copyright 1977-1993  REAL

(c) R. Richardson              0123456789ABCDEF       Pentium alpha 1.0  MODE

���������������������������������������������������������������ͻ

BEGIN  � 0000000000000000000000000000000000000000000000000000000000000 � DSseg

0000   � 0000000000000000000000000000000000000000000000000000000000000 � 8000

� 0000000000000000000000000000000000000000000000000000000000000 �

MEMORY � 0000000000000000000  do not use mouse if  0000000000000000000 � CSseg

0000   � 0000000000000000000  from protected mode  0000000000000000000 � 9000

� 0000000000000000000000000000000000000000000000000000000000000 �

VALUE  � 0000000000000000000  each page displays   0000000000000000000 � ESseg

0000   � 0000000000000000000  1024 bytes of memory 0000000000000000000 � B800

� 0000000000000000000  in ASCII & extended  0000000000000000000 �

EDIT   � 0000000000000000000  ASCII                0000000000000000000 � SSseg

MODE   � 0000000000000000000000000000000000000000000000000000000000000 � 9000

� 0000000000000000000    zero values are    0000000000000000000 �

DISAS- � 0000000000000000000  displayed as ASCII 0 0000000000000000000 � SP

SEMBLY � 0000000000000000000000000000000000000000000000000000000000000 � FCF6

� 0000000000000000000000000000000000000000000000000000000000000 �

HEX    � 0000000000000000000000000000000000000000000000000000000000000 � END

INPUT  ���������������������������������������������������������������͹ 03FF

�  <Enter> : Up 1024 bytes      �  <Arrows> : Move cursor       �

ADD    �  <Minus> : Down 1024 bytes    �   <Mouse> : <- ^ v ->         � F1

[Bx+Si]�     <F2> : Toggle modify mode �      <F5> : Change Seg 0 - F  � Help

AL     �     <F3> : Hex modify         � <Keypad+> : Disassemble Memory�

���������������������������������������������������������������ͼ

Exit EDMOD.OBJ and return to ENE.EXE in protected mode:

-------------------------------------------------------

In either EDMOD's edit or modify mode, pressing the Del (delete) key will

return you to protected mode with everything exactly the same as you left it.

This commences the following sequence:

EDMOD tests 0000:0300h for zero.  If so it beeps (nothing there, so ignore).

If not zero it saves ENE.EXE's code segment and then tests 0000:0382h for 'E'

denoting an .EXE file.  If so it then jumps far to ENE.EXE's code segment plus

20H which is located in the 15 each nops at line 39 and is part of Step 8A.

It then continues through Steps 9, 10, 11, 12, 13 and 14 re-entering protected

mode.  Since hold_video is not zero it calls rstor_video and uses sav_line and

savdi to restore the cursor to its previous position.  Everything now is as it

was before.

Pressing the Esc key in EDMOD will return you to the DOS> prompt.  Pressing

the Esc key in protected mode will return to real mode and give you the choice

of:

Press N to go to the DOS> prompt

Press Y to go to EDMOD

Press enter to return to protected mode

GOING TO EDMOD.OBJ FROM THE DOS COMMAND LINE:

---------------------------------------------

The little program, GOEDMOD.COM in C:\PROTECT will load EDMOD.OBJ to

9000h:8000h from the DOS> command line and then jump far to it.  GOEDMOD.COM

may be on any drive in any subdirectory as long as EDMOD.OBJ is in the same one.

Previous loading of EDMOD.OBJ has no effect since upon return to the DOS>

prompt, DOS 6.0's COMMAND.COM reloads itself into the 9000h:8000h region.  The

portion of DOS 6.0's COMMAND.COM that is loaded to 9000h:8000h is at its

offset 21,804.

CONCLUSION:

-----------

EN.EXE is strictly a demo of how to enter protected mode, exit protected mode,

re-enter protected mode, and go to and from EDMOD in real mode.  Since it uses

'para public use16' one cannot use the 80386, 80486 or Pentium's fabulous

ability of addressing up to 4 gigabytes of space with a single fell swoop.

Unfortunately the 'use16' and most all the protected mode initialization

routines limit segments to 64k.

Most programmers will admit that using the LIM (Lotus-Intel-Microsoft) 4.0

extended memory 64k page swapping scheme to use a single segment of extended

memory at a time is downtown crudesville, yet it has worked for years and is

used in countless successful programs.

As an obvious matter of fact, getting into and out of protected mode is not

exactly simplicity itself.....yet when one becomes accustomed to the new rules

of the protected mode game, it is not all that difficult.

HUGE.EXE and HUGE.ASM will shortly introduce the 'para public use32' version

of EN.EXE and EN.ASM in protected mode that will be able to address huge seg-

ments of memory far larger than 1024k.  Many of the protected mode initial-

ization routines are being rewritten to accomodate the huge segmentation.

Special thanks go to 'i386/i486 Advanced Programming' author, Shirley Her with

Nexgen Microsystems in Silicon Valley (San Jose), California for all her aid,

guidance and superior programming skills that made ENE.EXE possible.  Those

who were of assistance to Shirley Her while writing the book include:

Sen-Cuo Ro   - coauthor 'i386/i486 Advanced Programming' - Nexgen Taiwan

Rajvir Singh - Automata Publishing

Avatar Saini - Intel

Jay Sethuram - Intel

Hal Broome   - Sun Microsystems

Steve Thomas - Intergraph

Deepak Verma - VLSI Technology

DISPLAYING ASCII & EXTENDED ASCII CHARACTERS 0 THROUGH 255:

Press the Alt key and then the far right on the keyboard keypad's number

desired THREE times; i.e., to display 1, press 0 then 0 then 1. To display 2,

press 0 then 0 then 2.  THREE numbers must always be pressed.

- end -