A R B I T R A R Y   P R E C I S I O N   M A T H   R O U T I N E S

(0) Overview

To facilitate development of mathematical software on 8051-family of

processors, a set of routines has been developed that allows arithmetic

operations to be performed on data objects of arbitrary precision, and on

floating point data objects.  These routines will operate only on objects

located in the 8051 internal data memory.

(1) Arbitrary Precision Routines

All the following routines use the following registers:

R0 ... points to the destination data object located in internal memory.

R1 ... points to the source data object, if there is one.

R2 ... indicates the size of the data objects being operated on.

These are the routines.  In this list, the names "Src" and "Dest" are used to

indicate the source and destination objects.

ClearX:    "Dest = 0"

CopyX:     "Dest = Src"

CplX:      "Src = ~Src"

RolX:      "rotate Src left through the carry, C"

ShiftX:    "shift Src left, high bit goes into C"

LessX:     "C = (Src < Dest)"

ZeroX:     "C = (Src == 0)"

IncX:      "Dest++"

CarryX:    "Dest += C"

DecX:      "Dest--"

BorrowX:   "Dest -= C"

AddX:      "Dest += Src, carry goes into C"

SubbX:     "Dest -= (Src + C), borrow goes into C"

SubtractX: "Dest -= Src, borrow goes into C";

NegX:      "Dest = -Dest"

Inc10X:    "Increment Src in BCD format"

The BCD increment operation (Inc10X) would take a data object of value,

03 05 06 06 09 09h

for instance, and "increment" it to

03 05 06 07 00 00h

As an example of how these routines are used, if the following declarations

had been made:

X equ 30  ;;; allocate 3 bytes at registers 30-32 for X

Y equ 33  ;;; allocate 3 bytes at registers 33-35 for Y

and if X, and Y were initialized as follows:

mov X, #43h

mov (X + 1), #24h

mov (X + 2), #02h   ;;; X = 22443h;

mov Y, #24h

mov (Y + 1), #00h

mov (Y + 2), #20h   ;;; Y = 200024h;

then the following sequence:

mov R0, #X

mov R1, #Y

mov R2, #3

call AddX   ;;; X += Y;

will carry out the operation indicated in the command above, and the X

data object will have the value 222467h in the following format:

register  30  31  32

contents  67h 24h 22h

The carry flag, C, will be cleared.

(2) The Multiplication Routines

(A) MultiplyX

The MultiplyX routine is actually an "add-and-multiply" operation that

allows you to accumulate the products into a data object.

The operation performed is:

Z += X*Y;

The user defines the X data object by setting the variable ArgX to point to

the data object, and setting SizeX to indicate the size of the object pointed

to by ArgX.  The same thing goes with Y, with respect to the variables ArgY

and SizeY.  The data object, Z, is pointed to by ArgZ.

So, for example, if the following declarations were made:

Cycles equ 30 ;;; allocate 2 bytes at 30-31

Total  equ 32 ;;; allocate 4 bytes at 32-35

Product equ 36 ;;; allocate 6 bytes at 36-41

then the following will add the product of Cycles and Total into Product:

mov ArgX, #Cycles

mov SizeX, #2

mov ArgY, #Total

mov SizeY, #4

mov ArgZ, #Product

call MultiplyX     ;;; Product += Cycles*Total;

(B) MulByR1

This is a routine that allows you to multiply a data object ("Dest") by

a single byte.  The following registers are used:

R0 ... Points to the "Dest"

R1 ... Points to the multiplier byte

R2 ... Indicates the size of Dest.

A multiplication is performed, the result is placed in "Dest", and the carry

byte is placed in R3.  For instance, if the following declaration was made

X equ 30 ;;; allocate 3 bytes to X

with the initialization:

mov X, #7dh

mov (X + 1), #24h

mov (X + 2), #35h ;;; X = 35247dh;

then this operation:

mov R0, #X

mov R1, #10h

mov R2, #3

call MulByR1  ;;; X += 10h;

will multiply "X" by 10 (hexadecimal), so that X now will have the value

5247d0h, formatted as follows:

register 30  31  32

contents d0h 47h 52h

and R3 will have the carry (3).

(3) The Fixed Point Routines

(A) DivideX

DivideX will carry out an arbitrary precision division (up to 8 bytes).

The operand and divisor should be loaded into the registers Op, and Divisor.

After DivideX, the registers Quo, and Fract will contain the integer part

and decimal part of the quotient.  The effect is to carry out the operation:

"Quo.Fract = Op/Divisor"

(B) Sqrt64

Sqrt64 calculates the square root of an 8-byte data object located in

register W, and returns a 4-byte result in register Q:

"Q = sqrt(W)"

For instance, if the following initialization is carried out:

mov W, #21h

mov (W + 1), #43h

mov (W + 2), #45h

mov (W + 3), #23h

mov (W + 4), #01h

mov (W + 5), #00h

mov (W + 6), #00h

mov (W + 7), #00h  ;;; W = 123454321h

then, the operation

call Sqrt64

will load Q with the square root (which is 11111h):

register  Q   Q+1  Q+2  Q+3

contents 11h  11h  01h  00h

if W had be interpreted as a fixed-point quantity (12345.4321h), then

the square root would still be a valid result, when properly interpreted

(namely Q = 111.11h).  So this operation (and DivideX) works just as well

with fixed-point arithmetic.

(4) Allocating Registers

The assembler that I normally use does not handle relative addressing.  That

means that I have to allocate the registers defined above by hand each time

I include this library.  If your assembler does not have the ability to handle

relocatible segments, you will likewise have to do the mapping of register

variables by hand.  In the library provided, these registers have been mapped

as follows:

For MultiplyX: SizeX   equ 30h ;;; 1 byte

SizeY   equ 31h ;;; 1 byte

ArgX    equ 32h ;;; 1 byte

ArgY    equ 33h ;;; 1 byte

ArgZ    equ 34h ;;; 1 byte

For DivideX:   SizeX   equ 30h ;;; 1 byte

Op      equ 35h ;;; 8 bytes

Divisor equ 3dh ;;; 8 bytes

Quo     equ 45h ;;; 8 bytes

Fract   equ 4dh ;;; 8 bytes

Rem     equ 55h ;;; 8 bytes

For Sqrt64:    Q       equ 35h ;;; 8 bytes

D       equ 3dh ;;; 8 bytes

X       equ 45h ;;; 8 bytes

W       equ 4dh ;;; 8 bytes

Y       bit F0

The overlapping segments for DivideX and Sqrt64 are no major problem since these

routines are never executed concurrently in any of my applications.