TECMAR.BGI  --- V 1.13

This provides for support for the Tecmar Graphics Master in what

must be one of the weirdest setups around.  There is no way that this

configuration would have been supported without someone using it

having the ability to create drivers.  This sort of situation

represents the real flexibilty of a driver system.

This driver supports the Graphics Master in a VERY specific

configuration.  The card is configured to be mapped to the 64K

segment starting at the paragraph 0xA000 and ONLY that segemnt.  It

uses bank swithing to map the whole 128K into that 64K.  This was set

up so that it could co-exist with an Hercules card.  This also

partially explains its rather peculiar features.  It has no text mode

and only one graphics mode.  Because of the hardware on the Gaphics

Master that mode's colours are unchangeable.

MODES:

0 --- 640 x 400 x 16  -- 16 FIXED colours.

LIMITATIONS:

Block operations are slow.  They could be speeded up

considerably by recoding them in assembly language.

Flood fill is unimplemented at present.

Palette setting operations are null functions.  The colours on

the TGM are hardwired and unchangeable.

Error checking & reporting are almost non-existant.

Everything that can be emulated is.  This seems to be

reasonably fast so changing this is not a high priority.

KNOWN BUGS:

Line width is currently ignored.  This seems to be taken care

of by the kernal but should be changed.