DT2851.BGI  --- V 1.01

This driver provides support for the Data Translation DT2851

frame grabber.  This card came with some of the best documentation

I have ever seen for a PC add in card.

MODES

0 -- 512 x 480 x 256  with two pages.

LIMITATIONS:

This driver buffers and rewrites display lines into extended memory

as needed.  Because it buffers whole lines changes that run vertically

are very slow.

Does not check to make sure extended memory BIOS functions are

available.  This should not be a big problem since this board is

going to be difficult to use if they are not available.

Flood fill is unimplemented at present.

Error checking & reporting are almost non-existant.

Everything that can be emulated is.  This seems to be reasonably

fast so changing this is not a high priority.

KNOWN BUGS:

Line width is currently ignored.  This seems to be taken care of by

the kernal but should be changed.