C Communications Library ( CCL )

Version 1.0

14 January 1991

Copyright (C) 1991

By MarshallSoft

All rights reserved

MarshallSoft

PO Box 4543

Hunstville, AL 35815

(205) 881-4630

Page 1

Table of Contents

Chapter                                     Page

Introduction.................................3

Registration.................................4

Serial COM Ports.............................5

RS232........................................6

National INS8250 UART........................7

Revision History.............................7

Warranty.....................................7

Using the Library............................8

Compiling & Linking.......................9

Problems.................................10

Library Reference...........................11

SioBaud..................................12

SioBrkKey................................13

SioBrkSig................................14

SioCrtWrite..............................15

SioDTR...................................16

SioDelay.................................17

SioDone..................................18

SioError.................................19

SioGetc..................................20

SioKeyPress..............................21

SioKeyRead...............................22

SioLine..................................23

SioModem.................................24

SioParms.................................25

SioPutc..................................26

SioRTS...................................27

SioReset.................................28

SioRxBuf.................................29

SioRxFlush...............................30

SioRxQue.................................31

SioTimer.................................32

Error Code Summary..........................33

Page 2

Introduction

The  C  Communications  Library  (  CCL  ) is  an  asynchronous

communications  library   designed   for   experienced   software

developers  programming  in  Microsoft  C  or  Turbo  C.   An IBM

PC/XT/AT or compatible is required.  The CCL features:

o  16 communications functions + 6 support functions.

o  Receiver is interrupt driven.

o  Runs from 300 baud to 115,200 baud.

o  Supports COM1, COM2, COM3, and COM4.

o  Adjustable receive queues from 8 bytes to 16 KB.

o  Control-BREAK error exit.

o  11 comm error conditions trapped.

o  Supports all memory models.

o  Allows 2 ports to run concurrently.

o  Complete modem control & status.

o  Written in hand optimized assembly language.

o  Low overhead & very reliable !

A typical application program  using  CCL  might  look  like  the

following code outline:

+-------------------------------------------------------+

| #include "ccl.h"                                      |

|                                                       |

| char far buffer[1024];                                |

| main()                                                |

| {                                                     |

|  SioRxBuf(Port,Buffer,Size1024);                      |

|  SioParms(Port,NoParity,OneStopBit,SevenDataBits);    |

|  SioReset(Port,Baud2400);                             |

|                                                       |

|  ...application code...                               |

|                                                       |

|  SioDone(Port);                                       |

| }                                                     |

+-------------------------------------------------------+

A   simple   terminal  emulator  program CALLPGM.C is provided in

source code form as an example  of  the  use  of  CCL  functions.

CALLPGM  can  be  used  to  call  up  bulletin  board services or

mainframe computers.

If you find any problems with the  C  Communications  Library  or

have  any  suggestions  for improvement, please call or write.  A

Pascal version of CCL ( to be called PCL ) may  be  available  if

demand warrants.

Custom versions of CCL can be developed for special needs. Custom

communication packages can also be developed. Call for prices.

Page 3

Registration

The  shareware  version  of  CCL.LIB  is provided so that you may

personally determine the usefulness of the product for  yourself.

If you can use CCL.LIB, please register your use with us.

MarshallSoft

PO Box 4543

Huntsville, AL

35815-4543

Please  pay  by  check  in  US  dollars.  Payment  must accompany

purchase orders. Print the file CCL.INV if an invoice is  needed.

The registered package is $35 postpaid and includes:

o  CCL.LIB   C Communications Library - without shareware screen.

o  CCL.ASM   C Communications Library source code.

o  CCL.DOC   C Communications Library documentation file.

o  XMODEM.C  Source for the XMODEM file transfer code.

o  YMODEM.C  Source for the YMODEM file transfer code.

o  LZW.C     Source for Lempel-Ziv-Welch file compression code.

o  CRC.C     Source for cyclic redundancy check code.

o  CALLPGM.C Source for terminal emulator. Uses XMODEM & YMODEM.

o  Backbone bound printed users manual.

o  Telephone support for one year.

o  All future updates are $10

CCL.ASM  is  the  source code for the library. The source code is

copyrighted by MarshallSoft. The user is granted a license to use

the CLL object code in his own application only. CLL.ASM  is  not

shareware and may not be sold or given away to anyone.

XMODEM.C  is  an  implementation  of the industry standard XMODEM

protocol, including XMODEM/1K ( uses 1K blocks ) and XMODEM/CRC (

uses CRCs instead of checksums ). Virtually every bulletin  board

offers XMODEM file transfer.

YMODEM.C  is  basically  XMODEM/CRC/1K  with the  file  name  and

length   encoded in the first block. Thus, the receiver knows the

file  length  and  filename.  YMODEM  is also widely available on

bulletin board services.

LZW.C   is   an   implementation  of  the  Lempel-Ziv-Welch  file

compression algorithm.  It  is  used  in  many  file  compression

utilities available today. LZW can compress file to as much as 40

percent of their original size.

CRC.C is an implementation of the 16 bit Cyclic Redundancy  Check

algorithm   as   specified   by  the  Comsultative  Committee  in

International Telegraphy and Telephoney ( CCITT ). It is used  in

XMODEM/CRC  and  in  YMODEM. It will detect 99.99 percent of data

errors.

The  registered user will receive the  latest  version  of CCL by

return  mail.   A  5.25"  diskette  is  provided  unless  a  3.5"

diskette is requested when ordering.

Page 4

Serial COM Ports

IBM  PC  compatible  computers can have up to four serial  ports.

The  BIOS  table  located  at address 40:0000 has room  for  four

communication  port  addresses: COM1 to COM4. During boot up, the

COM1 and COM2 addresses are placed in the  BIOS  table  providing

that  the  hardware  is present. Unfortunately, the COM3 and COM4

addresses are not placed in the table in  many  IBM  compatibles.

This  can  be  corrected by using DEBUG to assemble the following

program SETCOM3 which should be added to the  AUTOEXEC.BAT  file.

When  executed,  this  program  will place the standard COM3 port

address 03E8 in the BIOS table. For COM4, change 03E8 to 02E8 and

[0004] to [0006].

PUSH DS

MOV AX,0040

MOV DS,AX

MOV AX,03E8

MOV [0004],AX

POP DS

MOV AX,4C00

INT 21

If you have never used DEBUG, refer  to  your  MSDOS  manual  for

detailed instructions.

Page 5

RS-232C

RS-232    is    the   name  of the serial data interface standard

used  to  connect  computers  to  modems.   Most  IBM  compatible

computers are built with at least one serial port and use  either

DB9 ( 9 pin ) or DB25 ( 25 pin ) connectors.

A summary of these pins and  their  function  follows.  For  more

detailed information, refer to one of the many books dealing with

RS-232 interfacing.

Signal Ground         Pin 7 (DB25),  Pin 5 (DB9)

The  SG line is used as the common signal ground, and must always

be connected.

Transmit Data         Pin 2 (DB25),  Pin 3 (DB9)

The TX line is used to carry data from the computer to the modem.

Receive Data          Pin 3 (DB25),  Pin 2 (DB9)

The RX line is used to carry data from the modem to the computer.

Data Terminal Ready   Pin 20 (DB25), Pin 4 (DB9)

The  DTR line is used by the computer to signal the modem that it

is ready.

Data Set Ready        Pin 6 (DB25),  Pin 6 (DB9)

The DSR line is used by the modem to signal the computer that  it

is ready.

Request to Send       Pin 4 (DB25),  Pin 7 (DB9)

The   RTS  line  is used to "turn the line around" in half duplex

modems, but is not necessary in full duplex modems.

Clear to Send         Pin 5 (DB25),  Pin 8 (DB9)

The CTS line, like the RTS line, in not necessary in full  duplex

modems.

Data Carrier Detect    Pin 8 (DB25),  Pin 1 (DB9)

The DCD line is used by the modem to signal the computer  that  a

data carrier signal is present.

Ring Indicator         Pin 22 (DB25), Pin 9 (DB9)

The RI line is asserted when a 'ring' occurs.

Page 6

National INS8250

The   C   Communications   Library   is  based  on  the  standard

National INS8250 UART. The 8250  consists  of  6  register  ports

based at the following standard addresses:

COM1 = 3F8H  COM2 = 2F8H  COM3 = 3E8H  COM4 = 2E8H

If  you are not familiar with the INS8250, several good books are

available.  Although  a knowledge of the 8250 is not necessary to

use CCL, a general  knowledge  of  the  theory  of  operation  of

Univeral  Asynchronous  Receiver  /  Transmitters  (  UARTs  ) is

recommended.

Offset R/W  Register

0       R/W   Receiver ( read ) / Transmitter ( write )

1       R/W   Interrupt Enable

2       R     Interrupt Identification

3       R/W   Data Format ( Line Control )

4       R/W   RS-232 ( Modem ) Control

5       R/W   Line Status

6       R/W   RS-232 ( Modem ) Status

Revision History

Version 1.0 -- 14 January 1991 -- original release.

Warranty

The  user  of this software assumes all liability for its use. In

no case shall MarshallSoft be liable for any  damages,  including

any incidental or consequential damages.

Page 7

Using the Library

The first thing  to  do  is  to  copy  all  the  files  from  the

CCL distribution disk to a working disk, and put the original CCL

distribution disk in a safe place.

The  C  Communications  Library  is  provided   as   the  library

file  CCL.LIB.   It  has  been  tested  with both Microsoft C and

Turbo C compilers.  CCL.LIB may be used with  any  memory  model.

However, your application must be certain to include CCL.H and to

specifically declare the receive buffers as FAR ( see SioRxBuf ).

Please examine the CCL.H file. Note that COM1 is defined as  port

zero,  not  port  one. The user must assume the responsibilty for

passing  the  correct  information  when  calling  CCL functions.

Function prototypes are not provided in CCL since some earlier  C

compilers  don't support them. However, the CCL.H file can easily

be modified to provide them if so desired.

For   an   example   of  CCL  use,  examine the terminal emulator

program  CALLPGM.C.   It  uses  most every CCL function. The user

should compile CALLPGM.C and link with CCL.LIB as a test  of  the

library.

If  you  have  two  computers, then you can connect them together

with a null  modem  cable  and  run  CALLPGM  on  both  machines.

Whatever  is typed on one machine should appear on the other, and

vice versa. Depending on the design  of  the  null  modem  cable,

CALLPGM.C  may  need  to be modified so that it does not wait for

DSR & CTS. This is clearly documented in the CALLPGM.C code.

If you have a modem, then use CALLPGM to  call  up  any  bulletin

board  system  (  BBS  ).  There  are  many  free BBSs around the

country. Look in any issue of "Computer Shopper" (  available  in

bookstores,  computer shops, and many grocery stores ) for a list

of current systems.

CCL functions can be called from any language  supporting  the  C

language  calling  convention  and FAR arrays and functions.  See

your compiler manual.

Page 8

Compiling and Linking

CCL  has  been  tested with both Microsoft C and Turbo C. Other C

compilers may work as well provided that they recognize  the  FAR

keyword.

Any  memory model can be used with CCL since all functions in CCL

are declared as  FAR.  Recall  that  the  receive  buffer  (  see

function SioRxBuf ) must also be declared as FAR.  If  two  ports

are  to  be  run  simultaneously, be sure to use seperate receive

buffers.

Registered users may  wish  to  assemble  CLL.ASM.  Use  the  /MX

switch  in  order to disable automatic conversion from lower case

to upper case.  If the /MX switch  is  not  used,  then  all  CLL

function  references in C code must be in upper case. To assemble

using the Microsoft assembler:

MASM CCL /MX;

Then make CCL.OBJ into a library file:

LIB CCL +CCL,CCL.LST,CCL.LIB;

When compiling with the Microsoft compiler, use the /Ze switch to

enable  recognition of the FAR keyword. To compile and link using

Microsoft C:

MSC -Ze TE;

LINK CALLPGM,CALLPGM,CALLPGM,CCL.LIB

To compile and link using Turbo C:

TCC CALLPGM CCL.LIB

Page 9

Problems

If you cannot get your application to run properly, first compile

and run  the  terminal  emulator  program   CALLPGM  provided  on

your distribution disk.

If  CALLPGM  runs  correctly,  then  you  have made a programming

mistake in your  application.   MarshallSoft  cannot  debug  your

application,  especially  over  the telephone!  However, consider

each of the  following  when  searching  for  an  error  in  your

application.

1.  Have you included the file CCL.H in your application ?

2.  Have  you declared your receive buffer to be FAR ? Your code

should include something like:

char far Buffer[1024];

SioRxBuf(Port,Buffer,Size1024);

3.  Is your receive buffer large enough ? If  you  are  using  1K

data  blocks in YMODEM, then your receive buffer should be 1K

or more.

4.  Have you selected too high a baud rate ?  Always  start  with

the   slowest   baud   rate.   If  only one COM port is being

run, you should be able to run at 38400 baud.

5.  Are  you  attempting  to  run  another  application  in  the

background  ?  Try running without any other programs running

in the background.

6.  If  you  are  running  two COM ports simultaneously, are you

using seperate receive buffers ? ( you should )

7.  Did SioReset return a zero value ? If not, then you must call

SioReset again. See CALLPGM.C for an example.

If   CALLPGM  does  not  run,  then  either  there  is a physical

connection problem or your computer isn't as  compatible  as  you

thought!  Registered users can always call (205) 881 - 4630 after

5 PM CST Monday through Friday for help.

Page 10

Library Reference

The remainder of this manual list all the  CCL  functions.  Every

library function will return a value as follows:

1.  Negative  values  for error conditions. See last page of this

manual for a list of error values and their meanings.

2.  Non-negative values when returning data ( eg: SioLine ).

3.  Zero otherwise.

When  debugging  an   application,   be  sure  to test all return

values. Use SioError to print the associated text for errors.

/*** example code segment ***/

int code;     /* MUST be 'int', not 'char' */

code = SioFunction(); /* any CCL function */

if(code<0)

{SioError(code);

}

Page 11

SioBaud

Function

Sets the baud rate of the selected port.

Syntax

int SioBaud(Port,BaudCode)

int Port;             /* Port selected (COM1,COM2,COM3,or COM4) */

int BaudCode;         /* Baud code */

Remarks

The  SioBaud  function  sets  the baud rate without resetting the

port. It is used to change the baud rate after calling SioReset.

Baud Code        Baud Rate        CLL.H Name

0                 300            Baud300

1                 600            Baud600

2                1200            Baud1200

3                2400            Baud2400

4                4800            Baud4800

5                9600            Baud9600

6               19200            Baud19200

7               38400            Baud38400

8               57600            Baud57600

9              115200            Baud115200

Returns

-2 : Port not enabled. Must call SioReset first.

-4 : Bad port selected. Value must be 0 to 3.

-11 : Bad baud rate code. See above code values.

Example

#include "ccl.h"

/* do auto baud detect */

for(Code=0;Code<10;Code++)

{SioBaud(Port,Code);

SioPutc(Port,'A');

SioPutc(Port,'A');

if(SioGetc(Port,18)=='A'))

{puts("Baud rate detected");

...do something here...

}

}

Page 12

SioBrkKey

Function

Return non-zero if the Control-BREAK key was pressed.

Syntax

int SioBrkKey()

Remarks

The  SioBrkKey  function returns a TRUE value ( non-zero ) if the

Control-BREAK key was pressed,  else  it  returns  a  zero.   Use

SioBrkKey  as  a safety exit form a polling loop.  Don't mix this

function up with SioBrkSig.

Returns

0 : Control-BREAK was pressed.

!0 : Control-BREAK was not pressed.

Example

#include "ccl.h"

int c;

while(1)

{if(SioBrkKey())

{puts("User typed Contrl-BREAK");

SioDone(Port);

exit(1);

}

if(SioKeyPress())

{c = SioKeyRead();

SioPutc(Port,c);

}

c = SioGetc(Port,0);

if(c!=-1)  SioCrtWrite(c);

}

Page 13

SioBrkSig

Function

Asserts, cancels, or detects  BREAK signal.

Syntax

int SioBrkSig(Port,Cmd)

int Port;             /* Port selected (COM1,COM2,COM3,or COM4) */

char Cmd;             /* 'A','C', or 'D' */

Remarks

The  SioBrkSig  function controls the BREAK bit in the line status

register. The legal commands are:

'A' to assert BREAK

'C' to cancel BREAK

'D' to detect BREAK

See CALLPGM.C for an example of the use of SioBrkSig.

Returns

-2 : Port not enabled. Must call SioReset first.

-4 : Bad port selected. Value must be 0 to 3.

-6 : Illegal command. Expected 'A', 'C', or 'D'.

>0 : BREAK signal detected ( READ only )

Example

/* Assert BREAK for 1 second */

SioBrkSig(Port,'A');

SioDelay(18);

SioBrkSig(Port,'C');

/* Detect BREAK */

if(SioBrkSig(Port,'D'))

{puts("BREAK signal detected");

/* do some more stuff */

}

Page 14

SioCrtWrite

Function

Write character to the screen.

Syntax

int SioCrtWrite(ch)

char ch;              /* character to write */

Remarks

The  SioCrtWrite  function  uses  the  BIOS  to  write  a  single

character to the screen at the current cursor location.

SioCrtWrite is faster than a call to the C library and  for  this

reason is included in CCL.

Returns

zero

Example

#include "ccl.h"

if( (c = SioGetc(COM1,18)) != -1)

{/* echo to screen */

SioCrtWrite(c);

}

Page 15

SioDTR

Function

Set, clear, or read the Data Terminal Ready ( DTR ) bit.

Syntax

int SioDTR(Port,Cmd)

int Port;             /* Port selected (COM1,COM2,COM3,or COM4) */

char Cmd;             /* command */

Remarks

The  SioDTR function controls the Data Terminal Ready ( DTR ) bit

in the modem control register. Commands used are:

'S'  to set DTR ( ON )

'C'  to clear DTR ( OFF )

'R'  to read DTR

Returns

-2 : Port not enabled. Must call SioReset first.

-4 : Bad port selected. Value must be 0 to 3.

-5 : Not one of 'S', 'C', or 'R'.

0 : DTR is OFF (READ Command).

>0 : DTR is ON (READ Command).

Example

#include "ccl.h"

/* turn DTR on for modem */

SioDTR(COM4,'S');

Page 16

SioDelay

Function

Delays one or more tics.

Syntax

int SioDelay(tics)

int tics;                     /* # tics */

Remarks

The  SioDelay  function  is used to delay one or more timer tics,

where each timer tic is approximately 55 milliseconds ( 18 to the

second ). See SioTimer also.

Returns

zero

Example

#include "ccl.h"

SioDelay(5*18);               /* delay 5 seconds */

Page 17

SioDone

Function

Terminates further serial processing.

Syntax

int SioDone(Port)

int Port;             /* Port selected (COM1,COM2,COM3,or COM4) */

Remarks

The  SioDone  function  terminates  further  serial   processing.

SioDone  MUST  be  called before exiting your application so that

interrupts can be restored to their original state. Failure to do

this can crash the  operating  system.  If  you  forget  to  call

SioDone before exiting, be sure to re-boot your computer.

Returns

-2 : Port not enabled. Must call SioReset first.

-4 : Bad port selected. Value must be 0 to 3.

Example

#include "ccl.h"

/* terminate processing for COM3 */

SioDone(COM3);

Page 18

SioError

Function

Displays error in text.

Syntax

int SioError(Code)

int Code;          /* Error code returned from a CCL function */

Remarks

The   SioError  function displays the error in text corresponding

to   the  error  code.  During  development  of  a communications

application, it is a good idea to always test return  codes,  and

print out their descriptions with SioError.

Returns

none

Example

#include "ccl.h"

code = SioReset(Port,Baud4800);

if(code<0) SioError(code);

Page 19

SioGetc

Function

Reads the next character from the serial line.

Syntax

int SioGetc(Port,Tics)

int Port;             /* COM1 to COM4 */

int Tics;             /* # timer tics */

Remarks

The SioGetc function reads the selected serial port. The function

will  wait  for  the  number  of  system tics given by the 'Tics'

argument before returning 'timed out'. There are 18 tics  to  the

second.

To specify no waiting, call SioGetc with Tics = 0.

Returns

-2 : Port not enabled. Must call SioReset first.

-4 : Bad port selected. Value must be 0 to 3.

-1 : If timed out.

>0 : Character read.

Example

#include "ccl.h"

int c;   /* MUST be 'int', not 'char' !!! */

if( (c=SioGetc(COM1,1)) != -1) printf("Character is '%c'\n", c);

else puts("Timed out");

Page 20

SioKeyPress

Function

Detects if keyboard has been pressed.

Syntax

int SioKeyPress()

Remarks

The   SioKeyPress function uses the BIOS to test the keyboard for

a key press.

Returns

zero

Example

if( SioKeyPress())

{

/* do something */

}

Page 21

SioKeyRead

Function

Reads the keyboard.

Syntax

int SioKeyRead()

Remarks

The SioKeyRead function uses the BIOS to read the keyboard.  Will

wait until a character is typed.

SioKeyRead is faster than using the C library.

Returns

character typed.

Example

if(SioKeyPress())

{

c = SioKeyRead();

}

Page 22

SioLine

Function

Reads the line status register.

Syntax

int SioLine(Port)

int Port;             /* Port selected (COM1,COM2,COM3,or COM4) */

Remarks

The   SioLine  function  reads  the  line  status  register.  The

individual bit masks are as follows:

0x20  = Transmitter Buffer Empty.

0x10  = Break detected.

0x08  = Framming error.

0x04  = Parity error.

0x02  = Overrun error.

0x01  = Data ready.

The above are documented in the file CCL.H.

Returns

-2 : Port not enabled. Must call SioReset first.

-4 : Bad port selected. Value must be 0 to 3.

>0 : Line status ( rightmost byte of word ).

Example

#include "ccl.h"

int rc;

rc = LineStatus(Port);

if(rc & (FramingError|ParityError|OverrunError))

{if(rc & FramingError) puts("Framing Error");

if(rc & ParityError)  puts("Parity Error");

if(rc & OverrunError) puts("Overrun Error");

}

else puts("No error");

Page 23

SioModem

Function

Reads the modem status register.

Syntax

int SioModem(Port)

Remarks

The SioModem function   reads   the   modem   register.  The  bit

definitions are as follows:

Bit    CCL.H Name   Function

7      DCD         Data Carrier Detect

6      RI          Ring Indicator

5      DSR         Data Set Ready

4      CTS         Clear To Send

3      DeltaDCD    Delta DCD   ( DCD has changed )

2      DeltaRI     Delta RI    ( RI has changed )

1      DeltaDSR    Delta DSR   ( DSR has changed )

0      DeltaCTS    Delta CTS   ( CTS has changed )

Bits 4 through 7 represent the absolute state of their respective

RS-232 inputs. Bits 0 through 3 repesent a change in the state of

their respective RS-232 inputs since last read.

The  above definitions are also in the CCL.H file for use by your

application program.

Returns

-2 : Port not enabled. Must call SioReset first.

-4 : Bad port selected. Value must be 0 to 3.

>0 : Modem status ( rightmost byte of word ).

Example

/* any change in DCD or CTS ? */

if(delta=SioModem(Port,DeltaDCD|DeltaCTS))

{status = SioModem(Port,DCD|CTS);

if(delta&DeltaDCD) {if(status&DCD) c='T'; else c='F';

printf("DCD=%c\n",c);

}

if(delta&DeltaCTS) {if(status&CTS) c='T'; else c='F';

printf("CTS=%c\n",c);

}

}

Page 24

SioParms

Function

Sets parity, stop bits, and word length.

Syntax

int SioParms(Port,ParityCode,StopBitsCode,WordLengthCode)

int Port;             /* Port selected (COM1,COM2,COM3,or COM4) */

int ParityCode;       /* parity code [0,1,2] */

int StopBitsCode;     /* stop bits code  [0,1] */

int WordLengthCode;   /* word length code [0,1,2,3] */

Remarks

The   SioParms  function  sets  the  parity,  stop bits, and word

length.  If the default parity ( none ), stop bits ( 1 ), or word

length ( 8 ) is not acceptable,  then  they  can  be  changed  by

calling  SioParms.  SioParms can be called either before or after

calling SioReset. See file CCL.H.

Value      Description    CLL.H Name

ParityCode:   *0        no parity      NoParity

1        odd parity     OddParity

2        even parity    EvenParity

StopBitsCode:  *0       1 stop bit     OneStopBit

1       2 stop bits    TwoStopBits

WordLengthCode: 0       5 data bits    FiveDataBits

1       6 data bits    SixDataBits

2       7 data bits    SevenDataBits

*3       8 data bits    EightDataBits

* = Default

Returns

-2 : Port not enabled. Must call SioReset first.

-4 : Bad port selected. Value must be 0 to 3.

-7 : Bad parity code selected. Value must be 0 to 2.

-8 : Bad stop bits code. Value must be 0 or 1.

-9 : Bad word length code. Value must be 0 to 3.

Example

#include "ccl.h"

SioParms(COM1,NoParity,OneStopBit,SevenDataBits);

Page 25

SioPutc

Function

Transmit a character over a serial line.

Syntax

int SioPutc(Port,c)

int Port;             /* Port selected (COM1,COM2,COM3,or COM4) */

char c;               /* character to send */

Remarks

The SioPutc function transmits one character  over  the  selected

serial line.

Returns

-2 : Port not enabled. Must call SioReset first.

-4 : Bad port selected. Value must be 0 to 3.

Example

#include "ccl.h"

char crc;

char buffer[128];

crc = 0;

for(i=0;i<128;i++)

{crc = crcupdate( buffer[i], crc);

SioPutc(Port, buffer[i]);

}

SioPutc(crc);

Page 26

SioRTS

Function

Sets, clears, or reads the Request  to  Send ( RTS ) bit.

Syntax

int SioRTS(Port,Cmd)

int Port;             /* COM1 to COM4 */

char Cmd;             /* command */

Remarks

The  SioRTS  function controls the Request to Send ( RTS ) bit in

the modem control register. Commands are as follows:

'S'  set RTS ( ON )

'C'  clear RTS ( OFF )

'R'  read RTS

Returns

-2 : Port not enabled. Must call SioReset first.

-4 : Bad port selected. Value must be 0 to 3.

-5 : Command is not one of 'S', 'C', or 'R'.

0 : RTS is OFF (READ Command).

>0 : RTS is ON  (READ Command).

Example

#include "ccl.h"

/* turn on RTS for modem */

SioRTS(Port,'S');

Page 27

SioReset

Function

Initialize a serial port for processing.

Syntax

int SioReset(Port,BaudCode)

int Port;             /* Port selected (COM1,COM2,COM3,or COM4) */

int BaudCode;         /* baud code */

Remarks

The  SioReset  function  initializes  the  selected  serial port.

SioReset should be called after calling SioParm and SioRxBuf  but

before  making  any other calls to CCL. SioReset uses the parity,

stop bits, and word length value previously set  if  SioParm  was

called, otherwise the default values ( see SioParm ) are used.

Recall that COM1 and COM3 share the  same  interrupt  vector  and

therefore  cannot  operate  simultaneously.  Similiarly, COM2 and

COM4 cannot operate simultaneously. Any other combination of  two

ports can be used.

See SioBaud for a list of the baud rate codes, or see "CCL.H".

Returns

-4 : Bad port selected. Value must be 0 to 3.

-11 : Bad baud rate code selected. Value must be 0 to 9.

Example

#include "ccl.h"

char far Buffer[128];

int rc;

SioRxBuf(COM1,Buffer,Size128);

rc = SioReset(Com1,Baud38400);

if(rc==0) puts("RESET ok");

else if(rc<0) SioError(rc);

else

{if(rc&OverrunError) puts("Overrun Error");

if(rc&ParityError) puts("Parity Error");

if(rc&FramingError) puts("Framing Error");

if(rc&BreakDetected) puts("Break Detected");

}

Page 28

SioRxBuf

Function

Sets up receive buffers.

Syntax

int  SioRxBuf(Port,Buffer,SizeCode)

int Port;             /* Port selected (COM1,COM2,COM3,or COM4) */

char far *Buffer;     /* Receive buffer */

int  SizeCode;        /* Buffer size code */

Remarks

The  SioRxBuf function passes the address and size of the receive

buffer to CCL. Recall that CCL requires a receive buffer for each

port in simultaneous operation  since  the  receive  function  is

interrupt  driven.  SioRxBuf passes the receive buffer to CCL for

both the primary ( COM1/COM3 ) and secondary ( COM2/COM4 ) ports.

It must be called before any incoming characters can be received.

SioRxBuf should be called before SioReset. Buffer size codes  are

listed in "CCL.H".

Size Code       Buffer Size    CLL.H Name

0              8 bytes        Size8

1             16 bytes        Size16

2             32 bytes        Size32

3             64 bytes        Size64

4            128 bytes        Size128

5            256 bytes        Size256

6            512 bytes        Size512

7           1024 bytes        Size1024

8           2048 bytes        Size2048

9           4096 bytes        Size4096

10           8192 bytes        Size8192

11          16384 bytes        Size16384

Returns

-2 : Port not enabled. Must call SioReset first.

-4 : Bad port selected. Value must be 0 to 3.

-10 : Bad buffer size code. Value must be between 0 and 11.

Example

#include "ccl.h"

char far RxBuf[128];

SioRxBuf( COM1, RxBuf, 128);

Page 29

SioRxFlush

Function

To flush the receive buffer associated with the specified port.

Syntax

int SioRxFlush(Port)

int Port;             /* Port selected (COM1,COM2,COM3,or COM4) */

Remarks

The  SioRxFlush  function  will  delete  any  characters  in  the

receive  buffer  for  the  specified  port.  After execution, the

receive  buffer  will  be empty. Call SioRxBuf after resetting  a

port in order to delete any spurious characters.

Returns

-2 : Port not enabled. Must call SioReset first.

-4 : Bad port selected. Value must be 0 to 3.

Example

#include "ccl.h"

char far buffer[1024];

/* setup receive buffer & reset port */

SioRxBuf(COM1,buffer,Size1024);

SioReset(COM1,Baud115200);

/* flush any spurious character */

SioRxFlush(COM4);

/* ready for serial processing ! */

Page 30

SioRxQue

Function

Returns the number of characters in the receive queue.

Syntax

int SioRxQue(Port)

int Port;             /* Port selected (COM1,COM2,COM3,or COM4) */

Remarks

The SioRxQue function will return the  number  of  characters  in

receive queue. It can be used to implement XON/XOFF flow control.

Returns

-2 : Port not enabled. Must call SioReset first.

-4 : Bad port selected. Value must be 0 to 3.

Example

#include "ccl.h"

#define XON 0x11

#define XOFF 0x13

int count;

char last = XON;

char far Buffer[128];

SioRxBuf(COM1,Buffer,Size128);

/* implement XON / XOFF */

count = SioRxQue(COM1);

if((last==XON)&&(count>120))

{SioPutc(COM1,XOFF);

last = XOFF;

}

if((last==XOFF)&&(count<8))

{SioPutc(COM1,XON);

last = XON;

}

Page 31

SioTimer

Function

Returns    the    number   of   system clock tics since midnight.

Syntax

long SioTimer()

Remarks

The SioTimer function will return the number of system clock tics

since midnight, at 18.2065 tics  per  second.  This  function  is

usefull for timeing various functions. Also see SioDelay.

Returns

timer tics

Example

#include "ccl.h"

long time;

time = SioTimer();

/* do some stuff */

printf("Elasped time = %ld tics\n", SioTimer() - time );

Page 32

Error Code Summary

Code        Description

0          No error.

-1          Timeout waiting for input. Only returned by SioGetc.

-2          Port not enabled. Must call SioReset first.

-3          No buffer available. Must call SioRxBuf before

calling SioReset.

-4          Bad port specified. Must be 0 to 3. Recall that COM1

is port 0, COM2 is port 1, etc.

-5          Expected 'S', 'C', or 'R' as second argument.

-6          Expected 'A', 'C', or 'D' as second argument.

-7          Bad parity code specified. Must be 0 to 7.

-8          Bad stop bits code specified. Must be 0 or 1.

-9          Bad wordlength code specified. Must be 0 to 3.

-10          Bad buffer size code specified. Must be 0 to 11.

-11          Bad baud rate code. Must be 0 to 9.

Page 33