Dirt Cheap Frame Grabber V2.02

by Michael Day

as of 5 January 1992

Public Domain

This device is free and released to the public domain.

If you actually find a use for it, all the more power to ya!

The purpose of the DCFG is to simply provide a very simple method

to grab video pictures and display them on a computer monitor.

Version 1.01 of the DCFG only provided 4 levels of gray scale.

Version 2.01 makes a modification which allows for 12 gray scales

by reading the port four times and interpolating the results.

Version 2.02 fixes a schematic error (DO3 is pin 5, not pin 4).

The DCFG requires at least a 80286 or better and a VGA

display to run. While it is possible to change the program to

allow operation on a 8086 and/or a different display, I'm not

doing it. If you want to, you can. My intent was to just prove

that it was possible.

The frame grabber program at this point is very simple, and as a

result has some problems. The heart of the program relies on a

very fast IO port to memory instruction. It can operate as fast

as the computer can shove data from the port to the memory. This

is as fast as DMAing the data could do.  This method is sensitive

to computer speed. As such, you may not get a stable picture at

first. If you don't, use the left or right arrow keys to adjust

the horizontal sample size. The up/down arrows adjust the

vertical sample size, but this normally doesn't need to be done.

You will probably notice some sparkles (pixels shifting left and

right one pixel position). This is caused by sample jitter and is

normal. All sampling systems have this problem. Some less than

others. Even your TV has the problem, but it is normally good

enough that you don't see it.

The jitter is caused by the fact that the sampler is not

perfectly synchronized to the incoming video signal. Your TV has

a lot of circuitry in it to achieve the synchronization. Being

that this is a dirt cheap frame grabber, there is only limited

synchronization of the picture, and no synchronization of the

sampler. If you want to try to improve on it, please do.

To use the Dirt Cheap Frame Grabber, plug it into a PC printer

port, and attach the other end to the video output from a VCR or

TV Monitor and run the program. Use the left or right arrow keys

to adjust the horizontal sample size to obtain a stable picture.

Do not attach the Dirt Cheap Frame Grabber to the computer with a

long printer cable. We are dealing with high speed signals here

(up to 4MHz). It requires as short a connection to the computer

as you can give it. I built my circuit inside the back shell of

the connector that plugs into the printer port on the computer.

Dirt Cheap Frame Grabber

(DB25P)

DO0 ---330ohm-------------*--->|--*-->|---*    IC1 = LM339

pin 2                  Vr |  red    1N914 |          quad

(2.4v) |  led          |        comparator

DO1 ---330ohm-------------*               |

pin 3                     |               |    * = connection

330ohm           |

1N914               |               |    ) = no connect

DO6 ---|<---*---1Kohm-----* Vc (2.21v)    |       (jump over)

pin 8                     |               |

1N914               |               | Vr = ref voltage

DO7 ---|<---*---510ohm----*------*        |      2.4 volts

pin 9                     |      |        |

(trimmer) 5Kohm<--*        | Vc = 00 = 1.65v

|               |      01 = 1.80v

DO2 ----------*-----------)------| |------*      10 = 1.95v

pin 4         |           |     .1uf      |      11 = 2.21v

3|           |               |

DC3           |/ |6       |               | Vz = 00 = 0.74v

(slct)     1 /  -|--------* Vz (1.0v)     |      01 = 0.81v

DS6 --------|IC1a|        |               |      10 = 0.88v

(ack)        \  +|----*   |               |      11 = 1.00v

pins 10,17     \ |7   |  360ohm           |

|   |               | Vy = 00 = 0.45v

DC2            / |4   |   |               |      01 = 0.54v

(init)     2 /  -|----)---* Vy (0.66v)    |      10 = 0.59v

DS5 --------|IC1b|    |   |               |      11 = 0.66v

(pe)         \  +|----*   |               |

pins 12,16     \ |5   |  220ohm           | Vx = 00 = 0.34v

|   |               |      01 = 0.37v

DC1            / |10  |   |               |      10 = 0.40v

(afd)     13 /  -|----)---* Vx (0.45v)    |      11 = 0.45v

DS4 --------|IC1c|    |   |               |

(slct)       \  +|----*   |               | Vs = 00 = 0.086v

pins 13,14     \ |11  |  360ohm           |      01 = 0.093v

|   |               |      10 = 0.101v

DC0            / |8   |   |               |      11 = 0.115v

(stb)     14 /  -|----)---* Vs (0.12v)    |  Assumes Vz set

DS7 --------|IC1d|    |   |               |  for 1.0v using

(busy)       \  +|----*   |               |  5Kohm trimmer

pins 11,1   12|\ |9   |  120ohm           |  with DO6, DO7

|       |   |               |  set high.

Gnd ----------*-------)---*---------------*------*--------*

pins                  |   |               |      |        |

18,19,20,21,          |  100Kohm          |     75ohm     |

22,23,24,25           |   |      .1uf     |      |        |

*---*-------| |-----)------*------o )

2N2222 |                   |             RCA

*---------C   E                   |            phono

|           B                     |            jack

pin 5   |           |      1N914          |

DO3 ----*--4.7Kohm--*-------->|-----------*

�-----------------------------------------------------------------

Parts List

1ea  DB25P connector

1ea  RCA phono jack

1ea  5K ohm trimmer pot

1ea  510 ohm resistor

1ea  1K ohm resistor

2ea  330 ohm resistor

2ea  220 ohm resistor

2ea  360 ohm resistor

1ea  120 ohm resistor

1ea  100K ohm resistor

1ea  75 ohm resistor

1ea  4.7K ohm resistor

2ea  0.1uF capacitor

1ea  red LED

4ea  1N914 diode (or 1N4148)

1ea  2N2222 transistor (NPN)

1ea  LM339 quad comparator

-----------------------------------------------------------------

Dirt Cheap Frame Grabber

Circuit Description

It is assumed that the video signal will be a NTSC type signal

complying to the RS170A video standard with negative going sync

and a signal level between 0.5Vpp to 2Vpp (1Vpp desired). This is

the typical signal that is available on the video output jack of

a VCR or TV monitor.

If you are using a VCR type video signal, you can get a quick

start by adjusting the bias trimmer (5Kohm pot) to obtain

1.0volt at location Vz while insuring that the data output lines

on the printer port are set to 0FFH. Once you get going you can

fine tune the reference voltage by adjusting the trimmer to

obtain the best picture.

The video input comes in on the RCA phono jack. The 75ohm

resistor provides the proper load for the connection. The .1mf

capacitor isolates the signal from the rest of the circuit and is

used as a part of the level shifter to clamp the video input to

ground reference on the computer. The 1N914 diode and 4.7K ohm

resistor provides a voltage reference 0.7V above ground for the

2N2222 clamp transistor. The transistor and diode get their

supply voltage from the DO3 output pin. Typically this is around

3.3volts and should have enough power for our needs assuming that

the output driver is a 74LS374 driver which is the standard part

for a PC printer port.

�The transistor provides a low impedance charge path for the

capacitor to clamp the sync tip to ground so as to insure the

video signal sync tip is referenced to ground. The 100K ohm

resistor provides a high resistance discharge path for the

capacitor. Enough to provide a discharge path for the capacitor,

but not enough to seriously affect the video signal level.

The clamped video signal is then applied to all four comparators

in the LM339. The LM339 gets its power from the DO2 output pin

Like the clamp transistor, normally there should be enough power

from the data output pin to supply the IC. A .1mf capacitor is

used to stabilize the power source. You can probably getaway

without using the capacitor, but I'm paranoid.

The four comparators receive their reference voltage from a

resistor ladder consisting of 360ohm, 220ohm, and 360ohm

resistors in series with the 120ohm resistor tied to the common

ground. The voltage level on the 120ohm resistor provides the

sync detector reference.  The other three resistor positions

provide three increasing levels of amplitude detection.

The resistor ladder voltages can be shifted up or down by the

computer using the DO6 and DO7 data output lines on the printer

port. This allows 12 gray scale levels to be detected instead of

the basic 4 levels that would otherwise be available.

The resistor ladder requires a regulated source voltage with an

adjustment to allow compensation for various input signal levels

depending on the signal source. The 5Kohm trimmer pot provides

for the bias adjustment which allows the circuit to be adapted to

the absolute signal source level. The regulated source voltage is

derived from the red LED in series with a 1N914 diode. A red LED

will have a typical voltage drop of about 1.7 volts and the diode

has a typical voltage drop of 0.7 volts giving a total of 2.4

volts for the reference.

The absolute level is not important since the trimmer can

compensate for minor differences. The LED gets its power through

the two 330ohm resistors attached to the D0 and D1 outputs. Two

outputs are used here to insure that there is enough power to

drive the LED and the resistor ladder while maintaining a

regulated voltage source across the LED.

The LED has a 1N914 diode in series with it to get the reference

voltage in the range that we need it. This is the one area where

I go beyond spec for the system. The 74LS374 driver is specified

to have an output voltage of greater than 2.5volts at 2.5ma. I'm

actually relying on the true output voltage for the part which is

typically 3.3volts. Should your printer port use a different part

than the 74LS374, the circuit may or may not work properly. A

74HTC374 cmos type part will pull up to 5volts. This is not a

problem, since all we need is 3 volts or better source voltage to

supply the circuit. If the output device supplies less voltage,

the circuit will not work because the LED and diode will not be

able to maintain the required 2.4volt reference source.�

The voltage reference is supplied through a 220ohm resistor to

the bias adjustment trimmer pot. The 220ohm resistor is a load

source for the 1Kohm and 510ohm gray level shift control

resistors attached to the DO6 and DO7 data output pins. The gray

level shift control lines change the reference voltage on the

resistor ladder to allow the computer to adjust the reference

voltage to four different levels. This allows us to multiply the

gray level detection by four by reading in four sequential video

frames at the four possible reference control levels and

selecting the highest determined video level from the four frames

that are collected. This gives us a total of 12 gray levels

instead of the four levels that would otherwise be available.

The down side to this is that we have to collect four separate

video frames to do this which means that motion will be blurred.

The comparators receive their pull-ups from the control lines DC0,

DC1, DC2, and DC3. In a standard printer port these lines are

open collector outputs with 4.7Kohm pull up resistors. It is

presumed that this is the case for this circuit. It saves us the

four pull up resistors. If you feel nervous about this, you can

attach 4.7Kohm pull up resistors to the comparator outputs and

attach two each to the remaining DO4 and DO5 output pins (pins 6

and 7). Note that you should not draw more than 2.5ma from any

data output pin or the output voltage may sag.

When wiring the circuit, remember that you are working with video

signals. That means you are working with signals in the RF range

(up to 4MHz). Keep all wiring as short as possible and the

components close together. All grounds should be tied to one

single point at the 25pin connector. The circuit must be built as

close as possible to the DB25P connector as possible. Do not

attach this device to the computer with a long cable. The DB25P

connector must plug directly into the computer to prevent signal

loss. Any attaching cable should be no more than six inches

long. If possible, I suggest building the circuit into the back

shell of the connector so that there is no cable involved.

Remember that we are dealing with high speed signals here.

Long cables don't work. Failure to keep the connection short will

cause the sync signal not to be detected and the picture to blur

or smear. The video cable attached to the RCA jack can be any

length (within reason) since it is not as sensitive to cable

length problems. Video cable lengths longer than 20 feet

may cause problems, I haven't tried them that long.

If you are good at this stuff, it is possible to wire the whole

circuit inside the connector shell (don't use a shielded shell,

it will short out the circuitry). It's a very tight fit, but if

you pick a large shell, and are very careful, it can be all

stuffed in there. The trimmer pot should be made available for

easy adjustment (I drilled a hole in the shell for access).

Also, the led should be visible (another hole to drill) to be

able to quickly determine if the circuit is working.

�To make it all work, simply plug the connector into a printer

port on the PC, and run the frame grabber program. When running

the program, the LED should turn on. That indicates that the

circuit has enough power to operate. If the LED does not come on,

or it is very dim, there may not be enough power, and you can't

use that printer port with the circuit. Try another port or a

different computer. Note that you may see some minor flickering

on the led when changing the DO6 and DO7 data output lines. This

is normal and indicates that the circuit is working properly.

Changing the DO6 and DO7 data output lines causes the current

flow through the voltage reference ladder to change. This means

that more or less current will be available to the LED, causing

it to change in brightness. The LED should not go out completely

nor become very dim. That would indicate that there is not enough

power available to drive the voltage reference or a problem in

your program.

You can operate in the simple mode by outputting a 0FFH to the

data output port which sets all the output lines high. This

provides power to the comparators, voltage reference, and clamp

transistor. It also disables the gray scale control lines.

To shift the gray scale reference voltage level, send either a

03FH (gray ref 0), 07FH (gray ref 1), 0BFH (gray ref 2), or 0FFH

(gray ref 3) to the data output port.

The resulting detected output from the port is as follows:

76543210

1000xxxx  sync

0000xxxx  level 0 (black)

0001xxxx  level 1 (dark gray)

0011xxxx  level 2 (gray)

0111xxxx  level 3 (white)

Note that bits 0-3 are unknown and should not be relied on as

being valid.

-----------------------------------------------------------------

Program Description

Thi� progra� require� � 28� o� bette� computer� � printe� por� �

wit� th� attache� devic� describe� abov� wit� � vide� signa� �

drivin� it� An� � MCG� o� VG� typ� displa� system� Yo� star� th� �

progra� u� b� enterin� it� nam� a� th� DO� promp� followe� b� th� �

printe� por� numbe� yo� wis� t� use�

Examples� DCFG� 1    <-� load� progra� fo� us� wit� LPT1.

DCFG� 2    <-� load� progra� fo� us� wit� LPT2.

I� yo� don'� giv� i� � number� th� progra� wil� defaul� t� LPT1�

�

Grabbing a Frame:

The program works by grabbing samples from the printer port in

frames. Once each time through the loop, it sucks in a video

frame from the video input.

Actually, more than a frame is grabbed. How much is grabbed

depends on the speed of access to the printer port on your

computer. On a typical computer this is 1.5us per byte. At the

1.5us access rate, 63.5us per scan line, and 265.5 scan lines per

video frame, that results in 11240 bytes per frame required to be

collected. The collection size is currently set to 20,000 bytes.

That provides enough space to handle buss speeds up to 12MHz which

should be adequate for most computers.

The down side is that we collect 210 extra scan lines on a normal

computer. In most situations this isn't a problem since three to

four frames will typically be lost while the collected video data

is being processed anyway. The extra partial frame has only a

slight effect on the over all collection time and results. The

effect is minimized on a fast machine since nearly the entire

array is filled anyway.

The maximum sample size allowed is 65520. More than that and we

run out of segment space in memory. Also during this period I

keep the system interrupts off so as to not have distortions of

the collected data caused by the system deciding to go off and do

something else.

In addition to the collection time, there is a pre-configure time

while the loop is waiting for the vertical sync pulse to come in.

I don't collect any data prior to the vertical sync pulse since

it would be ignored anyway.

The vertical sync is determined by watching the sync line. When

it goes on for longer than 10us, you are in the vertical sync

time. Less then 10us, and it is a horizontal sync pulse.

Once the video data is collected, I release the interrupts to

allow the computer to return to normal operation. Should no sync

come in during the vertical sync detection period, I terminate

the loop after a while to allow the computer to continue to

operate, and return an error condition so that the program will

know that no data was collected.

If you have an O'scope, you can verify the video collection time

by observing data output line DO7 (pin 9). This line should never

go high for longer than 50ms. Typically it should be around 20 to

40ms in length. This should be the same whether you have a signal

going into the video line or not. Though there may be a

difference between the timing when a signal is present verses

when it is not present.

�Video Data Translation:

Once I get the data, I convert it to in intermediate form to be

stored in the interpretation array. This array keeps the gray

level values collected on the previous frames. The display

routine will later interpret the values stored in the

interpretation array into an absolute gray scale level for each

display pixel.

The video data is converted by searching for the horizontal sync

pulse. Once found, we search for the end of the sync pulse.

Following the end of the sync pulse is the actual video data.

Each collected byte in the scan line is read and converted to the

intermediate gray scale level.  There are only four levels

involved; 00, 01, 10, and 11. Note that since sync is not video

data, it is not included in the gray scale level. Even if it were

included, it would be a 00 value. Once converted, the byte is

then shifted into it's proper frame position in the interp array

byte and stored in the array.

This is repeated until either the full width of the

interpretation array is filled, or a new sync pulse is

encountered. Should a sync pulse be encountered, the rest of the

interpretation array values are filled with 00. Should the full

width of the interpretation array be encountered, then the rest

of the scan line is discarded until an new sync pulse is

encountered. This sequence is continued until the entire frame

has been converted.

Displaying the Data:

Once the frame has been converted, the interpretation array is

then displayed. Each byte is read from the interpretation array

and translated to an absolute gray level using the translation

array located in the first 256 bytes of the interpretation array.

There are 12 levels of gray available, plus black. The

translation array was copied into the interpretation array at the

time the interpretation array was created.

�The translation array is formed from the following gray level

conversion chart:

gray level interpretation chart

frame data

gray    F3  F2  F1  F0   F3 = frame 3, F2 = frame 2

level:  76  54  32  10   F1 = frame 1, F0 = frame 0

12:  11  xx  xx  xx   each group of two bits

11: <11  11  xx  xx   represent the video level

10: <11 <11  11  xx   for the frame indicated

9: <11 <11 <11  11

8:  10 <11 <11 <11   xx = any bit pattern

7: <10  10 <11 <11   <11 = less than 11; (10, 01, 00)

6: <10 <10  10 <11   <10 = less than 10; (01 or 00)

5: <10 <10 <10  10   11, 10, 01, or 00 = the indicated

4:  01 <10 <10 <10                absolute bit pattern

3:  00  01 <10 <10

2:  00  00  01 <10   the gray level for the specified

1:  00  00  00  01   bit pattern is shown at the left

0:  00  00  00  00

There are no sync bytes in the interpretation array since they

were stripped out during the process of converting the video data

to the interpretation data. Note that it takes four video frames

to completely update the interpretation array. However, the

actual displayed data will reflect the highest gray level

determined from the four previous reads. This means that if the

video level shifts towards the white level, the change on the

display will be immediate. If the video level moves towards the

black level, the change on the display will take up to four frame

grabs to reflect the lower video level for that pixel.

One big problem is that since the number of scan lines collected

is much greater than the resolution of pixels collected in each

scan line, the aspect ratio of the picture would be out of whack

if we were to display the data pixel for pixel as we received it.

In order to get the aspect ratio back to a reasonable value, we

must throw data away. We do this by discarding three scan lines

for every one that we show. If you have a very fast buss, you may

want to change this to discard two scan lines instead of three

in order to get a more reasonable aspect ratio.

The other thing I do is to pull a little trick to improve the

horizontal resolution. Normally there is a typical horizontal

resolution of about 40 pixels. This is not very much, and it

would be useful if there were a better resolution. There is a

halfway solution.

What I do is display the even pixels on a scan line from the scan

line that I'm currently processing. I then display the odd pixels

from the next scan line on the succeeding video frame. What this

does is to provide a random selection on the odd pixels between

the normal two sequential scanned pixels that effectively

improves the display resolution by about 50%.

�The downside is that it worsens the jitter/twinkle problem since

the odd pixel is developed from a pseudo-random sampling

mechanism which is not consistent.

One thing to keep in mind is that the frame grab routine normally

takes between one and three video frames to collect data, and

during this time the interrupts are disabled. What that means is

that if you are operating on a computer which has a clock

interrupt faster than the normal 55ms, you will lose time, and

may experience problems.

Also note that since the interrupts are disabled, anything that

uses high speed interrupts during the time the video data is

being collected (such as RS232), will not work. The interrupts

will be lost.

The main factor that affects speed on slower machines are the

conversion and display routines. The INS instruction used to

collect the video data is relatively cpu performance independent.

The main thing that affects its operation is the buss performance

of the computer. The faster the buss, the more video pixels that

can be collected per scan line.

One possible consideration on slower machines would be to discard

the extra scan lines in the Video to interpretation array

conversion rather than in the interpretation to display routine.

That way the extra scan lines would not even be processed, thus

increasing the speed of the overall process loop.

---

Future considerations for someone to do if they wish to improve

on the design would be to improve the synchronization to reduce

the jitter problem. At this point I don't have any ideas on how

to improve the problem.

Another improvement that is possible to do would be to analyze the

captured video frame to determine the proper values to use for

capturing the data such as scan line length in pixels, vertical

sync length, total scan lines, etc.

A mechanism to increase the size of the displayed picture, and

maintain the best resolution possible while doing so.

Save an image to disk or a series of images to make a movie.

Be able to print a captured image.

Integrate the image with sound.

And of course integrating this stuff into Windows would be nice.

That's all folks!

<eof>�

#############################################################################