SBKEdit Version 1.0                                     02/06/92

FM Sound Card Timbre Editor

SBKEdit is a Timbre editor - it lets you create and modify instrument

sounds for the Sound Blaster and Adlib FM Cards.

This document is going to be rather brief:  I'm assuming that you know

how to operate and move around within a window based user interface.

I'll just mention things which might not be readily apparent.

Starting SBKEdit

Unzip all the files to a directory (You might want to create another

directory for your IBK bank files) and type: SBKEdit to load the

program.

Opening Files

Locate IBK files via the File/Open... menu.  You can double click on

one to select and open it (for those without a rodent, you can use

the cursor keys - then press enter).  The program prevents you from

opening the same file twice - this is to prevent loss of data involved

when modifying two copies of the same file.  You can open a copy of

the same file, by first saving the file with a different name

(using File/Save as...) and then reopening the original file.

Locating Files

You can move to a different current directory using the File/Change

Dir Menu selection, or you can navigate from the File/Open menu

(double clicking on the "\.." item).  New IBK file can be created

by using the File/New option.  When you do this, all the names are

blank, and a default Piano instrument is stored in each Timbre slot.

Saving Files

You can save an IBK file with either the name you opened it with

(File/Save) or to a new file name (File/Save as...)

Selecting Timbres

You can open multiple IBK files at the same time.  The current window

and the current timbre are highlighted.  Each IBK window has an Edit

and a Copy Button.  Pressing Edit brings up the Timbre window

containing the current timbre.

Copying Timbres

Timbres may be copied from one IBK to another, or within a single IBK.

The copy dialog box contains a list of open IBK windows.  After

selecting one of the IBK's from the list, you can select which position

to copy the current timbre to.  The number displayed in the dialog box

is the current timbre position in the target IBK.  This means you can

first select the target position, and then select the timbre to

actually copy (or select Cancel if you change your mind).

Editing Timbres

You get here by selecting edit from an IBK window.  From this dialog

box, you can change all the different parameters that comprise an FM

timbre. All the numbered fields have Spin controls: clicking on the up

arrow increases the value - the down arrow decreases.  You may also use

the + and - keys to achieve the same effect.  A tone may be auditioned

by pressing the Note or Chord buttons (the single letter short-cut keys

work on all fields except Timbre Name - for this field you must use Alt-N

or Alt-h as shortcut keys).  You can rename the timbre by entering a

different name into this field, but it will replace the original timbre

when you press Ok.  You may use any characters to name the Timbre, but

imbedding spaces and other illegal DOS filename characters will prevent

you from extracting INS and SBI files (Using SBANK or some other

extraction utility. To abandon changes press Cancel.

Configure

The default setting of the I/O Port should work on all Adlib, Sound

Blaster, Sound Blaster Pro, and any other compatible card.  This is

because all Adlib compatible cards are addressable at the 380 hex

I/O Port setting, regardless of the hardware setting on the card.

Note that the Port is given as 380 hex (NOT 388 hex).  All the FM

Chips begin at an xx8 address, but the Sound Blaster register set

begins on a xx0 (220, 240 ...).  All this is just to say: put an xx0

number in this field unless you have some oddball compatible scheme.

The program will determine that it can not find a card at startup, but

will not alert you to that fact upon changing the I/O Port address:

you won't hear any sound though.  You can verify this by opening the

About box on the system menu - if a card is located, a brief chordal

sequence is sounded.

The program supports 43/50 line modes on EGA and VGA graphics cards.

The higher resolution allows a much less cluttered view when many IBK

files are open at once (I personally like it best).  The current

settings can be saved to an SBKEdit.INI file by selecting Config/Save

Setup.  This file may be edited by hand, but is overwritten by

selecting the Save option.  The INI file is written in the same

directory as SBKEdit.EXE.

The amount of remaining memory is displayed in the About box - you

may want to monitor this if you have very many (10 or more) IBK files

open at once.

Parameters

Final Notes

To use your IBK files with Twelve Tone's Cakewalk Sequencer, copy

your IBK file to a file named ADLIB.IBK in the Cakewalk program

directory.  It will be automatically loaded by the FM card IPD

when Cakewalk starts up.

I'll try to expand on this when I find some time.  I've been really

busy just getting to this point...I hope you enjoy it.  Let me know

how it works out.