CDRILL

Version 3.2

May, 1994

Copyright (c) 1994

By:  Richard Zakas

WHAT IS IT ?

------------

CDRILL is a utility program that provides machinists and CNC

programmers an easy method to compute the depth to move a center

drill to produce a center of a given diameter.

LIMIT OF LIABILITY

CDRILL is distributed as-is.  The author disclaims all

warranties, expressed or implied.  The author will assume no

liability for damages either from the direct use of this product or

as a consequence of the use of this product.  This offer is a limited

time offer and pre-empts any previous offer.

SHAREWARE

This is a shareware version of CDRILL, and it is fully functional.

You are encouraged to try this SHAREWARE version, and if you find it

of value, to register your copy.  You are granted a limited license

to use this shareware version for a "reasonable" period of time.  The

registration fee is $12.50 U.S., and $17.50 outside the U.S.   This

registration fee entitles you to use CDRILL beyond a trial period.

Registered users will also be able to upgrade their copy of CDRILL,

by removing the registration screen.  When sending in your registration,

please mention the version you are registering, where you obtained

CDRILL, and the registration fee.  Any suggestions or comments are

always appreciated, so if you have something you want included or

changed in the program, please write.

Checks can be made to:

Richard Zakas

4205K Bland Rd.

Raleigh,  NC, 27609

USING CDRILL

CDRILL is run from the DOS command line.   Type CDRILL and then

press <ENTER>.  When the main screen comes up you will see a chart

of various center drill sizes and the center sizes each one

is capable of producing.  For example, if you needed to produce a

.330 dia center, looking at the chart you can use a # 5, 6, 7, or

8 center drill.  You pick  6           So.......

Enter Dia. of Center needed:  .330 <ENTER>

Enter Center Drill to use:  6 <ENTER>

and then the program calculates the drill depth needed to

produce a .330 center using a #6 center drill.

Drill Depth is : .37824

DISTRIBUTION

You to make as many copies as you wish for distribution

(in its unaltered original form, including all documentation) to others

on the same evaluation basis.