���������������������������������������������������������Ŀ

�       ASCII Documentation File for ... CPSTAT.EXE       �

�                       Version 9312                      �

���������������������������������������������������������Ĵ

�             A MS-DOS Software System for...             �

�                                                         �

�          ***  Taguchi Capability Indices  ***           �

�                                                         �

�     Capability Calculations, Bounds and Simulations     �

���������������������������������������������������������Ĵ

�            A Quality Assurance Training Tool:           �

�        Statistics Committee of the QA Section of        �

�      the Pharmaceutical Manufacturers Association       �

�                                                         �

�       Bob Obenchain, CompuServe User [72007,467]        �

�      5261 Woodfield Drive North, Carmel, IN 46033       �

�����������������������������������������������������������

Table-of-Contents

Introduction to Taguchi-like Capability Indices. . . . . . . . . . . 1

Capability Indices and Process Yield . . . . . . . . . . . . . . . . 4

USAGE of CPstat. . . . . . . . . . . . . . . . . . . . . . . . . . . 5

CPstat Input/Output File Naming Conventions. . . . . . . . . . . . . 6

Example CPstat Walk-Through. . . . . . . . . . . . . . . . . . . . . 6

Preserving Time-Series Order . . . . . . . . . . . . . . . . . . . .15

REFERENCES . . . . . . . . . . . . . . . . . . . . . . . . . . . . .15

CPstat Software Update History . . . . . . . . . . . . . . . . . . .17

�����������������Ŀ

�   INTRODUCTION  �

�������������������

Taguchi-like process capability indices, Cp , Cpk  and Cpm, are well

known summary measures of "process potential and performance,"

Boyles(1991).  Montgomery(1985,1991) and Kane(1986) are highly readable

sources of information about the Cp and Cpk forms of these indices;

these two reference works have apparently contributed to the current

widespread popularity of these indices in America.

The capability index introduced in Taguchi and Wu(1979) [and illustrated

in both Hsiang and Taguchi(1985) and Taguchi, Elsayed, and Hsiang(1989)]

was reinvented by Chan, Cheng, and Spiring(1988) and renamed "Cpm."  The

general formula for Cpm is:

Upper Specification Limit - Lower Specification Limit

Cpm  = -----------------------------------------------------

6 * Root Mean Square Deviation from Target

CPstat [Version 9312] . . . . . . . . . . . . . . . Page 2 of 17

Taguchi's early writings about Cpm (1979,1985) may have been widely

misinterpreted by American readers because he used the Greek symbol �

(lower case sigma) to denote what he called the "standard deviation

from the target."  In contrast, standard American statistical

terminology and notation is as follows...

Mean Value = the expected value of a random variable.  (The mean of a

probability  distribution is commonly denoted by �.)

= the arithmetic average of observed values.  (The mean of

a sample of "X" values is commonly called "Xbar".)

Variance = Mean Value of a Squared Deviation from the Mean

= Expected Value of ( Random Value  - � )�

= Average  Value of ( Observed X - Xbar )�

= ��  ...in standard American statistical notation.

Standard

Deviation = Square Root of the Variance

= �   ...in standard American statistical notation.

MSD = Mean Squared Deviation from a Target

= Mean Value of a Squared Deviation from a Target

= Expected Value of ( Random Value  - Target )�

= Average  Value of ( Observed X    - Target )�

There is a fundamental equation relating the above statistics:

MSD   =    ��  +   ( Mean - Target )�.

�����������������������������������������������������������������Ŀ

�  Taguchi's "standard deviation from the target" is the square   �

�  root of the MSD.  This value will coincide with the usual      �

�  standard deviation (from the mean) ONLY when the process is    �

�  exactly on target in mean value, i.e. NO long-term bias exists.�

�������������������������������������������������������������������

The two seemingly "closely related" capability indices that are

most commonly used in America today are...

Upper Specification Limit - Lower Specification Limit

Cp   = -----------------------------------------------------

6 * Standard Deviation

and

��                                           Ŀ

�   Upper Spec - Mean     Mean - Lower Spec   �

Cpk  =  Min �   �����������������  ,  �����������������   �

�     3 x Std. Dev.          3 x Std. Dev.    �

��                                           ��

CPstat [Version 9312] . . . . . . . . . . . . . . . Page 3 of 17

ANSI/ASQC A1-1987, states on page 16 that "standard measures of

process capability have not achieved consensus at this time."  This

publication also states on page 17 that "PCI = (UTI-LTI)/6� is

sometimes called Cp."

Boyles(1991) provides a highly readable discussion of differences

between Cp, Cpk, and Cpm.  In my opinion, he describes several HIGHLY

CONVINCING ADVANTAGES of the Cpm index over both Cp and Cpk.

Of course, if a process is "centered" exactly correctly, then its

mean will fall right at the intended target value; the process

can then be said to be "on-target-on-average."  And Cpm = Cp in

this case because the two definitions for the � symbol would then

coincide.  But this is a highly specialized sort of situation.

The numerical value of Cpm cannot exceed that of Cp because of

possible bias in the process away from its intended target.

Boyles also does a particularly good job of dispelling the common

misconception that Cpk measures process "centering."

Taguchi and Wu(1979, page 16) warn against attempts to artificially

inflate Cpm  by simply widening tolerance limits.  And Taguchi,

Elsayed, and Hsiang(1989),  page 12, say "we introduce a monetary

evaluation of the quality of products, assuming tolerances are

correct."  If the "Yield" of a process is defined to be the percentage

of product that falls within the stated tolerance limits, then

expected yield "usually" increases as Cpm increases.  However, as

shown by Boyles(1991), this property is guaranteed only when the

target value is "near" the midpoint between the upper and lower

specification limits!

Bissell(1990), Zhang, Stenback, and Wardrop(1990), Boyles(1991), and

Franklin and Wasserman(1992) discuss confidence intervals for Cp, Cpk

and/or Cpm indices.  Kusher and Hurley(1992) simulate the performance

of several alternative bounding methodologies and recommend Boyles'

combination of Chi-Squared and large-sample-normal approximations.

Therefore, CPstat implements the Boyles(1991) methodology...but also

simulates "exact" normal-distribution-theory performance of lower

bounds on Cpm.

The computational burden of estimating a process standard deviation

via brute-force manual calculations can be tedious.  This burden is

commonly reduced in quality control applications by computing,

instead, the observed Range (maximum minus minimum) and converting

this statistic [using "d(n)" factors tabulated by Tippett(1930) or

Pearson and Hartley(1954)] into an estimate of � as follows:

Range

� = �������� .

d(n)

CPstat [Version 9312] . . . . . . . . . . . . . . . Page 4 of 17

The d(n) notation emphasizes that the numerical value of the d()

factor depends, primarily, upon the sample size, n, used to estimate

the Range:

n �   2     3     4     5     6     7     8     9    10    11    12

���������������������������������������������������������������������

d � 1.128 1.693 2.059 2.326 2.534 2.704 2.847 2.970 3.078 3.173 3.258

This range-based method for estimation of � can be almost as efficient

as the brute-force (square root of average squared deviation from the

mean) method when the distribution being sampled is actually normal.

For example, the increase in Root-Mean-Squared-Error in the estimation

of � is no more than 15% for sample sizes 3 <= n <= 12.  Curiously,

this increase is 29% at n = 2.  And the increase exceeds 30% for

sample sizes larger than about 20.  On the other hand, CPstat assumes

the computational burden for you, and CPstat always uses the more-

efficient brute-force method of calculation.

����������������������������������������Ŀ

�  Capability Indices and Process Yield  �

������������������������������������������

Rodriguez(1992) summarizes many of the criticisms of Cp-type indicies

that have appeared in statistical literature.  In particular,

Rodriguez(1992) stresses that process yield calculations from Cp

indices assuming normality can be quite misleading.  Here we consider

some very simple (as yet unpublished!) results that drive this point

home.

The "yield" of a process is here defined to be the fraction of its

production units that fall within established Lower and Upper

Specification Limits, LSL < USL.

Suppose, first, that the process mean is on target ( � = T ) so that

Cpm = Cp.  Then, under the additional assumption that the target, T,

is at the specification midpoint, T=(USL+LSL)/2, a distribution-free

lower limit can be placed on process yield as a function of Cpm = Cp .

In these special cases, we have:

Prob( LSL < x < USL ) = Prob[ |x-T| < (USL-LSL)/2 ]

= Prob[ |x-T| < 3 * Cp * � ]

= 1 - Prob[ |x-T| >= 3 * Cp * � ].

Given finite values of � and �, the Bienayme-Tchebysheff inequality states

that the probability of an absolute deviation from � equaling or exceeding

k * � is less than or equal to 1/k�, for any positive, fixed k.  Therefore,

Prob( LSL < x < USL )  >= 1 - 1 / [ 9 * Cp� ] .

Furthermore, the Bienayme-Tchebysheff bound is "sharp" in the sense

that distributions exist that achieve equality (rather than simple

inequality) at the stated probability.

CPstat [Version 9312] . . . . . . . . . . . . . . . Page 5 of 17

The following is a tabulation that compares the Normal-theory yields

associated with various values of Cp with the yields given by the

above distribution-free lower limit...

=========================================

Yields for Processes with Mean on Target

and Target at the Specification Mid-Point

=========================================

Normal Theory   Distribution-Free

Cp=Cpm       Yield         Minimum Yield

-------      ---------        ---------

0.666        95.45%           75.00%

1.000        99.73%           88.89%

1.333        99.994%          93.75%

1.666        99.999?%         96.00%

2.000        99.9999?%        97.22%

Note that these lower bounds on yield are considerably lower than the

Normal-theory estimates!

���������������������������������������Ŀ

�    �����������������������������Ŀ    �

�    �   *** USAGE of CPstat ***   �    �

�    �������������������������������    �

�����������������������������������������

The CPstat software system is invoked from the MS-DOS operating system

by entering its 8-character name, CPstat, at your DOS prompt...

Prompt> cpstat

CPstat does not expect or accept any "command-line" arguments.

Instead, CPstat will prompt you to provide all information it needs to

perform analyses.  And each such prompt usually displays a [default

value] in square brackets that you may accept by simply pressing the

ENTER key.

WARNINGS:  Aberrant behavior may result if you attempt to execute

CPstat at any time that your system has less than 200kBytes of

available random access memory (RAM).  CPstat is actually a larger

program than it might appear to be.  Before compression with LZEXE

0.91 (c) Fabrice BELLARD, CPstat.EXE was 105kBytes in size.

CPstat assumes that your personal computer has a COLOR monitor and

uses colors in its text-mode windows/menus.  Screens with text

written in black on a green or cyan background will appear to be

BLANK if you do not have a color monitor.

CPstat [Version 9312] . . . . . . . . . . . . . . . Page 6 of 17

����������������������������������������������������Ŀ

�   CPstat Input/Output File Naming Conventions...   �

������������������������������������������������������

The ASCII (text) files used by CPstat for data input and output

consist of a filename (of at most eight characters) specified by the

user followed by a period (.) and one of two specific 3-character

filename EXTensions.

NORMAL.DAT  ...Batch input of Historical Data (at most 1000 obs.)

NORMAL.OUT  ...CPstat Output Save File.

��������������������������������������������Ŀ

�  ��������������������������������������Ŀ  �

�  �    Example CPstat Walk-Through...    �  �

�  ����������������������������������������  �

����������������������������������������������

The CPstat start-up screen will appear as follows...

����������������������� Capability Statistics ����������������������Ŀ

�                                                                    �

�                     CPSTAT.EXE...Version 9312                      �

�                                                                    �

�    ***   Capability Calculations, Bounds and Simulations  ***      �

�                                                                    �

�                 A Quality Assurance Training Tool:                 �

�         Statistics Committee of the QA Section of the PMA          �

�            Bob Obenchain, CompuServe User [72007,467]              �

�  ��������������������������������������������������������������ͻ  �

�  �   Press a KEY to start at the MAIN MEMU...                   �  �

�  ��������������������������������������������������������������ͼ  �

����������������������������������������������������������������������

And here is that menu...

���������������������������� Capability Statistics �����������������Ŀ

� ������������������������������������ͻ                             �

� �           Main Menu                �                             �

� �   A = Analyses of Sample Data      �                             �

� �   C = Capability Calculations      �                             �

� �   X = eXit CP-Stat                 �                             �

� �   choice --> A                     �                             �

� ������������������������������������ͼ                             �

����������������������������������������������������������������������

CPstat [Version 9312] . . . . . . . . . . . . . . . Page 7 of 17

If you have access to some sample observations on the process whose

capability you wish to quantify, you usually start with Menu Item A...

���������������������������� Capability Statistics �����������������Ŀ

� ������������������������������������ͻ                             �

� �           Main Menu                �                             �

� �   A = Analyses of Sample Data      �                             �

� ���������������������������������������������������������������ͻ  �

� Ⱥ        Will Data be Input via K = Keyboard ? ...or           �  �

�  �                               B = Batch File ?               �  �

�  �                                                              �  �

�  �        Press the   K   or   B   key now -->                  �  �

�  ��������������������������������������������������������������ͼ  �

����������������������������������������������������������������������

If you select Keyboard input, you will be prompted to type in all

historical data values using your keyboard.  But, in this case, CPstat

will also be creating a "Batch Input" file that you could use in

future CPstat runs to repeat and/or modify your initial analyses.

On the other hand, if you are familiar with a "screen oriented"

personal computer (ASCII) text editor, you might prefer to start by

creating your own "Batch Input" file.  This strategy can give you more

flexibility in data entry and validation than is possible inside of

CPstat.  Simply mimic the format of an example CPstat Batch Input

file.  For example, the Batch Input file could consist for the

following 6 records (rows), each starting in column 1:

1.09

2

13

4.7

3

q

where the letter Q at the start of the last record signals CPstat to

QUIT...i.e. the "end of data entry."

Once you respond to the initial CPstat prompt by pressing either the

K or the B key, the "Window" will clear itself, and you will either

(i) be prompted to specify a MS-DOS filename, consisting of at most 8

characters, for capture of Keyboard input or (ii) select an existing

Batch Input (.HST) file from the bounce-bar menu...

CPstat [Version 9312] . . . . . . . . . . . . . . . Page 8 of 17

����������������������� Capability Statistics ����������������������Ŀ

�                                                                    �

�   MENU of CPSTAT .DAT files in Current Directory.                  �

�                                                                    �

�  �FileNames�                                                       �

�  �CONTAMIN �      Arrow Keys: Move Highlight Bar Up / Down.        �

�  �NORMAL��ۺ                                                       �

�  �SAMPLE   �      Return Key: Selects the Highlighted File.        �

�  ���������ͼ                                                       �

�                   Escape Key: Abandon BATCH Input to CPSTAT.       �

�                                                                    �

����������������������������������������������������������������������

Whether input comes from your Keyboard or from a Batch file, the

CPstat Output File CAN have the same filename (of at most 8

characters) as does the Input File.  But the 3-character MS-DOS

filename EXTension for CPstat Output Files will always be ".OUT".

On the other hand, although the [default] output filename will always

be to use the same filename as the input file, there are circumstances

when you will probably want to use a DIFFERENT filename for your

output.  After all, if you always use the same output filename, that

file will get over-written and all previously saved results will be

lost!  Here, we illustrate changing the output filename to NORMAL2...

����������������������� Capability Statistics ����������������������Ŀ

�                                                                    �

�   At colon Prompts : ...simply press ENTER to get the [default].   �

�   --------------------------------------------------------------   �

�                                                                    �

�   Specify filename for Detailed Output [NORMAL.out] : NORMAL2 <<<<<<

�                                                                    �

�   The CPSTAT Output file is to be: NORMAL2.out                     �

�                                                                    �

�                                                                    �

�           Press Q to QUIT now...Other Key to Continue...           �

�                                                                    �

����������������������������������������������������������������������

CPstat [Version 9312] . . . . . . . . . . . . . . . Page 9 of 17

������������������������� Process Data Analyses ��������������������ͻ

�                                                                    �

�   Data Entry Complete.                                             �

�   Press a Key to Display a Data Listing...                         �

�                                                                    �

/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\

�                                                                    �

�          22              27              26              38        �

�          17              10              28              17        �

�          12              20              22              25        �

�          24              22              25              20        �

�          17              14              16               7        �

�          18              23              18              25        �

�                                                                    �

�   Number of Data Values   =  500                                   �

�   Maximum Data Value      =        38                              �

�   Minimum Data Value      =         1                              �

�   Data Mean Value         =     19.56                              �

�   Data Standard Deviation =  5.902406                              �

�   Press a Key to Display a 20-Line Data Histogram...               �

��������������������������������������������������������������������ͼ

������������������������� Process Data Analyses ��������������������ͻ

�    -------- --- -------------------------------------------------- �

�       36.15   1                                                    �

�        34.3   1                                                    �

�       32.45   6 ****                                               �

�        30.6  15 **********                                         �

�       28.75  16 ***********                                        �

�        26.9  16 ***********                                        �

�       25.05  14 *********                                          �

�        23.2  58 ****************************************           �

�       21.35  58 ****************************************           �

�        19.5  58 ****************************************           �

�       17.65  67 ***********************************************    �

�        15.8  71 ************************************************** �

�       13.95  47 *********************************                  �

�        12.1  17 ***********                                        �

�       10.25  22 ***************                                    �

�         8.4  20 **************                                     �

�        6.55   9 ******                                             �

�         4.7   3 **                                                 �

�        2.85   0                                                    �

�           1   1                                                    �

�    -------- --- -------------------------------------------------- �

�    LowLimit Freq       Press a Key...                              �

��������������������������������������������������������������������ͼ

CPstat [Version 9312] . . . . . . . . . . . . . . . Page 10 of 17

This returns you to the Main Menu.  Now press the "C" key...

���������������������������� Capability Statistics �����������������Ŀ

� ������������������������������������ͻ                             �

� �           Main Menu                �                             �

� �   A = Analyses of Sample Data      �                             �

� �   C = Capability Calculations      �                             �

� �   X = eXit CP-Stat                 �                             �

� �   choice --> C                     �                             �

� ������������������������������������ͼ                             �

����������������������������������������������������������������������

Here is what the "CPstat Spreadsheet" screen will look like when it is

initially activated.  (Assuming that you have just finished calculating

summary statistics for the data from the NORMAL.DAT file.)  The box in

the upper hight-hand corner of the spreadsheet identifies the FILENAME

used to define initial values for the sample size, mean, and standard

deviation...

������������������������  CPstat Spreadsheet �����������������������ͻ

���������������������������������Ŀ��������������������������������Ŀ�

�� Specification Limits...        �� Process Summary Statistics...  ��

��                                ��  Input Filename:   NORMAL      ��

�� Lower:  0.000  Upper:  100.000 ��     Sample Size:  500          ��

��                                ��      Mean Value:     19.560    ��

��        Target:    50.000       ��  Std. Deviation:      5.902    ��

��������������������������������������������������������������������ٺ

���������������������������������Ŀ��������������������������������Ŀ�

�� Capability Indices...          �� Approximate Lower Bounds on Cpm��

��                                ��                                ��

��       American  Cp  =  2.824   �� Desired Alpha Level =   0.050  ��

��       American  Cpk =  2.824   ��                                ��

��       Taguchi's Cpm =  0.538   �� Approx. Lower Bound = �����    ��

��������������������������������������������������������������������ٺ

���������������������������������Ŀ��������������������������������Ŀ�

�� Simulated Significance Level of�� Usage of Keys...               ��

�� ������ as a Lower Bound on Cpm ��  Escape key => Return to Menus ��

��                                ��  Arrow keys => Move between    ��

�� Number Monte-Carlo Replic=���� ��                Entry Fields    ��

�� Observed Fraction < Bound=���� �� Number Keys => Change Values   ��

��������������������������������������������������������������������ٺ

��������������������������������������������������������������������ͼ

Note that most of the other fields on this screen are initially BLANK

or contain default values...like the specification limits and target

value in the upper left-hand corner box.  WARNING: the capability

indices displayed in the middle-left box at this point in the analysis

are based upon these (potentially unrealistic) initial limits.

CPstat [Version 9312] . . . . . . . . . . . . . . . Page 11 of 17

The following 15 FIELDS are displayed on the "CPstat Spreadsheet":

Name                Location                   Status

=================   =======================    ===============

Lower Spec Limit    left  of upper-left box    parameter entry

Upper Spec Limit    right of upper-left box    parameter entry

Target Value        bottom of upper-left box   parameter entry

batch filename      2nd line upper-right box   display only

Sample Size         3rd line upper-right box   parameter entry

Sample Mean         4th line upper-right box   parameter entry

Sample Std. Error   5th line upper-right box   parameter entry

American Cp  Index  2nd line middle-left box   display only

American Cpk Index  3rd line middle-left box   display only

Taguchi  Cpm Index  4th line middle-left box   display only

Lower Alpha Level   middle line middle-right   parameter entry

Approx. Cpm Bound   bottom line middle-right   display only

Trial Cpm Bound     2nd line bottom-left box   parameter entry

Cum. Replications   3rd line bottom-left box   display only

Fraction < Bound    4th line bottom-left box   display only

Usage of KEYS on the "CPstat Spreadsheet" Screen:

=================================================

The arrow keys (up, down, left, and right) are used EXCLUSIVELY to

move between "parameter entry" fields.  If you edit an existing value

in a field and then press the ENTER key, the cursor will remain within

the field you just edited.  But, if you press an arrow key while

editing a field, you may change that field's value AND ALSO move on to

a different field!

Seven of the 8 "parameter entry" fields accept decimal values with up

to 3 places after the decimal point.  (The "Sample Size" field accepts

integers of at most 4 digits.)  Digits to the left and right of the

decimal place are edited somewhat differently:

Editing starts in "insert mode" with the cursor 1 place to the

left of the decimal (or in the right-most/units place of an

integer.)  You must first use the DELETE or BACKSPACE keys to

eliminate all unwanted digits to the left of the cursor!  Only

after these erasures should you start entering the digits you

want; initial digits will scroll left as later digits are added.

Press the "minus" (-) key to toggle negative values on / off.

Press the "decimal" (.) key to switch to editing of the digits

right of the decimal point...in "overstrike mode."  Here the

cursor moves right as you enter digits.  Pressing the DELETE or

CPstat [Version 9312] . . . . . . . . . . . . . . . Page 12 of 17

BACKSPACE key now will delete the last digit you typed...and

insert a blank space.  Any digits to the right of blank(s)

after the decimal will then be ignored when you press ENTER or

an arrow key.

The first step in using the "CPstat Spreadsheet" is, thus, usually to

evaluate capability indices that result from more realistic spec limits

than those given by the CPstat initial defaults.  (Because the numbers

in NORMAL.DAT were generated as normal variates with mean value 20 and

standard deviation 6, then truncated to integer values, approximate

3-sigma limits for these data are approximately 20-3*6=2 to 20+3*6=38.)

Editing the specification limits modifies the spreadsheet as follows...

������������������������  CPstat Spreadsheet �����������������������ͻ

���������������������������������Ŀ��������������������������������Ŀ�

�� Specification Limits...        �� Process Summary Statistics...  ��

��       ������Ŀ       ��������Ŀ��  Input Filename:   NORMAL      ��

�� Lower:� 2.000� Upper:�  38.000���     Sample Size:  500          ��

��       �������� ���������������ٳ�      Mean Value:     19.560    ��

��        Target: �  20.000 �     ��  Std. Deviation:      5.902    ��

����������������� ��������������������������������������������������ٺ

���������������������������������Ŀ��������������������������������Ŀ�

�� Capability Indices...          �� Approximate Lower Bounds on Cpm��

��                                ��                                ��

��       American  Cp  =  1.017 <<<� Desired Alpha Level =   0.050  ��

��       American  Cpk =  0.992 <<<�                                ��

��       Taguchi's Cpm =  1.014 <<<� Approx. Lower Bound = �����    ��

��������������������������������������������������������������������ٺ

/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/

��������������������������������������������������������������������ͼ

Note that entering the specification limits shown in the fields

outlined with "rectangles" causes the capability indices to change to

the values marked by the "<<<" symbols.

Next move the cursor to the "Alpha Level" field in the middle-right

box on the CPstat Spreadsheet.  Entering the value 0.05 here (the

default) asks CPstat to calculate an approximate 95% lower bound on

the true Taguchi Cpm index for the process given the sample mean and

sample standard deviation from a RANDOM SAMPLE of the stated SIZE (500

here.)  The Alpha Levels allowed here range from 0.01 to 0.50...i.e.

for lower confidence bounds of probability content from 99% to 50%.

CPstat [Version 9312] . . . . . . . . . . . . . . . Page 13 of 17

������������������������  CPstat Spreadsheet �����������������������ͻ

/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/

���������������������������������Ŀ��������������������������������Ŀ�

�� Capability Indices...          �� Approximate Lower Bounds on Cpm��

��                                ��                      ��������Ŀ��

��       American  Cp  =  1.017   �� Desired Alpha Level =�  0.050 ���

��       American  Cpk =  0.992   ��                      ���������ٳ�

��       Taguchi's Cpm =  1.014   �� Approx. Lower Bound =  0.961 <<<<

��������������������������������������������������������������������ٺ

���������������������������������Ŀ��������������������������������Ŀ�

�� Simulated Significance Level of�� Usage of Keys...               ��

��  0.961<<<<< Lower Bound on Cpm ��  Escape key => Return to Menus ��

��                                ��  Arrow keys => Move between    ��

�� Number Monte-Carlo Replic=���� ��                Entry Fields    ��

�� Observed Fraction < Bound=���� �� Number Keys => Change Values   ��

��������������������������������������������������������������������ٺ

��������������������������������������������������������������������ͼ

The lower bound will equal the observed Cpm when Alpha=0.50 and will

be smallest when Alpha=0.01 is selected.  Here are results for 0.20...

������������������������  CPstat Spreadsheet �����������������������ͻ

/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/

���������������������������������Ŀ��������������������������������Ŀ�

�� Capability Indices...          �� Approximate Lower Bounds on Cpm��

��                                ��                      ��������Ŀ��

��       American  Cp  =  1.017   �� Desired Alpha Level =�  0.200 ���

��       American  Cpk =  0.992   ��                      ���������ٳ�

��       Taguchi's Cpm =  1.014   �� Approx. Lower Bound =  0.987 <<<<

��������������������������������������������������������������������ٺ

���������������������������������Ŀ��������������������������������Ŀ�

�� Simulated Significance Level of�� Usage of Keys...               ��

�� 0.9867 <<<< Lower Bound on Cpm ��  Escape key => Return to Menus ��

��                                ��  Arrow keys => Move between    ��

�� Number Monte-Carlo Replic=���� ��                Entry Fields    ��

�� Observed Fraction < Bound=���� �� Number Keys => Change Values   ��

��������������������������������������������������������������������ٺ

��������������������������������������������������������������������ͼ

Bounds with a given Alpha Level also tend to get smaller if the SAMPLE

SIZE is reduced.  See Boyles(1991) for the Chi-Squared and normal-

theory approximations used by CPstat.

Move the cursor to the "Trial Cpm Bound" field in the lower-left box

on the CPstat Spreadsheet to initiate normal-theory simulations of the

exact Alpha-Level of a specified lower bound on Cpm.  The default

value displayed here is the most recent "approximate lower bound"

displayed in the middle-right box.  But enter any value here you might

want to evaluate and press the ENTER key...

CPstat [Version 9312] . . . . . . . . . . . . . . . Page 14 of 17

������������������������  CPstat Spreadsheet �����������������������ͻ

/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/

���������������������������������Ŀ��������������������������������Ŀ�

�� Capability Indices...          �� Approximate Lower Bounds on Cpm��

��                                ��                                ��

��       American  Cp  =  1.017   �� Desired Alpha Level =  0.2000  ��

��       American  Cpk =  0.992   ��                                ��

��       Taguchi's Cpm =  1.014   �� Approx. Lower Bound =  0.987   ��

��������������������������������������������������������������������ٺ

���������������������������������Ŀ��������������������������������Ŀ�

���������Ŀd Significance Level of�� Usage of Keys...               ��

��� 0.987 �s a Lower Bound on Cpm ��  Escape key => Return to Menus ��

�����������                       ��  Arrow keys => Move between    ��

�� Number Monte-Carlo Replic= 510<<<                Entry Fields    ��

�� Observed Fraction < Bound=.165<<< Number Keys => Change Values   ��

��������������������������������������������������������������������ٺ

��������������������������������������������������������������������ͼ

Here is what the screen looked like after I simulated drawing 510

samples, each of size 500.  Note that rather than an Alpha-Level of

0.20, a lower bound of 0.987 has an estimated Alpha-Level of only

0.165.  Simply press any key to halt the simulation; the cursor will

then return to a the "Trial Cpm Bound" field.

The following screen print illustrates the effects of (i) reducing the

sample size from 500 to 50, (ii) calculating a new approximate lower

20% bound, and (iii) simulating its true, normal-theory content...

������������������������  CPstat Spreadsheet �����������������������ͻ

���������������������������������Ŀ��������������������������������Ŀ�

�� Specification Limits...        �� Process Summary S����������Ŀ  ��

��                                ��  Input Filename: � <<none>> �  ��

�� Lower:  2.000  Upper:   38.000 ��     Sample Size: � 50       �  ��

��                                ��      Mean Value: ����19.560��  ��

��        Target:    20.000       ��  Std. Deviation:      5.902    ��

��������������������������������������������������������������������ٺ

���������������������������������Ŀ��������������������������������Ŀ�

�� Capability Indices...          �� Approximate Lower Bounds on Cpm��

��                                ��                       ������Ŀ ��

��       American  Cp  =  1.017   �� Desired Alpha Level = � 0.200� ��

��       American  Cpk =  0.992   ��                       �������� ��

��       Taguchi's Cpm =  1.014   �� Approx. Lower Bound =  0.923<<<��

��������������������������������������������������������������������ٺ

���������������������������������Ŀ��������������������������������Ŀ�

���������Ŀd Significance Level of�� Usage of Keys...               ��

��� 0.923 �s a Lower Bound on Cpm ��  Escape key => Return to Menus ��

�����������                       ��  Arrow keys => Move between    ��

�� Number Monte-Carlo Replic=3020<<<                Entry Fields    ��

�� Observed Fraction < Bound=.156<<< Number Keys => Change Values   ��

��������������������������������������������������������������������ٺ

��������������������������������������������������������������������ͼ

The simulation runs MUCH faster with this smaller sample size, so I

evaluated more than 3,000 samples of size 50.

CPstat [Version 9312] . . . . . . . . . . . . . . . Page 15 of 17

Press the ESCape key to return from the CPstat Spreadsheet to the Main

Menu.  Then press the X key to exit CPstat and return to DOS...

���������������������������� Capability Statistics �����������������Ŀ

� ������������������������������������ͻ                             �

� �           Main Menu                �                             �

� �   A = Analyses of Sample Data      �                             �

� �   C = Capability Calculations      �                             �

� �   X = eXit CP-Stat                 �                             �

� �   choice --> X                     �                             �

� ������������������������������������ͼ                             �

����������������������������������������������������������������������

REMINDER(S) :

The last output file created by CPSTAT was named... NORMAL.out

Use the DOS invocation... TYPE NORMAL.out | MORE

to review detailed computational results from

this last part of your CPSTAT run.

�������������������������������Ŀ

�  Preserving Time-Series Order �

���������������������������������

The primary statistical methodology implemented in CPstat involves

study of the "marginal distribution" of observed values.  I.E. we

disregard all information about the time-series order in which values

were actually observed.  On the other hand, there usually are very

good reasons for keeping your data stored in their proper time-series

order in .DAT Batch Input files.

���������������������������������������������������������������������Ŀ

�                            REFERENCES                               �

�����������������������������������������������������������������������

AMERICAN NATIONAL STANDARD ANSI/ASQC A1-1987.  "Definitions, Symbols,

Formulas, and Tables for Control Charts."  Milwaukee: American

Society for Quality Control.

BOYLES, R.A. (1991).  "The Taguchi Capability Index."  Journal of

Quality Technology 23, 17-26.

BISSELL, A.F. (1990).  "How Reliable is Your Capability Index."

Applied Statistics 39, 331-340.

CARR, W. E. (1991), "A New Process Capability Index: Parts Per

Million."  Quality Progress 24, no.8 (August), page 152.

CHAN, L.K., CHENG, S.W. and SPIRING, F.A. (1988).  "A New Measure of

Process Capability, Cpm ."  Journal of Quality Technology 20,

162-175.

CPstat [Version 9312] . . . . . . . . . . . . . . . Page 16 of 17

FRANKLIN, L. A. and WASSERMAN, G. S.  (1992).  "Bootstrap Lower

Confidence Limits for Capability Indices."  Journal of Quality

Technology, 24, 196-210.

HARRY, M. J. (1987), "The Nature of Six-Sigma Quality." (booklet)

Motorola Communications Sector, Schaumburg, IL.

HSIANG, T. C. and TAGUCHI, G.  (1985).  "A Tutorial on Quality Control

and Assurance - The Taguchi Methods."  Annual Meeting of the

American Statistical Association, Las Vegas, Nevada.  (188

page handout.)

KANE, V. E. (1986).  "Process Capability Indices."  Journal of Quality

Technology, 18, 41-52.

KRUSHLER, R. and HURLEY, P. (1992).  "Confidence Bounds for Capability

Indices."  Journal of Quality Technology, 24, 188-195.

L'ECUYER, P. (1988).  "Efficient and Portable Combined Random Number

Generators."  Communications of the ACM 31, 742-749,774.

MONTGOMERY, D. (1991).  Introduction to Statistical Quality Control,

Second Edition.  New York: John Wiley.

OBENCHAIN, R. L.  (1993).  "Cumulative Capability Curves."

Unpublished manuscript, Eli Lilly and Company.

OBENCHAIN, R. L. (1990-93).  "CAPQNT.EXE:  A Self-Extracting Archive

of MS-DOS Personal Computer Software for Process Capability

Quantification."  CompuServe IBMAPP Library 13; Uploaded by

User [72007,467].

PEARN, W. L., KOTZ, S. and JOHNSON, N. L.  (1992).  "Distributional and

Inferential Properties of Process Capability Indices."  Journal

of Quality Technology 24, 216-231.

PEARSON, E. S. and HARTLEY, H. O. (1954).  Biometrika Tables for

Statisticians, Vol.I. (Table 20.)  Cambridge University Press.

PRESS, W.H., FLANNERY, B.P., TEUKOLSKY, S.A. and VETTERLING, W.T.

(1988). "Numerical Recipes in C: The Art of Scientific

Computing."  Cambridge, Massachusetts: Cambridge University

Press.  {Source Code: Copyright 1987, 1988 by Numerical Recipes

Software, P.O.Box 243, Cambridge, MA 02238.}

RODRIGUEZ, R. N.  (1992).  "Recent Developments in Process Capability

Analysis."  Journal of Quality Technology 24, 176-187.

TAGUCHI, G. and WU, Y.  (1979).  Introduction to Off-Line Quality

Control. (English translation.) Central Japan Quality Control

Association, Nagaya, Japan.

CPstat [Version 9312] . . . . . . . . . . . . . . . Page 17 of 17

TAGUCHI, G., ELSAYED, E. A., and HSIANG, T. C.  (1989).  Quality

Engineering in Production Systems.  New York: McGraw-Hill.

TIPPETT, L.H.C. (1930).  "On the Extreme Individuals and the Range of

Samples from a Normal Population."  Biometrika, 17, 364-387.

WICHMAN, B. A. and HILL, I. D. (1981), "Algorithm AS 183: An Efficient

and Portable Pseudo-Random Number Generator."  Applied Statistics

31, 188-190.

ZEISEL, H. (1986), "Remark AS R61: A Remark on Algorithm AS 183."

Applied Statistics 35, 89.

ZHANG, N. F., STENBACK, G. A. and WARDROP, D. M. (1990).  "Interval

Estimation of Process Capability Index, Cpk."  Communications

in Statistics - Theory and Methods, 19, 4455-4470.

CPstat Software Update History:

===============================

Version 9203 ...Beta Test Version

Version 9206 ...Fix bug in calculation of Cpk

Version 9207 ...Embedd "Numerical Recipes Software" copyright notice

because object files compiled from their gser(), gcf(),

gammp(), and gammln() functions are bound into CPstat.

Version 9302 ...Fixed some window closing (memory allocation)

problems that apparently caused system "hangs."

...Changed CPstat Spreadsheet field name from "Monte-Carlo

Sample Size=" to "Number Monte-Carlo Replic=".

...Changed paths for cursor movements between fields on the

CPstat Spreadsheet in response to an arrow-key press

(left, right, up, & down.)

...Change random number generator from Witchman-Hill(1981)

to 32-bit L'Ecuyer(1988) algorithm.

Version 9312 ...Fixed memory allocation bug associated with reading more

than one batch input datafile in a single CPstat run.

...Update this documentation file to provide lower bounds

on process yield in cases where Cp=Cpm.