.ft e10

.pn 0

DOCUMENTATION ON

CREST: CHEMICAL REACTIONS AND EQUILIBRIUM THERMODYNAMICS

VERSION 5.00

developed by

Dudley J. Benton

August 10, 1991

.pa

.pn 1

_CONTENTS_

Page

Introduction . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .   2

Running the Program  . . . . . . . . . . . . . . . . . . . . . . . . . . .   3

Interpreting the Output  . . . . . . . . . . . . . . . . . . . . . . . . .   8

Defining Elements and Compounds  . . . . . . . . . . . . . . . . . . . . .   8

Creating a Plot of Mole Fraction . . . . . . . . . . . . . . . . . . . . .  10

References . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  11

Appendix 1 Listing of Error Messages . . . . . . . . . . . . . . . . . . .  13

Appendix 2 Listing of Data File CREST.GAS  . . . . . . . . . . . . . . . .

Appendix 3 Listing of Data File CREST.AQU  . . . . . . . . . . . . . . . .

Appendix 4 Sample Program Output . . . . . . . . . . . . . . . . . . . . .

Appendix 5 Program OXMOLES (computes combustion products vs. moles oxygen)

Appendix 6 Program FINDENTH (computes enthalpy of combustion products vs. T)

Appendix 7 Sample Plots of Computed Mole Fraction vs. Temperature  . . . .

.pa

_INTRODUCTION_

CREST  is an interactive computer program designed to solve chemical reactions

using equilibrium thermodynamics. CREST can handle  elements,  compounds,  and

ions.  CREST  can  compute  the  thermodynamic  properties  of  the  elements,

compounds, and ions at a  given  state  or  it  can  compute  the  equilibrium

composition  of  the  products of a reaction involving these. CREST solves the

simultaneous dissociation reactions implied by a reaction automatically. It is

not  necessary  to  specify  all of the possible dissociation reactions, CREST

will handle them for you.

CREST can handle many elements and compounds (currently set at 32 elements and

128  compounds).  Each compound can be composed of several elements (currently

set at 16). CREST can handle complex reactions (currently set at 128 reactants

and 128 products). Each of the dissociation species counts as a product of the

reaction. CREST can solve any number of independent reactions although it will

only  solve  one  at  a  time.  Simultaneous,  dependent  reactions  should be

specified as one reaction. Eventually, the capability of  multiple  sequential

reactions will be added (maybe Version 6).

You can specify the reaction to be solved interactively or in a data file. You

can have any number of data files.  CREST  will  allow  you  to  interactively

specify  under  what  conditions  (ie.  temperature and pressure) you want the

thermodynamic properties evaluated and the constraints on  the  reaction  (ie.

constant pressure or volume and specified temperature or heat transfer). CREST

will also allow you  to  interactively  change  the  molar  fractions  of  the

reactants.

Elements  and  compounds  are  defined  in  a database file (cf. CREST.GAS and

CREST.AQU) which cannot be altered by CREST. Any elements  or  compounds  that

appear  in  your  reaction  must be defined in the database before running the

program. You can get a list of  the  available  substances  by  selecting  the

appropriate  command  from  the  main  menu  or  from the built-in full-screen

editor. Additions to and modifications of the substances in the database  will

be covered in the section "DEFINING ELEMENTS AND COMPOUNDS."

Throughout  this  manual  and  within  the  program,  optional  parameters are

indicated by brackets, []. For example, the command B[atch]= can be  given  as

Batch=,  Batc=,  Bat=,  Ba=,  or B=. If you use more than the first letter, it

must  be  spelled  correctly  and  must  not  have  any  trailing   extraneous

characters. Case (upper/lower) is immaterial.

You  can  have  several  database  files.  Two  are  supplied  (CREST.GAS  and

CREST.AQU). To access a different database file just specify  it  in  the  run

command:  CREST  D[atabase]=CREST.AQU  or select the appropriate option in the

main menu. If none is provided, a file extension of ".DAT" will be assumed and

the file is not found without any extension.

CREST  works  in  either  English  or  SI units. The English units include the

lb-mole (pound-mole), the BTU, degrees Rankine, and psia  (pounds  per  square

inch  absolute).  The SI units include the gm-mole, Joule, degrees Kelvin, and

atmosphere. The units for each quantity are given by the program when  listing

results.  You can select the units from the run command by adding U[nits]=S[I]

or U[nits]=E[nglish]. English is the default.

.pa

_RUNNING THE PROGRAM_

Do not attempt to run CREST from BASIC. On a PC you should be in DOS with  the

DOS prompt. To run CREST just type CREST and hit Enter.

CREST will offer you a menu like the following...

Compute properties --> compute U,H,S,G for a given T,P

Database ------------> select another or list current database

Edit reaction -------> display or modify the reaction

File reaction -------> save the current reaction in a disk file

Graph ---------------> solve a series of reactions and plot results

Print=ON/OFF --------> begin/quit listing results to printer

Read reaction -------> open existing disk file and read contents

Solve reaction ------> determine equilibrium composition of products

Units=English/SI ----> select units for input/output

use ^v then Enter to select or Esc to exit

You  may  select  any  of  these by touching a single key corresponding to the

first upper case character in the menu line or by moving the  cursor/highlight

to  the  desired line with the arrow keys or mouse and then tap the Enter key.

If the menu entries begin with numbers (0-9) instead  of  upper  case  letters

(which  is the case with the plot menu), then select using the numbers. If you

have a Microsoft compatible mouse, then the left button can be  used  for  Yes

and  the  right button can be used for No or Esc. If you want to interrupt the

program during a long series of calculations then tap the space bar and wait a

few  seconds.  This will abort the current task. If you have a mouse, the same

thing can be accomplished by depressing both buttons at the same time. If  you

wish  to  exit  CREST,  follow  the  instructions  which appear on the screen.

Essentially, a few taps on the Esc key will get you out.

If you want the results to be printed select Print>ON from the  main  menu.  .

You  can  later  stop  printing by selecting Printer>OFF. Note that Ctrl-PrtSc

will not work with CREST, nor will standard  output  device  re-direct  (CREST

>PRN).  If you want to spool the output to a file instead of the printer, then

use the run command: S[pool]=spoolfile.ext. The spool file  must  not  already

exist  to  avoid  unintentional  overwriting. If you use the S[pool]= command,

then  CREST  will  automatically  set  Printer>ON.  If  you  use  the  command

S[pool]=PRN,  then CREST will send the output to the printer as usual;  but it

will also turn Print>ON. This is a useful when running in the unattended  mode

(ie., from a batch file or spawned by another program).

You  can  also  specify  the  reaction  file  in  the  run  command  with  the

R[eaction]=file.ext. If no file extension is given and the file is  not  found

without an extension, then CREST will assume .REA for the extension.

You  can  also  specify  the  database  file  in  the  run  command  with  the

D[atabase]=file.ext. If no file extension is given and the file is  not  found

without an extension, then CREST will assume .DAT for the extension.

You  can  run  CREST  interactively  or  unattended.  In  order  to run in the

unattended mode, you must use either the B[atch] or P[lot] run  commands.  The

B[atch] command has several configurations as listed below, where:

Tr=temperature of the reactants (if zero --> 537R/273K)

Tp=temperature of the products (if zero --> 537R/273K)

Pr=pressure of the reactants (if zero --> 14.7psia/1atm)

Pp=pressure of the products (if zero --> 14.7psia/1atm)

Q=heat transfer

B[atch]=100:200  <--- here Tr=Tp=100, Pr=Pp=200  this will compute

and list the properties, then exit

B[atch]=P[ressure]:T[emperature]:100:200:300  <--- here P tells

CREST to assume constant pressure (open system), T means

specified temperature of the products (ie., compute the heat

transfer), Tr=100, Pr=200, Tp=300

B[atch]=V[olume]:T[emperature]:100:200:300  <--- here V tells

CREST to assume constant volume (closed system), T means

specified temperature of the products (ie., compute the heat

transfer), Tr=100, Pr=200, Tp=300

B[atch]=P[ressure]:Q:100:200:300  <--- here P tells CREST to

assume constant pressure (open system), T means specified

temperature of the products (ie., compute the heat transfer),

Tr=100, Pr=200, Tp=300

B[atch]=V[olume]:Q:100:200:300  <--- here V tells CREST to

assume constant volume (closed system), T means specified

temperature of the products (ie., compute the heat transfer),

Tr=100, Pr=200, Tp=300

The P[lot] command also has several configurations, where:

Tp1=initial temperature of products

Tp2=final temperature of products

dTp=step size (ie., do from Tp1 to Tp2 in steps of dTp)

device=graphics device - must be one of the following:

=CRT   (CGA/EGA/VGA/HGA automatically selected by hardware)

=80    (HI-80 printer/plotter)

=7240  (HP-7240 printer/plotter)

=7470  (HP-7470 2-pen plotter)

=7475  (HP-7475 6-pen plotter)

=7550  (HP-7550 8-pen plotter)

=Star  (Star dot matrix printer)

=Epson (Epson dot matrix printer)

=IBM   (IBM dot matrix printer)

=Laser (HP LaserJet)

log=either L for log Y-axis (moles) or omit for linear Y-axis

P[lot]=P[ressure]:100:200:300:400:500:device:log  <--- here P

tells CREST to assume constant pressure (open system),

Tr=100, Pr=100, Tp1=300, dTp=400, Tp2=500

P[lot]=V[olume]:100:200:300:400:500:device:log  <--- here V

tells CREST to assume constant volume (closed system),

Tr=100, Pr=100, Tp1=300, dTp=400, Tp2=500

These  run  commands can be combined in any order. If either B[atch] or P[lot]

are specified, then CREST will run in unattended mode,  otherwise  CREST  will

run  in  the  interactive  mode.  Do  not  specify both the B[atch] and P[lot]

commands.

Two program (including source code in C) are  provided  which  illustrate  the

potential uses of the CREST unattended mode when spawning it as a process from

another program. The first of these is OXMOLES which uses CREST to compute the

products  of  combustion at constant temperature and pressure but with varying

moles of oxygen. The second is  FINDENTH  which  uses  CREST  to  compute  the

enthalpy  of  combustion  products with varying temperature. Listings of these

codes and sample results can be found in the Appendices.

_Computing Thermodynamic Properties_

There is a default reaction which is loaded into the  50  line  reaction  data

buffer.  This  happens  to be the dissociation of water into hydrogen, oxygen,

and hydroxl. You can examine this reaction by selecting menu  option  to  edit

reaction. CREST will list the reaction and some help information.

To  solve  the  reaction  you  must  first  return  to  the main menu. This is

accomplished by pressing F10. The menu should reappear. Now  select  the  menu

option to Compute thermodynamic properties.

CREST will run through a checking procedure and then look through the database

for the elements and compounds. It does this once every time  you  change  the

reaction. CREST lists some information while it is checking your reaction. You

need not be concerned if it flies by too fast to read. You only need  to  read

it  if you have made a mistake, in which case CREST will pause so you can read

it. It just throws the  information  on  the  screen  in  the  event  that  it

encounters an error so that it will be there if you need it.

After  a few seconds you should see a header listing the units and symbols and

a prompt to enter the temperature (T) and pressure (P). If you  enter  a  zero

for  either  temperature  or pressure CREST will assume the standard reference

values which are 537R/237K and 14.696psia/1atm. After you  type  these  values

(try  1000  14.7) hit Enter and CREST should list the internal energy (U/RTo),

enthalpy (H/RTo), entropy (S/R),  and  Gibbs  free  energy  (G/RTo)  for  each

reactant  and  product  under  the  specified conditions. The same prompt will

return and you can do this as many times as you like. To return  to  the  main

menu  just  hit  blank  Enter. If you select this option, CREST will assume an

equal number of moles for each product (as these  are  unknown  without  first

solving for the equilibrium.

If  CREST  takes  more than one second to compute the properties, you probably

don't have a math coprocessor in your PC. CREST will work even  if  you  don't

have one;  but I must warn you that there is a speed difference of a factor of

30 to  120!   Should  you  try  to  solve  a  long  reaction  without  a  math

coprocessor,  I  suggest you get a good book to read while you wait. Note that

if you don't know if you have a coprocessor and the program seems to be taking

forever  to  run, you can interrupt it at any time by striking the spacebar or

clicking both mouse keys at once.

_Solving a Reaction_

Now to solve the reaction select menu option  to  Solve  a  reaction.  If  you

haven't  changed the reaction CREST will not go through the checking procedure

again. You should see a header listing units and symbols and a  prompt.  CREST

wants  to  know  if  you  want to change the reactant mole fractions. That is,

something like:

C+O=CO

C+2O=CO

2C+3O=CO

You  can  change  the  coefficients  in  the equation as desired. If not, just

respond with a N.

Next you will be asked whether the reaction is constant pressure (open system,

steady-state   reaction,   etc.)  or  constant  volume  (closed  system,  bomb

calorimeter, etc.). Remember that for a  closed  system  the  conservation  of

energy  is  based  on  internal energy (U);  whereas, for an open system it is

based on the enthalpy (H). If you specify constant volume, CREST will  compute

the resulting pressure of the reactants.

Then  you  will  be asked whether you want to specify the final temperature of

the products or the heat transfer (Q). If you specify the temperature  of  the

products,  CREST  will  compute  the  heat  transfer  per  specified  moles of

reactants.  If  you  specify  the  heat  transfer,  CREST  will  compute   the

temperature of the products.

For  example,  a  bomb calorimeter would be a constant volume, adiabatic (Q=0)

reaction. An ideal furnace would be a constant pressure,  adiabatic  reaction.

Of course, real furnaces lose both pressure and heat. You can specify the heat

loss or addition, Q need not be zero.

CREST will flash messages on the screen as  it  solves  your  reaction.  These

indicate such things as the number of simultaneous nonlinear equations that it

is solving, the iteration number, temperature, pressure,  and  heat  transfer.

When  these  values  and  the  mole fractions of the reactants "level-out" the

iterations will stop and the results will be listed.

You can enter as many temperatures and pressures as you like, then  hit  Enter

to  return  to  the  constant  pressure/constant volume prompt. If you want to

change the reactant mole fractions just hit Esc. To get back to the main  menu

just hit Esc again.

_Using the Edit Features_

CREST also has a built-in full screen editor. You get into edit mode by making

the appropriate selection from the main menu. The reaction can take up  to  50

lines. Each line can hold no more than 78 characters making the maximum length

reaction 3900 characters. You can make whatever changes you want then exit  to

the  main  menu  by pressing F10. Use F1 to list the elements and compounds in

the database. Any lines in the stack that begin with an asterisk (*)  will  be

ignored as comments.

_Defining Reactions_

There  can only be one reaction;  but it can have many reactants and products.

You can read a reaction from a file (try COAL) and save a reaction in  a  file

(try MYREACT). You can have as many files containing reactions as you like.

The  reaction  is  defined by a group of elements and compounds followed by an

equals (=) and another group of elements and compounds. The first group is the

reactants and the second is the products.

The   mole  fractions  of  the  reactants  need  not  be  specified.  You  can

interactively change this later. This is done  by  preceding  the  element  or

compound with a number (which may be a decimal fraction). The mole fraction of

the products must not be specified (determination of the mole fraction of  the

products is the reason for running CREST in the first place).

One  of  the  important  applications  of  chemical equilibrium analysis is to

determine  dissociation.  Usually  this  requires  specifying  each   of   the

dissociation reactions separately. This is not necessary with CREST.

The  combustion  of  one  mole  of carbon (C) with 1.5 mole of diatomic oxygen

(Odia), would result in carbon, oxygen (O), diatomic oxygen,  carbon  monoxide

(Cmonox),  and  carbon  dioxide (Cdiox). This reaction could be represented in

proper CREST syntax as:

C+1.5Odia=C+O+Odia+Cmonox+Cdiox

There are actually several reactions represented by the above equation.  These

are typically written as:

C+Od<=>Cd+Od

Cd<=>C+Co+O

Co<=>C+O

Od<=>O

It  is  misleading  to represent each dissociation reaction separately because

they occur simultaneously and  are  interrelated.  CREST  solves  all  of  the

reactions  simultaneously to give you an accurate representation of the actual

chemical processes. Because CREST does all of this automatically,  it  reduces

your  chances  of  forgetting a crucial reaction as well as making the process

much simpler than the classical approach.

An example of a more complex reaction would be the combustion of coal...

Coal+6.3Air+.2Cacarbonate=Ar+H+C+Smonox+Sdiox+Hdia+Ndia+Odia+Water+

Nmonox+Ndiox+Coal+Cdiox+Hydroxl+N+O+S+Cacarbonate+Cahydroxide+

Casulfide+Casulfite+Casulfate

_Non-Ideal Behavior_

CREST is able to treat the reacting substances differently depending  on  what

you  tell  it  in  the  database  and  the  reaction  equation. There are five

different groups of substances which can be specified in the database (more on

databases  later).  These result in first order corrections to ideal behavior.

The five groups are:

1 = ideal gas             (Z=1.00, F=1.00)

2 = ideal gaseous ion     (Z=1.00, F=1.00)

3 = ideal aqueous ion     (Z=0.10, F=1.00)

4 = ideal liquid          (Z=0.10, F=1.00)

5 = ideal dispursed solid (Z=0.01, F=0.10)

where  Z is the compressibility and F is the activity coefficient.  Currently,

Z and A are the same for groups 1&2 and 3&4, there may  be  a  distinction  in

later versions.

Higher order correction can be specified by the !  operator. If a substance is

followed  by  a !, then it will be treated as a real substance. In the example

below, only CO and CO2 are treated as real (non-ideal) substances. All of  the

rest will be treated as ideal gases.

Methane + 2.5 Odia = O + Odia + H + Hdia + Ho + C + Cmonox! + Cdiox!

If a !  is found before the first reactant (as illustrated below), then all of

the reactants will be treated as real substances.

! Methane + 2.5 Odia = O + Odia + H + Hdia + Ho + C + Cmonox! + Cdiox!

If a !  is found after the = and before  the  first  product  (as  illustrated

below) then all of the products will be treated as real substances.

! Methane + 2.5 Odia = ! O + Odia + H + Hdia + Ho + C + Cmonox + Cdiox

Any  real  behavior  is  treated according to Redlich and Kwong's mixing rule.

The temperature dependence of  compressibility  and  activity  coefficient  is

computed  based  on a modification of the method described by Fuller, which is

in turn an improvement upon Soave's modification to the Redlich-Kwong equation

of state.

Because  of the versatility which CREST offers in being able to select the way

in which to treat each substance, it can be a powerful tool  in  learning  how

reactions  will  occur  and  what  difference the assumption of ideal behavior

makes in equilibrium calculations.

_INTERPRETING THE OUTPUT_

CREST computes several quantities, among  these  are:  temperature,  pressure,

internal  energy,  enthalpy,  entropy,  Gibbs  free  energy,  heat (or energy)

transfer, number of moles, and mole fraction.

Temperature is in degrees Rankine or degrees Kelvin. Standard  temperature  is

537R or 237K.

Pressure is in psia (pounds per square inch absolute) or atmospheres. Standard

atmospheric pressure is 14.696 psia or 1 atm.

Internal energy is in BTU/lb-mole or Joules/gm-mole.

Enthalpy is in BTU/lb-mole or Joules/gm-mole and is the internal  energy  plus

PV or ZRT. The enthalpy listed includes the enthalpy of formation.

For  ideal gases the difference between U/RTo and H/RTo will be one. For ideal

liquids and solids these will be essentially the same.

Entropy is in BTU/lb-mole/R or Joules/gm-mole/K and is referenced to  absolute

zero  as  required by the Third Law of Thermodynamics. Occasionally CREST will

compute a negative entropy. Absolute entropy can never be negative. This is  a

problem  that arises from having a relationship for specific heat which is not

accurate over the range of temperatures.

The Gibbs and Helmholtz free energies are defined by:

G = H - TS   or   G/RTo = H/RTo - TS/RTo

A = U - TS   or   A/RTo = U/RTo - TS/RTo

and is a measure of the available  or  useful  energy  (although  not  a  true

measure  as  is  availability).  These  have  the  same units and reference as

enthalpy and internal energy.

Heat (or energy) transfer is defined as being to or from the reactants. It  is

important  to  understand  the  concept of a system boundary as defined in the

thermodynamic sense. Here the system is the reactants and the surroundings  is

everything  else.  In a furnace for example, the furnace itself is not part of

the system. Only the coal and air are in the system.  Positive  heat  transfer

would  be  energy added to the coal and air from the surroundings and negative

heat transfer would be heat removed from the coal and air to the surroundings.

Heat transfer has the units of BTU or Joules.

_DEFINING ELEMENTS AND COMPOUNDS_

Any elements or compounds referenced in the reactions you intend to solve must

be in the database file before running the program. I  have  supplied  two  in

order to get you started. You may want to add more.

You  must have an editor of some type in order to make changes in the database

file. If you are going to use a word processor (e.g. PC-WRITE or  WordPerfect)

be sure to treat the file as ASCII.

When  reading  this next section it will be helpful to refer to the listing of

CREST.GAS found in Appendix and CREST.AQU found in an Appendix.

CREST allows you to define elements and compounds. Elements can't be  composed

of  anything  else  whereas  compounds  must  be  composed  of elements. It is

necessary  to  distinguish  between  the  two  in  order  to   determine   the

conservation  equations.  Elements and compounds are identified by syntax. The

order in which you define them doesn't matter;  but the syntax is critical.

_Defining Elements_

CREST allows you to define  your  own  elements  or  isotopes.  Each  must  be

identified  by a unique 12-character symbol. The first character of the symbol

must be an upper case letter (A-Z) and the  rest  can  either  be  lower  case

letters (a-z), or blanks (e.g. H, He, N, Na, Calcium), or one of the following

#$%&:;@^`{|}~ (H#, Ca~, Na^). You cannot use numbers or parentheses  (e.g.  H2

and  H(2)  are  not  permissible).  You  should  note  that hydrogen (H) is an

element;  but diatomic hydrogen (H) is a compound  composed  of  two  hydrogen

atoms.  In  order to reference diatomic hydrogen in a reaction you must define

the element hydrogen (H) AND the compound diatomic hydrogen (Hdia). Note  that

H2 for diatomic hydrogen or OH for hydroxyl is not permissible.

Following  the  symbol  defining  an,  element  you must specify nine numbers.

These are to be in standard free format.  That is, they can be in  any  column

and  can have mixed floating-point, integer, and exponential notation.  Again,

see the appendix for examples.

The first of these numbers is the enthalpy of formation in BTU/lb-mole.   Most

often  the  enthalpy  of formation is given in kilocalories per gram-mole (why

not kilogram-mole I have no idea).  To  convert  to  BTU/lb-mole  multiply  by

1800.   Note  that the enthalpy of formation of elements is zero by definition

(except diatomic gases).

The next number following the enthalpy of formation  is  the  entropy  at  STP

(standard  temperature  and pressure which is 537R/237K and 14.696 psia/1atm).

This must  have  the  units  of  BTU/lb-mole/R.   This  is  usually  given  in

gram-calories  per  degree C per gram-mole.  No conversion is required in this

case.

The next three numbers define the constant  pressure  specific  heat  (Cp)  in

BTU/lb-mole/R.   This  is  usually  given  in  gram-calories  per degree C per

gram-mole.  This also requires no conversion.  Cp is defined by the  following

equation:

Cp = C1 + C2*T + C3*T*T

For  constant  specific  heat  C2  and C3 are set to zero.  If Cp is given for

various temperatures, you must first curve-fit the data in order to  determine

the constants in the above equation.

The  next  number  must be an integer between 1 and 5 indicating the substance

groups listed in the previous section on Non-Ideal Behavior.

The next two numbers are the critical temperature  (degrees  R)  and  pressure

(psia).  If you don't know the correct values, set it to and CREST will use an

empirical formula to estimate these properties.

The last number is the molecular weight.

_Defining Compounds_

CREST allows you to define your own compounds. Just as for elements, each must

be  identified  by  a  unique  12-character symbol. The elements composing the

compound must follow immediately and be set off by  square  brackets  [].  The

number  of  moles  of  each  element per mole of the compound must follow each

element (viz. H2O2 for two hydrogen and two oxygen). These can be free  format

real  numbers  and  there  can  be  any number of spaces. Elements can also be

repeated (e.g. OHOH will be interpreted the same as H2O2).  You  might  define

coal using the following symbols:

Coal[C.2H.75S.05]

This  would  be  Coal  means coal and it is composed of 20% C, 75% H, and 5% S

(mole fraction). COAL[etc.] is not permissible because  there  is  no  way  of

distinguishing this from 1 mole each of C, O, A, and L.

Carbon monoxide as a compound could be defined as:

Cmonox[CO]

MEK (methyl-ethyl-ketone) could be defined by any of the following:

Mek[CH3 CO C2H5]

Mek[CHHHCOCCHHHHH]

Mek[C4H8O]

All are equivalent in CREST syntax.

The enthalpy of formation, entropy at STP, specific heat, substance group  and

critical  properties  follow as with the elements.  No molecular weight should

be specified  for  compounds  as  this  is  computed  based  on  the  elements

comprising the compound.

_CREATING A PLOT OF MOLE FRACTION_

CREST  will  create several files in order to assist you in creating a plot of

mole fraction as a function of temperature. These files are designed  for  use

with my plot program TPLOT. CREST will run TPLOT for you and display the plots

if TPLOT.EXE can be found in the current  directory  or  along  the  path.  If

HPGL2PRN.COM  is  in  the current directory or along the path, then CREST will

also use it to convert plots created by TPLOT to dot matrix or LaserJet  form.

CREST  will  also  let you edit the plot command file CREST.PLT if there is an

ASCII editor in the current directory or along the  path  named  EDITP.EXE  or

EDITP.COM (you must supply your own editor).

CREST  creates  a plot command file named CREST.PLT and at least one data file

containing the results of the equilibrium calculations. These files are  named

CREST0.OUT  through  CREST8.OUT. The reason for creating more than one file is

because TPLOT (and most other programs for that matter)  can  only  read  some

maximum  number  of  columns  (for TPLOT this is 200 characters). Because each

product requires a space and 12 characters for the name or  mole  fraction  in

exponential  notation,  it  is  necessary to break this up into several files,

each with no more than 15 products.

CREST by default removes the plot output files when it is  finished.   If  you

want  to  keep  them,  then  set  an  environment variable using the following

command at the DOS prompt:

SET CREST=O

Remember  that  unless you rename the output files, should you run CREST again

these will be overwritten.

.pa

REFERENCES

Abbott, M. M., 1973, "Cubic Equations of State," _AIChE Journal_ (19:596-601).

Benton,  D.  J.,  1987, "FEAST: Fast Estimation of Thermodynamic and Transport

Properties,"  a  computer  code  available  on  the  ASME/CIME   Bulletinboard

(608)233-3378.

Benton,  D.  J.,  1988, "CREST: Chemical Reactions and Equilibrium Statistical

Thermodynamics," loc. cit.

Benton, D. J., 1991, "Applications of a Hybrid Derivative-Free  Algorithm  for

Locating Extrema," SIAM-SEAS, Cullowhee, NC.

Benton,  D.  J.,  1991,  "Solution  of  Complex  Thermochemical  Equilibria in

Symbolic Form," Proceedings of the ASME WAM.

Chung, W. K., S. E. M. Hamam, and B. C.-Y. Lu,  1977,  Letter  to  the  Editor

regarding paper by Fuller (1976), op. cit. 16:494-495.

Cruise,  D. R., 1964, "Notes on the Rapid Computation of Chemical Equilibria,"

_Journal of Physical Chemistry_ (68:3797-3802).

Fletcher, R., 1987, _Practical Methods of Optimization_, John  Wiley  and  Sons,

New York, NY.

Fuller, G. G., 1976, "A Modified Redlich-Kwong-Soave Equation of State Capable

of Representing the Liquid State," _Ind. Eng. Chem. Fund._, 15:254-257.

Liu, Y., M. Wimby, and U.  Gren,  1989,  "An  Activity-Coefficient  Model  for

Electrolyte Systems," _Computers in Chemical Engineering_ (13:405-410).

More,  J. J. and D. C. Sorensen, 1984, "Newton's Method," _Studies in Numerical_

_Analysis_, G. H. Golub, ed.,  The  Mathematical  Association  of  America,  pp.

29-82.

Ortega,  J.  M.  and  W.  C. Rheinboldt, 1970, _Iterative Solution of Nonlinear_

_Equations in Several Variables_, Academic Press, New York.

Pierce,  F.  J.,  1968,  _Microscopic  Thermodynamics_,  International  Textbook

Company, Scranton, Pennsylvania.

Powell,  M.  J.  D.,  1977,  "Restart  Procedures  for  the Conjugate Gradient

Method," _Mathematical Programming_, Vol. 12, pp. 241-254.

Prausnitz,  J.  M., 1969, _Molecular Thermodynamics of Fluid-Phase Equilibrium_,

Prentice-Hall, Englewood Cliffs, New Jersey.

Redlich,  O.  and  J. N. S. Kwong, 1949, "On the Thermodynamics of Solutions,"

_Chemical Review_ (44:233-244).

Smith, W. R. and R. W. Missen, 1982, _Chemical Reaction  Equilibrium  Analysis:_

_Theory and Algorithms_, John Wiley and Sons, New York.

Stadler, H. P., 1989, _Chemical Thermodynamics_, The Royal Society of Chemistry,

Cambridge, U.K. (available in the U.S. through CRC Press).

van  Wylen,  G.  J.  and  R.  E.  Sonntag,  1973,  _Fundamentals  of  Classical_

_Thermodynamics_, John Wiley and Sons, New York.

Wagner,   H.   M.,   1975,   _Principles   of  Operations  Research_,  2nd  Ed.,

Prentice-Hall, Englewood Cliffs, New Jersey.

White, W. B., S. M. Johnson, and G. B. Dantzig, 1958, "Chemical Equilibrium in

Complex Mixtures," _Journal of Chemical Physics_ (28:751-755).

Wylie, C. R., 1975, _Advanced Engineering Mathematics_,  4th  Ed.,  McGraw-Hill,

New York.

.pa

APPENDIX 1

LISTING OF ERROR MESSAGES

.pa

ERROR#1: read error

You may have typed something in wrong, file is corrupt, or any other problem

which would result in a file read error.

ERROR#2: syntax error

This  may  be  due  to  incorrect  characters, use of parentheses, misplaced

+,=,[,] sign etc. A syntax error will  also  occur  if  something  like  the

following is found: "++", "+!", or "2!H".

ERROR#3: reaction has only one reactant and only one product

There isn't a reaction to solve.

ERROR#4: element does not appear on both sides of reaction

This  error occurs if an element appears on the left side of a reaction then

it must also appear on the right side and vise versa.

ERROR#5: reaction has no products

This can be due to a missing "=" sign or nothing on the right hand  side  of

the reaction.

ERROR#6: unexpected EOF (end of file)

CREST  was looking for something else and didn't find it before encountering

the end of data. Sometimes this error occurs when your reaction has  no  "="

sign in it or you have not specified a reaction.

ERROR#7: too many elements (max=24)

ERROR#8: redundant reactant/product

The more than one occurrence of the same substance was  found  on  the  same

side of the reaction, for example Air+Coal+Air=etc...

ERROR#9: too many compounds (max=128)

ERROR#10: reaction has no reactants

This can be due to an extra "=" sign or nothing on the left hand side of the

reaction.

ERROR#11: undefined substance

Every substance in the reaction must appear in the  database  as  either  an

element or a compound.

ERROR#12: unable to access database

This  can  be  due  to  it  not  existing  (e.g., you spelled it wrong), not

accessible  (e.g.,  hidden  or  locked),  too  many  files,  or  bad   PATH=

environment variable.

ERROR#13: compound contains no elements

Each  compound name in the database must be followed with the elements which

comprise the compound. These must be contained within brackets [].

ERROR#14: undefined mole fraction

You may have improper syntax resulting in CREST interpreting a  negative  or

zero mole fraction.

ERROR#15: unable to access data file

Either  the file does not exist, you typed the name in wrong, the disk drive

is down, or the file is read protected.

ERROR#16: reaction equations unstable

This may be due to an "ill posed" reaction, extreme temperature or  pressure

limits, or unreasonable specific heats.

ERROR#17: illegal file name

You  cannot  use spool or reaction filenames that begin with either CREST or

TPLOT. You can use data files that begin with either. In any case, never use

a  filename  of  TPLOT.OUT  as  this  will  be  destroyed  by  any  graphics

operations.

ERROR#18: compound contains too many elements (max=12)

ERROR#19: reaction contains too many reactants (max=128)

ERROR#20: reaction contains too many products (max=128)

ERROR#21: break detected

You must have hit a key while processing was in progress.

ERROR#22: reaction contains no elements

ERROR#23: reaction contains no compounds

ERROR#24: more than 128 simultaneous equations

The total number of simultaneous equations that must be solved in  order  to

determine  equilibrium is equal to the number of elements plus the number of

products. The number of elements adds to this total;   because  conservation

of these constitutes linear constraints which are implemented using Lagrange

multipliers. A 128x128 matrix of  single  precision  (REAL*4)  variables  is

accessible  through  large  memory  addressing.  Anything  bigger  than this

requires huge memory addressing  and  greatly  increases  the  overhead  and

subsequent  runtime.  For this reason, I don't even make this an option. You

will just have to make do with 128.

ERROR#25: unable to open plot data file CREST.OUT

ERROR#26: write error involving plot data file CREST.OUT

ERROR#27: unable to open plot command file CREST.PLT

ERROR#28: write error involving plot command file CREST.PLT

ERROR#29: illegal video mode... may be a hardware incompatibility

Your CRT should be in CGA or monochrome mode depending on the hardware.  Try

setting  the mode to "MODE CO80" if it is a CGA. If you have a CGA plasma or

LCD display and can't tell the highlighted or color lines from  each  other,

then try setting it to "MODE BW80".

ERROR#30: insufficient available memory

In  order  to solve the equations it is necessary to allocate a considerable

amount of memory. Your machine does not have sufficient available memory  to

process  the  request.  Either  solve  a  smaller  reaction  or find another

machine.

ERROR#31: substance element/compound ambiguity

This error will occur if a substance which appears in the reaction is  found

in  the  database as a compound and appears in itself or some other compound

as an element.

ERROR#32: undefined state code

The state code must be 0-5 which indicates respectively the following states:

unknown,  gas,  gaseous  ion,  aqueous ion, liquid, or dispersed solid. CREST

does  not  handle  precipitated  solids  which  have  a  vanishing   activity

coefficient.

.pa

.pn 0

APPENDIX 2

LISTING OF DATA FILE CREST.GAS

.pa

.de

.ad CREST.GAS

.ee

.pa

APPENDIX 3

LISTING OF DATA FILE CREST.AQU

.pa

.de

.ad CREST.AQU

.ee

.pa

APPENDIX 4

SAMPLE PROGRAM OUTPUT

.pa

CREST/V5.00: Chemical Reactions and Equilibrium developed by Dudley J. Benton

TVA Engineering Lab, Drawer E, Norris, TN, 37828 (615)632-1887

G.... Gibbs free energy [BTU/lb-mole]        H.... enthalpy [BTU/lb-mole]

S.... entropy [BTU/lb-mole/R]                Q.... heat transfer [BTU]

R.... gas constant [BTU/lb-mole/R]

Po... reference pressure [14.7 psia]         To... reference temperature [537R]

Pr... pressure of reactants [psia]           Tr... temperature of reactants [R]

Pp... pressure of products [psia]            Tp... temperature of products [R]

F.... activity coefficient                   Z.... compressibility (Z=PV/RT)

Y.... number of moles                        X.... mole fraction (X=Y/Ytotal)

Date: 08/10/91  Time: 2112     database: CREST.AQU        reaction: CEMENT.REA

*********************************** REACTION ***********************************

* aqueous reactions with cement and asbestos - use with database CREST.AQU

*

1000Water .5Cdiox .00001Odia .1Tricaaloxide .1Casilicateb .1Cawollastoni

.1Mgchrysotile .005Ca^^ .01Clmonox~

=

Water Cldia Cdiox Hdia Odia Algibbsite Tricaaloxide Calcite Caoxide

Casilicateb Cawollastoni Mghydroxide Mgchrysotile Sioquartz H^ Al^^^ Ca^^ Oh~

Cl~ Clmonox~ Ctriox~~ Hcarbonate~ Mg^^

********************************************************************************

solving reaction

elements=8, products=23, equations=31

iterations=164, seconds=35, dY=0.0001, dE=0.0001

Tr=580, Pr=14.7, Tp=580, Pp=14.7, Q/RTo=-33  (Isobaric)

substance    state       Y        X       Z     F       H/RTo     S/R      G/RTo

--------------------------------------------------------------------------------

Water         lqd1000.0000000 0.9990859  0.10  1.00    -114.5     9.11    -124.4

Cdiox         gas   0.5000000 0.0004995  1.00  1.00    -158.3    33.67    -194.6

Odia          gas   0.0000100 0.0000000  1.00  1.00       0.3    43.36     -46.5

Tricaaloxide  sol   0.1000000 0.0000999  0.01  0.10   -1444.5    27.63   -1474.4

Casilicateb   sol   0.1000000 0.0000999  0.01  0.10    -929.1    17.48    -948.0

Cawollastoni  sol   0.1000000 0.0000999  0.01  0.10    -658.4    11.57    -670.9

Mgchrysotile  sol   0.1000000 0.0000999  0.01  0.10   -1757.5    30.08   -1790.0

Ca^^          aqu   0.0050000 0.0000050  0.10  1.00    -218.9     5.82    -225.1

Clmonox~      aqu   0.0100000 0.0000100  0.10  1.00     -43.2    16.55     -61.1

--------------------------------------------------------------------------------

reactants        1000.9149170 1.0000000  0.10  1.00 -115077.9  9133.90 -124943.2

================================================================================

Water         lqd 999.5754395 0.9987319  0.10  1.00    -114.5     9.11    -124.4

Cldia         gas   0.0000000 0.0000000  1.00  1.00       0.3    61.67     -66.3

Cdiox         gas   0.0003200 0.0000003  1.00  1.00    -158.3    41.03    -202.6

Hdia          gas   0.0000000 0.0000000  1.00  1.00       0.3    44.79     -48.1

Odia          gas   0.0000000 0.0000000  1.00  1.00       0.3    59.48     -64.0

Algibbsite    sol   0.0999931 0.0000999  0.01  0.10   -1031.5    19.51   -1052.5

Tricaaloxide  sol   0.0000000 0.0000000  0.01  0.10   -1444.5    30.16   -1477.1

Calcite       sol   0.4803705 0.0004800  0.01  0.10    -485.8    12.70    -499.5

Caoxide       sol   0.0000000 0.0000000  0.01  0.10    -255.7     8.44    -264.8

Casilicateb   sol   0.0000000 0.0000000  0.01  0.10    -929.1    20.01    -950.7

Cawollastoni  sol   0.0001326 0.0000001  0.01  0.10    -658.4    12.23    -671.6

Mghydroxide   sol   0.0000000 0.0000000  0.01  0.10    -372.1    11.77    -384.8

Mgchrysotile  sol   0.0999777 0.0000999  0.01  0.10   -1757.5    30.08   -1790.0

Sioquartz     sol   0.1999004 0.0001997  0.01  0.10    -366.9     6.30    -373.7

H^            aqu   0.0000002 0.0000000  0.10  1.00       0.0    22.27     -24.1

Al^^^         aqu   0.0000000 0.0000000  0.10  1.00    -214.2    -4.16    -209.7

Ca^^          aqu   0.1245045 0.0001244  0.10  1.00    -218.9     2.60    -221.7

Oh~           aqu   0.2347642 0.0002346  0.10  1.00     -94.2     5.69    -100.3

Cl~           aqu   0.0100000 0.0000100  0.10  1.00     -67.4    18.31     -87.2

Clmonox~      aqu   0.0000000 0.0000000  0.10  1.00     -43.2    39.57     -85.9

Ctriox~~      aqu   0.0048516 0.0000048  0.10  1.00    -273.0     5.39    -278.8

Hcarbonate~   aqu   0.0144703 0.0000145  0.10  1.00    -279.0    22.12    -302.9

Mg^^          aqu   0.0000700 0.0000001  0.10  1.00    -188.2    -0.13    -188.1

--------------------------------------------------------------------------------

products         1000.8446655 1.0000000  0.10  1.00 -115110.8  9119.19 -124960.2

--------------------------------------------------------------------------------

difference         -0.0702515 0.0000000 -0.00 -0.00     -32.9   -14.70     -17.0

.pa

APPENDIX 5

PROGRAM OXMOLES

(COMPUTES COMBUSTION PRODUCTS VS. MOLES OXYGEN)

.pa

.de

.ad OXMOLES.C

.ee

.pa

APPENDIX 6

PROGRAM FINDENTH

(COMPUTES ENTHALPY OF COMBUSTION PRODUCTS VS. T)

.pa

.de

.ad FINDENTH.C

.ee

.pa

APPENDIX 7

SAMPLE PLOTS OF COMPUTED MOLE FRACTION VS. TEMPERATURE