Module User's Manual

Copyright (C) 1984-1990 by Mark F. Davis

All Rights Reserved

I. Introduction

Module is a program for designing and analyzing analog electronic

filters and equalizers.

It provides an interactive environment for entering a network  as

a  list of resistors, capacitors, and other elements; from  which

can  be  calculated the frequency and phase response,  poles  and

zeros, and tolerance sensitivity.  Families of frequency response

curves  can be generated corresponding to progressive changes  in

part  values. Additionally, one can specify a  desired  magnitude

&/or  phase  response  and an optimization  routine  will  adjust

network values, or poles/zeros, to best fit the desired response.

Module's  name  derives  from  its  ability  to  nest  previously

calculated  circuits  as  circuit  elements  called  modules  and

empiricals in other circuits; and thus model complex elements and

systems  in building block fashion. It is menu driven and  fairly

user  friendly; the best way to learn it is to poke  through  its

menus.  It is an adjunct to more traditional and general  purpose

analog  circuit analysis programs like Spice.  Module's  analysis

routines  currently deal only with a circuit's  incremental  (ac)

behavior,  on a linear basis, in the frequency domain.   It  will

not tell you a circuit's dc operating point, transient  response,

distortion,  or  temperature sensitivity, for example.   Nor  are

there  any  voltage, current, or power dissipation  limits.  (You

want  1 amp through a 1 Meg resistor? Fine.)  Module's  strengths

are its interactive environment and features like optimization.

It currently runs on the Ibm Pc and clones under MsDos,  requires

about  450K  bytes  of ram, and uses any  of  the  popular  IbmPc

graphics  cards  for  its graphical output.   Unix  and  Apple-II

versions  also exist.  Originally written in Basic,  the  current

version is written in C.  A companion program, Analyze4, provides

additional  plotting  and data manipulation  tools.   Module  and

Analyze4  do not require an 80n87 math coprocessor, but will  use

one if present.

Module  can be started by just typing its name, or the name of  a

Module  datafile  can be included on the  startup  command  line.

When terminated, Module attempts to store an environment file  in

the  active  directory  which includes the  name  of  the  active

datafile,  if any.  If subsequently restarted in  that  directory

without a command line filename, Module will retrieve the name of

the  last active datafile from the environment file, and  attempt

to read that datafile in as part of its initialization.

Module also stores temporary files while operating, so the active

directory/disk should be write enabled.

Module  and  windows: If an MsDos-compatible  window  environment

(such  as desqview or MsWindows) is available, it can be  helpful

to  dedicate one window to Module, a second window  to  Analyze4,

and  perhaps a third window to Dos or a word processor with  this

manual.

Running  multiple  simultaneous versions of Module  in  different

windows  is ok as long as each version of Module is running in  a

different directory, since each running Module will use temporary

files  in  its current directory, and  they  shouldn't  conflict.

II. Main Menu Selections in brief

Following  is Module's main menu.  In general, menu  options  are

enumerated,  and  single-key  selectable.   In  response  to  the

selection, the program will take some action, which in some cases

will  be  to put up a submenu, which in turn  may  have  sub-sub-

menus,  etc.  As a general rule, one can always get back  to  the

main  menu by repeatedly entering carriage-returns to any of  the

prompts.   Module  will usually indicate when it  is  awaiting  a

keypress or other user input with the caret prompt '>'.

Module's main menu:

Module

1 - Edit

2 - Compute freq resp

3 - Print

4 - Store

5 - Retrieve

6 - Calculation mode

7 - Desired resp

8 - Freqs

9 - Gain: Abs   Phase: Abs

A - Wiggle

B - Lp

C - Des <-> Calc

D - It

E - Plot

F - Wiggler: Simplex

G - Loop

H - Tolerance

I - dB per degree  0 / 1

J - Dir

N - Node xref

R - Reset wiggle

Q - Quit

>

Brief descriptions of the menu options:

1    Edit

This option invokes an editor; there are two.  The normal default

editor is the network editor, which allows entry and editing of a

circuit  as  a network list.  There is also a  pole/zero  editor,

which can be invoked by first setting Calculate mode 5 using main

menu option 6 ('Calculation mode', below).  The p/z editor allows

one  to enter and edit a list of poles and zeros, to be  used  by

Module in place of the network.

2    Compute frequency response

This will direct the program to calculate the frequency response,

magnitude  and  phase,  of  the  active  network,  providing  the

Calculation  mode  is  1 or 4.  In Calc  mode  5,  the  frequency

response  of the entered pole/zero array is  calculated  instead.

This option is discussed in greater detail below.

3    Print

Prints   the  current  network  list,  frequency  response,   and

poles/zeros  (if known) on the screen; waits for a  keypress  and

goes  back to the main menu.  The printout can be interrupted  by

hitting  ctrl-s, ctrl-numlock, or Pause, and resumed with  almost

any key.

4    Store

Stores  the existing circuit, including its responses  and  poles

and zeros (if known), in a user-named file.  Any existing file of

that  name  is overwritten.  The network must have at  least  one

element in it before a file can be saved.

The name of the last file either stored or retrieved, if any,  is

used  as the default filename when Module prompts for a  filename

to store the current data.  Hitting return to such a prompt  uses

the default filename and overwrites the file.

Module usually reminds the user to save the active data, but will

not automatically save a file except in exiting from a wiggle.

5    Retrieve

Retrieves  a  user-named  Module datafile from  the  disk.   This

includes the option of recalling only parts of a file (netlist  /

responses / poles-zeros).  If the file is not found, Module  will

so inform the user.

Except  when recalling only part of a file, the name of the  last

file  either stored or retrieved, if any, is used as the  default

filename when Module prompts for a filename to retrieve.  Hitting

return to such a prompt uses the default filename.

Once initiated, a file recall request can be canceled by electing

to recall the file in P(arts, then recalling none of the parts.

6    Calculation mode

Lets the user set which of 5 calculation modes the program uses:

1.   Computes poles and zeros from network,  then  frequency

response from poles & zeros.

2.   Not yet implemented.

3.   Not yet implemented.

4.   Numerical routine: Computes frequency response directly

from    network,   without    intermediate    pole/zero

calculation.

5.   Works  directly  with  user-entered  pole/zero list, no

network.  Causes pole/zero editor to be invoked by main

menu option 1.

For  some networks, Calc mode 1 is faster than 4, but 4 has  less

of a tendency to run into numerical overflow problems.  Calc mode

4  is  required (and will automatically be used) if  the  network

contains  any empiricals or modules, or if it  uses  differential

outputs.   See  section  VI, Calculating  response,  for  further

discussion.

7    Desired resp

Lets the user type in

- a desired dB magnitude response or

- a desired degree phase response or

- phase response weighting factors,

frequency by frequency.  Entering return to any of the  frequency

prompts is equivalent to leaving a value unchanged.

In  entering a desired dB magnitude response,  individual  values

can  be  designated  "minimum  acceptable  stopband  attenuation"

values by appending the letter 'm' to each value as entered.  The

error evaluation routine will count any corresponding  calculated

magnitude  response  value  of  the  Desired  value  or  less  as

contributing  0 error to the overall Rms error.  Stopband  values

are  indicated in the tabulated printouts by the  presence of  an

'm' next to the desired dB value.

The  phase  response  weighting  array  specifies  the   relative

contribution of individual phase errors to the overall Rms  error

when  wiggling to a desired response. The default values are  all

1.   This  feature is useful to design  filters where  the  phase

response is important over only part of the specified passband.

Stopband  responses,  indicated  with  an  appended  'm'  to  the

magnitude response (above), automatically ignore phase errors  in

the   error   calculation  regardless  of  the   weight  assigned

them.   Phase weights can be viewed by invoking this  option  but

not  changing any of the values. As yet there is   no   magnitude

weighting array.

8    Freqs

Lets  the user specify a list of frequencies for the  program  to

use.   The  frequency list is among the information  saved  in  a

Module data file (option 4) and restored when the  responses-part

of  the file is recalled with option 5.  Module uses the  31  Iso

standard 1/3 octave frequencies from 20 Hz to 20 kHz inclusive as

its default frequency list.  The maximum number of frequencies it

can currently handle is 50.

This  option  gives  the  user  6  ways  to  choose  a  list   of

frequencies, selectable by hitting the corresponding letter:

I - Iso standard audio freqs: 31 1/3 octave frequencies from  20

Hz to 20 kHz inclusive;

N - Nonstandard Iso freq range:  solicits for a first  frequency

and  a last frequency, and fills in a std value  1/3  octave

frequency list, with the specified endpoint values included;

M - Manual  entry:   solicits  for  individual  frequencies,  in

monotonically  increasing  order,   until  an  empty line is

entered;

L - Linear range: solicits for the number of frequencies to use,

then  solicits  for a pair of  endpoint  frequencies,   then

fills in a list of linearly spaced frequencies;

G - Logarithmic range: solicits for the number of frequencies to

use, then solicits for a pair of endpoint frequencies, which

should   both   be   positive,   then  fills in  a  list  of

logarithmically-spaced frequencies.

9    Gain

Requests in turn whether the user wishes to toggle the  program's

Magnitude and Phase Abs(olute/Rel(ative normalization flags.   In

relative  gain  mode, absolute gain errors are ignored,  and  the

overall  magnitude  response is gain shifted to  best  match  the

desired  response.   In  relative phase mode,  the  linear  phase

component of the calculated phase response is subtracted off,  so

that  only the deviation from linear phase is included  in  error

calculations.   This  latter mode is helful in  designing  linear

phase filters, using a mixed gain/phase criterion (option I).

Note  that proper operation of relative phase mode  requires  the

program to 'unwind' the phase, which in turn requires that  there

be  less  than  180 degrees of  phase  shift  between  successive

frequencies  in the band of interest.  Increasing the density  of

frequency values in regions of high phase shift may be  necessary

to  ensure this condition.  Proper operation also  requires  that

the  phase  shift at d.c. be 0 degrees - if  the  topology  being

optimized has 180 degrees phase shift at d.c., an inverter should

be cascaded with it (made up of, say, a pair of 10K resistors and

an  opamp  with 0 MHz GBw) during optimization to restore  the  0

d.c. phase shift condition needed by the program.  Make sure  the

tolerance  on  the two inverter resistors is 0,  so  the  wiggler

won't try to change them.

In general, the wiggler routines tend to converge a little faster

in Absolute gain/phase modes.

A   Wiggle

Invokes  the wiggler (optimizer) to adjust either network  values

(Calc  modes  1  & 4) or poles/zeros (Calc mode 5),  to  fit  the

Desired response.

Existing  network  (or pole/zero) values are used as  a  starting

point,  from which optimization proceeds.  The  initial  starting

data is stored in a Module datafile under the name  <filnam>.org.

Careful  choice of starting values can help the wiggler  converge

quickly.  When wiggling poles/zeros to fit a response (Calc  mode

5),  it can be helpful to avoid starting multiple poles or  zeros

with the same complex value.

Two  optimization routines are nominally supported; the one  that

gets  called is selectable with main menu option F.  The  Simplex

routine is recommended in most cases.

Once running, a wiggle can be interrupted by hitting any key.  It

may  take  a  moment  for  the  program  to  finish  the  ongoing

calculation   before   it  acknowledges   the   keypress.    Once

acknowledged,  the  program  saves the current  state  under  the

active filename, and returns to the main menu.

When started, the wiggler routine may ask whether the user wishes

to  'Continue from previous best soln (y/n, def n): '.  With  one

exception, the answer to this should always be 'no' (the 'n' will

suffice).   The  exception  is when an ongoing  wiggle  has  been

interrupted  by  hitting a key, then restarted  without  changing

anything,  in  which  case  continuing  from  the  previous  best

solution  can be a slight time saver.  Trying to  continue  after

changes  have  been made is likely to crash the  program,  or  at

least produce incorrect answers.

The  process of fitting a response can take from several  seconds

to many hours, depending on the circuit, the number of wiggleable

elements, the speed of the computer, etc.

Left  uninterrupted,  the wiggler will  eventually  converge,  at

which point it will save the file and return to the main menu.

Wiggler  routines  in general are subject to local  nulls,  which

means they may not converge to the solution you had in mind.  Try

to fit a five-pole response, for example, and it may move one  of

the poles out to zero or infinity (maybe an enormous resistor  or

infinitesimal  capacitor) and then try to use the rest to do  the

best 4 pole fit it can.  The user may have to negotiate with  the

routine:  from  an  initial starting point, let it  fall  into  a

possibly suboptimal solution, then freeze or restart certain part

values  as necessary so that it detects the desired solution  and

converges to it.

Optimization is no substitute for knowing what you're doing.

B    Lp

Lp stands for lineprinter.  This option prints a tabular  summary

of  the state of things on the lineprinter.  The  information  is

essentially  the same as that printed on the screen by main  menu

option 3.

C    Des <-> Calc

This swaps  the current Calculated magnitude/phase responses with

the Desired responses arrays, and returns to the main menu.

One use of this feature is in designing a circuit to imitate  the

response of another circuit: calculate the response of the target

circuit, swap the response into the active Desired array,  recall

the  netlist of the new circuit, (save the data in a  new  file),

and run the wiggler.

A second use is in manually specifying the response of a  module:

enter  the data as a Desired response (main menu option 7),  then

swap it into the active Calculated response array and save it  in

a file.

D    It

Resets  to  zero the iteration counter advanced  by  the  wiggler

routine.   The iteration counter has no real  significance,  it's

just  displayed  by the wiggler routines while  they're  running,

fyi.

E    Plot

Calls  up  the plot menu.  This allows  plotting  of  Calculated,

Desired,  or  Error ( = Calculated -  Desired)  magnitude  and/or

phase responses, in any combination, by manipulating plot toggles

6  thru  b.  Option 1 puts up the plot.  Option 2 allows  one  to

specify  values  for the vertical limits of  the  plot;  separate

limits are used for plotting dB and degree data.  Option 3 allows

specification of a marker frequency, which appears in the plot as

a  dotted vertical line.  Option 4 invokes a  graphic  screendump

routine. Option 5 toggles the autolimits routine on or off  ('on'

indicated with an asterisk).  Autolimits is automatically  turned

off if specific limits are specified with plot option 2.

The  horizontal  scale  can  either  be  linear  or  logarithmic,

selected  with plot option H.  Avoid the use of logarithmic  mode

with frequency lists that include frequencies of 0 or less.

On the Ibm Pc, plot menu option C is used to toggle the choice of

graphics monitor.

If screendumps are desired, choice of printer can be made  within

the screendump option 4.  Supported printers include Epson and Hp

ThinkJet  and DeskJet.  Hp Laser printers should also  work  with

the DeskJet settings.

F   Wiggler

Toggles  the  choice  of wiggle (optimizer)  routine  which  gets

called  when  menu  option  A  is  invoked.   Two  routines   are

available,  called Newton, and Simplex.  Newton (in this  version

at  least) tends to converge quickly with some circuits, but  can

get lost with others.  Simplex is more reliable, and will  rarely

fail to converge, but it's a little plodding at times.

When  wiggling  pole/zero  values  (Calc  mode  5)  or   networks

containing   modules  or  empiricals,  the  Simplex  wiggler   is

automatically used.

G   Loop

Brings  up the loop menu.  This menu allows the user  to  specify

one  or more parts from the network list, give initial and  final

values  and a loop count, and then run the loop.  The  loop  code

will then vary each element in the loop list from its starting to

its final value in Nloops steps, calculate the frequency response

for  each set of values, and store all the responses in  a  file.

The file stored by the loop option is not a standard Module  data

file, in that it has no netlist data, and an arbitrary number  of

frequency   responses.  Module  datafiles  contain  4   frequency

responses,  so  unless  Nloops happened to have  been  4,  Module

cannot  read the loop file.  Loop response files  with  arbitrary

number  of  frequency  responses can be  read,  manipulated,  and

plotted with Analyze4.

The loop menu options are:

1  -  Number  of  loops: sets number of  loops  to  be  executed,

typically 5 to 10;

2  -  Execute loop: executes the loop once the other  information

has been entered;

3  -  Add  element: solicits for the network  list  index  of  an

element,  lists its current network list  values,  solicits

for a starting loop value, a final loop value, and  whether

the loop increment should be additive (A) or multiplicative

(M), then adds that element to the loop list.

4  -  Edit  element: solicits for the network list  index  of  an

element  already  in the loop list, then solicits  for  new

starting  and  ending  values and  updates  the  loop  list

accordingly;

5  -  Delete element: solicits for the network list index  of  an

element already in the loop list, then deletes that element

from the loop list;

6  -  Loop filename: solicits for the name of a file in which  to

store the loop responses, which should be distinct from the

name  of the Module datafile name; the default name is  the

first name of the active Module file, appended with '.lps'.

Most  of  the loop menu operations refer to parts  by  the  index

number  they  are listed with by the network  editor,  main  menu

option  1.  It is sometimes necessary to temporarily come out  of

the  loop  menu and invoke the network editor to  see  what  that

index is for a particular part.  Once a part is selected for  the

loop  list, it is listed in the loop list with its  network  list

index.

Hint:  a  common  part  to loop  is  a  potentiometer,  which  is

represented in the network list as a pair of resistors sharing  a

common  node  (the  wiper).  Looping across the  range  of  wiper

settings  is generally a matter of entering both resistors in the

loop list, having one resistor increase (usually Arithmentically)

each time through the loop while the other resistor decreases  by

the  same  amount.   Remember that resistors in  Module  are  not

allowed  to  have values of 0, so one would  typically  vary  one

resistor  from  1 ohm to full value-1, while the  other  resistor

varies from full value-1 to 1.

H    Tolerance

Main  menu  option H will bring up the tolerance  submenu,  which

allows  choice  of one of two tolerance-related  routines:  Monte

Carlo and tolerance estimation.  The Monte Carlo routine randomly

varies parts values by their current tolerances, keeps statistics

on the variation in output response, and stores the high and  low

envelope responses in an Analyze4-compatible file. The  tolerance

estimation routine solicits for a dB bound on response variation,

and  calculates the required tolerances of the network  parts  to

meet the stated response criterion.  These routines are discussed

in greater detail in section VII, below.

I    dB per degree

This  option allows one to specify the relative weighting  of  dB

magnitude  response  error and degree phase response  error  when

trying  to fit a mixed  magnitude / phase desired response.   For

example, values of 1 / 10 would signify that 1 dB of error should

be given the same weight as 10 degrees of phase error.  Only  the

ratio  of the two numbers really matters.  If either quantity  is

0,  it means it is all important, and the other response  measure

should  be  ignored.   Values of 0 / 0 are  not  permitted.   The

default  values are 0 / 1, meaning ignore phase response and  fit

only to dB magnitude response.

J  Dir

Prints the contents of the currently active directory.

N  Node xref

When  there is an active netlist, this indicates which nodes  are

connected.

R  Reset from last wiggle

Reads the file <filnam>.org into Module's active memory,  without

changing  the default file name, or the contents of  the  default

file.  <filnam>.org is stored by the wiggler routine at the start

of  a  wiggle, so if the user is unhappy about the results  of  a

wiggle, it can be undone with this option.

Q    Quit

Exits  Module immediately back to the operating system.   Prompts

you if you forgot to save the active data in a file.

-----------------------------------------------------------------

The  next  section  of  the  manual  discusses  the  details   of

electrical  networks in Module. Following sections describe  some

of the menu options in greater detail.

-----------------------------------------------------------------

III. Networks

An  electrical  circuit is characterized in Module as  a  network

list,  entered and maintained with the network editor, main  menu

option  1,  w/Calcmode 1 or 4. Each element in the  network  list

consists of:

1. a part type - resistor, capacitor, inductor, transistor,

op-amp, empirical, or module;

2. a value, or two, if applicable;

3. 2 or 3 node numbers;

4. a tolerance.

Module's circuit analysis routines calculate the voltage transfer

function  as a function of frequency from the input node  to  the

output node(s).  If the circuit is in fact unstable, Module  will

just  calculate the response the circuit would have had,  had  it

been stable.  The main way to check stability is to note  whether

there  are  any right half plane poles (poles  with  non-negative

real parts), assuming Calcmode 1.

III.1 Nodes

Nodes  are  numbered  consecutively from 0.  Node  0  is  ground.

(Power  supply rails and reference dc sources act as ac  grounds,

and should be indicated, where they occur, as node 0.)

Node  1  is the input, assumed fed from a voltage  source.   (If,

e.g.,  one wanted to account for a 50 ohm source  resistance,  it

could  be connected from node 1 to (say) 2, and the rest  of  the

circuit  fed  from node 2).  Any number of parts can  connect  to

node 1 (or any other node) at the same time.

After nodes 0 and 1, the assignment of other nodes in the circuit

is the arbitrary choice of the user, except the numbering must be

consecutive.  (e.g. If node 4 is eliminated from a circuit with 5

nodes, all instances of node 5 must be renamed node 4 to  restore

consecutive  numbering.)   If  a node number  is  skipped  and  a

circuit analysis routine is called (either by the user with  main

menu  option  2, or by a wiggler, tolerance,  or  loop  routine),

Module  will usually stop, inform the user of the  missing  node,

and return to its main menu.  (Other times it just crashes).

As  noted,  Module's  circuit  analysis  routines  calculate  the

voltage transfer function as a function of frequency from node  1

to the output node(s).  Any node can be the output node, which is

selected  with  network editor menu option 4  ('Out  node').   If

Calculation mode 4 is active, differential output is possible  by

specifying two nonzero output nodes (otherwise the second  output

node should be entered as 0).

Hint:

A cheap and dirty way to avoid having to renumber nodes when  one

is  deleted  is  to  tie the unused  node  to  ground  through  a

resistor.  The analysis routines won't run as fast as if the node

were  deleted and the remaining nodes renumbered, but it  may  be

worth it as a quick fix.

III.2    Tolerances

Tolerances  are  (usually) non-negative quantities  expressed  in

units  of percent (%).  Module keeps track of tolerances only  to

the nearest tenth of a percent (0.1%).

Only  resistors,  capacitors  and inductors  can  be  assigned  a

tolerance.   When a new R, C, or L is added to the  network  list

via  the network editor's option 1, the editor solicits for  node

numbers and values of the new element, and automatically supplies

an  initial  tolerance  of 5%.  This can then  be  modified  with

editor option 2 ('Edit an element').

Once set, tolerances are left undisturbed by the various routines

in Module, except for the tolerance estimation routine (main menu

option H, submenu option 1), which resets all positive tolerances

to values necessary to meet the indicated response criterion.  If

if is desired to preserve the previously assigned tolerances, the

state  of the program should be saved in a separately-named  file

before running the tolerance estimation routine.

To a number of routines, tolerances acquire special significance.

Specifically,  the wiggler and tolerance routines will treat  the

value of any part with a 0 tolerance as fixed, and will otherwise

ignore  it  in  error or tolerance calculations.   It  is  common

practice  to temporarily zero some of the tolerances while  using

these routines, in part to help them run faster.

III.3   Part types

Module works with 7 basic circuit elements:

1. Resistor

2. Capacitor

3. Inductor

4. Opamp

5. Transistor

6. Empirical

7. Module

Module  expresses  and  expects values to  be  expressed  in  the

following units:

resistor:      ohms;

capacitor:     nano-farads (nF);

inductor:      milli-henries (mH);

op-amp:        (characterized   by  gain-bandwidth  product,

GBw, expressed in units of mega-Hertz (MHz));

transistor:    (characterized by quiescent collector current

Ic0    in   milli-amps   (mA),    and    beta

(dimensionless));

empirical:     real  and  imaginary  dB-ohms

(= 20 * log10 (ohms)) as functions of freq;

module:        dB response and degree response as  functions

of freq.

Most  values  can be optionally entered in  scientific  notation;

e.g. 4000 ohms or 4e3 ohms.

Values  of 0 ohms resistance or 0 mH inductance are not  allowed,

and will likely cause Module to crash.

III.3.a   Resistor

Resistors are two-terminal devices modeled by their resistance in

ohms, assumed to be non-zero and independent of frequency, and  a

tolerance.   There is no stray reactance assumed.  The  order  of

the two nodes is irrelevant.

v = i * R         v in volts, i in amps, R in ohms

Stray  inductance/capacitance  effects can be  accounted  for  by

adding  extra  L's and C's as desired, at the cost  of  increased

circuit complexity and calculation time.

There  is no explicit way provided to model  nonlinear  resistive

effects, such as the skin effect.

III.3.b   Capacitor

Capacitors   are   two-terminal  devices  modeleled    by   their

capacitance,  in nanoFarads and a tolerance.  They are  lossless,

and  have  no  stray reactance.  The order of the  two  nodes  is

irrelevant.

v = i / (j * 2 * pi * f * C * 1.e-9)

v in volts, i in amps, f in Hz, C in nF, j = sqrt (-1)

III.3.c   Inductor

Inductors are two-terminal devices modeled by their inductance in

milliHenries, assumed to be non-zero,  and a tolerance.  They are

lossless, and have no stray reactance. The order of the two nodes

is irrelevant.

v = j * 2 * pi * f * L * 1.e-3

v in volts, i in amps, f in Hz, L in mH, j = sqrt (-1)

III.3.d   Op-amp

Op-amps  are  three-terminal  devices  modeled  as   differential

voltage-controlled  voltage  sources,  where,  at  least  at  low

frequencies:

Vout = K * (Vin+ - Vin-)                     (1)

with      K = 100,000 = low frequency open loop gain

Op-amps have infinite input impedance, zero output impedance, and

unlimited output drive ability. It is not a good idea to  connect

the  output  of an opamp to nodes 0 or 1, or to the output  of  a

module or another opamp. Finite input and output impedance can be

modeled by explicitly adding appropriate shunt/series impedances.

Op-amp nodes are always expressed and solicited in the  following

order:

-  non-inverting (V+) input

-  inverting (V-) input

-  output

The single value associated with an op-amp is its  gain-bandwidth

product  (GBw),  expressed  in units of  MHz  (mega-Hertz):  that

frequency  where  the open loop gain becomes  unity,  assuming  a

single  pole  rolloff within the op-amp  continuing  to  infinite

frequency.  Typical values of GBw for modern day general  purpose

opamps are in the range 1 to 10 MHz.

The full transfer function of the op amp model is therefore

Vout = (K / (1 + j * (GBw / K))) * (Vin+ - Vin-)        (2)

(j  =  sqrt  (-1)),  i.e. a 1 pole low pass  filter  with  a  low

frequency gain of K = 100,000, a corner frequency of GBw / K, and

a gain of 0 dB at GBw MHz.

Module recognizes a magic value of GBw, namely zero, to allow the

user  indicate an op-amp with infinite (not zero)  gain-bandwidth

product.   An op-amp with Gbw set to 0 is just a diff amp with  a

gain  of K = 100,000 at all frequencies (eqn (1) instead  of  eqn

(2)).   Using  0  GBw  can  be  useful  for  observing  idealized

performance,  modeling  other  types of  circuit  elements  (e.g.

voltage-controlled voltage source) and for making the job of  the

pole/zero circuit analysis routine easier.

Module  does not allow the user to modify the op-amp lf  gain  of

100,000,  except  by  adding  attenuators  or  cascading  opamps,

(typically  some  or all with 0 Gbw). There is  explicit  way  to

specify infinite open loop gain.

Lacking  offsets, distortion or noise, Module's opamps work  just

fine  open loop, and can be cascaded to arbitrary  levels.   Real

opamps  may  not  be  as  cooperative.   As  noted,  Module  will

calculate the frequency response of circuits whether or not  they

are stable.

A perfect unity gain buffer, with infinite input impedance,  zero

output  impedance, and gain of 1.0000 at all frequencies, can  be

constructed as follows:

Output node:  3

Part           Nodes            Val            Tol

1. Op amp        1  2  3            0  mHz

2. Resistor         2  3            1 ohm          5 %

3. Resistor         2  0        99999 ohms         5 %

Module  does  not wiggle (try to optimize) any opamp  values;  it

ignores  any tolerance specification for op-amps,  and  generally

treats them as having 0% tolerance.  If op-amp sensitivity is  an

issue,  one should manually vary the GBw value of  suspected  op-

amps and recalculate the response.

Note that Module doesn't explicitly provide a  voltage-controlled

voltage source as a separate primitive building block, since  one

can  be built from the op-amp (possibly running open loop  and/or

with  '0'  Gbw) and suitable attenuation. The usual  problems  of

running op-amps open loop (noise, saturation, stability) are  not

a problem with models used in Module.

III.3.e   Transistor

The  transistor  is a three-terminal device  modeled,  using  the

simplest  possible hybrid-pi embodiment, as a  voltage-controlled

current source and a finite input resistance, both independent of

frequency.

Module  requests  two  values  for  each  transistor:  Ic0,   the

quiescent   dc   collector   bias  current   in   units   of   mA

(milliamperes), and beta (dimensionless), the current gain (ic  /

ib).  The default value of beta is 150.

From   the   value   of  Ic0  in  mA,   Module   calculates   the

transconductance, gm,  as:

gm = .04 * Ic0      (amps out /  volts in = mhos)

where:

Ic0 = quiescent collector bias current in mA,

.04 = kT/q at room temp

Module also calculates the base-emitter resistor Rpi as:

Rpi = beta / gm

For  example, Ic0 = 1 mA results in gm = .04 mho and, for beta  =

150, Rpi = 3750 ohms.

The voltage-controlled current source is connected from collector

to  emitter, its instantaneous value = ic proportional to the  ac

base-emitter voltage, vbe:

ic = gm * vbe                                (3)

Transistors  thus  have finite  input  (base-emitter)  resistance

(=Rpi),    infinite   output   impedance,   and    finite    gain

(transconductance = gm).

They  also  are assumed to have  unlimited  voltage/current/power

dissipation,  and remain linear and noiseless, regardless of  the

drive level.

Module doesn't care whether a transistor is NPN or PNP.

Transistor  nodes  are  always expressed  and  solicited  in  the

following order:

-  base

-  emitter

-  collector

Module  ignores any tolerance specification for transistors,  and

generally  treats  them as having 0%  tolerance.   If  transistor

sensitivity is an issue, one should manually vary the beta and/or

bias current values of suspected transistors and recalculate  the

response.

If more accurate transistor modeling is desired, several  passive

parts can be added to each transistor, including:

- a base-emitter capacitance, typically in the range

5-50 pf

- a collector-base capacitance, usually 1-2 pf

- a collector-base resistance, around 1 Meg

Module  doesn't explicitly provide a  voltage-controlled  current

source  as a separate primitive building block, since one can  be

built  from  the  transistor (possibly  fed  from  a  unity-gain-

connected op-amp to provide arbitrarily high input impedance) and

suitable  attenuation.  A handy collector bias current  level  to

remember  in  this regard is Ic0 = 25 mA.  At this  dc  collector

current  level, the transconductance is 1.0 amp out per  volt  in

(mho), regardless of beta.

Hint: Calculating impedance

To have Module calculate the input impedance of a circuit:

- rename the input node (1) to the next available node #

- connect a transistor with base = node 1, emitter = node 0, and

collector = input node, with Ic0 = 25 mA (i.e. gm = 1.)

- set Output node to be collector/input node

- Calculate response, which will be input impedance magnitude in

units of dB-ohms and phase in units of degrees.

To  measure  impedance between two points,  neither  ground,  use

strategy similar to above, with two transistors connected out  of

phase, one collector at each node, differential output voltage.

III.3.f   Empirical

An empirical is a two-terminal (complex) impedance  characterized

by  a  file of tabulated values rather than a model.  It  can  be

useful, e.g., for

- nesting  multi-element impedance networks within a  larger

network

- getting  empirically measured impedance data into  Module,

e.g. loudspeaker driver impedance curves

An  empirical  impedance  file  has  (at  least)  two   frequency

responses  in  Module/Analyze4  format  (see  appendix  A):   the

magnitude  of the impedance, in dB-ohms = 20 * log10 (ohms);  and

the  phase angle of the impedance in degrees.  Other  information

may follow these two curves, but it will be ignored when the file

is read as an empirical.

A datafile saved from Module can function as an empirical element

file if the impedance data is saved as the dB and degree  Desired

response  arrays, which are curves 1 and 2 in a Module  datafile.

Curves 3 and 4 in a Module datafile are the Calculated  magnitude

and phase responses.

To model the input impedance of an existing Module network as  an

empirical file, calculate the input impedance of the network (see

previous section), swap it into the Desired response arrays  with

main menu option C, and store under a new filename.

To use empirical data (ex: loudspeaker impedance):

- measure impedance magnitude (ohms) and phase (degrees) at  each

frequency of interest;

- convert ohms values to dB-ohms by taking log10 and  multiplying

by  20

(there's a handy accessory program, 'db', to do this;  typing

'db  {some  number}'  to the system prompt  will  return  the

equivalent number of dB's.  There's also an accessory program

called undb which does the reverse.)

-  enter  the  dB-ohms values in Module as  a  Desired  magnitude

response,  and  the  phase angle values  as  a  Desired  phase

response    (main menu option 7), and save the file.

Thereafter, the empirical can be embedded in any other network by

giving  the network editor (option 1 'Add') its filename and  two

nodes, whose order is irrelevant.

An empirical can be used multiple times within a network.  Module

doesn't actually read the empirical impedance file until asked to

calculate the response of the network the first time.  Thereafter

the impedance data is retained in memory until a new network file

is read in, or until Module is terminated.

The frequencies used in an empirical impedance file should  match

or be a superset of the frequencies used in a Module network file

in  which the empirical is embedded.  If Module fails to  find  a

particular frequency's response in an empirical file, or fails to

find the file, the results are uncertain and dubious.

Of  the two circuit analysis routines available to Module  -  the

pole/zero  routine used in Calcmode 1 and the  numerical  routine

used  in  Calcmode 4 - only the numerical routine  is  compatible

with  empirical impedance files.  Module will  automatically  use

the  numerical routine with any network containing an  empirical,

regardless   of  the  compute-mode  set  by  the  user,   without

announcing the fact.  You cannot get Module to tell you the poles

or zeros of any circuit containing an empirical.

Module treats empiricals as having 0 tolerance.

III.3.g   module

[Note:  Module  (cap  M) is a program, module  (little  m)  is  a

circuit element.]

A module is a circuit block having an input node, an output  node

(in  that  order)  and a  tabulated  mag/phase  voltage  transfer

function  stored  in a file. Modules are  typically  things  like

amplifiers  or filters, but they can be any ~linear system,  such

as  a  transducer.   Most circuit files produced  by  Module  the

program  can be used as modules within other  circuits,  allowing

circuits to be nested.

A module can be included within another network by specifying  to

the network editor:

- the module's filename

- the node within the current circuit to which the input of

the module is to be connected

- the node  within the current circuit to which the  output

of   the module is to be connected

When  Module  proceeds to calculate the response of  the  current

network,  it  will read the Calculated responses section  of  any

unread  module  file(s) referenced in the current  network  list.

This means:

-   it  is necessary to first calculate the  response  of  a

module, and save it as a file, before it can be embedded

in another circuit;

-   since input and output loading effects of a  module  are

ignored,  a  module  is  effectively  assumed  to   have

infinite  input  impedance and  zero  output  impedance.

[It's typically a stage fed from an op-amp and ending in

an op-amp.]

If the loading effects of a module have to be included, these can

often  be included as empirical impedance elements,  or  explicit

loading networks in the current circuit.

It  is  possible  to  contruct a  module  file  from  empirically

measured  data.   This can be a useful way, e.g.,  to  model  the

acoustic response of a loudspeaker driver.  To wit:

-  measure the transfer function, in dB and degrees, at each

frequency of interest;

-  enter  the measured data as Desired magnitude  and  phase

responses, via main menu option 7;

-  use  main  menu option C  to  swap  the  entered  Desired

responses into the Calculated responses arrays;

-  if  the network list is empty, swing through the  network

editor  and enter at least one element of any  type  into

the  network  list.  This hack is necessary  to  convince

Module to save the file;

-  save the file.

Once stored as a file, the module can be embedded in a network by

giving  the network editor its filename and the input and  output

nodes.

A  module  can be used multiple times within a  network.   Module

doesn't  actually read the module file until asked  to  calculate

the  response  of the network.  Thereafter  the  module  transfer

function  data is retained in memory until a new network file  is

read in, or until Module is terminated.

The  frequencies  used  in a module file should  match  or  be  a

superset  of  the frequencies used in a Module  network  file  in

which the module is referenced.

Of  the two circuit analysis routines available to Module  -  the

pole/zero routine and the numerical routine - only the  numerical

routine   is   compatible  with  module   files.    Module   will

automatically   use  the  numerical  routine  with  any   network

containing  a module, regardless of the Calculation mode  set  by

the user, without announcing the fact.  You cannot get Module  to

tell you the poles or zeros of any circuit containing a module.

Module treats modules as having 0 tolerance.

Note on file formats:

Where an empirical impedance is contained in curves 1 and 2  (the

Desired  response  arrays)  of a Module  datafile  (see  previous

section),  the tabulated magnitude/phase response of a module  is

stored  in the Calculated response array section, curves 3 and  4

in a Module file.

The next sections of the manual describe some of the menu options

in greater detail.

IV  The Network Editor

As  long  as  the Calculation mode is not 5,  hitting  main  menu

option  1 brings up the network editor, which allows the user  to

enter and modify a circuit as a network list.

The six things the network editor knows how to do directly are:

1. Add an element to the network list;

2. Edit an element in the network list;

3. Delete an element from the network list;

4. Specify the output node, or nodes;

5. Clear the entire active network from memory;

6. Set  the  tolerance  of all applicable network elements  to  a

value.

When  the  editor is invoked, it will print the  current  network

list, and a prompt which looks like:

1. Add  2. Edit  3. Delete  4. Out node  5. Clear  6. Set tols  >

Hitting  Return  at that point will return directly to  the  main

menu.   Entering a number from 1 to 6 will execute  that  editing

function.  The  prompt  is repeated after  each  of  the  editing

operations,  to which an entered Return will reprint the  network

list,  and a second Return at that point will return to the  main

menu.

The details of the six editing operations:

IV.1.  Adding an element

The  user is prompted for one of the 7 basic part types,  entered

by number; any other key returns up one level.  The prompt  looks

like:

Type  (1 Res  2 Cap  3 Ind  4 Opamp  5 Transis  6 Emp  7 Module):

The  user is then prompted for the appropriate number  of  nodes,

entered  one  per line.  The node order is  irrelevant  for  part

types resistor, capacitor, inductor, and empirical, each of which

has two nodes.  For an opamp, the node order is:

+ input, - input, output.

For a transistor, the node order is:

base, emitter, collector.

For a module, the node order is: input node, output node.

The  user is then prompted for a value for the element, with  the

expected units prompted in parens.

After the value is entered, the editor will assign a tolerance of

5%  if  the  element is an R, L, or C, and in any  case  add  the

element  to the network list.  Module automatically  numbers  the

elements  in a network list consecutively from 1. The order  that

circuit elements appear in the network list is irrelevant to  the

analysis routines.

IV.2. Editing an element

If network editor option 2 is selected, the user will be prompted

for the number in the network list of the element to be edited:

Element #:

The  user  should enter the number of the element  in  the  list.

Hitting  return  at that point will return to the  6-way  editing

prompt. If a legitimate number is entered, an indexed synopsis of

the elements values will be printed, e.g.:

1 - Type  2   Capacitor

2 - Nodes    2  3

3 - Value   10    nF

4 - Tol  5  %

>

Entering  a  number  from 1 to 4 to this prompt  will  cause  the

program  to solicit for new values of the  indicated  quantities,

and reprint the indexed data.  Hitting return to the prompt  will

return to the 6-way editing prompt.

IV.3  Deleting an element

Selecting network editor option 3 'Delete' will cause the program

to prompt for an Element #:.  Entering a number between 1 and the

number of elements will delete that element from the list. Return

without entering any number to cancel the delete command.

After  an  element is deleted, any higher numbered  elements  are

renumbered down one to eliminate the gap in the list.  This makes

deleting  multiple  elements  easier if carried  out  in  reverse

numerical  order.  For example, deleting elements 3, 5 and  7  in

reverse  order would involve deleting element 7, then 5, then  3.

In  forward order, it would be 3, then 4, then 5; because of  the

renumbering between each delete.

IV.4  Out node

Option  4 will solicit for 2 output nodes, entered one per  line,

across which the output voltage appears.  The second entered node

is  normally  0,  but  can be  meaningfully  set  to  a  non-zero

differential output node if the Calculation mode is 4.

IV.5  Clear

Completely  clears the active in-memory circuit.  Erases all  the

elements,  resets the output nodes to 0, and clears  the  default

filename.   Before taking this action, the program will give  the

user  one  last  chance  to return  without  clearing  ('Are  you

sure?').

IV.6  Set tols

This option will solicit for a value, and then set all R, L and C

tolerances in the list to that value.

IV.7  Miscellany

The  current maximum number of parts in a given network  list  is

50.   A circuit can have up to 30 nodes.  There can be up  to  55

frequencies in the frequency list.

Exiting  the  editor  and returning to the  main  menu  does  not

automatically save the data in a file; that is the responsibility

and option of the user.

V. The Pole/Zero Editor

If  the  Calculation mode is 5, hitting main menu option  1  will

bring   up  the  pole/zero  editor,  a  simple  environment   for

maintaining a list of poles and zeros.

A pole or a zero to Module is a complex number having a real part

and an imaginary part, both in units of Hertz.  If the  imaginary

part  is  unequal  to  zero,  Module  automatically  assumes  the

existence of the complex conjugate singularity, so it should  not

be explicitly entered.

The p/z editor prints the current active list of poles and zeros,

then  waits  for the user to indicate a desired action.  The  p/z

editor can do 11 simple tasks:

1 - Add zero 3 - Edit zero 5 - Delete zero 7 - Inc  zero 9 - G0

2 - Add pole 4 - Edit pole 6 - Delete pole 8 - Inc pole  a - F0

b - Lhp only

The  user  selects an action by entering a number from 1 to 9,  a

or   b,  or   enters   Return to get  back  to  the  main   menu.

Corresponding operations for poles and zeros are identical except

for the type of singalarity they affect.

-  The Add options will solicit for the real and imaginary  parts

of the singularity to be added:

Real, Imag:

These  two values should be entered in units of Hz.   The  editor

will  then  print the updated p/z list on the  screen  and  await

another directive.

-  The  Edit options allow one to enter new  real  and  imaginary

values for one of the active singularities in the list.  If there

is  more  than one singularity of the corresponding type  in  the

list,  the  user  will first be prompted for  the  index  of  the

singularity to be edited:

Index:

Unlike  the network editor, which prints the element  indeces  on

the screen, the p/z editor expects the user to supply the desired

index by inspection or counting, since there usually aren't  that

many  poles  or  zeros in the list.   They  are  each  enumerated

consecutively from 1, increasing left to right, top to bottom.

The  editor then indicates the current real and imaginary  values

for that singularity, and requests two new values:

Real     Imag

Current: -255      976

Revised:

The  user can enter two values (first real, then  imaginary),  or

enter return to leave existing values unchanged.

After  the two edited values are entered, the editor updates  the

screen display of the pole/zero list and awaits further commands.

If  one  attempts to edit poles or zeros when  there  aren't  any

already in the list, the program will just reprint the screen and

await another command.

-  The  Delete options of the p/z editor allow one to  delete  an

individual pole or zero from the list.  If there is more than one

of the indicated singularity in the active list, the program will

first prompt for the Index of the singularity to delete.

After deletion, any higher numbered singularities are  renumbered

down one, to fill the gap; so as with deleting network  elements,

it  may  be  easier  to perform  multiple  deletions  in  reverse

numerical order.

Any  attempt to delete an element for an already empty list  will

be treated as a no-op by the program.

- The Inc options allow the user to set the Incs, or  increments,

of  the active singularities.  Although an Inc can be set to  any

integer value, its only significance is to the wiggler  routines,

as  a flag: a zero Inc will freeze the corresponding  singularity

value, any nonzero value will let the wiggler move it.

Real and imaginary parts have separate Incs, to allow them to  be

individually wiggled or frozen.

In  addition  to  singularity parts with 0  Incs,  the  optimizer

routines  will decline to wiggle any value with a starting  value

of  0, in part to avoid turning real singularities  into  complex

conjugate pairs.

- G0 and F0: Since  pole/zero  frequency  responses (compute mode

5)  do  not specify  an  absolute  gain,  F0 and G0 are  provided

in  Absolute gain mode to  specify gain  normalization.  The  p/z

response routine will normalize the frequency response  to have a

value of G0 dB at a frequency of F0 Hz.   The value  of  F0  need

not  be one of the frequencies  in  the  active  frequency  list;

e.g. it is permissable to set a normalization  of 0 dB @ 0 Hz for

a  lowpass  filter,  even though the  frequency   list  has   all

nonzero  values.   [Putting a value  of  0  Hz  in  the frequency

list   will  cause problems for  the   plotting   routine,  which

uses   a   log  frequency scale.  The   rest   of   the   program

doesn't  care.]  In  relative gain  mode,  the  response  routine

adjusts  the  overall  p/z response to  best  match  the  desired

response,  and the values of F0 and G0 do not matter, so in  that

mode they aren't displayed by the p/z editor.

- Lhp poles only:  In the real world, stable circuits have  poles

strictly  in the left half plane, i.e. with real parts less  than

zero.  Module makes no particular distinction, and will calculate

the frequency response of any pole/zero array, regardless of  its

stability.  The wiggler can be effectively prevented from  moving

poles into the right half plane by toggling 'Lhp poles only' mode

"on"  via  P/z editor option 'b'. Indication that the  option  is

active  is provided by  the appearance in the P/z editor  of  the

legend 'Lhp poles only' after the listing of pole/zero values and

incs.   When  active, the Rms error of any p/z list with  one  or

more poles in the right half plane will be assigned an impossibly

high error value of 1 million, which will discourage the  wiggler

from  moving  them  there.  With 'Lhp poles only'  on,  the  user

should avoid starting the wiggler with poles already in the right

half plane, as this will confuse the wiggler and cause it to exit

without doing anything.

- Miscellany

The program can currently handle up to 22 complex entries in each

of the pole and zero active lists.

VI  Compute Frequency Response - main menu option 2

When  invoked,  this option calculates a frequency  response  and

returns  to the main menu.  The actual routine called depends  on

the  Calculation  mode, set with main menu option  6.   The  same

routine is generally used whether called directly from main  menu

option  3, or from some other routine.  Exception: the  numerical

analysis  routine associated with Calc mode 4 will be  used  with

any network containing a module or an empirical.

-In Calcmode 1, the poles and zeros of the circuit are extracted,

and the response then calculated from the poles and zeros.   This

mode  can be the fastest if the circuit contains many  nodes  but

only  a  few energy storage elements (L's, C's, and  opamps  with

nonzero  GBw).   The extra time spent solving for  the  p/z's  is

offset  by relatively fast calculation of the frequency  response

from the p/z's, which avoids matrix operations.

The  routine  can have trouble with networks having  many  energy

storage elements, especially if they cover a wide frequency range

(e.g.  sub-Hz  ac coupling caps in the same circuit  with  opamps

having multi-MHz GBw's).

The speed and accuracy of the p/z network routine can usually  be

enhanced   by  separate  high  and  low  frequency   analysis   -

alternately assume that either the high frequency capacitors  are

open or the low frequency capacitors are shorted.

If high frequencies are used - multi MHz on up - it may be useful

to  scale  quantities  to stay within  Module's  numeric  dynamic

range.   Instead  of using GHz, used Hz and scale  everything  by

1.e9.

When  the  p/z-network routine is called from the main  menu,  it

screen  prints  the values of the singularities it  finds  before

erasing the screen and returning to the main menu.  The pole/zero

display can be frozen by hitting any key (e.g. return) before the

routine  returns  to the main menu.  Hitting a  second  key  will

return to the main menu.

-Calculation modes 2 and 3 have not yet been implemented.

-In  Calcmode  4, the frequency response is  calculated  directly

from the information in the network list via a numerical analysis

routine,  without first calculating the poles and zeros, as  with

Calcmode  1.  At each frequency, a complex matrix is  constructed

which is Nnodes on each side, and that matrix is then essentially

inverted  to  calculate the response at each node.  Most  of  the

calculation time is spent inverting matrices, so the speed of the

routine  tends  to depend on Nnodes, the number of nodes  in  the

circuit,  rather  than  the  number  of  parts.   Calcmode  4  is

therefore preferred for networks with few nodes and many parts.

Compared to the p/z-network routine of Calcmode 1, the  numerical

routine  of  Calcmode  4  is more  numerically  robust  and  less

sensitive to details as the spread of corner frequencies.   Since

it works on a frequency by frequency basis, it can use  tabulated

data,  and  therefore  works  with  empiricals  and  modules   as

currently constituted.

-In Calcmode 5, the active list of poles and zeros is used as the

source  from which the frequency response is  calculated,  rather

than  the  network list.  This is basically the tail end  of  the

Calcmode 1 routine.

Poles  and zeros are complex singularities expressed in units  of

real and imaginary Hertz.  The existence of the complex conjugate

is presumed for any p/z with imaginary part unequal to 0.

It usually takes much less time to calculate a frequency response

from  a  pole/zero list than from a network list - there  are  no

matrix operations in the p/z calculation - so the fastest way  to

design a circuit to implement a desired response  can be to first

use the wiggler to fit the response with a pole/zero array  using

Calcmode  5,  then  divide  the p/z list  into  small  groups  as

necessary,  one  file  per group, use Calmode 1 or  4  to  design

individual  networks  to fit the p/z sections,  and  cascade  the

stages.

A  pole/zero array specifies a frequency response only to  within

an  arbitrary multiplicative constant - or dB offset.  Since  the

multiplicative  constant  can  be  negative,  there  is  also  an

arbitrary  180 degree phase flip.  When using Calcmode 1,  Module

internally augments its p/z information with a numerical analysis

of  the  network list to set the overall gain properly  (it  also

sets  the overall phase).  In Calcmode 5, Module  works  strictly

with  the  pole/zero  list.  Absolute  gain  information  can  be

provided with the F0 and G0 parameters, but there is no source of

reference  phase information.  The calculated phase response  may

therefore be off by 180 degrees from expected.

VII   Tolerance routines

Main  menu option H invokes the tolerance routines.  The user  is

given the choice of two routines:

1 -  Tolerance estimation: compute  tolerances given  a  response

bound;

2 -  Monte Carlo analysis: compute  a  response bound given a set

of tolerances.

VII.1   The tolerance estimation routine

Option 1 of the tolerance menu brings up the tolerance estimation

menu:

1 - Set output criteria

2 - Set magnitude specification

3 - Set phase specification

4 - Estimate tolerances

These options implement the following functions:

1  - Set output criteria: selects whether output criterion  will

be  magnitude (dB), phase (degrees), or both; also, if  magnitude

is a criterion, selects whether the calculated magnitude response

gain will be Absolute or Relative.  This latter option  functions

the  same way as the Abs(olute/Rel(ative flag set with main  menu

option 9, but applies only to the tolerance estimation routine.

The program will not allow the user to disable both magnitude and

phase criteria at the same time.

2  -  Set  magnitude specification: prompts the  user  with  the

current  value  of  the magnitude response variation limit  (e.g.

+/-  .5  dB),  and  allows  the user to  either   enter   a   new

value,  or  return to leave the existing value  unchanged.   This

number  will  be (approximately) the largest + or -  dB  response

deviation  allowed by the Estimate tolerances routine, option  4,

below,  assuming the Magnitude criterion was enabled with  option

1,  above.   If  the Magnitude criterion  was  not  enabled,  the

tolerance   estimation   will  be  based  only   on   the   phase

specification.

3  - Set phase specification: prompts the user with the  current

value of the phase response variation limit (e.g. +/- 5 degrees),

and  allows  the user to either  enter  a  new  value, or  return

to  leave  the  existing value unchanged.  This  number  will  be

(approximately)  the  largest + or -  degree  response  deviation

allowed  by  the Estimate tolerances routine,  option  4,  below,

assuming  the Phase criterion was enabled with option  1,  above.

If the Phase criterion was not enabled, the tolerance  estimation

will be based only on the magnitude specification.

4  - Estimate tolerances: From the information entered with  the

above  options and the network editor, this routine  will  assign

tolerances  to network list elements so as to meet the  indicated

response contraint(s).

The sign of the tolerance of each element in the network list  is

used  as  a  flag  by  this  routine.  Elements  having  positive

tolerances   when  this  routine  is  invoked  will  have   their

tolerances  reassigned.   Elements with negative tolerances  will

be treated as fixed, although their (positive) tolerance will  be

accounted  for  in assessing the error budget.  Elements  with  0

tolerances are treated as fixed, and otherwise ignored. The  user

can freeze a tolerance at a particular value by indicating it  as

a negative number using the network editor.

Each element in question is assumed to contribute equally to  the

calculated  response deviation.  Because the peak  deviations  of

different  elements  will in general be at  randomly  distributed

frequencies,  the  contributions  are assumed to add  in  an  rms

sense.   The  sensitivity of each qualifying element in  turn  is

evaluated by changing it a small amount and calculating the  peak

change in the frequency response.  From this information and  the

error  budget, the allowable tolerance variation of each part  is

calculated.

If  there  are any calculated tolerances above 20%,  the  routine

will fix them at 20% (i.e. assign them a tolerance value of -20),

and recalculate the remaining tolerances.

The  tolerances assigned are subject to the assumption  of  equal

output  contribution, and are thus not unique.  The intent is  to

indicate  the general order of things, and let the user pick  and

freeze  specific tolerance values successively, considering  such

factors  as cost and availability.  For example, in a  simple  RC

circuit, if the tolerance estimation routine initially  indicated

necessary tolerances of 3% each for the R and C, one might freeze

the  capacitor  tolerance at 5% (an  inexpensive  tolerance)  and

rerun  the  tolerance  estimation routine to see  how  tight  the

tolerance on the resistor became.

When the tolerance estimation routine finishes, it returns to the

4-option  tolerance  estimation  menu.   In  order  to  see   the

tolerances  that have been assigned, one must double-return  back

to  the  main menu, and then (typically) drop  into  the  network

editor (option 1) to see (and optionally edit) the newly assigned

tolerance values.

Once  finished using the tolerance estimation routines, the  user

should reset any negative value tolerances to positive values, to

avoid confusing the other routines in Module.

VII.2   The Monte Carlo routine

The Monte Carlo routine varies the value of each element randomly

within  its  indicated  tolerance,  collects  statistics  on  the

variations in calculated output response, and stores the  central

and hi/low envelope responses in a plottable file.

Seven  user options are provided, shown below with their  initial

settings:

1 - Magnitude sense (absolute or relative) A

2 - Component distribution (flat or worst case) W

3 - Number of trials - 0

4 - Histogram limits (auto or manual) A

5 - Run Monte Carlo

6 - Print histograms on screen

7 - Print histograms on printer

Details:

1   -  Magnitude  sense  (absolute  or  relative)  A:  a   local

relative/absolute gain flag which in the Monte Carlo  calculation

overrides the flag set with main menu option 9;

2  - Component distribution (flat or worst case) W: in the  flat

position,  the  Monte Carlo routine will select  values  randomly

with  equal  probability  within  their  corresponding  tolerance

range.  In the worst case position, only values at the  tolerance

extremes are used.  This means that for a given element, the only

decision made each iteration is whether to use the upper or lower

tolerance limit value.  Either option is likely to be at least  a

little  more pessimistic than actual practice, where  probability

distributions are likely to follow a bell curve.

3 - Number of trials - 0: sets the maximum number of  iterations

allowed  to the Monte Carlo routine before it exits back to  this

menu.   Typical  practice might be to start with a  small  number

like 5, checking the statistics and the envelope responses  (with

Analyze4) to see if everything looks ok, then doing some moderate

number  of  iterations, like 2 to 4 times  2-to-the-n'th,  n  the

number of variable elements.  One quick way to check the validity

of  the results and whether you've used enough iterations  is  to

rerun  the  routine  and see if you get  substantially  the  same

answers.

4  -  Histogram limits (auto or manual) A: Allows  the  user  to

enter  specific,  symmetric  dB  and  degree  limits  over  which

statistics are taken during the Monte Carlo run.  In most  cases,

the Auto option will make reasonable decisions about this.

5  - Run Monte Carlo: Given the information set with  the  above

options,  this  will  run the Monte  Carlo  tolerance  evaluation

routine.   The routine initially evaluates the central  response,

using  the values in the network list at the time it  is  called.

It then repeatedly picks a set of element values randomly  within

their  corresponding tolerance limits, calculates  the  response,

and  updates its internal statistics and error  response  arrays.

It will continue until it reaches the loop limit set with  option

3,  or can be manually interrupted at any time by hitting a  key,

such  as return, in which case it will return to the Monte  Carlo

menu after the current response calculation is complete.

When  the routine finishes, it saves two files on the disk,  both

having the same first name as the current Module datafile: a .hst

file  contains the text of the histogram printout (see option  5,

below),  and a .tol file is an Analyze4-compatible  response/plot

file containing the central, and high and low envelope responses.

The  .tol file also contains a fourth response of all 0's.   This

was  included partly to provide a handy reference, and partly  so

that  the  file would have 4 responses, and  would  therefore  be

readable by Module as if it were a datafile.  This latter feature

is  a bit of a kludge to allow Module to plot the contents  of  a

.tol  output  file: if the file is read in as a  datafile,  using

main menu option 5, the central response will be plottable as the

'Desired'   dB  response,  the upper envelope  response  will  be

plotted as the 'Desired' degree response, and the lower  envelope

response  will  be plotted as the 'Calculated'  dB  response.  In

Analyze4, these are simply the first three responses (of four) in

the file. The fourth, padding response of all 0's will show up in

Module  as the 'Calculated' degree response; in Analyze4 it  will

be the fourth curve in the .tol file.

6 - Print histograms on screen: after the Monte Carlo routine is

run,  this will print the dB and degree histogram information  on

the screen.  The is a tabulation of the number of trials, out  of

the  n  loops which were performed, that were X dB or  Y  degrees

peak  from center, as functions of X and Y.  22  discrete  values

(bins)  each  are  provided between the + and -   dB  and  degree

limits.  The same information is automatically stored in the .hst

file,  readable with any text editor or a 'type' command  to  the

MsDos prompt.

7  -  Print  histograms on printer:  this  sends  the  tabulated

histogram   data  to  the  printer.   The  same  result  can   be

accomplished outside of Module by 'print'ing the .hst file to the

lineprinter using the MsDos print.com program.

Appendix A: Analyze4

Analyze4  (file  analyze4.exe  on the Ibm pc) is  a  program  for

plotting and manipulating frequency responses. It can communicate

with  Module  by  virtue of  compatible  datafile  formats:  each

program  can read the other program's files, subject  to  certain

restrictions. Analyze4 is useful for:

.making annotated plots;

.plotting multiple responses;

.manipulating  and preparing desired response curves for use  by

Module.

Analyze4   performs   in-memory   processing,   using   a  buffer

capable  of  holding  50 curves of  50  values  each (200/200  on

the unix version).   Data can be either typed in or read in  from

disk  files.   The program keeps track of how many   curves   are

in  the  buffer.    As  each  new  curve   is  entered,   it   is

appended  to the list.  It is  the  user's responsibility to keep

track  of  which  curves came from which files;   the  only  file

information Analyze4 keeps track of is the last filename  written

or read. Curves in active memory can also be selectively deleted,

among other manipulations.

The  underlying abscissa values are initially assumed to  be  the

31  ISO standard 1/3 octave audio center frequencies from 20   Hz

to 20 kHz inclusive.   This list can be  changed  by reading   in

a   file   which   uses  a  different    list    of  frequencies.

Unfortunately,  there is no menu option provided for  entering  a

list  of  frequencies directly into Analyze4.  There is  such  an

option in Module, so one can save the Module datafile and read it

into  Analyze4 to have a custom list of frequencies. All   curves

in memory at any one time  should have the same frequency basis.

Some understanding of the Module and Analyze4 datafile formats is

helpful  in using the programs together.  An  Analyze4-compatible

file  is a text file containing a list of frequency responses  It

can contain any number of frequency responses, but all  responses

in  one file must use a common set of frequencies, which is  also

included in the file.

A Module-compatible datafile starts with 4 frequency responses in

Analyze4 format, which are then typically followed by information

about  the  network  list, poles/zeros, and  other  data.  The  4

frequency responses are, in order,

1. The Desired dB response

2. The Desired degree response

3. The Calculated dB response

4. The Calculated degree response.

When  Analyze4 reads a Module datafile, it finds the 4  frequency

responses and ignores the rest of the file.  If the data is  then

written  back  to the disk from Analyze4, the file  will  contain

only  frequency response data and be devoid of any network  data,

so be sure to save it under a different filename.

Module  will  read  as a datafile any file  saved  from  Analyze4

providing  it  has 4 frequency responses in it.  Since  the  file

contains  no  network  or p/z  information,  Module  will  retain

whatever  net/p/z information is in active memory, including  the

active datafile default name, if any.

The Analyze4 file format:

An  Analyze4 file contains one or more frequency responses  (more

generally  a  rectangular  matrix of values  with  an  associated

abscissa).  It is a text file consisting of  numbers, one  number

per  line.   Numbers  can be expressed with  or  without  decimal

points, or in scientific notation (1.2e3).

The   first   number in the file is the number   of   frequencies

(abscissa values) in the file. The second number is the number of

curves  in the file. This  is followed by a list of  frequencies,

followed  by the actual data values of each curve,  typically  in

units of dB or degrees (phase).

ISO option:

If the first number in the file (the number of values per  curve,

or the number of frequencies) is 0,  it signifies the  use of the

ISO  standard  1/3  octave center audio  frequencies,  which  the

program   will   supply automatically.   In  this   case,   there

should  be  no list of frequencies in the file,   just   0,   the

number of curves, and the curve data values.

The Analyze4 menu:

Analyze4

1 - Enter data                        E - Hard copy

2 - Type data                         F - Plot

3 - Plot all                          G - Delete

4 - Difference                        H - Directory

5 - Sum                               I - Auto limits *

6 - Clear                             J - Weighting

7 - Normalize                         K - Gain Offset

8 - Rms                               L - Average

9 - Invert                            M - Extrapolate

A - Store                             N - Pairwise +/-

B - Retrieve                          O - Screen Ibmcolor

C - Marker                            Q - Quit

D - Vert range   20.00 / -20.00 dB

>

The   program   comes  up displaying its main   menu,   with   25

options labeled 1-9, A-O plus Q, followed by the prompt '>'. This

prompt  is  used when the  program  is  awaiting  user  response.

A   given  menu option is selected by hitting  the  corresponding

key.   After the corresponding  operation  is carried  out,   the

main  menu is redrawn and a  new  option solicited.

Following is a brief explanation of each of the menu options:

1 - Enter Data

Allows  the  user to key in a curve by entering a  value  at each

(prompted)  frequency.   After  the  values  at   all frequencies

are   entered,   the  active  curve  counter  is incremented   by

1   and  the entered curve  becomes   the   new  highest-numbered

active curve.

2 - Type Data

Prints  on  the screen the values of all  responses  in   current

memory in tabular form, to 2 decimal place accuracy.

3 - Plot all

Erases   the   graphics screen and plots all  active  curves   in

semilog   format:   linear vertical scale  and   log   horizontal

scale;  (e.g. dB or degrees versus log Hz).

To return to main menu after viewing plot, hit return.

If there are no active curves when option 3 is invoked,  the axes

alone are plotted.

The  number of curves in active memory is displayed next  to this

option on the main menu, providing it exceeds 0.

4 - Difference

Requests   the   number  of  a curve  in   active   memory,   and

subtracts   this  curve,  point-by-point,  from  all  curves   in

active memory, including (optionally) itself.

If   a   file was previously stored  or   recalled,   the   menu-

displayed  filename  (see options B/C) has  the  word  'Modified'

appended to it as a reminder.

[One   application   of this option is  deriving  the   frequency

response  of a system for arbitrary input signal,  which  can  be

obtained  by  subtracting  the input  from  the  output,  leaving

the change induced by the system.]

5 - Sum

Adds a user-specified curve to all curves, point-by-point.

[Summing  two  dB- and/or degree-valued  frequency  responses  is

roughly  equivalent  to  cascading  the  corresponding   systems.

Summing is  also  useful for deriving weighted figures by  adding

a  weighting curve (e.g.  A,  B,  C, CCIR, etc.),  then  deriving

the (weighted) RMS value using option 9.]

6 - Clear

Sets the active curve counter to zero,  effectively clearing  the

memory buffer. The active filename is also cleared.

7 - Normalize

Adds   a   constant value to each curve in memory  so  that   the

middle value of each curve is 0.

If the Marker Frequency is not 0 Hz (option C),  this option will

use  the  Marker  Frequency as the 0 dB point,  rather  than  the

middle frequency.

Normalize   is good for lining up a bunch of curves  quickly,  to

get a look at their relative differences.

8 - Rms

Prints   the   overall Rms amplitude of each  curve   in   active

memory,  derived  by summing the squares of the elements  of each

curve,   dividing  by  the number of elements,   and  taking  the

square root.

9 - Invert

Inverts the sign of all curves in memory.

A - Store

All   curves   in the active memory buffer are  written   to   an

Analyze4-compatible file.

If,   when  the program prompts for a filename,   the  user  just

enters  a carriage return, the most recently entered filename  is

used.  If there is no previous filename, the data are not  stored

and the program returns to the main menu.

The previous filename is displayed after the Store option on  the

menu until next disk action, or Clear (6).

B - Retrieve

Recall an Analyze4-compatible data file from the disk.

Requests   a  filename and reads the file from  the   disk   into

active   memory,   without  altering the file.   If   there   are

already   active   curves,   the  new  ones   are   appended   at

successively higher curve numbers.

If   in   the  course of a retrieval   operation,   the   maximum

active storage capacity is reached,  the file will be closed  and

a message will be displayed briefly to the effect  that the  data

file  has not fully been loaded.  Any curves already loaded  from

that file will remain in active memory.

Analyze4  reports if it is unable to find or otherwise  open  the

file.

The   last   name typed in is displayed on the  option   B   line

until another disk action or Clear (6).

C - Marker

The  frequency  marker is mostly used to  indicate  a  particular

frequency   on  a graph.  When this option   is   selected,   the

program will prompt for a value of the Marker Frequency.

Setting   the  Marker to a non-zero value will  cause  a   dotted

vertical  line  to be drawn on any further plots  at  the  Marker

Frequency.

A  non-zero Marker will also be used by the Normalize option  (7)

as the normalization frequency.

The  value  of the Marker is displayed on the  menu  after   this

option, providing it is not 0.

D - Vertical range

Requests   the   values of the top and bottom  of   the   plotted

graph,   prompting  with the current values.   These  values  are

otheriwse   displayed   next to this option on  the  main   menu.

Entering carriage return leaves a value unchanged.

If   either   value is changed and the  Autolimits   option   was

enabled (I), that option is turned off.

E - Hard copy

This  option  is for producing graphic  screen  dumps.   Printers

currently   supported  are  Epson,  HP  Thinkjet,  Deskjet,   and

LaserJet, and TTX.

When   selected,   the  option  will prompt  the   user   for   a

caption,  which can be as many lines as desired.  After each line

is  typed in,  the program sends it to the printer, then  returns

to the screen with another prompt.

To stop sending caption lines, enter a blank line.

The   program   will  then send the plot to   the   printer   and

return to the menu.

Printer   selection   is  toggled by   entering   ctrl-p   before

entering the caption.

F - Plot

This   is   a  selective plot option which  allows   plotting   a

subset of the curves in active memory.

After   invoking the option,  enter the numbers of the curves  to

be  plotted,  either one curve per  line,  separated  by carriage

returns,  or a range of curves per line,  specified as a pair  of

numbers  separated  by  a space. After  the  numbers  have   been

entered,  enter an empty line (just  a  carriage return) and  the

curves   will  be  plotted.   They  are  plotted  in   increasing

numerical  order,   regardless of the order  typed  in.   Entered

curve numbers outside the range  of  currently active curves  are

ignored.

If   after   invoking  this option an empty   line   is   entered

without  indicating any curve numbers,  the previously  used  set

of  curve numbers will be replotted. If the last plotting  option

was  3  (plot  all),  all  of  the  curves  will  be replotted.

G - Delete

Delete one or or a range of curves from active memory.

To  delete a range of curves,  enter the lower curve  number then

the   upper  curve number,  on one line separated  by   a  space.

All   numbered  curves in the indicated range will   be  deleted,

including   the   entered  limits,   and   any    higher-numbered

curves   remaining  will  be   squeezed   (their   curve  numbers

reduced) to occupy contiguous curve numbers.

If   the   second entered limit is either equal  to   the   lower

limit   or   omitted,  only the single curve  indicated  by   the

lower limit (first number entered) is deleted.

If   the   second entered limit is less  than   the   first,   no

action is taken.

Be   careful  in deleting multiple ranges,   since  the   squeeze

operation   moves   curves  around after each   delete.   It   is

usually   preferable  to delete multiple  ranges  in   decreasing

order,  so  the lower ones don't move before you  try  to  delete

them.

H - Directory

Prints  the  directory of the current disk on  the  screen,  then

waits for any key to be hit before returning to menu.

I - Autolimits

Toggles  the  autolimits option.   If enabled  (indicated  by  an

asterisk next to the option on the main menu),  the  program will

choose the vertical limits before each plot.

J - Weighting

Not implemented at this time.

K - Gain offset

Moves   all   curves in memory up or down  by  a   user-specified

amount. Solicits the user for a signed gain value, then adds this

value to all curves in memory.

L - Average

Produces  a new curve calculated as the average of all curves  in

active memory, on a frequency-by-frequency basis.

The   resulting   average  curve  is  made   the   next   highest

numbered active curve,  assuming there is room in the buffer  for

it.

M - Extrapolate

Not working at this time.

N - Pairwise +/-

Will   either  add or subtract the first n/2  curves  in   memory

to/from  the  second n/2 curves in memory, producing  a  new  n/2

curves  in  memory as a result,  assuming there is  room  in  the

buffer.    Will not work if there is an odd number of  curves  in

memory.

O - Screen

Toggles   between   using  CGA, EGA,  VGA,  Monochrome  (Ega)  or

Hercules graphic cards.

(Only on IBM PC version).

Q - Quit

Terminates   the   program.   Contents  of  the   active   memory

buffer are lost.

Appendix B: Miscellaneous routines

A  couple of functions have never quite gotten put  into  Module,

and are tossed in as independent routines runnable from dos.

Oneprcnt will solicit for a value and print the nearest  standard

1% value.

db and undb each take a numerical command line argument and print

the appropriate calculated quantity:

db 2

2 = 6.021 dB

undb 10

10.000 dB = 3.1623

Appendix C: Example: 1 pole highpass

Let's  design a 1 pole 1 kHz highpass filter.  The first step  is

to  generate the desired response. One straightforward way to  do

this is to select Calc mode 5 (p/z mode) via main menu option  6,

then  use the pole/zero editor to enter a pole at -1000 Hz and  a

zero  at  0 Hz.  If Absolute gain mode is active, set G0  to  be,

say,  0 dB at F0 = some large frequency re: 1 kHz, e.g. 200  kHz.

Calculate  the response w/ main menu option 2, and swap  it  into

the  desired response area with option c (Des <->  Calc).  (note:

since we're only fitting the magnitude response in this  example,

it  doesn't  matter whether the pole is at 1000 Hz or  -1000  Hz:

that  only  affects  the  phase  response  and,  of  course,  the

stability).

Having generated the desired response, go back to Calc mode 1 and

use the network editor to enter a starting network, for example:

Output node:  2

Part           Nodes            Val            Tol

1. Capacitor        1  2           10   nF         5 %

2. Resistor         2  0        10000 ohms         5 %

At this point, the wiggler can be run ('n' to the 'Continue  from

best?' question), which should fit the response:

Output node:  2

Part           Nodes            Val            Tol

1. Capacitor        1  2     12.57199001312256   nF         5 %

2. Resistor         2  0     12659.669921875 ohms         5 %

Capacitors  of value 12.572 nf are a little hard to buy  off  the

shelf,  but in this circuit R and C scale arbitrarily,  so  let's

reset  the capacitor value to something convenient, like  10  nf,

fixed  (set tol to 0), and just wiggle the resistor  value.   The

result:

Output node:  2

Part           Nodes            Val            Tol

1. Capacitor        1  2           10   nF         0 %

2. Resistor         2  0     15915.62 ohms         5 %

Checking with program oneprcnt, the nearest 1% value to  15915.51

is 15800:

Output node:  2

Part           Nodes            Val            Tol

1. Capacitor        1  2           10   nF         0 %

2. Resistor         2  0        15800 ohms         5 %

The calculated response of this circuit differs from the ideal  1

pole 1 kHz response by .06 dB at low frequencies.  In a practical

circuit, one would likely add a buffer of some sort.

Let's check the tolerances necessary to hold an absolute response

deviation  of less than 0.5 dB.  First set the tolerance  of  the

capacitor  to  some positive value, so the  tolerance  estimation

routine  will  estimate  its tolerance along  with  that  of  the

resistor.   Then  select the tolerance estimation  routine  (main

menu   option  H,  submenu  option  1),  indicate   a   magnitude

specification  of  0.5  dB,  and  run  the  tolerance  estimator.

Returning to the network editor, the estimated tolerances are:

Output node:  2

Part           Nodes            Val            Tol

1. Capacitor        1  2           10   nF       2.4 %

2. Resistor         2  0        15800 ohms       2.4 %

Can  it  hold the tolerance if we use a 5%  tolerance  capacitor?

Freeze  the tolerance of the capacitor at 5% by setting it  to  a

negative number:

Output node:  2

Part           Nodes            Val            Tol

1. Capacitor        1  2           10   nF        -5 %

2. Resistor         2  0        15800 ohms       2.4 %

Now rerun the tolerance estimator.  Result:

Output node:  2

Part           Nodes            Val            Tol

1. Capacitor        1  2           10   nF        -5 %

2. Resistor         2  0        15800 ohms         0 %

Doesn't  look  good  -  you'll need  a  perfect  (0%)  or  better

resistor.   Actually, with a 1% resistor and a 5% capacitor,  256

iterations  of  the  Monte Carlo routine  yielded  a  worst  case

absolute peak error of about 0.9 dB.  Using a 2.5% capacitor  and

1%  resistor  reduced  the worst case absolute  deviation  to  an

acceptable level of about 0.4 dB.

Note:  since in this circuit there is really only one  degree  of

freedom,  the desired response need only have been  specified  at

one  frequency.   For example, a one pole filter is known  to  be

down  -3.01  dB at its corner frequency.  The  circuit  could  be

designed  without recourse to poles/zeros by specifying a  single

frequency:  1000  Hz,  using the manual suboption  of  main  menu

option  8  (Freqs), and a desired response of -3.01  dB  at  that

frequency  with  main  option 7.  Running the  wiggler  with  the

capacitor fixed at 10 nf results in:

Output node:  2

Part           Nodes            Val            Tol

1. Capacitor        1  2           10   nF         0 %

2. Resistor         2  0     15916.75 ohms         1 %

Once  the response has been fit at 1 kHz, a full  frequency  list

can be dialed up with main option 8 and the full network response

optionally calculated and plotted.