Tue  07-14-1992

Greetings!  Just a few words about the TIESTEST .BIN files

in this archive.  These will test whether your modem

supports the standard Hayes-compatible escape sequence, or

the time independent escape sequence (TIES) instead.

The smaller one will work as long as your terminal program

does not substitute characters for the escape sequences,

which means most common terminals can use it.  The longer

one will defeat such techniques, and can be used with

terminal packages that cannot use the shorter one, or if you

want to really give your modem a workout!

Note that the transfer must be FROM your modem to test it;

receiving these files without any problems proves nothing.

To test your modem, send either of the .BIN files to a bbs

or other remote modem.  Use xmodem, xmodem-1k, or ymodem

batch for the test.  If the transfer just plain stops, your

modem does NOT have the Hayes compatible escape sequence.

In fact, if you abort the transfer when this happens, you

will note that your modem is in command mode, and your

terminal will not send anything to the remote!  To get back

online with the remote, type ATO and hit enter.

If you use your modem to send data to a remote often, and it

lacks the Hayes escape sequence, you must either only send

archived data, or you must completely disable the escape

sequence characters (see your manual, but S2=255 will

disable it).  Note that if it is disabled, you will NOT be

able to send commands to your modem while you are online.

Have fun!

Bob R.

SysOp, The Anonymous BBS

1:154/40

+1 414 251 2580