GPrint 2.03

Black and White Graphics Print Utility for Epson FX printers.

Copyright (c) 1988, Peter Cherna

Documentation as of October 6, 1988.

Contents:

1.   Introduction

1.1. Changes Since Version 2.00

1.2. Overview

1.3. Features

1.4. Using GPrint

2.   Print Densities

2.1. Low Density

2.2. Medium Density

2.3. High Density

2.4. Ultra Density

3.   Print Qualities

3.1. Draft Quality

3.2. Good Quality

3.3. Special Smoothing

4.   Summary of Print Modes

4.1. Physical Print Densities

4.2. Relative Time for a Given File

4.3. Relative Time for a Given Area

5.   Other Options

5.1. Offset Image

5.2. Center Image

5.3. Invert Image

5.4. Form Feed

5.5. Wide-Carriage Printers

5.6. Suppress Reverse Feeds

5.7. Full Blackness

5.8. Aborting a Print

6.   Technical Notes

6.1. Printer Requirements

6.2. Paper Movement

6.3. Perforations

7.   Future Enhancements

7.1. Printer Driver

7.2. Workbench Version

7.3. Improved Abort

7.4. Other Printers

8.   Final Notes

1.   Introduction

1.1. Changes Since Version 2.00

A single bug was discovered in GPrint 2.00.  If the width of

the picture to be printed is not a multiple of sixteen

pixels, then GPrint would not print correctly in Smooth

quality (option S).  This has been corrected in version 2.03.

Now, the picture or brush to be printed may be any arbitrary

size.

A special algorithm has been implemented which reduces the

amount of blackness in Smooth quality (option S).  The amount

of ink which is placed in solid areas is reduced by up to one

half, without any change in the picture quality.  The

additional processing needed for this algorithm does slow

GPrint down slightly, however the algorithm is so successful

that it has been implemented as the default.  If you prefer

the full blackness as was found in GPrint 2.00, use option B.

(See Section 5.7).

GPrint 2.03 now processes blank lines in a picture (such as

often remain at the bottom of a picture file) about four

times faster than it did in version 2.00.

1.2. Overview

GPrint is a utility that takes any two-colour IFF picture

file as input, and prints in on an Epson FX-series or

compatible printer.  The philosophy behind GPrint is to offer

the highest possible quality of graphics printing with the

greatest ease.  A cornerstone of GPrint is that it provides a

direct pixel-for-pixel copy of the picture file on the

printer, so that parallel lines, for example, remain of equal

thickness.

To provide maximum flexibility, GPrint offers a total of nine

print modes, consisting of four print densities and three

printing qualities (not all combinations are allowed).  The

print densities range from 80 by 72 screen pixels per printed

square inch all the way up to 240 by 216!  The highest print

quality incorporates a special smoothing algorithm to greatly

improve the output look.  It is especially effective on text,

curves, and diagonal lines or boundaries.

GPrint is an essential utility for anyone with an Epson

compatible printer who has a need to print graphics,

diagrams, charts, or who uses desktop publishing packages

such as Gold Disk's PageSetter.  It is offered as shareware,

meaning that if you find this program useful, you are asked

to register by sending a contribution of $10 to:

Peter Cherna

5511 Westbourne Ave.,

Cote St. Luc, Quebec

Canada

H4V 2G9

Registered owners will be kept abreast of any new versions or

updates.  Without your support, I cannot afford to work on

improvements or other software, so please register!

1.3. Features

GPrint is designed to provide the maximum printing speed and

quality.  For speed, for example, it doesn't ask the print-

head to pass over white space to the right of the picture.

Nor does the print-head move laterally over completely blank

lines.  In addition, if there are several blank lines in a

row, GPrint queues up this information, and issues one or

more large paper feeds instead of a stream of little paper

feeds.

For the best possible printed look, GPrint avoids asking the

printer to perform minuscule paper feeds of the order of one

or two hundredths of an inch, which the printer can only

perform with some potential for misalignment.  Instead, using

a special algorithm, it layers its print passes evenly,

avoiding tiny paper feed commands, thereby providing a more

clean-looking output.

GPrint provides numerous print densities and qualities to

meet various needs.  The resulting print densities go from

the relatively low (80 by 72 pixels per square inch) all the

way up to the very high (240 by 216 pixels per square inch).

The user can then trade off picture size and picture quality

vs. time needed for the print.  Additional options include

setting a left margin, centering the picture, inverting the

picture, and issuing a final form feed after printing.

Providing the ability to print a pixel-for-pixel copy of a

picture file gives the ability to get the best possible print

quality.  Every printed pixel can be controlled;  it is just

taken from the picture file.  Using the medium print density

for example, a full 8" by 11" page can be prepared in Deluxe

Paint II from Electronic Arts (960 pixels across by 792 down)

or taken from the output of the PageIFF utility of

PageSetter.

However, this is not even the highest print density an Epson

printer is capable of.  At the highest density, such a

picture file would occupy only one-sixth of a printed page!

How can one prepare a large image at a high print density

without resorting to an unwieldy sized file (1920 by 2376

pixels) that many programs can't handle?  (Deluxe Paint II

can't, PAR's Express Paint 2.0 can, given enough memory.)  To

allow such work, GPrint offers its Special Smoothing mode,

which doubles the horizontal and vertical print densities

without changing the size of the final printed image or

requiring a larger picture file.  The simplest way to do this

would be of course to just repeat each printed dot

horizontally and vertically, but this would produce slightly

blocky results.  The smoothing mode produces somewhat more

pleasing results, as demonstrated on the letter "g" from the

Topaz 8 font:

*** **       ******  ****       *****   ***

**  **        ******  ****      ***   * ***

**  **      ****    ****       ***     ***

****       ****    ****       ***     ***

**   **      ****    ****       ***     ***

*****       ****    ****        ***   ***

********           *******

********          *     ***

****      ****     ***       ***

****      ****      ***     ***

**********         *********

**********

Original   Repeat horiz&vert    Smoothing

To really appreciate the power of the smoothing algorithm,

try out these modes on some of the sample files!

1.4. Using GPrint

GPrint runs from the CLI.  To activate it, type:

GPrint filename [L|M|H|U] [D|G|S] [Onn|C] [I] [F] [W] [R]

Where "filename" is the name of the IFF picture or brush file

you wish to print.  Only the first bit-plane of the file is

significant, so normally a single-bit-plane file (2-colour

picture) would be used.  The file may be of any size, memory

permitting, so oversize bitmaps from Express Paint 2.0 or

Deluxe Photo Lab may easily be used.

Where the options are as follows:

L:   Low Density (80 x 72 ppsi)

M:   Medium Density (120 x 72 ppsi)

H:   High Density (120 x 108 ppsi)

U:   Ultra Density (240 x 216 ppsi)

D:   Draft Quality (use with L, M, H, or U)

G:   Good Quality (use with L, M, or H only)

S:   Special Smoothing (use with M or H only)

Onn: Offset nn tenths of an inch (eg. O5 gives 1/2 inch)

C:   Center Image

F:   Issue form feed after printing

W:   If you are using a wide-carriage (13.6") printer

R:   Suppress reverse feeds (for MX or RX printers)

B:   Full-strength blackness (allowed with S only)

Notes:  - Options can be given in any order, and in either

upper or lower case.

- 'ppsi' stands for screen pixels per square inch,

horizontal x vertical.

- The print mode defaults to M,D.

Example:  To print the picture "df1:MyPicture" in high density

with special smoothing, indented 1.5 inches, with a form feed

issued after printing, you would type:

GPrint df1:MyPicture h s f o15

2.   Print Densities

GPrint offers four print densities, the choice of which will

determine how large a particular picture will appear on the

printed page, as well as its aspect ratio (whether it will appear

squashed, stretched, or about the same as it looks on the screen.)

2.1. Low Density

Low density (option L) puts 80 screen pixels per printed inch

horizontally, by 72 vertically.  On an 8" printer, a picture

640 dots across will span the page.  The aspect ratio of this

density is with 5% of the aspect ratio of a 320x200 or

640x400 screen, so objects that are square on the screen will

be almost exactly square on the printer.

At this density, it is possible to print in draft or good

quality, but not with special smoothing.

Example:

GPrint df1:MyPicture l

2.2. Medium Density

Medium density (option M or by default) puts 120 screen

pixels per printed inch horizontally, by 72 vertically.  When

combined with draft quality this produces the print density

normally thought of as "standard" on Epson printers.

Printing a file produced by PageSetter's PageIFF utility in

GPrint's medium draft mode will produce the same print look

as the direct printed output of PageSetter.  Up to 960 screen

pixels will fit on an 8" line.  This is a good choice of

print density for general work, with the one caveat that to

create a square on the printer, you must draw a rectangle

that is 5 dots wide for every 3 dots tall, which won't appear

square on the screen.

At this density, all three qualities (draft, good, and

special smoothing) are available.  Try printing an existing

PageSetter file (saved via PageIFF) in medium density with

special smoothing.  Compare it to draft output.

Example:

GPrint df1:MyPicture m   or   GPrint df1:MyPicture

2.3  High Density

High density (option H) is another good choice for general

work, and puts 120 screen pixels per horizontal inch, and 108

vertical.  The big advantage of high density is that like low

and ultra density, its aspect ratio is within 5% of that of a

320x200 or 640x400 screen.  Thus, the horizontal stretch of

the printed copy is barely noticeable.  (Squares stay square

and circles stay round.)  It is possible to make a picture

file big enough to fill a full 8" page in this resolution in

software such as Deluxe Paint II.

As is the case with medium density, all three qualities are

possible at high density.

Example:

GPrint df1:MyPicture h

2.3. Ultra Density

Ultra density (option U) gives a pixel-for-pixel image at the

maximum physical print resolution obtainable on the FX-series

printers, namely 240 by 216 dots per square inch.  This

produces beautifully smooth curves, but be aware that the

largest picture that Deluxe Paint II can handle fills only

about a quarter page at this resolution, and it can be very

difficult to manipulate images large enough to be readable in

this mode.  Also, you will want to use some of the largest

fonts available.

As ultra density uses the maximum available print density,

there are no additional printable pixels which could be used

to implement the higher quality modes.  Thus, ultra density

only works with draft quality.  (But the results are still

excellent, because the density is so high.)

You may find that a solid black area in ultra density is "too

black", and gets smeared where the print-head has passed over

it.  If this is the case, use an older, lighter ribbon (which

yours will quickly become if you persist in printing solid

black graphics - a square inch of solid black in ultra

density or smooth quality high density mode uses as much ink

as a double-spaced page of draft-quality text!).  The other

choice is to pull the print-head away from the printer, using

the paper-thickness lever on your printer.  Of course, you

could also decrease the amount of black in the picture file!

Example:

GPrint df1:MyPicture u

3.   Print Qualities

In order to boost the print quality without forcing the user to

switch to a higher density and a larger (possibly unwieldy picture

file), different print qualities are available.  It is important

to note that while selecting different print qualities may change

the physical PRINT density, the number of SCREEN pixels per square

inch remains unchanged for a given density.  In other words,

changing print quality but not print density will change the look

of the output without changing its size.  Higher qualities

naturally take longer to print.  Not all qualities are available

in all densities.  If you specify an invalid combination, then the

desired density will be taken along with the highest allowed print

quality.

3.1. Draft Quality

Draft quality (option D or by default) can be combined with

all four print densities.  This is the most straightforward

density, in which each screen pixel is represented by

precisely one printed dot on the page.  It provides the

fastest output for a given density, and is useful in many

cases, including previewing what a final copy (at a higher

quality) might look like (in terms of size).  When used with

medium density, it produces a "standard" print resolution,

like that used in PageSetter, for example.

Example:

GPrint df1:MyPicture d   OR   GPrint df1:MyPicture

3.2. Good Quality

Good quality (option G) improves the appearance of a printed

output by printing each row twice, with the second run just

below the first.  GPrint automatically sets the printer for

double the vertical printed dot resolution for that density

in order to make room for these extra dots without changing

the resulting picture size.  Good quality may be combined

with low, medium, or high densities, but not with ultra

density.

Example:

GPrint df1:MyPicture g

3.3. Special Smoothing

Think of special smoothing (option S) as being the "near-

letter-quality" of graphics printing.  Without any extra work

on the part of the user, and with just a bit of extra

patience for the longer printing times involved, special

smoothing can dramatically increase print quality.  Special

smoothing, which works only with medium and high densities,

produces the best possible output by calculating intermediate

dots both horizontal and vertical, based on a special

smoothing algorithm.  GPrint automatically sets the printer

for double the vertical and double the horizontal printed dot

resolution for the selected density, in order to make room

for these extra dots without changing the resulting picture

size.  For an idea of the power of special smoothing, examine

the sample letter "g" given in Section 1.3, but for a real

idea, print a file with this option.

You will probably use special smoothing along with either

medium or high density for most final work.  A brief

discussion is necessary on one potential pitfall.  If GPrint

prints a file containing something (such as text) on a

screened background (a light grid of dots), you will find

that the grid dots adjacent to the text appear to "grow onto"

the letters.  This is a function of the smoothing algorithm,

which can't tell that these dots shouldn't really be attached

to the letters.  To get around this, ensure that there is a

single-pixel white space around each letter.  In PageSetter,

for example, it suffices to specify any text placed on a

screened background as being both "reversed" (\r) and

"outlined" (\o), producing black letters (as desired) with

the required white outline (as required).  In a Paint

program, the same white space can be created in a number of

ways.

As is the case with ultra density, you may find that a solid

black area in smooth mode is "too black", and gets smeared

where the print-head has passed over it.  If this is the

case, use an older, lighter ribbon.  The other choice is to

pull the print-head away from the printer, using the paper-

thickness lever on your printer.  Of course, you could also

decrease the amount of black in the picture file.

4.   Summary of Print Modes

4.1. Physical Print Densities

The four densities, low, medium, high, and ultra, put 80 x

72, 120 x 72, 120 x 108, and 240 x 216 SCREEN pixels per

square inch respectively, regardless of the selected print

quality.  However, the different print qualities mean that

the physical PRINT densities vary from this.  The following

table shows the physical print densities in dots per square

inch (horizontal x vertical) as a function of the mode:

Draft          Good          Smooth

Low        80 x  72       80 x 144          -

Medium    120 x  72      120 x 144      240 x 144

High      120 x 108      120 x 216      240 x 216

Ultra     240 x 216          -              -

Note:  Very few other printing programs or drivers use 144

dots per inch vertical density, and perhaps none use 108 dots

per inch.  However, such densities ARE possible on Epson FX

printers and compatibles, and are in fact very useful!

4.2. Relative Time for a Given File

The various print densities and qualities are provided to

give the user maximum flexibility.  Of course, for a given

file, some modes take longer than others to print, so the

following table can be used as a guide for how long each mode

takes, normalized so that medium density draft quality is

considered to be unity.  (Of course, a given file will

produce a different sized picture depending on the density

chosen.)

Draft          Good          Smooth

Low          1.5             3              -

Medium       1               2              4

High         2               2              4

Ultra        1               -              -

4.3. Relative Time for a Given Area

Since some modes print more compactly than others, the

following table can be used to estimate the time each mode

would take to fill a given printed area (say, for example,

one square inch), normalized so that medium density draft

quality is considered to be unity.

Draft          Good          Smooth

Low          1               2              -

Medium       1               2              4

High         1.5             3              6

Ultra        6               -              -

5.   Other Options

GPrint provides several options for added flexibility.

5.1. Offset Image

The Offset Image option (option O) can be used to specify a

desired left margin in tenths of an inch.  Use the letter 'O'

(or 'o') followed immediately by the number of tenths of an

inch desired.  (No intervening space is allowed;  in other

words, "O5" and "o10" are allowed, but "o 5" won't work.)  An

offset will cause the printed image to be moved over by that

amount.  (If the offset great enough, then the picture will

be clipped on the right.)  To get an offset 1.5 inches, use

the following command:

GPrint df1:MyPicture o15

The Offset Image option cannot be combined with Center Image

(see below).  If both are specified, only the last one will

be considered.

5.2. Center Image

The Center Image option (option C) can be used to position

the printed image in the horizontal center of the page.  Its

use is shown as follows:

GPrint df1:MyPicture c

The Center Image option cannot be combined with the Offset

Image option.  If both are specified, only the last one will

be considered.

5.3. Invert Image

The Invert Image option (option I) reverses the

interpretation of black and white in the picture to be

printed.  GPrint does not consult the colour palette to

determine which colour is black and which is white.  GPrint

by default prints colour 1 as black and colour 0 as white.

This works correctly with later versions of PageSetter's

PageIFF.  In a default Deluxe Paint II two-colour screen,

colour 1 will be white, so then anything white on the Deluxe

Paint screen would print in black.  If you prefer the "look"

of paper on the screen, exchange the two colours in the

palette (so that colour 0 is black (or dark) and colour 1 is

white (or light)).  If you've set things up backwards, or

have an older version of PageIFF, then GPrint will print a

mostly black picture, which not only kills your ribbon very

quickly, but it tends to blot out fine white details such as

small white text on a black background.  Use the Invert Image

option to switch this back to normal, as follows:

GPrint df1:MyPicture i

5.4. Form Feed

The Form Feed option (option F) asks GPrint to issue a form

feed after having printed the picture.  Note that GPrint does

not reset the printer at the beginning of a print, so the

top-of-form (to which the form feed advances the paper)

remains wherever you set it (i.e. at the paper position when

the printer was last reset or turned on.)  To invoke the form

feed option, type:

GPrint df1:MyPicture f

5.5. Wide-Carriage Printers

This option (option W) informs GPrint that you are using a

wide-carriage (13.6") printer, such as the FX-185 or 286.

GPrint will then adjust itself for the extra width, allowing

wider pictures to be printed, and centering pictures

accordingly.  If you leave it out, then all pictures will be

truncated at the eight-inch mark. To inform GPrint that you

have such a printer, use:

GPrint df1:MyPicture w

5.6. Suppress Reverse Feeds

This option (option R) is to be used in conjunction with

printers that cannot perform reverse paper feeds, such as the

Epson MX and RX series.  GPrint uses reverse paper feeds to

perfectly align the paper before and after a picture is

printed.  If your printer does not support reverse paper

feeds, then a bit of garbage will appear (typically a lower-

case "j").  Use option R to avoid this.  (However, then there

may be a bit of extra space above and below a picture,

depending on the print mode chosen.)  If you have such a

printer, use:

GPrint df1:MyPicture r

If you are having difficulty with the print-head snagging the

paper if you start printing at the very top of the paper

(just below the perforation), suppressing reverse paper feeds

will be of help.  (See Section 6.3, Perforations).

5.7. Full Blackness

New in GPrint 2.03 is a special algorithm that reduces the

blackness in solid areas of the printout.  The algorithm

applies only in Special Smoothing mode, and is on by default.

It helps to save the ribbon, and produces a cleaner-looking

output with less chance of smudging by the print-head or by

fingers.  If you prefer the full blackness as was found in

GPrint 2.00, simply select option B:

GPrint df1:MyPicture s b

Printing with full blackness on is a bit faster, as GPrint

does not need to employ the de-blackening algorithm.  The

full blackness option does not apply and is ignored unless

Special Smoothing is selected.

5.8. Aborting a Print

If in the course of printing a picture you wish to cancel the

operation, click on the "Abort Print" gadget in the GPrint

window that opens in the upper right corner of the screen.

GPrint will then stop sending graphics to the printer, and

terminate gracefully.  Of course, the printer buffer may

already contain several lines of graphics, and thus the

printer will continue printing even after GPrint stops

sending.  You may either wait for the printer to finish

printing what it has already received in its buffer, or you

may shut off your printer.

GPrint will exit when it has finished sending graphics to the

printer.  If the printer is still printing from the buffer

when GPrint finishes sending, either wait it out, or shut off

the printer.

If the "Abort Print" gadget is pressed while the computer is

waiting for the printer (eg. if the printer is not on or if

it is out of paper), the abort will not take effect unless

and until the printer error is corrected (eg. by turning on

the printer or loading paper).  Otherwise, after about a

half-minute of waiting, the printer driver will put up a

requester announcing printer trouble.  Click on RETRY if

you've fixed the problem, or click on CANCEL to abort and

exit GPrint.

6.   Technical Notes

6.1. Printer Requirements

As discussed in Section 4.1, various print densities are used

to provide the nine different print modes that GPrint

supports.  If you have an FX or LX printer or something else

relatively recent, all these modes will work.  If you have

something older like an MX-80 with Graftrax-Plus, then some

modes will work, while others won't.  This section explains

which graphics print modes are used by GPrint, so you can

check your printer manual to see if your printer can do what

GPrint asks of it.

GPrint Modes        Epson Code     Description

LD, LG              ESC '*' 4      80 dots/inch

MD, MG, HD, HG      ESC 'L'        120 dots/inch

MS, HS, U           ESC 'Z'        240 dots/inch *

*    in this mode, the printer does not allow two adjacent

dots to be printed in a single pass.  GPrint uses two-

pass printing to overcome this.

For example, an MX-80 with Graftrax Plus does not support the

ESC 'Z' and the ESC '*' 4 graphics modes.  Therefore such a

printer can only be used in medium or high density, in draft

or good quality.

If your printer does not support reverse paper feeds (code

ESC 'j' n), be sure to use option R (Suppress Reverse Feeds).

(Note that some early Star-Gemini printers are only mostly

Epson compatible, and graphics printing was one of the

incompatibilities.)

6.2. Paper Movement

GPrint causes the paper to do a funny dance at the beginning

of every printed picture, and sometimes at the end, also.

This is done for two reasons:  at the beginning of the

picture, it removes any slack in the paper feed mechanism

caused by manual paper movements (thus preventing unwanted

gaps or overlaps in print passes), and second, GPrint

sometimes requires that the paper be rolled back a tiny

fraction of an inch before and after printing, depending on

the print mode chosen.

When GPrint encounters a picture which has a large blank

section (say at the bottom of the picture, or possibly a band

in the middle), instead of sending dozens of little paper

feed commands to the printer (for that machine-gun effect),

it queues them up internally and issues them as one or more

very large paper feeds, which is less annoying, and faster.

Sometimes, especially when printing a wide picture (that has

such a gap) with special smoothing, it seems like GPrint may

be twiddling its thumbs, but it is actually processing this

blank space, and storing up the paper feed commands.  It may

be momentarily disconcerting, but it is still faster than

having GPrint issue a steady flow of small paper feeds.

6.3. Perforations

Two problems can be encountered when printing on or near the

perforations in fan-fold paper.  First of all, slight

alignment errors are introduced when the printer is printing

near the perforation.  This corrects itself when the

perforation has passed, and is usually small enough to pass

undetected when printing text.  When printing graphics,

however, the alignment error usually shows up as a small gap

or overlap between consecutive print passes that should be

just touching.  The second problem is more rare, and is that

some printers get into trouble when reverse paper feeding

near the perforation.  If your printer jams or otherwise gets

unhappy, start the printing a bit farther down the page if

possible, or else use option R to suppress reverse paper

feeds.

7.   Future Enhancements

GPrint has come a long way since version 1, which was used only by

myself and a few friends.  Since version 1, the high and ultra

modes were added.  As well, the printer was trained not to send

the print-head farther right than the last dot on the line, and

not to waste time passing over blank space.  All the options

listed in Section 5 were added, and the program was made to run

several times faster, and is now about half the size of the

original.

There are still a few things that could be added to GPrint, but

Version 2 is here and useful.  To give you an idea of what I hope

to add, here are a few points.

7.1. Printer Driver

GPrint is currently a stand-alone program that prints IFF

files.  For a fair amount of extra work, the algorithms that

GPrint uses could be cast into a printer driver that could

then be placed with any program and used to get superb output

without having to go through an IFF file to do it.  The

output from GPrint is still better than the much improved and

much ballyhooed EpsonX driver from Workbench 1.3.  (The Epson

driver from 1.2 doesn't come close to either GPrint or the

EpsonX driver!)

7.2. Workbench Version

GPrint is the perfect pal for us CLI fans.  However, people

who freeze at the sight of the little CLI icon also have the

right to great print quality.  A Workbench version of GPrint

with a menu and gadget interface offering the same print

quality and options can be arranged.  This too would be a

fair amount of work.

7.3. Improved Abort

Why doesn't the "Abort Print" gadget work if the computer is

waiting for the printer to clear its error (out of paper, not

on, etc.)?  I spent many late nights trying to get this

feature to work, but I usually ended up freezing the computer

or visiting the Guru.  I've now got a pretty good

understanding of how the printer device and the parallel

device work, and I'm not convinced that this is the same as

how they ought to work and are supposed to work according to

the documentation.  If ever I discover that the problem is

mine and the devices actually do work as documented, or if

the problem lies with the device drivers and corrected

versions are issued, then this will be done right away.

The other part of the Abort function that could be improved

is the question of graphics that made it to the printer's

buffer before the "Abort Print" gadget was clicked.  The

printer keeps going for a while, even though GPrint has

stopped sending stuff, as the user asked.  There doesn't seem

to be any Officially Approved Way (OAW) to cause a hardware

reset of the printer, which would arrest the printing

immediately.  My best attempt was too successful;  it reset

the printer and the computer!  If someone shows me such an

OAW, I'd put it in in a flash.  If there's a dirty way of

doing it, I'd at least consider it.  If there isn't, then it

would be nice if Commodore were to provide one.

7.4. Other Printers

Some or all of the techniques used in GPrint would apply to

other models of printer as well.  If there is sufficient

interest, I may be willing to create other versions of GPrint

that support those printers, for example the Epson Q series,

or HP LaserJets and DeskJets.

8.   Final Notes

I hope that you find GPrint handy and useful.  I've kept it to a

small size so that you can tuck it on any disk where it might be

handy.  I've worked hard at testing it to pieces, so there can't

be many bugs left. (never say zero!)  It has a bright future

(there are several interesting enhancements which could be done -

discussed in Section 7).  At this stage, I have to make one final

appeal:  there can't be future versions of GPrint nor any

stablemates released into the public domain or as shareware if

those who find it useful don't register!  So please, keep

developers developing, and send $10, along with any comments,

suggestions, bug reports, etc. to:

Peter Cherna

5511 Westbourne Ave.,

Cote St. Luc, Quebec

Canada

H4V 2G9

Thank you and enjoy!