KFFT.DOC               KFFT version 1.1               Jerry Kallaus

14 Feb 1989

KFFT V1.1 (C)Copyright 1989, Jerry Kallaus.  All rights reserved.

May be freely redistributed for non-commercial use (FREEWARE).

Jerry Kallaus

993-D Mangrove

Sunnyvale, Ca  94086

Feedback welcome.

---------------------------------------------------------------------

KFFT -- Fast Fourier Transform

CONTENTS

1.      Introduction

2.      Files

3.      Words

4.      Stack Diagrams

5.      Conditional Compile Controls

6.      Run Time Controls and Global Variables

7.      Data Representation

8.      Performance

9.      Future Versions

10.     Miscellaneous

1.  INTRODUCTION

The following documents a Fast Fourier Transform (FFT) program

which is written in JForth* for Commodore Amiga computers.  A

common computationally intensive bottleneck in a wide variety of

digital signal processing (DSP) applications is the transformation

of data back and forth between the time and frequency domain using

the FFT.  The original motivation for writing this program was to

satisfy my curiosity as to how fast an FFT could be performed on a

stock Commodore Amiga computer with a 7.14 MHz Motorola 68000

microproccesor.  Another motivating factor was the idea that a

good FFT program is probably the most massively useful thing to

have lying around when doing DSP applications.

One set of source code is used which conditionally compiles

variations of the same basic algorithm, namely a radix-2

Cooley-Tukey FFT.  A choice had to be made as to whether to have

one set of source code cluttered with conditional compilation

statements, or to maintain umpty-dozen sets of source code.  The

former was chosen.  Some of the FFT variations achieve a speed

which is probably within about twenty percent of what could

ultimately be achieved by implementing the features described in

the section on future versions.  Currently, times as good as 0.54

seconds for a 1024 point complex fixed-point FFT, and 6.5 seconds

for a 1024 point floating-point FFT have been achieved.  (see

section on Performance).

While JForth does a good job of supporting Forth standards, some

"features" of JForth may have been used which may or may not make

this code difficult to port to other Forths.

The current FFT version works with JForth version 2.0 and 1.2.

As part of cleaning up some of the clutter in future FFT versions,

support for JForth v1.2 will probably be dropped altogether.

The first few sections of this document consists of rather terse

reference material, with more elaboration given in later sections.

* JForth is a product of

Delta Research

P.O. Box 1051

San Rafael, Ca 94915

2.  FILES

The file KFFT1.ZOO contains the following files:

fft.doc      this file

cmplx.doc    documentation for complex arithmetic support words

fftasm.doc   documentation for assembly language support words

fftinc       INCLUDEs for compile

fftcontrols  Conditional compilation controls

fft1         Forth FFT code for complex data

fftrc        Forth FFT code for real data

fftmisc      Forth miscellaneous support words

cmplx        Complex arithmetic and data type Forth words

fft.asm      FFT assembler support words

wtable       cosine, sine table for FFT

makewtable   utility for making wtable

testfft      simple test code for fft

fft1.for     Fortran source which concisely depicts algorithm.

3.  WORDS

FFT               fft - complex data in, complex data out

FFTRC             fft - real data in, complex data out

IFFT              inverse fft - complex data in, complex data out

IFFTCR            inverse fft - complex data in, real data out

CHECK.INPUTS.FFT  sanity check of fft arguments

BIT.REVERSAL      performs bit reversal reordering operation

INIT.MAP.FFT      makes bit reversal map for quick-reversal

QUICK.REVERSAL    internal faster bit reversal, used with init.map.fft

See also cmplx.doc and fftasm.doc

4.  STACK DIAGRAMS

FFT               ( data-address  log2-fft-size  -- )

FFTRC             ( data-address  log2-fft-size  -- )

IFFT              ( data-address  log2-fft-size  -- )

IFFTCR            ( data-address  log2-fft-size -- )

CHECK.INPUTS.FFT  ( data-address  log2-fft-size -- )

BIT.REVERSAL      ( data-address log2-fft-size  -- )

INIT.MAP.FFT      ( reversal-map-address  log2-fft-size  -- )

QUICK.REVERSAL    ( data-address  reversal-map-address -- )

Note that for FFTRC and IFFTCR, the argument log2-fft-size

represents the number of data points in the real array.  Also, for

2N real data points, FFTRC computes a transform with N+1 real

numbers, and N-1 imaginary numbers.  The first and Nth imaginary

parts are zero; rather than working with odd ball array sizes,

FFTRC puts the N+1 real part in the imaginary part of the first

complex number of the transform.  Likewise, IFFTCR expects to find

it there.

See also cmplx.doc and fftasm.doc

For example usage, see file testfft; words tfftrc and tifftcr.

5.  FFT CONDITIONAL COMPILE CONTROLS

The file fftcontrols contains the following flags which control

the conditional compilation (* indicates default):

FLOAT_FFT?        true  => version for floating point data

* false => version for fixed point data

W_TABLE_FFT?    * true  => use table for cosine, sine values

false => compute cosine, sine values as needed

ASM_FFT?        * true  => use assembler code for some basic operations

false => use JForth code for everything

INNER_ASM_FFT?  * true  => use assembler code for inner loop of FFT

false => don't use assembler for inner loop of FFT

AUTO_SCALE_FFT? * true  => fixed pt version using block floating point

falue => no auto scaling

Certain combinations of flags are not supported and logic is present

which precludes those combinations.  Specifically,

if FLOAT-FFT? is false, then W_TABLE_FFT? will be forced to true

if FLOAT-FFT? is true,  then INNER_ASM_FFT? will be forced to false

if FLOAT-FFT? is true,  then AUTO_SCALE_FFT? will be forced to false

In other words, for a fixed point version, table trig values must

be used; and there is no assembler code for the inner-loop or the

auto-scale feature for the floating version.  The logic which

forces the above flag conditions is provided primarily as a

convenience which allows switching from a fixed-point version to a

floating-point version or vice versa by simply changing FLOAT_FFT?

and not bothering with the other flags.

6.  RUN TIME CONTROLS AND GLOBAL VARIABLES

The following global variables provide run time controls and fft

output scaling control and information.  Use of any of these is

optional, and except for REVERSAL-FFT, all are applicable only to

fixed-point versions.

REVERSAL-FFT    addr => pre-formed reversal map address

EVENS-FFT       true => divide by 2 on even fft stages

ODDS-FFT        true => divide by 2 on odd  fft stages

INSHIFT-FFT     value specifying right shift of input to first stage

OUTBITS-FFT     value specifying number of significant output bits

SHIFTS-FFT      value returned specifying number of shifts done on data

BLK-EXP-FFT     adjusted block floating point exponent

Explanation:

Part of the FFT computation involves reordering the data by a

process called bit-reversal, so called because pairs of elements

are exchanged whose indices are the bit-reversal of each other.

The FFT algorithms provided here will by default perform this

bit-reversal logic each time an FFT is computed using a word named

BIT.REVERSAL.  However, the word INIT.MAP.FFT may optionally be

used to eliminate most of this.  As an example, suppose the user

wishes to compute many 1024 point FFT's, then the following code

could be used,

512 ZARRAY mymap           ( note only need half as many elements )

0 mymap  10 INIT.MAP.FFT   ( 2**10 = 1024, 0 mymap is mymap begin addr )

The word INIT.MAP.FFT will compute and store into mymap a

bit-reversal map consisting of index swap pairs and a zero

terminator at the end of the list.  A little less than the 512

elements are actually used.  The address of mymap is stored in the

global variable REVERSAL-FFT.  Later, when the FFT algorithms are

used, the non-zero value in REVERSAL-FFT will indicate that a

bit-reversal map exists and where it is located; and instead of

using the normal BIT.REVERSAL word, a word named QUICK.REVERSAL

will be used.  The FFT of any data set of the same size may be

taken using the same map and the user simply uses the FFT

algorithms as normal.  If different sizes of FFT's are desired,

simply use INIT.MAP.FFT for each FFT size, providing a different

array for each size.  Only now, just before each use of the FFT

algorthms, stuff the appropriate map array address into the

variable REVERSAL-FFT.

The following applies to the scaling of output data from the

various FFT algorithms.  For floating FFT's, scaling the output is

left to the user.  For fixed-point non-auto-scaled FFT's, the

scaling of the output data is left to the user, but the use of the

variables INSHIFT-FFT, EVENS-FFT, and ODDS-FFT will affect the

scaling.  For auto-scaled FFT's, the code uses a block floating

point concept to maintain maximum accuracy without overflows

throughout the computation.  For auto-scaled FFT's, the variables

INSHIFT-FFT, SHIFTS-FFT, BLK-EXP-FFT, and OUTBITS-FFT are used to

control and inform the user of the the scaling of the output data.

Also, a number of independent functions are provided for

determining and changing the scaling of arrays of fixed point

data.

The casual reader may wish to skip to the last paragraph of this

section, as the following tends to go on ad nauseum about scaling

considerations when doing fixed point fft's.

A radix-2 fft computes the FFT in log2 n stages; for example, a

1024 point FFT takes 10 stages.  The root mean square magnitude of

the complex elements of the FFT increase by the square root of two

on each stage, and specific elements may double (note that the RMS

value doubles for every two stages).  This isn't really a problem

with a floating point FFT; however, for a fixed point FFT, this

can create arithmetic overflow problems.

For a FFT fixed point version which does NOT use auto-scale, the

following is applicable.  The flags EVENS-FFT and ODDS-FFT can be

set to scale-down (shift) the data by a factor of two on even or

odd stages or both.  For this purpose, the stages are counted zero

through M-1, where M is the number of stages.  The scale-down

occurs on the input of the stage.  The default is for scale-down

on odd stages.  When taking forward or inverse FFT's, if it is

known or suspected that the data is impulsive in nature in the

domain being transformed to, scale-down on both even and odd

stages should probably be used to avoid arithmetic overflows.

Unfortunately, when taking large fft's, the preceding capability

has serious shortcomings.  If a shift is done on every stage,

compete loss of significance can occur.  If a shift is done on

every other stage, overflows can occur.  Both situations are

unacceptable.  The auto-scale feature now described addresses that

problem.

The basic operation done on each element on each stage is of the form

An+1(i) = An(i) + W * An(j),

where n refers to the old stage, n+1 refers to the new stage, i

and j are array indices, and A and W are complex numbers.  W is

the cosine and sine of some angle, and complex multiplication by W

represents a rotation about the origin in the complex plane.

Assume that a fixed-point representation is used with an implied

binary point such that the magnitude of each An is less than one,

then the maximum magnitude of each An+1 will be less than two.  So

it would be possible to compute the magnitude of each An+1, pick

the maximum of these, and if it is greater than one, right shift

all the data by one on input to the next stage, which would ensure

that all values remain less than one.  Unfortunately, the

additional multiplies, adds, compares, etc., required would nearly

double the fft time.

A much more efficient solution is as follows.  If the magnitude of

each real and imaginary part of each An is limited to be less than

one, then the maximum magnitude that the real or imaginary part of

any An+1 can be is 1+sqrt(1+1), or approximately 2.414, and would

occur when W corresponds to a 45 degree rotation.  The

implementation of this is as follows.  The absolute value of the

real and imaginary parts of all the An are bitwise logically OR'd

together in a register.  If the resultant value is less than one,

no shift is done on the input to the next stage.  If the value is

between one and two, the next stage input is right shifted by one.

If the value is greater than two, the next stage input is right

shifted by two.  This algorithm only adds about ten percent to the

fft time.

It should be mentioned here that on machines which have the same

word length for everything, say 16-bits, the implementaion of this

algorithm would lose one bit of significance relative to the

algorithm which tracks the complex magnitudes.  This happens

because the real and imaginary parts would have to be kept smaller

than one-half to prevent overflows.  However, on the M68000

microprocessor, even though fixed point multipliers and

multiplicands are limited to 16-bits, everything else may be

32-bits.  Thus, it is permissable for the operation to overflow by

two bits between stages, and there is no loss in significance.

The variable:

INSHIFT-FFT   may be used to specify a right shift ammount

for the data going into the first stage of the fft.

OUTBITS-FFT   may be used to specify the number of significant bits

to use for the output of the fft.

SHIFTS-FFT    an output value specifying the total number of shifts

performed on the data.

BLK-EXP-FFT   input and output value which represents the block

floating point exponent of the data.  It is updated

by the FFT code simply by adding shifts-fft to the

input value.

7.  DATA REPRESENTATION

Each value uses one cell (long word, 4-bytes, 32-bits).  For

complex numbers, the following conventions apply.  On the stack,

the imaginary part is on top with the real part under it.  In

memory, the real part is at the lower memory address with the

imaginary part at the adjacent higher memory cell address.

For fixed point data, the following applies.  Although 32-bits are

used to hold each value, the signed value must be contained within

the low-order 16-bits, with the high-order 16-bits being the sign

extension of the low-order 16-bits.  The variable INSHIFT-FFT is

primarily provided for handling data with more than 16-bits

significance, including sign bit.  The functions OR.ABS.ARRAY and

NSBITS may be used for rapidly determining the magnitude of an

array of fixed point data.

8.  PERFORMANCE

Times in seconds for 1024 point fft for complex (FFT, IFFT) and

real (FTRC,IFFCR) data.  The run-time quick-reversal option was

used in all cases.  Add one-tenth of a second for the time without

this option.

complex  real     float  asm-fft  w-table  inner-asm  auto-scale

6.92    3.72       T       F        F         F         F

3.92    2.00       F       F        F         F         F

1.24     .60       F       T        T         F         F

.54     .34       F       T        T         T         F

.62     .38       F       T        T         T         T

9.  FUTURE VERSIONS

The following are only possibilities, and will be probably only be

developed if and when the need arises.  The timing performance

improvements are very rough guestimates.

Add option to use table for all trig values, as opposed to their

recursive computation.  (5-10 percent)

Provide additional inner-loops which eliminate multiplies by one's

and zero's, and possibly combine multiplies by sqrt(2).  (5-10

percent)

Probably will not develop radix-4 fft algorithm (10-20 percent) or

other/mixed radix algorithms.

Provide some commonly used windowing functions.

Change to algrithm(s) which do the bit-reversal after the

transform, rather than prior to the transform.  This would be

preferable for high speed convolution (filtering) and correlation

applications which do not necessarily require any bit-reversal.

10.  MISCELLANEOUS

A discussion or derivation of the fast fourier transform is beyond

the scope of this document.  Numerous texts on digital signal

processing are available for this purpose.  I would particularly

recommend "Digital Signal Processing " edited by Lawrence R.

Rabiner and Charles M.  Radar, IEEE Press.