Kevin's Amazing Echo Machine

Written and Copyright (c) 1988, 89 by Kevin Kelm

(719) 392-1023, CIS 71071,2726

but in the Public Domain

--------------------------------------------------

WHAT'S THE BIG DEAL?

SuperEcho does something I've never seen another program do:  given an

audio input to the left channel of an audio digitizer in the parallel port,

SuperEcho will perform a variety of LIVE special effects on that signal,

including echos and pitch changes without speed changes!

---------------------------------------------------------------------------

WOW!  HOW DO I MAKE IT WORK?

There are a 12 gadgets that allow you to change to parameters of what

is going on.  The are listed below with their descriptions.

# VOICES (1...4):   Lets you set how many channels will be involed.

BUFFER (8..65535):  You can specify how much memory the buffer is to take

up.  Pitch changes should be done with a small buffer,

say 127, and echos can be made with larger (16384+)

buffers.

FREQ (L|R)(1|2) (124..600):  These are the left/right channels 1 and 2

frequency settings.  A low number means a high-

pitched voice, and a high number means a deep

voice.

ECHO DELAY:    The number to count to before starting each channel.  This

(0...65535)    creates the separation between each channel's playback which

creates echos.  A low number means a short separation and a

high number means a long separation.

VOLUME(L|R)(1|2): These sliders independently set the volume of each

channel.  Fading echos can be made with decreasing

volumes.

SAMPLE DELAY (0..20): This lets you set the number the computer will count

to before each sampling of the digitizer.  Low numbers

mean better signal quality, while larger numbers can

shift the frequency of playback higher (sort of like

an octave shift).

GO! :  Clicking this will start the echo effect with the current parameters.

The system will seem to freeze, but as soon as your mouse pointer

stops moving (it may take a sec to get set up), SuperEcho will start

processing signals.  You can press the left mouse button to stop.

QUIT:  Exit SuperEcho.

---------------------------------------------------------------------------

WOW!  TELL ME MORE!

I have my own set of favorite parameters.  One is the default, which

creates a LOW, BOOMING ECHOING voice, which does WONDERS to scare the

kiddies on Halloween!  I like to hook the digitizer up to a microphone and

speak directly in.  Another fun activity is to plug in a radio, say a talk

show, and set up the parameters to be #V=1, Buf=4095, FreqL1=160 or 170,

Samp=0.  I think that's right.  This will make the radio announcer

st-st-stutter like Max Headroom!  with small frequencies, like 124, you can

create the "Mooooommmmmmyyyy!" voice from the TV in Poltergeist!  Also try

mixing a range of frequencies, like 124, 200, 160 and 180 and a small buffer

to sound like a whole group of people!

Anyway, have fun and enjoy!  Above all, spend some time with SuperEcho,

plugging different values in and trying new things.  I'm sure there are even

some combinations that **I** haven't even heard.  Try to keep a running list

of your favorite parameters.  It would be great if someone came up with a

good list of effects and their parameters!

Who out there is ready to use channel modification modes, etc. to make

robot effects?  What ELSE can you come up with?  I'd love to see the results

of YOUR audio hacks!

--------------------------------------------------------------------------

If you like, I can be contacted at the following address:

Alternate Realities

c/o Kevin Kelm

7080 Roaring Springs Avenue

Colorado Springs, CO

80817