%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

Examples of ACSII-hex formats for :-

INTEL

MOTOROLA

TEKTRONIX ( inc extended format )

Note: the INTEL example (data length 16dec bytes) is the reference.

: in following "byte" =  binary value of 2 ASCII-hex charas

hi nibble = 1st byte

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

Format :  INTEL

:LLAAAATTdd..ddCC

LL   = number of data bytes

AAAA = 16 bit address

TT   = type

0 = data

1 = end record

2 = Upper Segment Base Address (USBA)

3 = start address record ( data length = 4 bytes)

CC   = checksum ( = -ve sum of record : ie sum of all byte-pairs = 0 )

dd = ASCII-hex charas

USBA format = :02000002ssssCC

( following data records will be have (16 * ssss) added to their load addr)

----------------------------------------------------------------------------

:020000020500??                                type 2 record , base = 500H

:1012340090FAFCE96B002020202020202020202090    type 0 record (data)

(load addr = 1234H)

:00000001FF                                    type 1 record (end)

above converted to :-

:1062340090FAFCE96B002020202020202020202040    <<EXAMPLE

:00000001FF

@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@

MOTOROLA - all use S9 as end record

Format :  Motorola ( S1 , 16 bit address )

|<-------------- LL ---------------->|

S1LLAAAAdd..ddCC                             << Format

LL = 2 + (number of data bytes) + 1

AAAA = address

CC = ~( LL + (AA .. + AA) + ( data bytes ))  : Note 1's complement!

S1LLAAAAdd..ddCC                              << Format

S113623490FAFCE96B00202020202020202020203C    <<EXAMPLE

S9030000FC

@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@

Format :  Motorola ( S2 , 24 bit address )

S2LLAAAAAAdd..ddCC                              << Format

S21400623490FAFCE96B00202020202020202020203B    <<EXAMPLE

S904000000FB

@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@

Format :  Motorola ( S3 , 32 bit address )

STLLAAAAAAAAdd..ddCC                              << Format

S3150000623490FAFCE96B00202020202020202020203A    <<EXAMPLE

S90500000000FA

@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@

Format :  Tektronix

Note checksum algorithm

/AAAALLRRdd..ddCC    << Format

AAAA = load address

LL   = record length ( number of data bytes : 0 = last record )

RR   = load checksum ( = sum of previous 6 de-ASCII'd CHARAS )

CC   = data    "     ( sum of de-ASCII'D CHARAS )

/AAAALLRRdd..ddCC                              << Format

/6234101090FAFCE96B002020202020202020202079    <<EXAMPLE

/00000000

@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@

Format :  Tektronix (extended)

Note checksum algorithm

%LLTCCky......ydd..dd    << Format

LL = length ( all charas after % )

T  = type chara ( 6 = data , 8 = end )

CC = checksum   ( sum of all de-ASCII'd CHARAS = 0 )

k  =   of address charas, max 8 ( = 4 bytes )

y...y = 'k' address charas

xx =  data as usual

%LLTCCky......ydd..dd    << Format

%2E6A680000623490FAFCE96B0020202020202020202020    <<EXAMPLE

%0E81E800000000

*