Virus Memory Kill V1.0  Chris Hames.  10th June 91.

(REMEMBER! no virus can copy itself to a write-protected disk.)

This utility is different to the previous versions in that it no

longer directly detects any virus.  Instead it is now the most powerful

tool for detecting new viruses.  It checks a heap of things that viruses

use and tells you when they have changed.

Firstly it checks CoolCapture, ColdCapture, WarmCapture, KickTagPtr

and the KeyboardReset to find anything that is trying to survive reset.  If

any of these are abnormal it will alert you including a display of the area

of memory that they are pointing to.  You can look for words describing was

the thing is and then decide whether to do nothing or do a cold reset(note

this is much more that just a normal reset) which should clear memory of the

virus.

Secondly it checks the jump tables of all resident libraries, devices

and resources and warns you if any are not pointing to ROM.  It will give

you a message describing what isn't pointing to rom and where it is actually

pointing.  Most systems will get at least a few of these warnings.  Setpatch

causes a few and other legit programs do as well.

FOR PEOPLE WHO DON'T UNDERSTAND A WORD I AM SAYING:-

This program is very technical I agree but a general user can just

have it in their startup-sequence and notice the messages it gives.  If

they change and you haven't changed your system get the latest best Virus

Killer(One that checks your disks and files) and run it to check out your

system.

ALERTS THAT ARE CAUSED BY LEGIT PROGRAMS

Please note some legit programs will cause alerts.

SetPatch r (Workbench 1.3 command for 1/2 meg chip uses with RAD)

Changes the coldcapture.

RAD changes KickTagPtr

Stopping Alerts/Warnings that are caused by legit programs:-

You can stop a alert/warning by giving the full cause of the alert

which is best idea

eg   -$01E(graphics.library)=$66666666

eg   KickTagPtr=$77777777

You can stop a alert/warning by giving the full cause without the

location of the alert which is second best idea

eg   -$01E(graphics.library)

You can stop a alert/warning by giving just the description of the

of the alert which is the worst idea

eg   KickTagPtr

eg   (keyboard.device)

Usage: VMK -ca alerts

-c will cold reset(this should kill any virus from memory)

-a will make library/devices/resources warnings into alerts

with memory display.

Examples of use:-

VMK -c		; Resets your machine safely!!

; (Should kill ANY virus from memory)

VMK KickTagPtr	; Stops alerts about the KickTagPtr (Say if you use rad)

VMK KickTagPtr=$00000700	; Stops the specific alert at this location

; eg If RAD always at same location use it

VMK (dos.library)	; Stop all warnings/alerts about the dos library

VMK (dos.library) -$01E(graphics.library)	; no dos &

; no -$01E graphics alerts

If you find a new virus send it to:-

Erik Lovendahl

Snaphanevej 10

4720 Prst

Denmark

Bye,

Chris Hames (Available for any Amiga work)

Internet:	bytey@phoenix.pub.uu.oz.au

ins760z@monu4.cc.monash.edu.au (1991 only)

FidoNet:	3:633/353