TEAC SCSI FLOPPY OVERVIEW DOC		    July 2/91

Overview.doc

Index

-----

0) What it is

1) Command Lines

2) Setting up the device

3) Mountlist Entries

4) Using the Device

5) Gotchas

6) Getting SCSI floppies & docs

7) SCSI Direct

8) Included files

9) Caveats, Disclaimers etc.

10) Email addr

What it is

----------

The TEAC SCSI Floppy (hereafter TSF) [FD-235JS] needs to have its

floppies formatted by the SCSI controller. This is accomplished by

sending an appropriate MODE_SELECT and then a FORMAT command via SCSI

Direct. SCSI Direct is an Amiga method of piggybacking raw SCSI

commands on a hard disk device (see include.i/scsidisk.i).

Every time you put a disk of different capacity, you need to change

the mode of the controller. This is done by sending an appropriate

MODE_SELECT command via SCSI Direct. You also need to send a DiskChange

to the handlers involved. If you put in a disk of the same capacity,

you still need to send a DiskChange command.

The different capacities and filesystems are reflected in a series of

devices.

SD4:	4 Meg AmigaDOS

SD2:	2 Meg AmigaDOS

SD1:	1 Meg AmigaDOS

SI4:	4 Meg MSDOS

SI2:	2 Meg MSDOS

SI1:	1 Meg MSDOS

In an ideal world, none of this would be necessary. One filesystem

could control the various capacities taking note when disks &

capacities are changed. But the FastFileSystem knows nothing about

changing the SCSI controller's various modes. Neither does it know when

disks are changed. Neither does it have the ability to change the

capacity of devices once mounted. One solution would be to write a

special filesystem, a challenging enterprise which I may attempt.

In the meanwhile, this is a suite of programs for dealing with TEAC

SCSI floppy drives. The suite includes:

TEAC SCSI FLOPPY MODE_SELECT		TSFM1, TSFM2, TSFM4

TEAC SCSI FLOPPY FORMAT 		TSFF1, TSFF2, TSFF4

TEAC SCSI FLOPPY AUTODISKCHANGE 	TSFADC

TEAC SCSI FLOPPY EXERCISER		TSX

Command Lines

-------------

The TSF suite expects to be run from the CLI. Except for TSX, the

general template is

TSFxxx [xxx.device scsi_id]

The .device and scsi id are optional, depending on your hardware.

The defaults are harddisk.device and 1.

For TSX you do not need to worry about passing the .device and scsi

id as parameters, because there are menu items which allow one to set

these.

Examples:

TSFF4 ? 				/* Prints format */

TSFF2					/* USES DEFAULTS */

TSFM2 ? 				/* Prints format */

TSFM1					/* USES DEFAULTS */

TSFM4 scsi.device 3

TSFADC					 /* USES DEFAULTS */

TSFADC scsi.device 3

TSX

There is more detailed info regarding command lines in each of

the TSxxxx.docs.

Setting up the device

---------------------

The FC-1 has a 50 pin standard scsi connector so it is straight

forward if your controller uses the 50 pin connector as well. If your

controller uses the CBM A2090x which has a Mac style 25 pin connector

you will need a special cable. These are usually available from Mac

dealers.

At the back of the TSF, there are pins for the SCSI ID. See the

Teac docs for more info on setting the SCSI ID. As far as I know the

LUN is fixed at zero. Once you have the drive connected (with the power

off, yes?) the mountlist entries below need to be added to your

mountlist.

Mountlist Entries

-----------------

/*

*

* SCSI Floppy under the Xetec driver 720K, 1.44M, 2.88M

*	 BlocksPerTrack = 36  (for 2.88M)       SD4:

*	 BlocksPerTrack = 18  (for 1.44M)       SD2:

*	 BlocksPerTrack = 9   (for  .72M)       SD1:

*/

SD1:	 Device = harddisk.device

FileSystem = l:FastFileSystem

Unit = 1

Surfaces = 2

LowCyl = 0

HighCyl = 79

Reserved = 2

BlocksPerTrack = 9

Buffers = 10

BufMemType = 3

Stacksize = 6000

Priority = 10

GlobVec = -1

MaxTransfer = 0x0001FE00

Mask = 0x00FFFFFE

DosType = 0x444F5301

#

/*

*

* SCSI Floppy under the Xetec driver .72M

*	 Using MSDOS File System

*	 BlocksPerTrack = 36  (for 2.88M)       SI4:

*	 BlocksPerTrack = 18  (for 1.44M)       SI2:

*	 BlocksPerTrack = 9   (for  .72M)       SI1:

*/

SI1:	 Device = harddisk.device

FileSystem = l:MSDOSFileSystem

Unit = 1

Surfaces = 2

LowCyl = 0

HighCyl = 79

Reserved = 2

BlocksPerTrack = 9

Buffers = 5

BufMemType = 5

Stacksize = 6000

Priority = 10

GlobVec = -1

MaxTransfer = 0x0001FE00

Mask = 0x00FFFFFE

DosType = 0x4D534400

Mount = 0;

#

Using the device

----------------

Once you have mounted the capacity device which you are going to

use, the general sequence is to scsi format [TSFFx] the disk, then to

AmigaDOS format the disk.

For example, to use the drive with a 2 Meg AmigaDOS disk

1) Mount SD2:

eg. use the command line

1> Mount SD2:

2) SCSI Format with TSFF2

eg. use the command line

1> TSFF2

3) AmigaDOS Format

eg. use the command line

1> Format drive SD2: name Test.0 noicons quick

4) Test if the system sees it

eg. use the command line

1> info

1> list sd2:

There are more detailed examples in each of the TSxxxx.docs.

Gotchas

----

It is very easy to corrupt disks because the FastFileSystem [or the

MSDOSFileSystem] has no way of knowing if you have changed disks. You

either have to be very careful about doing DISKCHANGEs or use TSFADC.

This is a daemon which does occasional Test Unit Readys. If there is a

change in the SCSI status, it sends a disk change packet to the

handler.

Getting SCSI floppies & docs

----------------------------

There are several different TEAC SCSI JS floppies available. The

names are of the form FD-235JS-xxx. The xxx in the name designates the

model of drive which the FC-1 controller board is strapped to. I have

seen mention of 300/302/401/650 & I have the 660-U.

FD-235HS-302	3.5" 1.44Mb SCSI floppy (using FC-1)

FD-235JS-401	3.5" 2.88Mb SCSI floppy (using FC-1)

FD-55GS-704	5.25" SCSI floppy (using FC-1)

I have used this software only on my FD-235JS-660U model. There are

different versions of the FC-1 around, which TSX will show you with the

INQUIRY command. There are also other programs around such as Xetec's

SCSI_LISTER which will tell you the version you have.

The TEAC drive documentation available is:

FC-1 Spec	    Part number 10530468-01 @ US$20.00

FD235J-650 Spec Part number 10530403-50 @ US$ 7.00

FD235HS-300Spec Part number 10530472-02 @ US$ 7.00

FD235JS-   Spec Part number  ?	    @ US$ 7.00

They also want $4.00 shipping & handling per item. (My copy of the

FC-1 docs was missing pages & had pages out of order, but there was

enough info there to get the thing working.)

The SCSI specs are available from the ISO rep in each country. See

the file SCSI.docs for more info.

The easiest & most useful way for a hacker to get hold of the SCSI

specs is downloading from the SCSI BBS. As far as I know, there is no

internet site which carries these docs.

SCSI BBS  (316)-636-8700.

SCSI Direct

-----------

If you are bit banging on a SCSI Controller chip, normally you set

up your CDB and then diddle the status/control of the chip until you

can send your commands to the target. Then you send your commands and

monitor the chip for various responses.

On the Amiga, you set up your CDB, you set up a SCSI_CMD to point

to the CDB and you set up a standard IO Request Block to point to the

SCSI_CMD. You then issue a HD_SCSICMD to the device using that IO

Request Block. This method allows one to send any SCSI CDB to any

device, a harddisk laser printer, or whatever.

For more info on SCSI Direct see devices/scsidisk.i|h.

Included files

--------------

If you want a look at the source (in assembly) drop me a line.

Caveats, Disclaimers etc.

-------------------------

This software is provided for testing purposes only.

I cannot be held reponsible for what you do on your computer.

Under no circumstances will I or MetaMedia Productions be held

reponsible for any damage problems etc. which may arise as a result

of running this Beta software.

Email Addr

----------

Harvey Taylor      Meta Media Productions

uunet!van-bc!rsoft!mindlink!Harvey_Taylor

a186@mindlink.bc.ca

The TSF suite and documentation is all Copyright   1991    by Harvey Taylor