SetRamsey V1.50 by Nic Wilson

-----------------------------

This program is freely distributable with no conditions

except that the program and this text file must remain

intact without any modification.

THE FOLLOWING MUST BE STATED

** FLAME ON **

Some engineers have stated that SetRamsey is not required or

performs incorrectly or have said "throw it away".  They have

even stated the incorrect terminology of the 'static column mode'

name.  To set the record straight.  This name was taken straight

from their own Devcon manuals.  It does not matter wether you call

it 'page detect' or 'static column' the function is the same,

and the name debatable.

Even though they do not agree, I have found by hundreds of

users with many Rev 8.9 boards that work fine, 100% of the

time with SC Drams and this mode enabled.  We DO get an

increase in speed EVEN on benchmark programs in the range

of 15 to 20%, if SCM is on but BURST off, which is another

problem again with the Ramsey chip so I am told.

The problems started occuring on the Rev 9 boards. Instead of

arguing the necessity for SetRamsey, time should be spent

finding the actual cause of the problems and fix them or at least

state the whole story of why these modes operate incorrectly on

some Amiga's instead of beleiving they already know why.  Your

current explanations do not hold water with what is actually

happening here in the field.

SetRamsey can also be used to read the current settings,  they

never supplied a utility for this purpose.

They stated to some users "don't take for granted the OS is wrong,

setting SCM on Page Mode Drams would cause problems".  My reply to

this is "don't take for granted that SetRamsey is wrong, it does

the CBM recognised test for SCM Drams before allowing changes".

If you belevie that SetRamsey performs incorrreclty in some

way then contact me and I will fix it!!

EMAIL {cbmvax!cbmehq}!cbmaus!wilson!nic@uunet.uu.net

** FLAME OFF **

REQUIREMENTS

------------

Amiga A3000     (nothing less)

FUNCTION

--------

Allows manipulation and or reading of the RAMSEY ram controllers

registers that toggle various modes in an Amiga 3000.

USAGE

-----

SetRamsey STATICON, STATICOFF, BURSTOFF, BURSTON, WRAPON, WRAPOFF

REFRESHON n, REFRESHOFF

or    SetRamsey ?   Gives USAGE string

SetRamsey     Gives current settings of the RAMSEY chip registers.

NOTE

----    The use of STATIC COLUMN or BURST MODES require that you have some

SCRAM installed. (Static Column Random Access Memory).  A test is

made before setting these modes, and error occurs if page mode ram

found.

STATICON        -       Turns Static Column Mode on

STATICOFF       -       Turns Static Column Mode off

BURSTON         -       Respond to *CBREQ input and do burst cycles

BURSTOFF        -       Turns Burst mode off

WRAPON          -       Allows all 4 longwords of a burst to take place

WRAPOFF         -       Burst will only continue A3,A2 are increasing

Burst will not be allowed to WRAP A3,A2=00

In other words,  the 68030 always reads 4 long

words, but if the start address is on a non-quad

boundary it will loop back and read the words

prior to the start address.  Since this will rarely

be needed, Ramsey can stop the burst cycle after

the forward words are read in.

REFRESHON n     -       Sets refresh to value specified by n

n must equal 154, 238 or 380

REFRESHOFF      -       Turns refresh off

DONT USE NORMALLY, WILL CAUSE UNPREDICTABLE RESULTS.

All enquiries to

Nic Wilson Software

138d South Street

TOOWOOMBA QLD 4350

Phone +61 76 358522  Work hours

Fax   +61 76 358384  Work hours

Phone +61 76 358539  After hours

EMAIL {cbmvax|cbmehq}!cbmaus!wilson!nic@uunet.uu.net