S t o p W a t c h

Version 3.0

December 06, 1991

by

Christian Danner

M A N U A L

===========

PRINCIPLES

"StopWatch" is  a stop  watch application, which makes a programmable precision

of more than the by the  system  time  offered  1/50  second  possible  (with a

standard Amiga 500/1000/2000 e.g. a resolution of one millisecond occupies only

little processor time,  with  a  turbo  board  this  value  can  furthermore be

increased).

The program  reacts on  the status of the joystick button. It gets its timebase

from the CIA B timer  A  and  in  the  last  resort  from  the  7.16 MHz system

frequency  by  interrupt  programming.  After  many - by reason of inaccuracy -

frustrating experiments with the CIA A timer A, which ought to be  the only one

available for  free access  according to the Addison Wesley Hardware Manual and

other publications, I  used  this  pretendedly  by  EXEC  continuously occupied

timer. Till  now no  incompatibilities occurred. The interrupt priority level 6

prevents  the  timer  program  from  being  overrun  by  other  interrupts  and

guarantees a high degree of precision.

I tested  the program on different conditions. During car races I'm using it in

the background, where the input data come from a hardware interface producing a

short circuit  to the  according joystick  port wires when the time measurement

beam  of  a  light  barrier  is  interrupted.  At  the  same  time  a Superbase

application is  running on  this machine,  communicating with another Amiga via

the serial  device  for  displaying  the  score  lists  and  serving  a printer

connected to  the parallel  port. In  comparison with a professional stop watch

running simultaneously,  I couldn't  find out  any difference  in the resulting

times. Corresponding  to my  tests, normal Intuition interfacing, file handling

etc. might not corrupt  the timer.  So far  I only  found one  program, a sound

generator, which caused deviations.

FEATURES

The program  displays the  single lap  times and the total times in a window on

the Workbench screen. You can determine  the number  of laps  per race  and the

period of  time, within  which no  incoming signal  is processed  after a valid

event. In an uncomplicated  manner  you  can  adjust  the  measurement  on your

equipment (I  have not  yet tested, whether it's necessary or not, but it might

be required depending on  the  quartz  base).  Optionally  for  the  purpose of

transmitting the results to other applications, the data can be saved to a disk

file in ASCII-format. You can specify the filename freely, a  selection between

only saving  total times  or saving  all lap-times is possible, just as you can

choose whether the file only has to contain the last time or a time  list shall

be generated. Also an ARexx-port for remote control with parameter modification

and a more comfortable result handling  is  implemented.  With  Version  3.0 an

additional screen  can be  opened as  a time display e.g. in combination with a

genlock interface. Text and primitive graphics  can  also  be  written  to this

screen via Rexx-commands.

HARDWARE

As I  explained earlier,  the program  scans the status of the joystick button,

which is a simple switch connecting Pin 6 with Pin 8 of the joystick  plug when

it is  closed. Pressing  the button  is a  valid event  for the  timer. You can

trigger the timer manually or by an interface,  which "presses  the button" for

you  by  a  switch  (mechanic  or  optoelectronic),  e.g. controlled by a light

barrier.

If the signal to detect is active only for a  period of  time shorter  than the

interval  between  two  scans  of  the  input it might be ignored by the timer.

Therefore you have to delay the  release  of  the  switch  (with  an electronic

circuit) for a reliable measurement of such signals.

CALIBRATION

First  you  have  to  select  the  resolution  of  the  timer,  which means the

frequency of timer increase (e.g. 1000 for a resolution of 1/1000 second). This

value should  not be  excessively high,  because you  can slow  down the system

rapidly by the timer interrupts.

Now  you   have   to   tune   the   timer   by   changing   the  VIACYCL-Values

("VIA-Cycles/Timercycle" in  the Workbench window), so that the resulting times

are mostly valid in comparison  with  an  already  calibrated  stopwatch. Doing

this, the "Correction-factor" has to be 0.

For a  fine adjustment  a correction factor can be defined, which is equivalent

to the time deviation divided by the total period of time where it occurred. To

be  able  to  supervise  this  adjustment,  the  resulting  correction value is

displayed in terms of milliseconds with  every measurement  in the  last of the

three columns.

PARAMETERS

All the  parameters can  be transmitted to the program when starting it. I have

implemented certain commands  for  the  startup  from  the  CLI,  ToolTypes for

running  "StopWatch"  from  the  WorkBench  and  Rexx-commands to influence the

already started application.

ToolType             CLI   PrgWindow        Explanation

---------------------------------------------------------------------------

General:

--------

LAPS=number          -l..  Number of Laps   number of laps per race

REFRACT=number       -r..  Refract.-Time    refractory time after a pass

CYCLSEC=number       -s..  Resolution       number of timercycles per second

VIACYCL=number       -v..  VIA-Cycles / Ti. coarse tuning of the timer

CORRECT= number      -c..  Correctionfactor timer fine tuning factor [* 1E-6]

DISPLAY=number       -d..  not implemented  number of result lines in display

BEEP=ON              -b    not implemented  display beep after timer events

FILE=path/filename   -f..  not implemented  filename for saving the results

APPEND=ON            -a    not implemented  generate result-list

PASS=ON              -p    not implemented  save also lap-times

Display Screen:

---------------

SCREEN=number        -o..  not implemented  display screen depth

WIDTH=number         -w..  not implemented                 width

HEIGHT=number        -h..  not implemented                 height

COLORS=hexnumbers    -k..  not implemented                 colors

ACTTIDIG=number      -(..  not implemented  actual   time   digits   after  the

decimal point

TOTTIDIG=number      -)..  not implemented  total /  lap time  digits after the

decimal point

TIFGCOLOR=number     -e..  not implemented  time display color of characters

TIBGCOLOR=number     -g..  not implemented                        background

TIMEFNAME=fontname   -n..  not implemented  time font name (disk font !)

TIMEFSIZE=number     -z..  not implemented            size

TEXTFNAME=fontname   -m..  not implemented  text font name (disk font !)

TEXTFSIZE=number     -q..  not implemented            size

TOTLEFT=number       -t..  not implemented  coordinate time total  left

TOTUP=number         -u..  not implemented                         up

LAPLEFT=number       -i..  not implemented                  lap    left

LAPUP=number         -j..  not implemented                         up

ACTLEFT=number       -x..  not implemented                  actual left

ACTUP=number         -y..  not implemented                         up

Parameter           Valid data range                                 Preset

---------------------------------------------------------------------------

LAPS                    1 .. 99999                                        1

REFRACT                 1 .. 99999                                      500

CYCLSEC                 1 .. 50000                                     1000

CORRECT             -9999 .. 99999                                     -525

VIACYCL                50 .. 50000                                      708

DISPLAY                 2 ..    12                                        5

BEEP                  OFF ..    ON                                      OFF

APPEND                OFF ..    ON                                      OFF

PASS                  OFF ..    ON                                      OFF

SCREEN                  1 ..     4                        no display screen

WIDTH                 640 ..   767                                      640

HEIGHT                400 ..   766                                      400

COLORS                                          66AFFF000D000D000BDD0D0B0DB

ACTTIDIG                0 ..     1                                        1

TOTTIDIG                0 ..     3                                        3

TIFGCOLOR               0 .. number of colors - 1 (1/3/7/15)              1

TIBGCOLOR               0 .. number of colors - 1 (1/3/7/15)              0

~LEFT                   0 .. WIDTH  - TextWidth( "00:00:00.000")        150

~UP                     0 .. HEIGHT - TextHeight

ACTUP                                                                   150

LAPUP                                                                   180

TOTUP                                                                   210

Time-/Text-Font                                            no font selected

Examples:

---------

~FNAME              RUBY (extension ".font" not allowed)

~FSIZE              15

COLORS              00AFFF000C00444DD00D000A...

\_/

rgb color 0 = blue (hexadecimal Red/Green/Blue-values 0..F)

\_/

rgb color 1 = white

\_/

rgb color 2 = black

\_/

rgb color 3 = red

...

Command line for CLI-Startup:

StopWatch -l2 -r500 -s1000 -v708 -c-525 -d5 -b -fCLIPS:TimeList  -p -a

-o3 -w704 -h460 -k44afff000c00 -x495 -y350 -i250 -j350 -t495

-u350 -nstopwatch -z26 -e1 -g2

To modify the parameters in the Workbench window you first have to click on the

"STATE"-button  or  to  select  the  pulldown menu "Project/Parameters". Then a

modification of the values in the parameter field is possible.  When done click

on the  same button  again. This  sets the  new values  and zeroes the timer. A

reset also can be processed simply  by selecting  the according  pulldown menu.

The state  of the  system is displayed in an area above the "STATE"-Button. For

preventing the timer from  analyzing uninteresting  signals a  locking function

can be  activated by  pressing the  "LOCK"-button or  sending the corresponding

Rexx command.  You return  to the  normal timer  action by  pressing the button

again or using the Rexx command "UNLOCK".

Notes:   Even  if  the  continuous  time  display  skips  some  values and runs

irregular  (not  enough  system  preformance  for  the  defined  timer

settings)  the  resulting  times  are valid! Time measurement and time

display are different, parallel running processes.

A VIACYCL-value < 150 may freeze a system without accelerator  card by

occupancy of a great amount of calculating time for handling the timer

interrupts! You could check  this with  a performance  monitor. If you

intend to  use this program only as a Rexx-controlled display unit set

VIACYCL to its maximum value (10000)  for the  least possible  loss of

system preformance.

To protect  the timer  from being  triggered repeatedly by an instable

signal when the joystick button is pressed or released, it is possible

to define  a period of time, during which no check of the input signal

is performed after an event. E.g. in car races with  longer refractory

times this  feature can also be useful to prevent the timer started by

the front wheels from being stopped by the rear wheels.

The strategy: 1. After a valid event (button  pressed  =  signal  on =

pins connected)  the input is scanned for the release

of the button.

2. At that moment the refractory time,  which determines

the period,  during which  a signal is ignored by the

timer, is set to  the user-defined  REFRACT-value and

counted down.

3. If  there  is  another  event  within  this time, the

program ignores it and  repeats  the  whole procedure

beginning  at  1  (scanning  for  the  release of the

button...).

This loop is performed  until no  event occurs during

the time of delay.

4. Subsequently the timer is active for the next event.

The  power  led  indicates  the  state  of  this feature. If it shines

brighter, the  input is  inactive. For  a premature  activation with a

running  timer  you  have  to  lock  /  unlock the input. A reset also

activates the input, but the actually measured time is lost.

The format of the  actual  and  total  /  lap  time  displayed  in the

separate  screen  is  shortened  depending  on  the defined TIMEDIGITS

values:            3         "00:00:00.000"

2         "00:00:00.00"

1         "00:00:00.0"

0         "00:00:00"

The time strings transmitted to the stopwatch window,  the output file

and Rexx are never truncated to make the further processing easier.

When  using  proportional  fonts  for  time display all digits and the

characters " ", "." and ":"  must  have  the  same  width  for correct

output and  deletion. If  necessary modify  the respective font with a

font-editor.

The display screen  depth  determines  the  maximum  number  of screen

colors. Depth  1/2/3/4 means  2/4/8/16 colors  (colors = depth^2). The

time needed for text-  and graphics  output increases  with the screen

depth, so  avoid unused bitplanes! Toggle between the "StopWatch"- and

the Workbench screen with the keys <Amiga>-N and <Amiga>-M.

Caution: On my  Amiga 2000B PAL  with  Agnus 8361 / Denise 8362  some baselines

(below  line  455)  are  not  shown  on  a  screen  with  the depth of

4 bitplanes, but they are present and can be dumped to  the printer by

the use  of a  screen grabber.  Maybe this fault does no longer appear

in combination with the ECS-Chipset and  1 MB chip  memory, but  I was

not yet able to check this hardware.

REXX-COMMANDS

The ARexx-port of "StopWatch" can be accessed via its name "RexxStopWatch".

So far the following commands are implemented:

General:

--------

CLOSE          ends the program

parameters: none

LOCKEXIT       forbids ending of the program (except via REXX)

parameters: none

returncode: warn:  LOCKEXIT already active

UNLOCKEXIT     resets the LOCKEXIT-command

parameters: none

returncode: warn:  LOCKEXIT not active

RESET          resets the timer

parameters: none

LOCKTIMER      disconnects the timer from the joystick port

parameters: none

UNLOCKTIMER    resets the LOCKTIMER-command

parameters: none

LAPS           number of laps per race

parameters: <number>

returncode: error: no parameter

parameter out of range

REFRACT        refractory (delay) time after a pass

parameters: <number>

returncode: see "LAPS"

CYCLSEC        number of timercycles per second

parameters: <number>

returncode: see "LAPS"

VIACYCL        coarse tuning of the timer

parameters: <number>

returncode: see "LAPS"

CORRECT        timer fine tuning factor [* 1E-6]

parameters: < number>

returncode: see "LAPS"

BEEP           display beep after timer events

parameters: [ON]/[OFF]

returncode: error: no / wrong parameter

NEXTTOTAL      returns the next total time

parameters: none

format of result:  "00:00:00.000"

(depending on "CYCLSEC" filled with "0")

returncode: warn:  timer overflow (> "99:59:59.999")

error: rexx result handling disabled

(no "OPTIONS RESULTS" command)

NEXTLAP        returns the next additive lap time

parameters: none

format of result:  see "NEXTTOTAL"

returncode: see "NEXTTOTAL"

NEXTSINGLE     returns the next single lap time

parameters: none

format of result:  see "NEXTTOTAL"

returncode: see "NEXTTOTAL"

PREVTOTAL      returns the previous total time

parameters: none

format of result:  see "NEXTTOTAL"

returncode: see "NEXTTOTAL"

PREVLAP        returns the previous additive lap time

parameters: none

format of result:  see "NEXTTOTAL"

returncode: see "NEXTTOTAL"

PREVSINGLE     returns the previous single lap time

parameters: none

format of result:  see "NEXTTOTAL"

returncode: see "NEXTTOTAL"

Display Screen:

---------------

SCREENADDRESS  returns the pointer to the display screen structure for external

screen manipulations (before using this feature  you should call

the LOCKEXIT-command  to avoid  a closure of the screen from the

workbench, which might cause a system crash)

parameters: none

result:     <address in decimal text format>

returncode: warn:  timer overflow

error: rexx result handling disabled

(no "OPTIONS RESULTS" command)

TIMEDIGITS     changes the number of  digits following  the decimal  point when

the actual and total (lap) time is displayed

parameters: <digits actual time> <digits total / lap time>

returncode: see "LAPS"

TIMEPOS        changes position of actual, total and lap time

parameters: <xact> <yact> <xtot> <ytot> <xlap> <ylap>

returncode: error: position(s) out of screen range

-> no change

warn:  text too large for correct display

-> position(s) shifted

TIMECOLOR      defines foreground- and background color of time display font

parameters: <foreground color> <background color>

returncode: error: no / wrong parameters

TIMEFONT       changes the  time display  font. It also clears the locations of

the actual, lap and total time!

parameters: <name> <size>

returncode: error: no / wrong parameters

diskfont not available

TEXTFONT       changes the text display font used by the ~TEXT~-commands

parameters: <name> <size>

returncode: see "TIMEFONT"

BOXO           draws an opaque filled box

parameters: <left> <up> <width> <height> <color>

returncode: error: no / wrong parameters

parameters out of range

(object has to fit into the screen)

BOXT           draws a transparent filled  box  (by  alternation  of  pixels in

foreground- and background color)

parameters: <left> <up> <width> <height> <color>

returncode: see "BOXO"

DRAW           draws a dot, line or polygon

parameters: for dot      <color> <x-pos.> <y-pos.>

for line     <color> <x1> <y1> <x2> <y2>

for polygon  <color> <x1> <y1> <x2> <y2> <x3> ..

returncode: see "BOXO"

FILL           fills  an  area  with  a  plain color beginning at the specified

position, delimited by the determined outline color

parameters: <x-pos.> <y-pos.> <color>

returncode: see "BOXO"

ELLIPSE        draws an opaque filled ellipse

parameters: <center x> <ct. y> <radius x> <rd. y> <color>

returncode: see "BOXO"

LOGO           displays copyright logo

parameters: <left> <up>

returncode: see "BOXO"

CLRLOGO        clears copyright logo

parameters: <left> <up>

returncode: see "BOXO"

TEXTR          text output from the defined position to the right

parameters: <left> <up> <textcolor> <backgroundcolor> <text>

returncode: see "BOXO"

if  necessary  text  is  shortened  without  error

message !

TEXTL          text output from the defined position to the left

parameters: <left> <up> <textcolor> <backgroundcolor> <text>

returncode: see "TEXTR"

TEXTC          text output centered horizontal relative to the position

parameters: <left> <up> <textcolor> <backgroundcolor> <text>

returncode: see "TEXTR"

CLRTEXTR       clears text written with TEXTR

parameters: <left> <up> <text>

returncode: see "TEXTR"

CLRTEXTL       clears text written with TEXTL

parameters: <left> <up> <text>

returncode: see "TEXTR"

CLRTEXTC       clears text written with TEXTC

parameters: <left> <up> <text>

returncode: see "TEXTR"

SHADOW         sets shadowtext coordinates

(in pixels relative to normal text position = 0 / 0)

parameters: <x-pos shadow 1> <y-pos shadow 1> <x2> <y2> ..

preset: "-1 -1 0 -1 1 -1 -1 0 1 0 -1 1 0 1 1 1"

restriction: max. 30 shadows (60 x/y-values) allowed

returncode: see "BOXO"

TEXTRS         text output to the right with shadows

parameters: <left> <up> <textcolor> <shadowcolor> <text>

returncode: see "TEXTR"

TEXTLS         text output to the left with shadows

parameters: <left> <up> <textcolor> <shadowcolor> <text>

returncode: see "TEXTR"

TEXTCS         text output centered horizontal with shadows

parameters: <left> <up> <textcolor> <shadowcolor> <text>

returncode: see "TEXTR"

CLRTEXTRS      clears text written with TEXTRS

parameters: <left> <up> <text>

returncode: see "TEXTR"

CLRTEXTLS      clears text written with TEXTLS

parameters: <left> <up> <text>

returncode: see "TEXTR"

CLRTEXTCS      clears text written with TEXTCS

parameters: <left> <up> <text>

returncode: see "TEXTR"

The ARexx returncode values are 0 for OK, 5 for WARN and 10 for ERROR.

The  valid  data  range  for  the  commands LAPS, REFRACT, CYCLSEC, VIACYCL and

CORRECT is already described above.

For a correct sizing  of the  CLRTEXT~-commands you  have to  activate the same

text display  font you used for the corresponding TEXT~-command which has to be

wiped out. In the same manner for clearing a text  displayed with  a shadowtext

command (CLRTEXT~S) the shadows must be set to their correct values.

For  further  details  I  refer  to the example program "StopWatch.rexx", which

comes along with this package.

The abortion of NEXTTOTAL, NEXTLAP or  NEXTSINGLE with  "Abort RexxCmd" returns

an error  level 10  in RC.  If the "OPTIONS RESULTS" command is not active when

commands expect a return value (~TOTAL, ~LAP, ~SINGLE, SCREENADDRESS), an error

level 10 is returned.

IMPLEMENTATION

Most of  the code is written in M2Amiga Modula 2 V3.1d (really antique version)

by AMSoft. I have  implemented the  interrupt routines  in assembler  using the

Aztec C 3.6a Assembler.  Thanks to CygnusSoft for their excellent editor, which

helped me to keep the source together.

HISTORY

V 2.0  - First released version of  the  program,  which  already  supports the

different Workbench fonts of AmigaDOS 2.0.

V 3.0  - Continuous time display in the program window.

- New  timer  REXX-Commands:  PREVTOTAL,  PREVLAP, PREVSINGLE, LOCKEXIT,

UNLOCKEXIT, LOCKTIMER, UNLOCKTIMER, BEEP.

- REXX-Commands GET... renamed to NEXT... !!

- Optional display-beep on all screens with the timer  event. Activation

by the CLI-Option -b and the ToolType BEEP=ON.

- Correct 3D-Display also with the color palette of AmigaDOS 2.0.

- Assembly routines now coded and optimized with the Aztec Assembler, no

longer with the Data Becker Profimat-Assembler.

- REXX interface improved (unknown commands are sent back  to the server

and don't cause Guru Meditations any longer..).

- Display Screen for genlock interaction.

Features:  - Up to  16-color screen.  Number of  bit-planes and colors

definable by the user.

- Variable screen dimensions. Overscan supported.

- Free font selection. Color fonts in  combination with the

"ColorText"   program   from  the  Calligrapher  software

package possible.

- Position of actual, lap- and total time not fixed.

- Text- and  graphics  output  via  Rexx-commands (TIMEPOS,

TIMECOLOR, TIMEFONT, TEXTFONT, BOXO, BOXT, ELLIPSE, DRAW,

FILL, TEXTR, TEXTL, TEXTC,  CLRTEXTR, CLRTEXTL, CLRTEXTC,

SHADOW,  TEXTRS,  TEXTLS,  TEXTCS,  CLRTEXTRS, CLRTEXTLS,

CLRTEXTCS,  LOGO,  CLRLOGO).  SCREENADDRESS  makes screen

manipulations by external programs possible.

V 3.0S - Shareware  version  of  StopWatch  V3.0.  The sometimes on the display

screen appearing "Danner Timing"-logo  reminds you  to order  the full

version and pay your registration fee.

The package consists of:

StopWatch       Version 3.0S of the executable file

StopWatch.doc   the user manual

StopWatch.rexx  a demo file for ARexx-interfacing

StopWatch.pic   a  snapshot  of  the StopWatch screen while running

StopWatch.rexx (for those who do not yet use ARexx)

COPYRIGHT

Copyright (c) 1991 by Christian Danner. All rights reserved.

The author is not responsible for any damage caused by  the use  of the program

"StopWatch".

The shareware  versions 2.0  and 3.0S of this program may only be redistributed

on non-commercial base without changes together with their documentation files.

They must  not be  distributed on  commercial base  without my explicit written

permission.  If  you  use  these  versions  more  frequently,  especially  in a

professional manner, I won't refuse a little fee.

Version  3.0,  which  is  neither  in  the public domain, nor shareware, can be

obtained via my  address  for  the  amount  of  50 US$  or  80 DM  (only bills)

including airmail  postage and registration. Already registered users only have

to send me 5 US$  or  10 DM  for  forwarding  expenses  to  receive  the latest

version. Because  of the external generation of the system frequency in genlock

mode I can not  guarantee a  correct time  measurement in  combination with all

kinds of  genlock interfaces.  So please  check your  hardware with  one of the

shareware versions of this program before ordering.

For further improvements please let me know of  all the  bugs you  find and all

the options  you miss. Every attempt will be made to solve your problems within

a reasonable amount of time.

My address:    Christian Danner

Rie 10

8106 Oberau

Germany

Phone: 08824/8122