** Hitachi 9500 and 9550 players. This version UNTESTED! I couldn't find   **

** anyone willing to test it!                                              **

** Communication protocol:                                                 **

** Computer activates RTS (pin 4) on rs-232 port, (amiga 7-line handshake) **

** then sends "remote" command (68H) to switch player from local to remote **

** mode. Then send "play" command (25H) and expect back "job end" (A5H).   **

** The player may take up to 15 seconds to spin up! Commands that are sent **

** may return either a "job end" code which is the original command + 80H, **

** or echo back the original command (like status ). The Hitachi players   **

** don't support STATUS messages to find out if it is READY for a command. **

** ASCII digits used for frame search numbers, etc. BUT, frame numbers are **

** returned as two byte binary, high byte first, then low byte. Usual baud **

** rate is 9600.                                                           **

COMMANDS       CHAR     HEX    DECIMAL  JOB-END(hex)    COMMENT

----------------------------------------------------------------------------

play-fwd        %       25      37      A5              Play forward

play-rev        B       42      66      C2              Play reverse

pause           *       2A      42      AA

slow-fwd        #       23      35      A3              Slow forward

slow-rev        (       28      40      A8              Slow reverse

still-fwd       $       24      36      A4              Still forward

still-rev       )       29      41      A9              Still reverse

fast-fwd        !       21      33      A1              Fast forward

fast-rev        &       26      38      A6              Fast reverse

scan-fwd        "       22      34      A2              Scan forward

scan-rev        '       27      39      A7              Scan reverse

search          +       2B      43      B0

enter           A       41      65      41 (echo back)  Terminate groups of cmds

P. frame        k       6B      107     6B (echo back)  Get current frame #

reject          /       2F      47      AF              Park disc

(and a few others......)

SERIAL PORT CONNECTION:

computer   player

--------   ------

TxD 2      2 command

RxD 3      3 data

RTS 4      4 host ready

CTS 5      5 player ready

GND 7      7 GND

-----------------------------