** communication protocol:                                               **

** sony: Computer activates DTR (pin 20) on rs-232 port,                 **

** (amiga 7-line handshake)                                              **

** then sends a command and expects ACK back. ASCII digits used for      **

** addresses,etc. Expect ACK back after EACH digit sent. Returns frame#  **

** as ASCII digits. Player is single character oriented.                 **

COMMANDS        CHAR    HEX     DECIMAL              COMMENT

----------------------------------------------------------------------------

ACK                     0A      10          Acknowledge receipt of a command

ADDR INQ        `       60      96          Get frame number

Clear Entry     A       41      65

Ch-1 on         F       46      70          Audio channel commands

Ch-1 off        G       47      71

Ch-2 on         H       48      72

Ch-2 off        I       49      73

Chapter # INQ   v       76      118         Get chapter number

Chapter # MODE  i       69      105         Set chapter mode

Clear All       V       56      86

Completion              01      1

Continue        a       61      97          Resume mode prior to STILL

Eject           *       2A      42          Open the disc compartment

Enter           @       40      64          Terminate a command

Error                   02      2

Forward Fast    ;       3B      59

Reverse Fast    K       4B      75

Forward Play    :       3A      58

Reverse Play    J       4A      74

Forward Scan    >       3E      62

Reverse Scan    N       4E      78

Forward Slow    <       3C      60

Reverse Slow    L       4C      76

Forward Step    =       3D      61          Variable speed forward play

Reverse Step    M       4D      77          Variable speed reverse play

Fwd Step Still  +       2B      43          Advance one frame forward

Rev Step Still  ,       2C      44          Advance one frame backward

Frame # Mode    U       55      85          Set frame mode

Index on        P       50      80          Set on-screen display ON

Index off       Q       51      81

NAK                     0B      11          Reject receipt of a command

Not Target              05      5           Target frame not found (in search)

Repeat          D       44      68          Playback a designated sequence

Search          C       43      67          Locate a target address

Status INQ      g       67      103         Inquire about status of player

Still           O       4F      79          Still picture

Stop            ?       3F      63          Halt video and audio playback

Video on        '       27      39

Video off       &       26      38

(and a few others.......)

SERIAL PORT CONNECTION:  (A standard DB25 serial cable with NULL-MODEM)

computer   player       (adapter will work.)

--------   ------

TxD 2      3 RxD

RxD 3      2 TxD

RTS 4      5 CTS

CTS 5      4 RTS

DSR 6      20 DTR

GND 7      7 GND

DTR 20     6 DSR

------------------