MUSIC IN THE KEY OF C

By Mark Lewis Baldwin

*****************************************************************************

NOTICE

This article and the attached program and code are Copyright (c) 1986 by

Mark Lewis Baldwin.  It is realeased as shareware.  It may be copied and

distributed freely so long as this notice and the copyright remain

intact.

If you feel that the article/program is of value, please send me a

pricely sum of $1 for its use.  If the code (or a substatial part

thereof) is used in a commercial product, I ask that a sum of $100 be

sent to me for licensed use of the code.  My mailing address is:

Mark Baldwin

Beacon Technical Services

7711 S Curtice Way, #E

Littleton, CO 80120

I can also be reached through GEnie at M.L.BALDWIN and through

Compuserve at 73637,3032.

I hope that you find the article of value.  Enjoy.

Mark Baldwin

*****************************************************************************

Over the past months, there have been many articles about using the

MIDI for music on the Atari ST.  But the ST has an internal sound chip

as well capable of a wide range of sound and music.  Lets have a look at

this sound chip and figure out how to make beautiful music with it.

What I'm going to do is describe the sound chip and how to talk to it.

Then we'll look at a program I wrote for the chip which allows us to use

three independent voices and even describe instrument ADSR envelopes.

Before I talk about the program, let us look at the sound chip itself.

The Chip and Its Registers

The ST sound chip is a General Instruments AY-3-8910 programmable

sound generator.  It has three tone generators and a noise generator

which can be mixed and matched on three output channels (or voices).  It

communicates with 16 registers, each an 8 bit word in length, from which

the user can communicate with the chip.  These registers are numbered 0

through 15.  We are actually only interested in the first 14 registers

(0-13) since the last two are used to communicate with the I/O port.

Each of the first 14 registers has a specific purpose which are

described below:

Register 0 and 1 - These registers control the frequency (or tone) of

the first voice (Channel A) of the sound chip.  A 12 bit word is

necessary to define the frequency, therefore the lower 8 bits of the

word are stored in register 0 and the upper 4 bits of the word are

stored in the lower half of register 1.  See the next section to find

out how to convert musical notes to the proper frequency.

Register 2 and 3 - These registers control the frequency of the second

voice (Channel B).

Register 4 and 5 - These registers control the frequency of the third

voice (Channel C).

Register 6 - The frequency of the noise generator is controlled through

this register.  Only the first 5 bits are used so acceptable values are

between 0 and 31.

Register 7 - This register is actually 8 on/off flags used to control

mixing of the tone and noise generator plus control of the I/O ports

(registers 14 and 15).  Bits 0 through 2 control mixing the tone signals

for channels A, B and C respectively.  A 1 in the bit position will turn

the tone for that channel off while a 0 will turn it on.  Bits 3 through

5 do the same thing for the noise generator.  Note that by turning on

both tone and noise, you can mix both signals together.  This may be a

little confusing, so let's try an example.  Lets say we want to turn the

tone on for Channel A and B plus merge the noise signal as well on

Channel B and leave Channel C off.  As a result, we need bits 0, 1, and

4 to be 0 while 2, 3 and 5 should be set to 1.  Or in binary it looks

like this 101100 (the right most bit is bit 0).  101100 in binary

converts to 44 decimal which we would place in register 7.

Register 8 through 10 - These registers control the volume for channels

A, B and C after mixing.  Values of 0 through 15 are acceptable with 0

being no sound and 15 being the loudest.  However, if bit 4 is set (i.e.

a value of 16), the volume is calculated from the sound envelope data

described in registers 11 through 13.

Register 11 through 13 - These registers describe a wave form that can

be used to control the volume of a voice should the voices respective

volume register be set to 16.  Registers 11 and 12 contain a two byte

word (lower byte in register 11) describing the length of the wave form.

This is calculated by taking the incoming clock frequency (2 Megahertz),

dividing by 256, and then dividing that result by the contents in

registers 11 and 12.  Register 13 describes the shape of the wave form

as shown in Figure 1.

Register 14 and 15 - As mention before, these registers have nothing to

do with producing sound but are used for the I/O ports of the ST so let

us leave them alone.

Converting Musical Notes For The Registers

Any musical note can be calculated as follows:

period = 284.0909091 * ( 1.059462 ** index )

After rounding 'period' to an integer, the lower byte would be

stored in the first register (i.e.  registers 0, 2, or 4) while the

upper byte would be stored in the second register (i.e.  1, 3, or 5).

The value of index is determined by counting the number of notes there

are between the desired note and a 440hz "A".  When counting, higher

notes are negative.  Remember that there are 12 notes to an octave, not

7 because of the sharps.  Therefore a "C" in the same octave as "A"

would have an index of 9 (with register values of 222 and 1) while a "B"

would be -2 (with register values of 253 and 1).  If you look in the

attached program listing, the note_f and note_c arrays contain register

data for 5 octaves around middle "A".

Talking to the Sound Chip

Now that we know what makes our chip sing (or at least croak), how

do we get the data into the chip registers.  Atari has solve that for us

by giving us two routines to communicate with the sound chip.

The simpler of the two routines is 'Giaccess'.  The C form is

char Giaccess( data, register )

char data

int register

This routine allows us to read or write to any single register in the

sound chip.  If you wish to write to a register (like putting in the

volume for a channel), 'data' would contain the value you wish to write

and 'register' would contain the register number plus 128.  The added

128 (which turns on bit 7) tells the routine that this is a write and

not a read action.  If you wish to read a register, just use the

register number itself, and the value will be returned.

The second routine for accessing the sound chip is 'Dosound'.  This

routine actually allows you to send an array of byte commands to the

chip and then forget about it.  Therefore, it is a little more complex.

The C call is of the form:

Dosound( ptr )

char *ptr

where *ptr points to a byte array containing the sound commands.  The

following commands are allowed for the byte array:

Commands 0 - 15 ($00-$0F): These commands are perceived as register

numbers in which the next byte will be inserted as data.  Therefore the

sequence 8 10 would put the value 10 in register 8.

Command 128 ($80): This command loads the next byte into a temporary

register.

Command 129 ($81): This command uses the next three bytes as data.  The

first byte is the number of the register in which the contents of the

temporary register will be loaded.  The second byte is a 2's compliment

value to be added to the temporary register.  The third byte is the

termination value.  The sound register will be loaded once each cycle

until the temporary register equals the termination value.

Command 130-255 ($82-$FF): The byte following this command either

terminates the byte array (with a 0) or defines the number of 50HZ

cycles before the next update.

The example program includes an example of using 'Dosound'.

Entering the Score

Now that we know how to produce sound out of the sound chip, how can

we turn that into music.  That's what the attached example program is

for.  I designed this program to play a three voice score using multiply

designed instruments.  You can use the program just as it is or include

it in you own programming projects.  Let's go through it step by step

and see how we use it to make the sound chip sing.

Since we want to be able to include music easily, I had to invent a

simple language to describe the musical score.  Two scores are included

named 'interstel' and 'spake'.  'interstel' is the theme for software

produced by The Interstel Company while 'spake' is "Thus Spake

Zarathustra" by Richard Strauss.  I used these two songs because I

previously used them in Star Fleet I for the ST and had the scores lying

around.  Note that although the Interstel Theme is copyright, you are

welcome to use it for your personal use with this program.  Each song is

an integer array where each value has a meaning to the score processor.

To make things simple, I've defined all of the parameters we need to

construct the score at the start of the program.  The following score

instructions are available:

SPEED n, - Change the song speed to n.  The larger the number, the

slower the song.  Start the score with a SPEED value, and insert it

again if you need to change speed part way through the song.  Remember

to put a comma after the number n.

<instrument> - There are three instruments defined in the program being

ORGAN, HORN and SNARE.  They indicate that all note entries following

will be played with that instrument until a new instrument is used.  A

little later I'll show you how to define your own instrument.

<note duration> - The following durations are available, WH, HF, QU, EI,

SX and TH representing WHole, HalF, QUarter, EIghth, SiXteenth and

THirty second.  They indicate that all note entries following will be of

the designated length until a new length is defined.  The length values

can be modified for dotted and tripleted notes by adding a D or T to the

end of the duration (i.e.  EID means eighth note dotted).

<note> - Five octaves have been defined.  Two above and two below the

middle octave.  A note is designated by the note (C,D,E,F,G,A or B), a

optional S for sharp, and the octave (2-6) where 4 is the middle octave.

Therefore FS3 would be F sharp below middle C.  The note will use the

currently active instrument for the currently active note duration.

REST - Rest (no sound) for the current note duration.

P - Play the current notes you've defined since the last P.

VU - Increase the volume of all following notes by one.

VD - Decrease the volume of all following notes by one.

VN - Return the notes to normal volume.

DO n, - Repeat the following music until the ENDDO n times.

Remember to put a comma after the n.

END - Terminate the music.

All you have to do to create a score is to build a data array using the

above defined parameters.

The Instruments

We also have a capability of defining instruments to a limited

extent in the program.  We can define the ADSR envelope for each

instrument.  ADSR stand for Attack, Decay, Sustain and Release and are a

means of defining a sound envelope for the instrument.  See figure 2.

The magnitude of each phase of the envelope is contained in the array

'inst_mag'.  Values of 0 to 15 are allowable.  These correspond to the

volume levels allowed on the sound chip.  The array 'inst_tim' describes

the time allowed for each phase.  Values of 1 to 15 are allowed here

with one exception.  If a value of 16 is used for the sustain phase (3rd

value), the program will calculate the length of the sustain based on

the length of the note being played.  Therefore, long notes will be

drawn out while short notes will be cut short.  Go ahead and play with

the values or add as many more instruments as you wish.

Since we can also mix the noise generator with the tone generator,

two more arrays have been defined.  The first is 'inst_tone'.  A value

of TRUE mixes in the tone generator.  Secondly, we have 'inst_noise'

which will mix in the noise generator if TRUE.  Note that either one can

be turned on or even both of them at once as I did for the SNARE

instrument.

The Code

The program first calls 'init_music' to initialize some

precalculations and clear the sound chip.  It then starts a song by

entering 'play_music' passing a pointer to the song array.  'play_music'

actually processes the song array separating out the notes, durations,

instruments, etc.  When it's ready to play some notes, it calls

'inst_sound' with arrays containing the note(s) it wishes to play, the

instrument assigned to each note, the duration of each note, and the

time 'inst_sound' should play until returning to 'play_music'.  Note

that if less than three notes are passed and 'inst_sound' still has some

of a note left over from a previous call, it will continue to play the

old note as well as the new ones.

'inst_sound' actually builds the data array that we are going to

pass to the sound chip through 'Dosound'.  The function 'sound_build'

takes the register and value data and places them in 's_array' which is

our data array.  The function 'calc_s_vol' used toward the bottom of

'inst_sound' is the routine which actually calculates where we are in

the ADSR envelope for each of our voices, and returns a volume level.

Finally, 'sound_go' takes the sound array and passes it to 'Dosound'.

Play It Again, Sam

I hope that now that you've been exposed to the Atari sound chip,

you'll see that there are a great deal of possibilities for it.  The

program will play it's two songs as much as you wish, but I hope you dig

into the program yourself and add more songs or instruments.  Or even

improved on the processing code itself.  I can already think of half a

dozen ways to improve upon the program.  If you come up with any

improvements, I'd love to see them, so drop me a line.  Until then,

happy sounds to you.