Using the BSE Flashdrive with the Atari Portfolio

-------------------------------------------------

You will need to copy the files: ATFLASH.SYS and ATPART.EXE to the

Portfolio.  Put ATFLASH.SYS in the root directory on RAM drive C:,

ATPART can be placed on a RAMcard.  Connect the Flashdrive to the

Portfolio using the Portfolio Smart Parallel Interface port.  Use a

cable that is as short as possible.

Preparing the Flashdrive for use:

---------------------------------

The Flashdrive hard disk must be prepared before it can be used: There

are two basic steps to preparing it.  First, the drive must be

partitioned.  This defines one or more areas on the disk that will be

assigned logical drive letters (D:, E:, etc.).  The largest partition

allowed is 32 megabytes, so if the basic capacity of your drive is

larger than 32 meg, you will need to define multiple partitions to be

able to use all of that capacity.  The second step in preparation of

the disk is formatting.  Formatting creates a directory structure in a

partition that allows DOS to locate files.  Each partition must be

formatted before it can be used.

Important: If you have formatted your Flashdrive on a computer using a

version of DOS 3.0 or greater, it WILL NOT WORK properly on the

Portfolio, which uses DOS version 2.11.  You will need to reformat it

with the ATPART.EXE program before it can be used.

To partition and/or format the Flashdrive, run the ATPART.EXE program.

After a message describing the configuration of the Flashdrive appears,

press any key to get to the main menu, where you can choose to create,

delete, format or view the current partition(s).  From the main menu,

pressing <ESC> will exit the program and return to DOS.  If any

partitions have been created or modified, you will be asked if you want

to save the changes.  If you respond with 'N' the changes you made will

not be saved.  If you press 'Y', the new partition data will be saved,

effectively erasing any data that may have been on the disk.  If you

choose neither 'Y' or 'N' and press <ESC> again, you be returned to the

main menu.

Creating a partition: Select main menu option 1 to go to the Create

Partition sub-menu.  There are two types of partitions you can create:

DOS and EXTended partitions.  A DOS partition is limited to a maximum

size of 32 megabytes.  An extended partition may be any size, and

contains one or more logical DOS partitions of 32 meg or less.  The

first partition on a disk must be a DOS partition.  Up to three more

DOS partitions may be defined, or an extended partition may be defined.

After defining an extended partition, logical DOS partitions must be

defined to fill the space it occupies.

After selecting the type of partition you want, the maximum number of

disk cylinders available to allocate to the partition are displayed at

a prompt.  You can press ENTER to select this default value, or type in

a smaller number.  (When defining an extended partition, always select

the maximum amount -- you can subdivide the partition later).

Examples of typical partition configurations:

1. 20 megabyte drive, one 20 meg partition:

Select the first option: Create DOS partition

Press ENTER to select the maximum number of cylinders

2. 60 megabyte drive, three 20 meg DOS partitions:

Select the first option: Create DOS partition

Look at the TOTAL number of cylinders on the disk displayed at the

top of the screen.  Divide this number by three and enter the result

as the number of cylinders to allocate.

Repeat two more times.

3. 60 megabyte drive, one 32 meg DOS partition and a 28 meg extended

partition containing two 14 meg partitions.

Select the first option: Create DOS partition and use the maximum

number of cylinders (32 meg).

Select the second option: Create EXT partition and use the maximum

number of cylinders.

Select the third option: Create logical partition

Take the number of cylinders displayed at the prompt and divide by

two and enter the result.  This will create a 14 meg DOS partition

in the extended partition.  After you press ENTER, you will be

prompted again to enter the number of cylinders to allocate to the

next logical DOS partition.  This will continue until all available

cylinders have been used.

If you define four partitions or less, there is no real advantage to

using extended partitions, and you might as well use four standard DOS

partitions.  However, if you want more than four partitions you must use

an extended partition.

Deleting partition(s):

Select main menu option 2 to go to the Delete Partitiion menu.  You can

delete the LAST partition defined, or delete all partitions.

After deleting or creating partition(s), the changes are not actually

saved to disk until you exit the program, or choose to format the

partition(s).  At this point you have the choice of saving the

partition data or quitting without saving.

Displaying partitions:

Main menu option 4 allows viewing the current partition status.  You

can see up to four partitions on the screen at one time.  You will see,

starting from the left, the partition number, the starting and ending

cylinders of the partition, the partition size in cylinders, and in

megabytes, and the partition type.  If any extended partitions are

defined, you are offered the option of seeing the extended partition

contents.  A number followed by a + sign at the right shows how many

logical partitions are in the extended partition.  Extended partitions

do not have a partition number; only DOS partitions and logical DOS

partitions are numbered.

Formatting a partition:

After you have defined a partition structure for the disk, you will

need to format the partition(s).  Select main menu option 3 to reach

the Format Partition menu.  Here you can choose to format a single

partition by number (see Displaying Partitions to find out how to

determine the number of a partition).  Select menu option 1, then enter

the partition number and press ENTER.  The partition will be formatted,

erasing all data in it.  If you like, you can choose to format all

partitions at once using menu option 2.  This will effectively wipe out

the entire disk data area.

It is possible to run the ATPART.EXE program without changing the

partition structure if you want to re-format a partition.

Using the prepared Flashdrive:

------------------------------

For the Portfolio to recognize the existence of the Flashdrive, the

ATFLASH.SYS device driver must be installed.  This is done by placing

ATFLASH.SYS on drive C: and editing the CONFIG.SYS file on drive C: (if

you don't have a CONFIG.SYS file already, you will need to create one).

Using the Portfolio's built in text editor, add the following line to

the CONFIG.SYS file:

DEVICE = C:ATFLASH.SYS /P=8078 /T=15

The /P=8078 option tells the driver to use port address 8078 (hex) to

access the parallel port.

The /T=15 option sets the disk drive timeout delay.  This helps save

battery power in the Flashdrive by turning the motor off when not in

use.  The motor will turn back on automatically when the disk is

accessed.  You can change this number i#ς##O�##��π#π###O�#��ς#ς##pp��#�#π##�ρ��O���#�O��O��###ω�##ώ##�###�

##��@�p�p�#��#τ##�#�O���O�##��#@�##O��#�Aπ###O�##�O���O���##���#�O�#O��#��	π#�p�#�χ��##O�#���##��#�#ρ�O��

�##ρ��O��#���#�#@�ς##O�#�##�τ##O�pp���#Ì�###q#�C)#�#�++q##+�#�K#Ks9#�C)##K�Y##�K�)#k{#+a##s!#�+�K#`hSs�k#+�a##�#�+ca##�#	##+�#�K��K{q#{1#�C)###�##K��#{1#+##A#c{;K##a##�K�)q#hRK1##q#+��{################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################to switch the Flashdrive on, do so BEFORE turning on the Portfolio.  If

the Flashdrive has been disconnected from the Portfolio, remember to

press CTRL-ALT-DEL to reboot the computer after turning them on.

umber of cylinders

2. 60 megabyte drive, three 20 meg DOS partitions:

Select the first option: Create DOS partition

Look at the TOTAL number of cylinders on the disk displayed at the

top of the screen.  Divide this number by three and enter the result

as the number of cylinders to allocate.

Repeat two more times.

3. 60 megabyte drive, one 32 meg DOS partition and a 28 meg extended

partition containing two 14 meg partitions.

Select the first option: Create DOS partition and use the maximum

number of cylinders (32 meg).

Select the second option: Create EXT partition and use the maximum

number of cylinders.

Select the third option: Create logical partition

Take the number of cylinders displayed at the prompt and divide by

two and enter the resu