_#I_#n_#t_#r_#o_#d_#u_#c_#t_#i_#o_#n

Passm, a PAL Assembler,  allows  the  user  to  write  boolean

equations  describing  the outputs of a PAL and generates a JEDEC fuse

file. This assembler supports most 20 and 24 pin PALs. Currently,  the

list of parts supported are:

20 Pin PALs             24 Pin PALs

------------            -----------

10L8, 10H8               12L10

12L6, 12H6               14L8

14L4, 14H4               16L6

16L2, 16H2               18L4

16L8                     20L2

16R4                     20L10

16R6                     20L8

16R8                     20R4

20R6

20R8

22V10

The sources for PASSM are included so that customizing for  additional

parts  can be done. The program has been writen in Pascal and conforms

with ISO Pascal, except for two areas. One, it  uses  the  'otherwise'

descriptor for the CASE statements, and two, it uses packed arrays.

The following is a description of PASSM syntax.

_#1.  _#L_#a_#n_#g_#u_#a_#g_#e _#D_#e_#s_#c_#r_#i_#p_#t_#i_#o_#n

Before describing the syntax for PASSM, let  us  go  thru  the

notation  used  in  describing the language. When showing a PASSM key-

word, it will be enclosed using angle brackets, > and <  .  The  ASCII

string  enclosed  within these bracket must always appear in such form

(ie. use the indicated case, as well as spaces, etc.).  When  a  given

portion,  or  section,  is optional, it will be enclosed with { and }.

With this in mind, let us start.

A PASSM source file  consists  of  three  main  sections,  all

enclosed  within a module description.  These three sections are: one,

the device specification (ie. what type of PAL), two, a pin list  (ie.

symbolic  names  to  be  attached to a particular pin), and three, the

equations section (ie. a boolean description of a given output).   All

comments in PASSM start with a double quote >"< and are assumed to end

with the end of line terminator (ie.  carriage  return).  Also,  valid

PASSM delimiters are tabs or space characters.

Every source file MUST have  a   'module'  declaration.   This

module  declaration informs PASSM that it must start assembling a par-

ticular PAL description. A module declaration consists of the  follow-

ing:

Passm                                                           Manual

>module< module_name{;}   {optional_sections}

device_type_declaration

pin_list_section

boolean_equations

>end<

The module_name is an identifier, no more than  15  characters,  which

must  start with a letter. The set of valid characters for an identif-

ier is : a..z, A..Z, underscore '_', and the minus  sign  '-'.  Notice

that the semicolon ending the module name is optional.

The next element within a module is the optional_section. This

section  is being reserved for future features, such as flags indicat-

ing what type of reduction to do with the boolean  equations,  include

files, etc. At the present time, no optional features are enabled.

The next section is the device_type_declaration. This  section

informs PASSM as to what type of device is being targeted, ie. a 16r4,

etc. This section syntax is as follows:

device_name  >device<  >'<part_type>';<

The device_name, an identifier, is the symbolic name  with  which  the

user  may  refer  to the PAL. The part_type is the actual device being

programmed.  The following is the list of available devices:

20 Pin Pals     24 Pin Pals

-----------     -----------

p10l8, p10h8     p12l10

p12l6, p12h6     p14l8

p14l4, p14h4     p16l6

p16l2, p16h2     p18l4

p16l8            p20l2

p16r4            p20l10

p16r6            p20l8

p16r8            p20r4

p20r6

p20r8

p22v10

Notice that the part_type must appear as shown above.

The next section in a module description is the pin list. This

section provides a symbolic reference between the signal names used in

the equations, and the device physical pins. The syntax followed is:

signal_name {, signal_name}  >pin< pin_numbers >;<

Page 2