The  MUSSCALE  program is an example of a way  to  play  musical

notes  from a Pascal program.   After playing with  several  BASIC

programs that played music on the internal synthesizer,  I thought

that  BASIC was simply doing a little bit fiddling to produce  the

notes. I quickly found out that there is more to it than that.

Although  I  have  two books devoted to ST sound  and  the  Abacus

Internals book,  I couldn't figure out how to get a musical scale.

There  was some non-trivial conversion required.  As a  matter  of

fact,  Internals has some information that is actually  wrong.  It

says  the  period information is divided into  a  'set  frequency'

field and a 'step size' field.  Not so.    It is one single 12 bit

number.

The  scale we use for music divides an octave up into 12 notes. Two

notes  F0 and F1 where F1 is one semitone higher than F0 have  the

relationship:  F1  :=  F0 * 2 ^ (1 / 12).   To  establish  a  base

frequency,  I  got  a  zero beat between  the  synthesizer  and  a

keyboard.   From this I established middle C as having a period of

478  on the ST.   With this background,  the program  should  make

sense  to  you.   Note      that    the sound chip can  produce 27

frequencies *between* middle C and the adjacent B!

I  got the number 478 by beating against a Yamaha PSR-11  keyboard

which  has the pitch still at the factory setting.   Someone  with

better  ears and/or equipment could, perhaps, refine that  figure.

Please let me know of any improvements you come up with.  The

program includes an easy way to tune the synthesizer channel. You

can almost 'hear' a string getting tighter and looser as you tune.

The  program  has  code to turn the key clicker off  and  back  on

automatically   without  using  the   CONTROL.ACC.   The   clicker

interferes badly with sound production.