Yamaha TX81Z Editor Librarian

Version 2.01

by John Davis

This program is a completely GEM-based voice and  performance

editor  and  librarian  for the Yamaha TX81Z  FM  Tone  Generator.

Though not tested,  I believe some of the functions will also work

with the Yamaha DX21/27/100 synthesizers.   The voice data is  not

compatable with the Yamaha FB01 Tone Generator.

The  program  allows you to edit all  voice  and  performance

parameters  directly from the ST.   It also allows you  to  create

and  edit banks of 32 voices or 24 performances,  and  save  those

banks to disk or the synthesizer.

Getting Started

After  unArcing  TXLBR2.ARC,  you should  find  the following

files on your disk:

TXLIBR21.DOC   The file you're reading.

TXLIBR20.PRG   The program file.

TXLIBR.RSC     Resource file for voice editing functions.

TXPERF.RSC     Resource for performance editing.

INTERNAL.PRF   Bank Performances containing the Performances

initially supplied with the TX81Z.

INTRNALA.BNK

INTRNALB.BNK   Banks A and B of the TX81Z internal voices.

README         Additional hints, bug documentation, etc.

It  is  important that the files  TXLIBR.RSC  and  TXPERF.RSC

remain in the drive you boot the program from.   The program loads

the resources as needed,  and will fail if the necessary  resource

file  isn't  found.   If  you  are  sure  you  will  not  use  the

performance  editing functions,  the program disk can  be  removed

from the drive after booting.   When  I  get the  new  version  of

Megamax,  the program will be redone so that only 1 resource  file

is needed,  and data will not be lost when switching from voice to

performance editing.

Voice Librarian

After  getting  the program started,  you will see  a  screen

containing two empty banks of voices, an editing box between them,

and  a  status  box at the bottom of the  screen.   The  menu  bar

contains Desk, File, and Performance Editor entries.

At all times,  a simple Midi-Thru function is in effect.   If

you  have  a keyboard connected to the Midi-In port  of  your  ST,

playing  it will cause the notes to be echoed throught the ST  and

to  the TX81Z.   Clicking the left mouse button will cause a  Note

Off event to be sent.  One problem I have encountered is selecting

a  voice or performance while holding a key down.   The  TX81Z  is

silenced,  but sometimes the keyboard will freeze up,  requiring a

reset  to start working again.   I don't know if this  problem  is

unique  to Casio,  or if it will affect other keyboards  the  same

way.   Also,  this  only seems to occur when selecting  voices  or

performances - not while editing the voice or performance.

The  first thing you should do is select FILE from the  menu,

and choose one of the following actions:

SYNTH -> BANK A

SYNTH -> BANK B

Transfers  voices  from Bank I of the synthesizer  to  either

Bank  A or Bank B of the program.   The synthesizer  must  be

properly connected and ready to transmit,  or the status  box

will report an error.

BANK A -> SYNTH, BANK B -> SYNTH

Transfers the voices from Bank A or B of the program to  Bank

I of the synthesizer.  You must first manually set the Memory

Protect on the synthesizer to off, or the transfer will fail.

BANK A -> DISK FILE, BANK B -> DISK FILE

Saves the voices from Bank A or B to a disk file.

DISK FILE -> BANK A, DISK FILE -> BANK B

Load Bank A or B with voices from a disk file.

QUIT

Quit the program and return to the desktop.

After  loading a bank with voices,  you can make a  selection

from the edit box in the center of the screen.   Point to a voice,

and click once to select it.   As soon as a voice is selected,  it

is instantly sent to the edit buffer of the synthesizer.

EDIT   Transfer the selected voice to the edit buffer,  and  begin

editing.

COPY   Make a copy of the selected voice in another location.  The

status  box  will ask you to select a destination  for  the  copy.

Point  to any voice slot in either bank,  and  click  again.   The

program will prompt for confirmation,  then transfer a copy of the

selected voice into the new location.

MOVE   Just like COPY,  but will delete the original voice  after

making the copy.

SWAP   Exchanges the contents of two voice slots.

DELETE Deletes the data for the selected voice.

Voice Editor

Select  a  voice,  then click on EDIT to  begin  editing  the

voice.  The selected voice will be transferred to the synthesizer,

and the editing screen will be shown.

To edit any parameter,simply point at it,press the left mouse

button down,  and drag the mouse.  Dragging down will decrease the

value,  and dragging up will increase it.   Any parameters changed

during  the current edit will be redrawn in red (or  whatever  you

have  selected for color 2 in the control panel).   When  changing

parameters near the top or bottom of the screen, you may find that

you  don't have enough room to drag the mouse for the  change  you

want.   If so, drag the mouse quickly in the opposite direction of

the desired change,  then move it slowly in the desired direction.

Because of the way the program samples the mouse position, data is

changed  much more quickly with slow mouse movements.   Note  that

the program will not allow you enter a value greater or less  than

that  allowed by the TX81Z.   All data is represented  exactly  as

shown in the TX81Z's edit window, with the exception of coarse and

fine  frequency ranges.   I still haven't figured this one  out  -

you're on your own here!

On  the right side of the screen,  you will find another  box

with the following buttons:

INITIALIZE

Sets all data to the initial values shown in the TX81Z owners

manual.  Useful if you want to start from scratch.

RANDOMIZE

Will  randomize the values for the voice  being  edited.   To

produce the most consistant results,  the program gets all of  its

random values from the voices located in Bank A.   Note that  this

applies even if you are editing a voice from Bank B - Bank A  must

be  filled with valid voices.   This feature can be used  to  your

advantage in several ways.   For example,  if you are looking  for

string-type sounds,  load Bank A with string voices, then throw in

a  few horns or something for variety.   Or,  if you have a  voice

that's  'almost right',  fill all but 3-4 slots in Bank A  with  a

copy of that voice,  then add a few voices which have the type  of

change your looking for.   Now,  when you randomize,  most of  the

parameters will remain unchanged.   Depending on how many  'other'

voices  you added,  you will find that some of the parameters  get

changed to the values in your 'other' voices.

MIDIMOUSE

This  is a simple function that will allow you to send  Note-

On,  Note-Off data to the synth.   Pressing the left mouse  button

will  sample the mouses position,    and send a Note-On event with

the pitch and velocity corresponding to the mouses position. Going

from left to right will increase the pitch, and top to bottom will

increase the velocity.   Releasing the button will send a Note-Off

event.   You can press and hold the mouse button, then drag it for

some  interesting  effects.  A  Note-On  will  be  sent,  followed

immeditely  by  a  note  off (as long as  the  mouse  is  moving).

Clicking the right mouse button will cancel Midi_Mouse.

RESTORE

This will restore all voice data to the values when you first

began editing.

CANCEL

Ignore all changes made to this voice and return to the  Bank

Editor screen.

FINISHED

Save  all  changes made to the voice selected from  the  Bank

Editor screen, and return to the Bank Editor.

There  are 3 parameters in the Edit Screen which don't  apply

to the TX81Z,  but were included for any DX21/27/100  owners.  The

parameters  SUSTAIN,  PORTAMENTO,  and CHORUS (drawn in  blue,  or

color 4) can be edited,  but the changes are ignored by the TX81Z.

OPERATOR ON/OFF changes are sent to the synth,  but are not  saved

with  the  voice data,  and are always set to ON  when  you  begin

editing a voice.

Performance Librarian

The  Performance Editor/Librarian is activated  by  selecting

PERFORMANCE  EDITOR from the drop down menu.   Make sure that  the

file  TXPERF.RSC is located on the disk in the   boot  drive,  and

that you have saved all changes before continuing.

As with the Voice Editor/Librarian,  Midi-Thru is always  on,

and  selecting  any performance will immediately send  it  to  the

synthesizer.

The Performance Editor/Librarian functions identically to the

Voice  Editor/Librarian.   The Menu entries are the  same,  except

that there is no QUIT selection,  and PERFORMANCE EDITOR has  been

replaced with MAIN MENU.   Selecting MAIN MENU will let you  leave

the Performance Editor and return to voice editing.   Once  again,

make  sure  that  TXLIBR.RSC is located on the disk  in  the  boot

drive, and that all changes have been saved.

Limited  error  checking  is done while  in  the  Performance

Editor.   It  will  not allow you to have more than 8 notes  at  a

time.   If you try to increase the total count to more than 8, the

program  will  stop your editing at the maximum  allowable  value.

Since Bank I could contain anything, the voice names for that bank

have been replaced with BANK I -1 to BANK I - 32.  All other banks

will show the correct voice name.

While in the Performance Editor, you will see 5 buttons which

have been de-activated - DELAY,  PAN,  CHORD,  MICROTUNE KEY,  and

MICROTUNE  TABLE.   These functions will be incorporated into  the

version 3.0 of this program.

-----------------

Well,  that's it.   Hope you find this program  useful.   The

program is still evolving, keep an eye out for version 3.0!

Don't send money or anything,  just upload all those great patches

and performances to my BBS!

L.A.C.E. II BBS

(806)792-3317

Online 24 hours a day with over 110 meg

of disk storage!

Also,  I can be reached on Compuserve with PPN 72465,227,  or

on GEnie with J.W.DAVIS.