TABLE OF CONTENTS

InfraRed.library/IRmain

InfraRed.library/IRgetlen

InfraRed.library/IRconrad

InfraRed.library/IRdma

InfraRed.library/IRsetversion

InfraRed.library/IRmain                            InfraRed.library/IRmain

NAME

IRmain -- Send ir data

SYNOPSIS

IRmain( data , puffer , send , type , channel )

A0     A1       D0     D1     D2

void IRmain( char *, char *, int , int , int )

FUNCTION

Sends previousely learned data.

INPUTS

data    pointer to data for the ir signal.

puffer  pointer to a memoryblock which will be used as puffer.

Its size must be at least GetLen( data ) bytes.

send    Number of impulses which should be send.

type    type of signal, supported types are:

TYP_KHZ37

TYP_KHZ45

TYP_NOPULSE

TYP_KHZ66

There are also other types but they can't be used

with IRmain().

channel the output channel:

0   send signal to the joystickport (master) hardware

>0  send signal to the parallelport hardware, too:

bit 1 == 1st sender

bit 2 == 2ng sender

...

bit 8 == 8th sender

RESULTS

NOTES

SEE ALSO

BUGS

InfraRed.library/IRgetlen                          InfraRed.library/IRgetlen

NAME

IRgetlen --  Get the size of memory needed to send this command.

SYNOPSIS

size = IRgetlen( data )

int IRgetlen( char * );

D0          A0

FUNCTION

This routine returns the required puffer size to send this command.

To senda command you must use GetLen to get the puffersize, then

allocate the puffer and at last use IRmain or IRdma to send the

signal.

INPUTS

data        pointer to ir data (must be learned, of course).

RESULTS

size        Size of required puffer to send this command

NOTES

SEE ALSO

BUGS

InfraRed.library/IRconrad                          InfraRed.library/IRconrad

NAME

IRconrad  --  Send Conrad-Relaicardspezific ir signals

SYNOPSIS

IRconrad( mask )

void IRconrad( int )

D0

FUNCTION

This routine sends specific already learned in ir signals for a

relaicard from conrad electronics.

Please contact the authors for more information.

INPUTS

mask        bitpacked information what to send (contact the authors).

RESULTS

NOTES

SEE ALSO

BUGS

InfraRed.library/IRdma                             InfraRed.library/IRdma

NAME

IRdma -- Send ir data via audio hardware

SYNOPSIS

IRdma( data , puffer , send , type  )

A0     A1       D0     D1

void IRdma( char *, char *, int , int )

FUNCTION

Sends previousely learned data via the audio hardware.

INPUTS

data    pointer to data for the ir signal.

puffer  pointer to a memoryblock which will be used as puffer.

Its size must be at least GetLen( data ) bytes.

send    Number of impulses which should be send.

type    type of signal, supported types are:

TYP_KHZ37

TYP_KHZ45

TYP_NOPULSE

TYP_KHZ66

There are also other types but they can't be used

with IRmain().

RESULTS

NOTES

SEE ALSO

BUGS

InfraRed.library/IRsetversion                      InfraRed.library/IRsetversion

NAME

IRsetversion -- Set version of hardware

SYNOPSIS

IRsetversion( version )

D0

void IRdma( int )

FUNCTION

Set version of joystickport hardware.

INPUTS

version     0 means hardware is designed for IR-Master 1.00 to 2.61

1 means hardware since version 3.0 of the IR-Master

RESULTS

NOTES

SEE ALSO

BUGS