S P L O T  -  The Pen Plotter Simulator V2.51

---------------------------------------------

Copyright (C) 1989-93 by Alexandr NOVY

All rights reserved.

U S E R ' S   M A N U A L   F O R   MS-DOS   V E R S I O N

----------------------------------------------------------

July 1993

1. INTRODUCTION

---------------

The SPLOT program is  a highly effective tool  intended for

simulating the pen plotter on the display of an IBM PC.   The

SPLOT program is  the very thing  you need to  make your work

with the plotter more effective. The SPLOT program will  give

you a guarantee you will always draw on your plotter what you

really want. That is why it will save your nerves, your time,

your plotter, pens and paper, that means your money.

The SPLOT program can offer:

- active support of about 50 HP-GL instructions (according

to the type of your plotter).

- active support of 20 DXY-GL instructions (for ROLAND

plotters).

- possibility of exporting a  picture to the printer and  in

this way to obtain an output of a high quality. The  SPLOT

program uses the  best graphics resolutions  available for

each printer  that it  supports and  optionally uses  data

packing to improve transfer speed.  In this way the  SPLOT

program will  make a  fast and  excellent plotter  of your

printer. It supports all usual types of printers including

color ones.

- exporting  a  picture  to  the  HP-GL,  HP-GL/2,  DXY-GL,

PLOTCALL,  DXF  and  PostScript  formats.

- possibility of making picture cuts (the ZOOM function). In

this way you can scan,  draw on the plotter or  print only

the selected part of the picture.

- possibility  of  completing  the  picture  with  a    text

including multilingual characters - in this way the  SPLOT

program will teach  your plotter German  (Spanish, French,

etc.)  easily  and  quickly  without complicated switching

between  character  sets!   The  built-in  HP-GL or DXY-GL

exports respectively, will make it possible. Due to  these

exports it is also possible:

-  to  offer  a  highly  optimized output for your plotter

(where each pen is grasped only once).

- to  teach  your  plotter  the advanced instructions  you

would get only  if you buy  a more expensive  plotter or

additional  hardware  (e.g.,   the  SPLOT  program   can

simulate  connecting  an  additional  HP 17440A graphics

enhancement cartridge to the HP 7440A ColorPro plotter).

The SPLOT program can be used by:

- anybody who wants to work with his plotter in the WYSIWYG

style (What You See Is What You Get).

- anybody  who  wants  to  develop  his  own plotter  program

applications.   You  can  reach  it  with the SPLOT program

easily even if you do not own the plotter at all!

- anybody  who  wants  to  use  his  printer  as  a   plotter

occasionally or constantly. In  many cases using a  printer

may be faster and moreover, this may be an alternative of a

high quality.

- anybody who wants to program a printer in graphics mode and

at  a  high  level  and  who  does  not  want  to   develop

complicated software which would enable it. In these  cases

using  the  HP-GL  language  in  connection  with the SPLOT

program  seems  to  be  very  advantageous for creating own

graphics outputs. The HP-GL  language can  be learned  very

easily and in addition, it offers unexpected possibilities.

The  SPLOT  program  is   available  in  various   versions

according to  the type of simulated plotter. This  means that

each SPLOT.EXE  module is  designated for  a certain  plotter

type.   This  principle  gives  the  maximum  reliability  of

simulation of the given plotter type. At present versions for

the following plotter types are at your disposal:

Plotter type                          | Plotter syntax

------------------------------------------------------------

ROLAND DXY 1100/1200/1300             | DXY-GL / RD-GL I

HP 7470A                              | HP-GL

HP 7475A                              | HP-GL

HP 7440A ColorPro                     | HP-GL

HP 7550A                              | HP-GL

HP 7570A/7575A/7576A DraftPro DXL/EXL | HP-GL

HP 7595A/7596A DraftMaster I/II       | HP-GL

An UNREGISTERED COPY  of the program  is available only  in

the version for the HP 7475A plotter. The versions for  other

plotter types can  be received only  on the basis  of regular

registration. See next chapter  for licensing, evaluation and

registration information or see file LICENCE.DOC for details.

The  SPLOT  program  is  designated  for  using with IBM PC

computers  and  compatibles,  and  for  the MS-DOS and PC-DOS

version  3.00  and  higher  ones ( the MS-WINDOWS version  is

available too ).  In the  DOS version  graphics  cards EGA or

VGA  are  required.  To  run  the  program  comfortably it is

recommended to use a Microsoft Mouse or a compatible one.

2. LICENSING, EVALUATION and REGISTRATION INFORMATION

-----------------------------------------------------

SPLOT  is  distributed  as  "Shareware"  or  "User  Supported

Software"  and  is  fully  copyrighted.  If  you register the

program you will receive these benefits:

1) The latest version of SPLOT.

2) A printed user's manual.

3) A printed technical reference which describes

all plotter instructions supported by SPLOT.

4) An end to the nag screens and opening banners.

5) Deep discounts on the next major versions.

6) A collection of useful utilities:

- BPRINT.COM - prints binary files on a printer

on time background with many options.

- BPLOT.COM  - plots files on a pen plotter on time

background with many options.

- CLPT.COM - redirects printer ports to a disk file.

- CPORT.COM - redirects serial ports to a disk file.

The  single  user  registration  fee  for SPLOT is $70.00 US.

The multisystem site fee for SPLOT is $150.00. Users  outside

of   Europe   should   include   an   additional   $5.00  for

international airmail.

Author can accept payment in these three ways:

1) In cash (money enclosed) - this is the most quick method,

but at your risk.

2) Credit card  -  MasterCard, VISA and American Express are

accepted.  Please,  fill out  carefully  the  credit card

information in order form.

3) Bank or travelers cheque enclosed - please make  a cheque

payable in US funds.

Since the SPLOT program is always distributed in the  version

adapted for a certain plotter type, do not forget to  mention

the  plotter  type  with  which  you  want  to  use the SPLOT

program.  A list of plotters you can choose from is given  in

the  first  chapter  or  in  the  PLOTTERS.DOC  file.  In the

PLOTTERS.DOC  file  there  are  also  given the parameters of

these   plotters   together   with   a   list   of  supported

instructions. It makes possible to choose a suitable  plotter

type even if  your plotter is  not listed (most  plotters are

compatible with some plotter from the list).  Of course,  you

can order more SPLOT  program versions for different  plotter

types.  In this case the single user registration fee for any

further version is  $20.00 (+ $5.00  outside Europe) and  the

multisystem site fee is $40.00 (+ $5.00 outside Europe). This

principle  is  also  valid  for  an  upgrade.   If  you  have

registered at  least one  SPLOT program  version at  the full

price of $70.00 ($150.00), now and in the future only  $20.00

($40.00) is to be paid for all other versions.

Special offer:

Registered  users  of  the  MS-Windows  version  of the SPLOT

program  can  order  the  MS-DOS  version  for  $50.00   only

( $100.00 for the multisystem site licence ) (+ $5.00 outside

Europe). In this  case fill out  your serial number  in order

form and mark  it as `upgrade  from Windows version'.

Note:  The latest MS-Windows version of the SPLOT program  is

available  on  all  SIMTEL20  archives  in   <MSDOS.WINDOWS3>

directory as SPLTXXXW.ZIP file (recently SPLT220W.ZIP).

To register, please use the registration/order form  supplied

with this package (as provided in the file ORDER.FRM).

Please send the completed form along with a payment to:

Alexandr Novy

Havlickova 2209

390 01 TABOR

Czech Republic

NOTE: Czech users of SPLOT should contact Czech distributor,

SOFTECH, P.O.B. 69,  323 00 Plzen 23,  Czech Republic,

Tel./Fax  0042-19-226294.  A  special  Czech  language

version of the package is available from them as well.

EVALUATION AGREEMENT

SPLOT  is not  and  has never been  public  domain  software,

nor is it free software.

Non-licensed users are granted a limited license to use SPLOT

on  a  14-day  trial  basis  for  the  purpose of determining

whether SPLOT is suitable for their needs.  The use of SPLOT,

except for the initial 14-day trial,  requires  registration.

The use of unlicensed copies of SPLOT by any person,business,

corporation or any other entity is strictly prohibited.

No one may modify or  patch the SPLOT executable file  in any

way, including but not limited to decompiling, disassembling,

or otherwise reverse engineering the program.

A limited  license is  granted to  copy and  distribute SPLOT

only  for  the  trial  use  of  others,  subject to the above

limitations, and also the following:

1) SPLOT must be  copied in unmodified form,  complete

with the file containing this license information.

2) The full machine-readable SPLOT documentation  must

be included with each copy.

3) SPLOT  may not  be distributed  in conjunction with

any other product without a specific license to  do

so from author.

4) No  fee,  charge,  or  other  compensation  may  be

requested or accepted, except as authorized below:

a) Operators of  electronic bulletin board  systems

(sysops)   may   make   SPLOT   available    for

downloading as long as the above conditions  are

met.   An overall  or time-dependent  charge for

the  use  of  the   bulletin  board  system   is

permitted as  long as  there is  not a  specific

charge for the download of SPLOT.

b) Vendors of user-supported or shareware  software

may  distribute  SPLOT  after  obtaining written

permission  from  author.  Such  permission   is

usually granted. Vendors may charge a reasonable

disk duplication and handling fee.

LICENSING AGREEMENT

Licensed user are bound to the following licensing agreement.

You may not transfer your license without written  permission

from author.  A single  user license  permits a  user to  use

SPLOT only on a single computer at a time.

This product is distributed  AS IS.  The  author specifically

disclaims all  warranties, expressed  or implied,  including,

but not limited to, implied warranties of merchantability and

fitness for a particular  purpose with respect to  defects in

the diskette and  documentation, and program  license granted

herein,  in  particular,  without  limiting  operation of the

program license with  respect to any  use or purpose.   In no

event shall the author  be  liable for any loss of  profit or

damage  including  but  not  limited  to special, incidental,

consequential or other damages.

By licensing SPLOT you agree to the above licensing agreement.

3. THE SPLOT WORKING PRINCIPLES

-------------------------------

The plotter  simulation consists  in  interpreting  plotter

instructions contained  in a  disk file.   The SPLOT  program

reads these instructions from the file and plots them on  the

screen of the computer in  the same way as the  plotter would

do  on  paper.  The  file  is  plotted  in  the  window which

represents the plotting paper of the plotter.  The boundaries

of this window normally correspond to the hard clip limits of

the plotter (i. e., to the physical limits in which a plotter

pen  can  move  for  the  selected  paper  size). If the ZOOM

function is used,  the chosen plotting  area is displayed  in

this window.

4. WORKING WITH THE PROGRAM

---------------------------

The SPLOT program can be  operated using the keyboard or  a

mouse. From the keyboard the program is operated by  pressing

the keys the list of which is given in chapters 4.2 and  4.3.

If you have a mouse installed, its cursor will appear on  the

screen.  The shape of the cursor is a cross which appears  in

the plotting  area (paper)  (see chapter  4.1), outside  this

area it is a rectangle of  one letter size.  The function  of

this cursor is to choose, activate and change items displayed

visually. You can make a  choice by moving the cursor  on the

selected item  and by  pressing any  button of  the mouse. At

this moment a  tick off character  appears on the  cursor and

only after releasing the pressed button the action associated

with the selected item is  realized. If you choose from  more

items in the menu (e.g., choosing a file to be plotted),  you

have to make a double  click since the first pressing  serves

to highlighting an item.

The program can perform plotting the chosen file on one  of

two pages, they are called "a small view" and "a large  view"

here.

A small view is used for setting the chosen parameters  and

selecting  the  file  to  be  plotted.  The  file can be also

plotted in a small area.

The choice of the file to be plotted can be made either  by

entering its name  or by choosing  it from the  menu. In this

menu there are all the files of the given directory with  the

extension  .PLT  displayed  and   there  are  also  all   the

subdirectories the contents of which can be displayed  simply

by choosing a subdirectory from the menu. Any file group  can

be displayed also  by means of  the wildcards *  and ? (e.g.,

'*.*' will display all the files in the given directory).  As

soon as the file to be plotted is chosen, its plotting starts

on  the  page that is set up  by the F1 key (A SMALL VIEW  or

A LARGE VIEW).

Plotting the file can be  stopped by pressing the keys  F1,

Esc, Ctrl-C or  Ctrl-Break at any  time. If the  plotting was

performed in a large view, after finishing plotting you  will

stay on the page of a large view.

Returning from the large view page can be performed by  the

F1 key.

4.1 ZOOMING

-----------

With the cursor run by the mouse or the keyboard resp., you

can make and enlarge the chosen cuts of the plotted area. The

control of the cursor is realized by the mouse control or  by

pressing the appropriate keyboard keys.

There are two possible zoom modes named SLACK and EXACT.

In the SLACK mode the aspect ratio of the displayed cut  is

identical to the aspect  ratio of the original  plotting area

regardless of the aspect ratio of the selected cut. To ensure

proportional aspect ratio of  the displayed cut in  the SLACK

mode the  size of  the displayed  cut area  is usually larger

than the size of the selected cut area.

In the EXACT mode the aspect ratio of the displayed cut  is

exactly identical to  the aspect ratio  of the selected  cut.

To ensure proportional aspect  ratio of the displayed  cut in

the EXACT mode the screen plotting area is usually reduced.

You can set the default zoom mode in the OPTIONS menu  (see

chapter 6). However you  can apply the opposite  mode without

this setting (see below).

The movement of  the cross cursor  in the plotting  area is

realized by the mouse movement or by pressing the  Ctrl+arrow

keys (or plain  arrow keys in  large view).   By pressing any

mouse button or the Ins(ert) key when the cross cursor is  in

the plotting area you define first corner of the zooming box.

The next  mouse movement  or the  pressing of  the arrow  (or

Ctrl+arrow) keys expands and contracts the zooming box around

the area you desire to expand. If the Scroll Lock is switched

on or  the Shift  key is  pressed, or  if you  press the left

mouse button with the right mouse button simultaneously,  the

box moves as a whole.  By pressing the right mouse  button or

by pressing  the Enter  key you  can start  zooming. Starting

zooming with the Ctrl key depressed selects the opposite zoom

mode to  the default  (i.e., when  the default  zoom mode  is

SLACK  (EXACT),  you  must  press  Ctrl+right mouse button or

Ctrl+Enter key to perform zooming in EXACT (SLACK) mode). The

left button or  the Esc key  or the Ins(ert)  key cancels the

box cursor mode and  you get back to  the mode of the  simple

movement of the cross cursor.

To expand the picture into the original size press the  Tab

key or  press both  the mouse  buttons simultaneously  in the

cross cursor mode.

If you have a  three button mouse with  appropriate driver,

pressing the  middle mouse  button is  equivalent to pressing

the right and left buttons simultaneously.

Note  that  in  the  small  view  the  Enter and arrow keys

normally serve  for control  of the  file menu  and the  edit

line. However in  the mode of  defining the zooming  box they

serve for this operation.

4.2  LIST OF KEYS IN "A SMALL VIEW"

-----------------------------------

F1 - It selects "A SMALL VIEW" or "A LARGE VIEW" for plotting

the file.

F2 - The choice of a disk.

F3 - It replots the selected file in the given picture box.

F4 - The choice of parameters. In the menu initiated by  this

key it  is possible  to set  various program  parameters

concerning  also  exporting  a  picture to the supported

formats. See chapter 6 for more details.

F5 - It changes the paper size according to the plotter  type

and the selected paper norm.

Alt_F5 -  It switches  among the  paper norms (ISO,ANSI,ARCH)

defining the paper size according to the plotter type.

Ctrl_F5, Alt_E - It switches  the EXPAND mode on and  off (on

condition it is supported for the given plotter type).

F6 - In the ROLAND plotter version it switches between  RD-GL

and   DXY-GL   command   mode   (Note: the  RD-GL syntax

corresponds with the HP-GL syntax). In some versions  it

makes  a  choice  of  the  plotter  type (e.g. in the HP

DraftPro   version   it   switches   among   the   types

7570A/7575A/7576A). In the version for HP 7440A ColorPro

it switches on  or off the  simulation of the  HP 17440A

graphics enhancement cartridge.

Alt_F6,Alt_U - In the  ROLAND plotter version in  DXY-GL mode

it changes the  plotter unit size  from 0.1 mm  to 0.025

mm.  In RD-GL mode  or HP-GL mode the plotter  unit size

is always 0.025 mm.

Alt_F7, Alt_F - In the  ROLAND plotter version the choice  of

an initial font (the font set up after switching on  the

plotter or after its initialization).

F7 - It switches on or off a multilingual TEXT filter in  the

LB command of HP-GL mode (or in the P command of  DXY-GL

mode  of  the  ROLAND  plotter).  See chapter 7 for more

detailed information.

F8 - It activates  the export  of the  displayed part of  the

HP-GL or DXY-GL  file to alternative  formats (including

printing).  See chapter 5 for more details.

F9 - It switches  on or  off the  tracing of  the  individual

plotter commands during plotting  the file. It is  valid

only  for  "A  LARGE  VIEW".  First  every  command   is

executed,  then  it  is  displayed  in  the  information

window.  The  next  command  is  executed after pressing

Enter  or  F7.  In  this  mode  the  S key or the F8 key

switches on  and off  automatic tracing.  The tracing is

disabled during printing or exporting.

F10 - It stops  the program.  The program  can be stopped  by

using the F10 key practically at any moment.

Alt_1 - Alt_0 - The identification of pens. See chapter 9 for

more details.

Alt_P - The information about assigning colors to pens.

Alt_S - It saves the  selected palette of colors assigned  to

individual pens to be registered in the setup file.

Alt_R - It restores the original palette unless the saving of

the palette has been already done.

Alt_I - The initialization of graphics mode (it can be  used,

e.g., if the screen graphics mode is damaged -  possibly

by a popup resident program).

Alt_V -

Alt_F10 - Video configuration. More details in chapter 8.

4.3  LIST OF KEYS IN "A LARGE VIEW"

-----------------------------------

F3, F4, F8, F9, F10,

Alt_F10, Alt_V

Alt_P, Alt_S, Alt_R, Alt_I - See "A SMALL VIEW".

F1 - The return to the page of "A SMALL VIEW".

F2 - Opens a file selection dialog. Pressing F2 in this

dialog opens a drive selection dialog.

1 - 0

Alt_1 - Alt_0 - See the identification of pens (chapter 9).

F5 - Inserting a text  into the picture and the  manipulation

with it (see chapter 7 for more detailed information).

Page_Up, Page_Dn - It switches between informative pages.  On

the  first  page  there  is  the  size  of a plotted

picture box expressed  in mm units.   On the  second

page there is the  position in the plotted  file and

the  position  of  the  cursor  in the plotting area

expressed in plotter units. On both the pages  there

are  the  most  important  parameters describing the

state of the plotter.

5. EXPORT

---------

The SPLOT program  enables to export  a plotted picture  to

the following formats:

- HP-GL

- HP-GL/2

- DXY-GL (ROLAND)

- PLOTCALL

- DXF (AutoCAD)

- POSTSCRIPT (color)

-----------------

- IBM Graphics (IBM 9 pin printer)

- IBM X24 (IBM 24 pin printer)

- EPSON LX

- EPSON EX/FX (color)

- EPSON LQ (color)

- NEC 24 pin (color)

- HP LaserJet

- HP DeskJet (color)

- HP PaintJet (color)

- HP QuietJet

- HP ThinkJet (HP mode)

The HP-GL format executes  the export of a  plotted picture

to the HP-GL  plotter. It can  be employed, for  example, for

plotting a picture  cut on the  plotter, for the  output of a

multilingual text on the plotter (see chapter 7), or in  case

your  plotter  does  not  support  all  the  HP-GL   commands

supported  by   the  SPLOT   program.   The   last  case   is

characteristic especially  of the  situation you  own the  HP

ColorPro  7440A  plotter  and  you  have  not  the additional

graphics enhancement cartridge HP 17440A.

The HP-GL/2 format can be  used for exporting a picture  to

any HP-GL/2 device. It may be a pen, ink-jet or electrostatic

plotter as well as a printer of any type. The HP-GL/2 is  the

standardized next generation  of the HP-GL  ( Hewlett-Packard

Graphics Language ).

The DXY-GL format realizes the  export of a picture to  the

DXY plotter (usually the ROLAND plotter).

The PLOTCALL  format is  used in  the GoldenSoftware's PLOT

program which  supports a  wide range  of peripheral  devices

(printers, plotters).

The DXF format converts a  picture to a form applicable  by

the AutoCAD program. From AutoCAD you can read this DXF  file

via the DXFIN command. The exported picture is written in the

form  of  LINES  where  each  pen  of the plotter has its own

LAYER. Pen  1 has  a layer  with the  name PEN1,  pen 2 has a

layer with the name PEN2, etc.

The PostScript format can be used by any peripheral  device

which  has  implemented   PostScript  interpreter  (such   as

PostScript printer).

The function of the other formats is to export a picture to

the corresponding peripheral devices - printers. The  (color)

word  indicates  a  possibility  of  color  printing  for the

appropriate format. Naturally, all formats can be used on all

printers compatible with these ones.

The picture  can be  exported either  to the  disk file  or

directly to the corresponding peripheral device (a plotter, a

printer).  If  your  peripheral  device  is  connected to the

parallel interface, use PRN or LPT1 resp., LPT2, etc. as  the

output file. If it is connected to the serial interface,  use

COM1  or  COM2  respectively.  Set  the  parameters  of   the

corresponding serial interface by  the DOS MODE command  with

the parameter "p" beforehand, like this:

MODE COM1:9600,,,,p

If  you  export  a  picture  to  a disk file, which is less

time-consuming, you can use  the enclosed  programs BPLOT  or

BPRINT resp.,  for exporting  this file  to the corresponding

peripheral device  (in the  registered version  only).  These

programs  perform  exporting  files  to  the chosen serial or

parallel interface, on the time background.  Please,  realize

that it  is not  possible to  use the  DOS PRINT  command for

exporting binary files  (the files containing  graphics data)

to the printer.

The export of the picture is started with the F8 key. In the

EXPORT menu you can specify:

- an exporting format,

- the size of the resultant picture,

- resolution (the size of a plotter unit) - in the case of

the DXY-GL export only,

- print density - if exporting to the printer,

- the output file name.

You can choose from the following export sizes:

- ZOOM size,

- ORIGINAL size,

- A3, A4 or A5 size,

- USER size.

The ZOOM size is offered only if a zoom cut is exported. It

is exactly identical  to the original  size of the  displayed

cut (1:1 scaling). When  the HP-GL, HP-GL/2 or  DXY-GL export

is performed it is possible to keep the original position  of

the  selected  cut  (note  that  the optional left and bottom

margins are added to this  position too - for specifying  the

left and  bottom margins  see chapter  6).   This way you can

make an HP-GL, HP-GL/2 or DXY-GL picture which contains  more

selected cuts positioned as on original picture.

The ORIGINAL size is the whole size of the selected plotter

paper size (as defined by the F5, Alt_F5 or Ctrl_F5 keys). If

you use  this size  for exporting  the original  (non-zoomed)

image, you ensure the 1:1 scale for that export.

The A3, A4 and A5 sizes are the predefined sizes.

To  ensure  proportional  aspect  ratio  of  the  resultant

picture for ORIGINAL, A3, A4 and A5 sizes, only the one  side

from  selected  size  is  used.  This  determinant  side   is

displayed in bright color. The other side is counted so  that

the resultant picture would be proportional.  By default  the

choice  of  the  determinant  side  is  performed so that the

resultant size  lies entirely  within the  selected size. You

can override this by selecting the other determinant side. To

do this click  on the non-bright  side item or  press the Tab

key.

The USER size serves for setting of the user defined  size.

If you set your  own size of the  output format, you have  to

give  only  the  size  of  the  one  side , the other side is

counted automatically so that the resultant picture would  be

proportional. Use the Tab key  or mouse control to switch  to

the editing of the desired side.

The size is  given in cm  units or inch  units according to

the choice  from the  OPTIONS menu  (see chapter  6). The DXF

format is an exception -  here the output size is  understood

in units to be defined in AutoCAD.

The  chosen  size  corresponds  with  the  real size of the

resultant picture realized by  exporting. It is necessary  to

consider it as the maximum  limits into which a picture  seen

in   the   program   plotting   area   can   be   transformed

proportionally. Therefore, for example, if you choose A4 size

paper  as  the  output  size  for  printing and if you have a

printer  of  the  same  size,  you  will reach a satisfactory

result only if you choose the LANDSCAPE orientation. For  the

PORTRAIT orientation it is necessary to choose the maximum A5

size for the  output so that  the resultant picture  could be

positioned on A4 size paper.

The other parameters influencing the look of the  resultant

picture  can  be  set  in  the  menu activated by F4 key (see

chapter 6).

6. OPTIONS

----------

With the menu activated by the F4 key it is possible to set:

1. DEFAULT EXTENSIONS - this item sets default extensions

of the following file types:

1.1 PLOT FILES - the files given in the menu of files from

which  the  files  for  plotting  are  selected. For

example, if this extension is PLT, the displaying of

all  the  files  with  the  PLT  extension  will  be

standard. If this extension is *, the displaying  of

all the files will be standard.

1.2 HP-GL EXPORT -

1.3 HP-GL/2 EXPORT -

1.4 DXY-GL EXPORT -

1.5 PLOTCALL EXPORT -

1.6 DXF EXPORT  -

1.7 POSTSCRIPT EXPORT -  sets standard  extensions  of the

output files according to  the  type of  export. The

name of the output file is composed of the base part

derived  from  the name of  the exported file + this

standard extension.  However, the  file name  of the

output file can be  changed before the beginning  of

any export.

1.8 PRINT - sets  standard extensions of the  output files

during  exporting  a  picture  designated  for   the

printer.

1.9 TEXT - sets the standard extension of the output  file

determined for storing texts on the disk.

2. SORT ORDER FOR FILES - the choice of order in which  files

are displayed. The sorting can be done by the  NAME,

EXTENSION or SIZE.

3. BACKGROUND - switches between NORMAL  and BRIGHT intensity

of background white color  of plotting area  (paper)

(in color modes only).

4. VIDEO SETUP - more detailed information in chapter 8.

5. SOUND - switches on and off sound signals.

6. DEFAULT ZOOM - sets the default zoom mode as SLACK or

EXACT (see chapter 4.1 for details).

7. PEN WIDTHS  -   this  menu  item  enables  you  to  manage

a variable  pen widths feature  provided by the SPLOT

program:

7.1 SET PEN WIDTHS -  It  determines  the line  width   for

individual  pens.  By  default  the  line  width   is

determined  by  the  PT  (pen thickness) instruction.

The   line   width   is   controlled   by  the  HP-GL

instructions and   will  vary   according  to   value

issued  in  PT  instruction.  However note that  each

SP (select pen) instruction  resets  the  PT  to  its

default  value (mostly 0.3 mm).

The other way is to set the line width to fixed  user

defined value.  Unlike the  other measurement  values

the line width is expressed in millimeters (mm).

7.2 USE ON SCREEN - Check  this  option if you want  to use

variable pen widths for screen drawing.

7.3 USE ON PRINTER - Check  this option if you want to  use

variable pen widths for printing and for HP-GL/2  and

PostScript exporting (the variable pen widths are not

supported  for  HP-GL,  DXY-GL,  PLOTCALL and AutoCAD

exports).

7.4 ON PRINTER -  Select here  a mode  for use  of the  pen

widths when printing or exporting. It can be:

Absolute  -  a  real  printed  line  width  will   be

absolutely  equal  to  the  specified  pen

width.

Relative - a real printed line width will be computed

from the specified pen width relatively to

the  original  plotter  page  size (or its

printed cut).

Only  checking  of  the  relative  mode  can   ensure

printing in  the WYSIWYG  style. However  by checking

the absolute mode  along with PT  width type for  all

pens you get a native plotter simulation.

Note  that  on  screen  the  pen  widths  are  always

displayed in relative mode.

7.5 ZOOM PEN WIDTHS  -  Check  this option  if you  want to

enlarge the pen widths  along with enlarging an image

cut (see the ZOOMING section).

8. TEMPORARY  PATH -  this menu  item sets  the directory  in

which  the  SPLOT  program  will  create its auxiliary

files  in  some  types  of  picture  exporting. If the

picture   exporting   stops   with  an  error  message

"DISK FULL", it means,  that there  is not  sufficient

disk space for the  created output or auxiliary  file.

In the latter  case you can  try to set  the temporary

directory to another disk.  You can choose from  three

possibilities:

8.1 The directory defined by the environment variable  TMP

by means of the DOS SET command (e.g., SET TMP=D:\).

8.2 The directory defined by the environment variable TEMP.

8.3 A current work directory i.e., the directory given  on

the  information  board  of  the  program  in "a small

view".

9. USED UNITS  - it defines  units used for  setting the size

and distances during picture exporting. The units may

be given in cm (centimeters) or inch (inches).

A. EDIT USER COLORS - sets the RGB values of the user colors.

They can be used in the PEN --> COLOR assignments for

PostScript and  HP-GL/2 exports.  The RGB  values are

expressed in percents.

B. Parameters for the HP-GL EXPORT :

B.1 PEN --> PEN - by  means of this item it is  possible to

assign each  pen on  the input  a pen  on the output.

Thus  it  is  possible,  for  example, to convert the

picture that uses more pens to a single colored  one,

or it is  possible to exclude  using some pen (if you

assign PEN 0 as the output pen to it).

B.2 PEN OPTIMIZATION - if ON, the number of the grasping of

pens when the picture is exported will be  optimized.

This means that when the resultant  picture is  being

drawn, each pen is grasped only once.

B.3 PEN ORDER - specifies the sequence of pens according to

which the optimized picture will be drawn.

B.4 OUTPUT FORMAT can be like this:

* NORMAL - every HP-GL  command is given on a  separate

line and it is ended with a character ";".

* PACKED - all the HP-GL  commands follow closely   one

after  another  without  any separators. The

number of uses of PU and PD instructions  is

minimized.

B.5 ORIGIN IN CENTER - Activate this parameter only if  the

resultant HP-GL file is designated for the plotter in

which the origin of the coordinate system is situated

in the center of the plotting area (it is usual  with

large paper size  plotters,  e.g. HP  DraftPro or  HP

DraftMaster).

B.6 LEFT MARGIN -  specifies   the offset of  the  exported

picture from the left margin - it is on the X axis  -

it can also be a negative number.

B.7 BOTTOM MARGIN - specifies the  offset of the   exported

picture from the bottom margin - it is on the Y  axis

- it can also be a negative number.

B.8 90 DEGREES ROTATION - It determines  the orientation of

an  exported  picture.  The 90  degrees  rotation  is

forced by the RO90 instruction.

B.9 PG AT END - specifies if the PG (Page Feed) instruction

will be attached to the end of the output file.

C. Parameters for the HP-GL/2 EXPORT :

C.1 DEVICE TYPE - select here to which HP-GL/2 device type

you will export. It can be:

* PEN PLOTTER

* MONOCHROME (B&W) RASTER DEVICE (e.g. electrostatic

plotter, laser printer etc.)

* COLOR RASTER DEVICE (e.g. color ink-jet plotter or

printer etc.)

C.2 DUAL CONTEXT DEVICE - for raster devices it  determines

if  the  device  supports  along  with  HP-GL/2   the

Hewlett-Packard's  printer  command  language  (PCL).

Check  your  printer/plotter  documentation  to   see

whether or not your device supports this extension.

C.3 PEN --> PEN - see the HP-GL export.

Valid for PEN PLOTTER device type only.

C.3 PEN --> COLOR  -  It  assigns   the  output  colors  to

individual pens for the RASTER COLOR device type.

C.3 PEN --> SHADE  -  It  assigns   the  output shades   to

individual pens for the RASTER B&W device type.

The shades are expressed in percents. The value of 0%

represents black pen, the  value of 100%   represents

white pen  and other  values represent  corresponding

gray levels.

C.4 PEN OPTIMIZATION - see the HP-GL export.

Valid for PEN PLOTTER device type only.

C.5 PEN ORDER - see the HP-GL export.

Valid for PEN PLOTTER device type only.

C.6 OUTPUT FORMAT - see the HP-GL export.

C.7 LEFT MARGIN - see the HP-GL export.

C.8 BOTTOM MARGIN - see the HP-GL export.

C.9 90 DEGREES ROTATION - see the HP-GL export.

C.A USE PS INSTRUCTION - switches  on or off  using of  the

PS (plot size) instruction in  the  HP-GL/2   output.

Allowing  the  use  of  PS  instruction is especially

useful  with  plotters  drawing  on rollfeed media to

avoid paper waste  when  drawing  a  small size plot.

However using PS instruction on a dual context device

can scale the resultant picture by the picture  frame

scaling factor (that is the ratio of the PCL  picture

frame size to the HP-GL/2 plot size).

C.B PLOT FRAME ADVANCE  - Set  this option  to VERTICAL  if

your  plotter  frame  advance  is  parallel  with the

longer side of paper.  Set this option to  HORIZONTAL

if your  plotter frame  advance is  parallel with the

shorter side of paper.

C.C ENABLE CUTTER -  enables   or  disables  the  automatic

cutter  function  of  your  HP-GL/2  plotter.  If the

cutter is enabled,  cutting is done  after exporting.

Note:  Not  all  devices  have  an  automatic cutter.

Rollfeed devices that do not have a cutter may draw a

line where the paper should be cut.

C.D MEDIA TYPE - selects  the type of media loaded  in your

HP-GL/2 device. Your device uses this information  to

change  resolution,  drop  volume,  alter  plot/print

speed,   change   from   bidirectional   printing  to

unidirectional  printing   etc.  according   to   its

plotting  or  printing   technology,  quality   level

setting, internal knowledge of  the pen type and  any

control-panel setups.

C.E QUALITY LEVEL -  specifies the  quality level  of  your

output from 0 (draft quality) to 100 (presentation or

final quality).  On raster  devices  it  can optimize

a toner usage  when final  quality  is not necessary.

For a pen plotter, quality level is primarily a speed

control. The pen speed  set by the plotter's  control

panel  is  the  maximum  speed  which  corresponds to

quality level of 0. If the pen speed is not set,  the

plotter  determines  the  maximum  speed based on its

knowledge of media type  and pen type. Minimum  speed

(corresponding to quality  level of 100)  is linearly

interpolated between the media and pen types.

Note: The default quality level in the SPLOT  program

is 0  (maximum speed)  for pen  plotters and  100 for

raster devices.

D. Parameters for the DXY-GL EXPORT:

D.1 PEN --> PEN - see the HP-GL export.

D.2 PEN OPTIMIZATION - see the HP-GL export.

D.3 PEN ORDER - see the HP-GL export.

D.4 OUTPUT FORMAT can be like this:

* NORMAL - every DXY-GL command is given on a separate

line.

* PACKED - all the DXY-GL commands follow closely  one

after another  without any  separators. The

number of uses of  M and D instructions  is

minimized.

D.5 ORIGIN IN CENTER - activate this parameter only if  the

resultant DXY-GL file  is designated for  the plotter

whose origin of the  originate system is situated  in

the center of the plotting area.

D.6 LEFT MARGIN - see the HP-GL export.

D.7 BOTTOM MARGIN - see the HP-GL export.

E. Parameters for the PLOTCALL EXPORT:

E.1 NUMBER OF DECIMAL DIGITS - It determines the number  of

figures  written  after  the  decimal  point  at  the

numbers on the output.

E.2 PEN --> PEN  - see the HP-GL export.

E.3 LEFT MARGIN - see the HP-GL export.

E.4 BOTTOM MARGIN - see the HP-GL export.

F. Parameters for the DXF EXPORT:

F.1 NUMBER OF DECIMAL DIGITS - see the PLOTCALL export.

F.2 PEN --> PEN  - see the HP-GL export.

F.3 LEFT MARGIN - see the HP-GL export.

F.4 BOTTOM MARGIN - see the HP-GL export.

G. Parameters for the POSTSCRIPT EXPORT:

G.1 PEN --> COLOR  -  It  assigns   the  output  colors  to

individual pens for the color PostScript export.

G.2 PEN --> SHADE  -  It  assigns   the  output  shades  to

individual  pens  for  the  black & white  PostScript

export. The  shades are  expressed in  percents.  The

value of  0% represents black pen,  the value of 100%

represents  white  pen  and  other  values  represent

corresponding gray levels.

G.3 LEFT MARGIN - specifies   the offset of  the   exported

picture from the left margin - it is on the X axis  -

it can also be a negative number.

G.4 BOTTOM/TOP MARGIN  -   specifies  the   offset  of  the

exported picture from the bottom margin (when the  90

degrees  rotation  is  suppressed)  or  from  the top

margin (when the 90  degrees rotation is demanded)  -

it is  on the  Y   axis -  it can  also be a negative

number.

G.5 90 DEGREES ROTATION - It determines the orientation of

an exported picture.

H. ALL PRINTERS - It sets   up various  parameters valid  for

printing on all kinds of printers:

H.1 ORIENTATION -   It  determines  the  orientation  of  a

printed  picture on the paper.  It may be either  the

LANDSCAPE orientation (breadthwise)  or  the PORTRAIT

orientation (heightwise).

H.2 FRAME - It  determines if  a printed  picture is  to be

situated in the frame limiting the plotted area.

H.3 PEN FOR A FRAME - It determines  the number of the  pen

with which the frame  will be drawn. Its  function is

important for color printers, where the color of  the

frame is determined by assigning PEN --> COLOR.   The

frame is drawn always with a thin line regardless  of

the current pen widths settings.

H.4 PAGE BREAK AT END - specifies  if the page break  (form

feed) command  will be  attached  to the end  of  the

output file.

H.5 LEFT MARGIN - specifies the distance from the left stop

of  the  printhead  to  the  start  printing position

independently of the orientation.

H.6 TOP MARGIN - specifies the size of shift of the printer

platen before starting printing independently of  the

orientation.

The  LEFT MARGIN  and  TOP MARGIN  parameters  enable to

place the printed picture in any place on the paper.  At

laser printers which have neither printheads nor platens

in a classic  sense of word,  the terms LEFT  MARGIN and

TOP MARGIN always  relate to the  paper oriented in  the

vertical   position   independently   of   the   picture

orientation on the paper.

I.  DOT MATRIX PRINTERS -  In  this  item it is  possible  to

increase the  number of  the printhead  passes during

printing.  It can be used for emphasizing printing or

when the ribbon is written up.

J. Parameters for IBM X24 - it sets parameters for the IBM

24 pin printers:

J.1 OUTPUT FORMAT can be like this:

a) NORMAL - without use of relative shift instructions.

b) PACKED - the instructions for the relative shift  of

the  printhead  are  used  for reducing the

output  code  size.

J.2 PANASONIC X24 - switches on and off Panasonic X24  mode

in which is used a different format for selecting  a

base line feed unit. This format is not strictly IBM

compatible and should be used with Panasonic 24  pin

printers in X24 mode.

K. Parameters for EPSON - it sets parameters for the EPSON

printers:

K.1 OUTPUT FORMAT can be like this:

* NORMAL - without use of relative shift instructions.

* PACKED - the instructions for the relative shift  of

the  printhead  are  used  for reducing the

output  code  size  (not  used for EPSON LX

printers), but on  some older types  it may

cause substantial reducing print rate.

K.2 PEN --> COLOR  - It   assigns   the  output  color   to

individual pens  on the  color printers  of the EPSON

EX/LQ type. It is possible to choose from a fixed set

of 7 colors.

L. Parameters for NEC 24 - it sets parameters for the NEC 24

pin printers:

L.1 OUTPUT FORMAT can be like this:

* NORMAL - without use of relative shift instructions.

* PACKED - the instructions for the relative shift  of

the  printhead  are  used  for reducing the

output  code  size.

L.2 PEN --> COLOR  - It   assigns   the  output  color   to

individual pens on the NEC 24 pin color printers.  It

is possible to choose from a fixed set of 8 colors.

M. Parameters for HP LaserJet - it sets parameters for the HP

LaserJet printers:

M.1 OUTPUT FORMAT can be like this:

* NORMAL - no special coding used.

* PACKED - special  coding   is used for  reducing the

output code size. This coding is NOT a

standard feature of the HP LaserJet and

compatible printers and you must be sure

that your laser printer supports this (it

must support ESC*b1M instruction - I have

tested this on the HP LaserJet IIIp model).

N. Parameters for HP DeskJet - it sets parameters for  the HP

DeskJet printers:

N.1 OUTPUT FORMAT can be like this:

* NORMAL - no special coding used.

* PACKED - special  coding   is used for  reducing the

output code size. This coding is a standard

feature of the HP DeskJet printers.

N.2 PEN --> COLOR -  It  assigns   the   output   color  to

individual pens for color printing on the  HP DeskJet

printer. It is possible to choose from a fixed set of

7 colors.

O. Parameters for HP PaintJet - it sets parameters for the HP

PaintJet printers (but not for XL300 model):

O.1 OUTPUT FORMAT can be like this:

* NORMAL - no special coding used.

* PACKED - special  coding   is used for  reducing the

output code size. This coding is a standard

feature of the HP PaintJet printers.

O.2 PEN --> COLOR -  It  assigns   the   output   color  to

individual pens for color printing on the HP PaintJet

printer.  It  is  possible  to  choose  from  a fixed

(standard) or variable (extended) set of colors. Both

color sets have 7 colors.  The extended color set can

be redefined by user from offer of 17 colors.

O.3 COLOR SET  -   switches  between standard and  extended

color  set.  The  selected  color  set  is  used  for

defining assignment PEN --> COLOR.

O.4 EXTENDED COLOR SET - It is used  for setting individual

colors in the  extended color set.   If the  extended

color set is  selected as used  color set, then  if a

color  in  this  set  is  changed,  the corresponding

assignment  PEN --> COLOR  is changed  automatically.

O.5 MEDIA TYPE -  It selects  output media type -  paper or

foil.

P. Parameters for HP PaintJet XL300 - it sets parameters for

the HP PaintJet XL300 printer:

P.1 OUTPUT FORMAT can be like this:

* NORMAL - no special coding used.

* PACKED - special  coding   is used for  reducing the

output code size. This coding is a standard

feature of the HP PaintJet XL300 printer.

P.2 PEN --> COLOR -  It  assigns   the   output   color  to

individual pens for color printing on the HP PaintJet

XL300 printer.  It is possible to choose from a fixed

set of 7 colors.

Q. TEXT FILTER  - It switches on or off  a multilingual  TEXT

filter in the LB command  of HP-GL mode (or in  the P

command of  DXY-GL mode  of the  ROLAND plotter). See

chapter 7 for more detailed information.

R. CODE PAGE SELECTION  -  It  sets a  code page  for a  text

filter (see chapter 7).  The code page can  look like

this:

- United States

- German

- Multilingual

- Portugal

- Canada - French

- Norway

7. INSERTING TEXTS INTO PICTURES

--------------------------------

The SPLOT program is able to process a multilingual text in

your HP-GL (or DXY-GL) files. As known, plotters receive only

the ASCII characters in the range of 0 to 127.  Therefore, if

the instruction for the output of the text (LB at HP-GL and P

at DXY-GL resp.) contains the ASCII characters with the value

higher  than  127,  the  plotter  will  not draw them or will

ignore  the  highest  bit  of  characters  and  will draw the

corresponding characters  from the  lower half  of the  ASCII

table.  For  example, if you  use the following  statement in

GW-BASIC (handle #1 represents the plotter),

PRINT #1, "LBGr��e";CHR$(3)

the plotter will draw only the text

Gre  or  Grae  respectively.

If you want to draw the  text correctly, you have to use  the

following statements, e.g.:

PRINT #1, "CS0;CA33;SS;LBGr";CHR$(3)

PRINT #1, "SA;LB|~";CHR$(3)

PRINT #1, "SS;LBe;CHR$(3)

However,  e.g.,  if  you  own  the  HP ColorPro 7440A plotter

without an  additional HP  17440A graphics cartridge, it will

not help you either.

The  above  mentioned  example  concerns  the use of a German

text,  but  similar  problems  occur  also in other languages

which use  the ASCII  characters from  the upper  half of the

table.

Therefore, the SPLOT program offers the TEXT FILTER  function

with the  F7 key.  If you  switch on  this function,  all the

ASCII  characters  from  the  value  of  128  to  175  and  �

character will be plotted according to the selected code page

(see chapter 6).  And  it does not matter what  character set

of  the  plotter  is  just  set.  This will enable  to  write

multilingual  texts  in  your  HP-GL  (DXY-GL) files directly

without complicated  switching of  character sets.  The HP-GL

(DXY-GL) file created in this  way must be exported to  a new

HP-GL (DXY-GL)  form through  the HP-GL  (DXY-GL) export (see

chapter 5) with the TEXT filter switched on, and only such  a

newly originated file can be drawn on the plotter.

To demonstrate  using this   method of  writing  multilingual

texts  we   recommend  you   to  study   over  the   enclosed

demonstration file ASCII.PLT and display it at first with the

TEXT filter being switched off and then on.

Another function which the SPLOT program can offer for your

work  with  the  HP-GL  text   is  the  function  of   direct

(interactive) inserting texts into a picture. To do it, it is

necessary to define its  position (XY), size (SI),  direction

(DI), slant (SL), the character set number (CS), the selected

pen (SP) and, naturally, the text itself.

By means of the menu activated by the F5 key on the page of

a  large  view  you  can  choose  the  following   individual

functions necessary for the text manipulation :

- define a new text item

- edit already existing items

- read the text items from the disk file

- save the text items to the disk file

On defining, editing or reading texts the SPLOT program  is

brought  into  the  initial  state  which  corresponds   with

executing IN;RO;IP;IW; instructions. The user's scale (if  it

was set before) and rotation,  if there is any,  is cancelled

then. The  position of  the text  is given  in plotter  units

absolutely, the size  of the text  is in cm  (SI) absolutely,

the direction of the text is also in degrees (DI) absolutely,

the slant is  understood as a  tangent of the  required angle

(SL).

The position of the text and its size can be defined either

by setting numeric  values from the  keyboard or by  moving a

quadrangle which  demonstrates the  outline of  the text  and

which is  controlled by  the mouse  in a  similar way as when

making picture cuts (zooming). To do this it is necessary  to

move the cursor  of the mouse  to the plotting  area. Then by

pressing the  left button  and moving  the mouse  at the same

time the position of the text changes, by pressing the  right

button and moving the mouse at the same time the size of  the

text changes. The other values are set in a numeric form from

the keyboard.

Of course,  the text  itself can  be written  in a  desired

language  with  the  ASCII  characters  >  127.  In the HP-GL

(DXY-GL) picture these characters will be displayed according

to the  code page  you will  choose in  the OPTIONS menu. The

corresponding  support  of  the  national environment must be

installed in MS-DOS  (it must support graphics modes) if  you

want to have the same characters on the edit line.

The  function  of  direct  inserting  a  text  accepts  the

following keys :

a) when  defining  a  new  text item or correcting  already

existing text items:

Alt_T - setting the text string for plotting

Alt_X - setting the position of the text (X,Y) [pu]

Alt_S - setting the size (X,Y) [cm] (i.e. the size  of

one text character)

Alt_D - setting the direction of the text [deg]

Alt_L - setting the slant of the text

Alt_C - setting the character set number

Alt_P - setting the  number of a pen for  plotting the

text

F1    - finishing  the editing of  the text with   the

changes saved

F2    - the redrawing of the edited text

F9 or Esc - finishing   the editing  of the text  with

canceling the changes

b) when correcting already existing text items :

F3 - skip to the correction of the previous text  item

(if it exists)

F4 - skip to the correction of the next text item  (if

it exists)

F6 - the choice of a text item for correcting (if  you

have defined more than one text item)

F8 - the  canceling (erasing  from memory)  of a  text

item according to your choice

In this  way it  is possible  to define  any number of text

items  that  are  loaded  into  memory  and  after  finishing

plotting the respective file they are drawn.

It is  supposed that  the set  of the  text items  you have

created relates to the plotted file. That is why when  coming

to the plotting of  a new file (i.e.  when the plotting of  a

file is  activated otherwise  than by  the F3  key or  F8 key

resp.), all  the text  items are  always erased  from memory.

Before it happens, the program enables to put the text on the

disk if you did not do that before.

There are two ways how to save the created texts :

1) In the  HP-GL (or DXY-GL  resp.) form in  the plotted file

(according to the item in the menu in the ORIGINAL  file).

Respective commands for plotting the defined text items on

the plotter are attached to  the end of the plotted  file.

After the texts are saved on the disk in this manner, they

are always erased from memory of the program, even if  you

do not start plotting a  new file. This is the  final form

of storing texts  which you can  use on condition  you are

already satisfied with the  created texts and you  want to

attach them to the displayed file for ever. If you want to

display the texts  together with the  plotted file on  the

plotter or on  the printer and  you do not  want to attach

the texts to the original file for ever, use directly  the

appropriate export format  with the F8  key (as the  texts

loaded in the  memory of the  program are also  exported).

Then  you  can  put  the  texts  on  the disk in the other

described way.

2) In a special format in a EXTRA file.  Only the texts saved

in such a  way can be  read into the  program memory later

again and thus you can continue in their editing.

8. VIDEO CONFIGURATION

----------------------

The  SPLOT  program  is  a  graphics  program  requiring  a

graphics  card  EGA  or  VGA  and  supporting  the  following

graphics modes:

1) monochrome mode

- EGA 640x350

2) 16 color modes

- EGA 640x350

- VGA 640x380

- SUPER EGA 640x480, 640x258, 800x600

- SUPER VGA 800x600, 1024x768, 1280x1024

The choice  of the  required mode  can be  made in the menu

which is initiated by the Alt_F10 key or Alt_V key resp.   In

this menu there are the following items :

- AUTOMATIC MODE  - is determined  to set up  the video mode

automatically according to the type of a video card and

according to this it sets up the following modes:

- EGA mono 640x350

- EGA 16 color 640x350

- VGA 16 color 640x480

- BIOS MODE - is determined  to set up the mode you  require

according  to  the  number  and  resolution.  The  mode

numbers together with the appropriate resolution should

be given  in the  documentation of  your graphics card.

The BIOS  mode   number  is  to   be  set  in   decimal

units standardly, if you want to set it in  hexadecimal

units, write the H character  at  its end. This  way of

setting the video mode is useful if you own an EGA card

or  VGA card with high  resolution (SUPER EGA  or  VGA)

and  you will  not find it in  the list of SUPER  cards

offered  by the SPLOT program.

- SUPER VIDEO  CARDS - contains  the lists of  SUPER EGA or

VGA  cards  divided  according  to resolution (640x480,

640x528, 800x600, 1024x768, 1280x1024).  Even  if  your

card is not  in the list,  it is  possible that it will

function when you choose  another card of the list.  In

the other case you can try to set the  required mode by

means of the BIOS MODE item  (except the resolutions of

1024x768  and  1280x1024  which  are  highly   hardware

dependent).

Notice - not all listed SVGA cards have been tested.

Only standard VESA video modes are supported (not OEM).

- the other items of  the menu contain standard EGA  or VGA

modes  in  which  the  SPLOT  program  can work (EGA mono

640x350, EGA color 640x350 and VGA color 640x480). It  is

possible to set any of  these three modes on VGA,  but on

the  EGA  cards  it  is  possible  to  set only EGA modes

according to the type  of the connected monitor  (mono or

color).

- the last item of the menu called `VGA GRAY-SCALE'  serves

to  switch  ON/OFF  the  gray-scale  summing on VGA video

adapter.  You will need switch ON this option in case you

have an VGA video adapter with mono monitor and your  VGA

adapter   doesn't   perform   the   gray-scale    summing

automatically.   The  gray-scale  summing  enables use of

color video modes  along with VGA  mono monitor.   If the

gray-scale summing is  active, each color  is transformed

to  corresponding  gray  level  and  thus you are able to

distinguish  individual  colors  even  when  using a mono

monitor.   If the  gray-scale summing  is inactive,  some

colors are  undistinguishable on  mono monitor  (e.g., in

SPLOT program you may not see some pen colors unless  you

change its color value). The `VGA GRAY-SCALE' item is not

displayed  if  your  video  adapter  or active video mode

doesn't support it.

9. PEN IDENTIFICATION

---------------------

The identification of pens  serves to emphasizing parts  of

the picture drawn  with the same  pen and to  informing about

the number  of pens  used during  drawing the  given picture.

This information is useful namely for the correct loading  of

the plotter pens in case you decide to draw the given picture

on the plotter.

The keys from Alt_1 to Alt_0  in a small view and the  keys

from 1 to 0 in a  large view are determined for indicating  a

part of the picture drawn by the same pen.  If you press  the

appropriate  key,  a  part  of  the  picture  drawn  by   the

appropriate pen will begin to  flash and it will be  flashing

until you press the key corresponding with the identification

of another pen or if you do not cancel the identification  by

pressing the Alt_0  key or 0  key resp.   This identification

functions only in color video modes.

Example: If you press  the key Alt_1 (in  a small view) or  1

(in a  large view)  resp., the  part of  the picture

drawn by the pen number 1 will begin to flash (Alt_2

or 2 resp. by the pen number 2, etc.).

In color modes in the SPLOT program each pen is  associated

with appropriate  color. It  is possible  to choose  from the

palette of 62 colors for  each pen. You can change  the color

of the pen when the identification of this pen is switched on

by pressing  the Gray-Plus  key or  the Gray-Minus  key resp.

(increasing or decreasing the number of the color  associated

with this pen). When  the pen identification is  switched on,

information about the color number of the pen in a large view

is given in the right  bottom corner of the screen.  In large

view you can change pen colors also after pressing the  Alt_1

key, the Alt_2 key etc.  In this case there is no flashing of

the part of the  picture as it is  in a small view,  but only

information about the color number  of the pen, which can  be

changed in  the described  way, appears  in the  right bottom

corner.   You can  save the  palette of  the colors  you have

chosen by  pressing the  Alt_S key,  or you  can restore  the

original palette by the Alt_R key.

The information panel containing the individual pen numbers

serves to identify  the numbers of  pens used during  drawing

the picture. At the beginning of drawing all the pen  numbers

are  gray,   during  drawing   the  color   of  the    number

corresponding with the  pen just being  used is white,  after

finishing drawing the color of the numbers corresponding with

all the used pens is white.

10. NOTES

---------

- When  the  SPLOT  program  is  started, it is in the  state

corresponding with the state of the simulated plotter  just

switched  on.  When  coming  to  plotting  a  new file, the

program performs only the  operation of changing paper.  In

this case the program (plotter) is in the state in which it

has been left by the commands of the previous file. That is

why  it  is  generally  recommended  to place an initiating

command sequence  to the  beginning of  each file.  It will

bring the plotter  to the well-known  state.  Otherwise  it

can happen that the same  file can be drawn in  another way

every time,  dependently of  the contents  of the  previous

plotted files. The author recommends to use the  initiating

sequence IN;RO;IP;IW; (in some plotters the IN command does

not influence the setting of rotation).

In  other  cases  the  program  is  brought  to the state

corresponding with the switching off and on the plotter  in

these cases:

- at any change of the format or the paper size,

- at the change of the language syntax (HP-GL <--> DXY-GL),

- at  switching  on/off  the  simulation  of the  graphics

cartridge HP 17440A (ColorPro).

- After  finishing  the  SPLOT  program  saves  the important

parameters (a palette of colors for pens, a choice of  view

for plotting, a size of  paper, command mode, etc.) in  the

SPLOT.INI file in  the directory in  which the program  EXE

file is situated.  When starting the program, the SPLOT.INI

file is used for resetting these parameters. If you want to

set a permanent configuration of the program which will not

be changed,set on the READ ONLY attribute of this file.

NOTICE

-  Hewlett-Packard, HP-GL, HP-GL/2, HP, DraftPro,

DraftMaster, LaserJet, DeskJet and PainJet are

registered  trademarks of the Hewlett-Packard Company.

-  Microsoft,  MS,   MS-DOS  and  GW-BASIC  are  registered

trademarks  and  Windows  is  a  trademark  of Microsoft

Corporation.

-  IBM and PC-DOS are registered trademarks of

International Business Machines Corporation.

-  EPSON is a registered trademark of EPSON Corporation.

-  AutoCAD is a registered trademark of Autodesk, Inc.

-  PostScript is a trademark of Adobe Systems, Inc.