Super VGA card drivers for Cinema

___________________________________________

NOTE: The super VGA drivers provided with Cinema are identical to

the ones used with Graphic Workshop, another program by Alchemy

Mindworks Inc. The other drivers included with this package -

CGA.DRV and HERC.DRV - are not compatible with Graphic Workshop,

and will not load into it.

The following drivers are provided with this version of Cinema.

If you have an EGA only system, use GRAFDRV.DRV and be sure not

to include any files with more than 16 colours in your Cinema

programs.

TRIDENT.DRV	This is a driver for the Trident VGA graphic

cards. It drives the card in its 640 by 400, 256

colour mode, and will work with cards having 256

kilobytes or more of onboard memory. We

originally found the Trident card this driver was

developed on in a Sun Moon Star 386SX we were

loaned. It turned out that the monitor provided

with this computer would not sync in the 640 by

400 256 colour mode, even though the card was

doing fine. If you have this system, you probably

won't be able to use this driver unless you

spring for a multisync monitor.

VIDEO7.DRV	This is a driver for the Headland Technologies

Video Seven VRAM and Video Seven 1024i cards. It

supports the 640 by 480 line 256-colour mode. It

uses the standard VGA and EGA 16-colour and

monochrome modes... we decided not to implement

the 1024 by 768 16-colour mode. There are very

few 16-colour images around which could take

advantage of this mode, but there are a lot of

monitors which cannot.

PARAPLUS.DRV	This is a full featured driver for the Western

Digital Paradise Plus card. This driver also

works with the Dell Super VGA card, which is

actually a Paradise 16-bit card.

DELLVGA.DRV	This is a version of PARAPLUS.DRV, above. It has

been modified not to use the 800 by 600 pixel EGA

and monochrome modes of the Paradise card, as

this causes the super VGA monitor on a Dell to

click a (loud) internal relay every time it

changes modes. If you don't mind the relay, or

you want the 800 by 600 modes, use PARAPLUS.DRV,

above.

Note: rumour has it that some Dell systems have

been shipped with Headland Video 7 cards rather

than Paradise cards. If your Dell system is so

equipped... or if you can't get GWS to work in

the super VGA modes with one of the above two

drivers... try VIDEO7.DRV.

ATIWONDR.DRV	This is a driver for the ATI VGA Wonder and ATI

VGA Edge cards. It drives the 256 colour screen

in the 400 line mode, and will work with cards

having 256K on them. The upper 256K on half meg

cards will be ignored by this driver.

ATI512.DRV	This is a driver for ATI Wonder cards having 512K

of memory on board. It will display pictures in

up to 800 by 600 pixel resolution, selecting

among the three available 256 colour display

modes based on the dimensions of the image to be

viewed.

GRAFDRV.DRV	This is a stock VGA card driver. You might want

to use this if there is no super VGA card driver

available for your card, however, as it handles

the 16-colour and monochrome modes in potentially

more useful ways than the internal VGA driver of

Graphic Workshop does.

VGA400.DRV	This is a "gadget" driver for a stock VGA card

which reprograms the beast into an undocumented

320 by 400 line mode. It should work on any VGA

card, but being undocumented one cannot be sure.

In any case, while interesting, this mode has

such a squashed aspect ratio that it's not really

useful for anything. In addition, having heavily

interlaced memory, it's painfully slow to update.

VGA480.DRV	This is another undocumented mode. It gets still

more on the screen, with still more distortion.

To use one of these drivers with Cinema, place an OpenDriver

command with a path to the appropriate .DRV file at the beginning

of each Cinema program you write.

Writing your own driver

Also included with this version of Cinema is GRAFDRV.ASM, the

source code for GRAFDRV.DRV. This file can be used as the

boilerplate for writing your own super VGA card driver. You will

need a pretty good grasp of assembly language and the hardware

specific details of the card you want to write a driver for to

get this together. This file should be assembled with Microsoft's

MASM assembler, version 4.0 or better.

The following batch file is useful in assembling

GRAFDRV.ASM into GRAFDRV.DRV... or any other driver you happen to

write.

MASM %1 %1 NUL NUL

IF ERRORLEVEL 1 GOTO PROBLEM

LINK %1 @AUTOLINK

EXE2BIN %1 %1.DRV

DEL %1.EXE

DEL %1.OBJ

:PROBLEM

The file AUTOLINK must be present... it contains four

carriage returns, that is, four blank lines.

In future releases we hope to include a number of SVGA

drivers with Cinema and Graphic Workshop. If you write one using

the GRAFDRV.ASM skeletal driver and care to send us the source,

we'll be pleased to include it with future releases, with

suitable credit.

Registered users of Graphic Workshop or Cinema who want

to write their own drivers are welcome to the source code for all

the above drivers... you might find some of the techniques used

in them helpful in writing your driver. The only conditions are

that you must be a registered user and we'd like to have the

source code for the driver you write when you get it working...

we'll include the object file in with future releases of Cinema

and Graphic Workshop.

We'll be happy to give you whatever help we can in

writing your driver, too.

If you want the driver source files, please phone or

write us and we'll send 'em off to you.

Custom drivers

We've been asked on a few occasions whether we'll write custom

drivers for Graphic Workshop and Cinema on behalf of various

interested parties. We might be able to, depending on time and

whathaveyou. If you really want a specific SVGA driver for one of

our packages... for example, if you want to licence it for

inclusion with a particular card or computer... and you can't get

it together yourself, contact us. Our address is in CINEMA.DOC.

#########################################################################################