-----------------------------------------------------------------------------

***                   Jolly Box v4.00 Construction Plans                  ***

***                       Software Manual for v4.3.                       ***

***                      Latest revision: 1993-06-25                      ***

-----------------------------------------------------------------------------

THE CREATION OF THE

___  _                  _   _

| | | |   |   \_/ __ |_| | | \/

\_| |_| |__ |__  |     |_| |_| /\

Created By:

___          _

/_  //    /_)_  _  _  _

(_/(_)//(_/ / \(_)(_)(_'/

/       (_/

Major Software Improvements/Additional Programming

and Hardware Improvements by:

Zaphod Beeblebrox of Control Team/I.C.S.

Hardware Version: v4.00, Software Version: v4.3

This has been a German/Swedish Co-Production between:

Beeblebrox Industries Unlimited (Sweden) / Unlimited Tollfree Access (Germany)

-----------------------------------------------------------------

For Ideas Bugreports additional developements and changes

we can be contacted on the following BBS's:

Eagles Nest:                               +46-18-262804

Nightbox                                   +49-5861-8795

Jolly Roger understands both German and English, and I, Zaphod Beeblebrox

understands Swedish and English, and also a very little bit German. Just

feel free to contact any of us if you need help or just want to chat a bit!!

-----------------------------------------------------------------

General Info

The Jolly-Box is a hardware project realized by Jolly Roger. The first

version in release was v3.1, which I downloaded from a BBS. I started

thinking about making a more advanced version, and contacted Jolly Roger

about it, and here we are again, now with the German-Swedish Co product-

-ion of The Jolly-Box v4.3!!! All original hardware design was made by

Jolly Roger, as well as the main structures of the software for the 80C39

Intel CPU. The improvements to the hardware, and the programming for adding

all the new features to the v4.3 software was made by Zaphod Beeblebrox of

ICS/Control Team. The Jolly-Box is now the most advanced hardware box of

any kind, which plans are released to the public. We have included the

sourcecode for the CPU software, together with sources in GFA basic for

an assembler/dissassembler for 80C39, and software for calculating the

sine-wawe samples for the different tones. All of these GFA basic programs

were written by Jolly Roger on the Amiga, I only added some features for

compatibility with the Atari ST/STE series of computers. You'll find all

these sources in ASCII format also in this archive so that you can convert

them if you wanna use a Pc for example instead of an Atari or Amiga....

However, I must notify you of the fact that you'll have to find out what

trunk-frequencies you should use yourself, since we will only include the

standard CCITT #5 ones, so if you need anything special you'll have to code

them yourself!!!!

The  hardware  will  cosists  out  of  two single-sided boards, a

keypad and a phone speaker. Or, you might put a 3.5 mm phono jack on the box

and use a pair of freestyle headfones. I use that myself, with excellent

results.... Also, this looks more unsuspicious if you use it in a payfone...

Now, the rest is up to you. There is still some room left in the eprom for

additional routines, so you might add whatever you want to the box....

(Yeah, if you use a 2732 (4 Kb eprom) there is actually the whole second

2 Kbyte rom bank left for you, but I do think that we already have coded

most useful boxing routines needed for the box. If you feel that we left

something out, feel free to contact us about the improvements!!!!

-----------------------------------------------------------------

*** WARNING ***

The use of a bluebox is a criminal offence, and it's *YOUR* problem if you

get caught using one. We will accept no responsibility for your actions, we

just show you the theory behind the construction of a CPU controlled bluebox

and the use of this device on any phoneline without permission is strictly

forbidden!!!!!!! Now it's all up to you...........

Building this box is not a simple task. If you do not have good experience of

soldering together complicated circuit, do not attempt the constyruction of

this device. Ask somebody who does have experience to do it for you!!!!

-----------------------------------------------------------------

Parts List for Jollybox v4.00:

==============================

No:        Description:             Elfa Order #:

===        ============             =============

01        80C39 CPU                73-802-49

01        27C64 EPROM              73-729-31

01        27C010 EPROM             73-736-32

01        74LS373                  73-726-00

02        74LS393                  73-729-15

01        741 OP.                  73-010-21

01        8-PIN SOCKET, 7.62MM     48-135-49

02        13-PIN SOCKET, 7.62MM    48-135-64

01        20-PIN SOCKET, 7.62MM    48-136-22

01        28 PIN SOCKET, 15.24MM   48-137-54

01        32 PIN SOCKET, 15.24MM   48-137-62

01        40 PIN SOCKET, 15.24MM   48-138-12

01        12-KEY TELEPHONE KEYPAD  35-678-07

01        1 POLE SPST SWITCH       35-355-31

01        3.5MM PH. JACK /W SWITCH 42-700-47

01        146*90*37MM PROJECT BOX  50-381-04

01        7805 5V POS.V. REGULATOR 73-090-08

01        10 MHZ CPU CRYSTAL       74-505-05

01        RED LED, 3MM             75-037-09

02        10 UF, ELECTROLYTIC      67-008-19

02        15 PF, CERAMIC           65-691-23

01        0.1 UF, CERAMIC          67-776-43

01        0.33 UF, CERAMIC         67-777-18

01        100 KOHM POT, LOG.       64-064-09

01        270 OHM, 5%              60-104-25

03        10 KOHM, 5%              60-106-15

03        100 KOHM, 5%             60-107-30

01        10 KOHM, 1%              60-734-23

01        20 KOHM, 1%              60-737-46

01        40.2 KOHM, 1%            60-741-08

01        80.6 KOHM, 1%            60-744-54

01        160 KOHM, 1%             60-747-77

01        324 KOHM, 1%             60-751-21

01        634 KOHM, 1%             60-754-85

01        1.3 MOHM, 1%             60-757-33

02        PCB'S /W PHOTORESIST     49-576-19

01        16 LEAD RIBBON CABLE,1M  55-660-54

-----------------------------------------------------------------

Building the Main-Board

This will be the hardest part!  If you make one mistake, you will

not get any result, so if it doesn't work, check the Layout! Etched

boards can be obtained from Zaphod if you can manage to get hold of him,

for the cheap price of 150 Sek for a set of ready made circuitboards...

'8049BB1P.PCX' is the picture showing where on the PCB the components

should be placed. '8049BB1.PCX' and '8049BB1M.PCX' is the circuitboard as

seen from the back in normal and mirrored format. The same pictures exist

for the sample board too, but the they're called #2 instead of #1....

To get films for photo etching, either make your own layout from these pics,

or if you have a standard epson 9 or 24 pin printer, or a HP Laserjet

compatible laser, you can just print the files in the PCB_PRNT.LZH archive.

*.P9      Epson 9-Pin printer image files.

*.P24     Epson 24-Pin printer image files.

*.HPL     HP-Laserjet printer image files.

*.HPD     HP-Deskjet printer image files.

*.PCB     PCB Layout files for PCB Layout v1.x on Atari ST.

Power up the main circuit board and test it with a power supply of

9V, 500mA. A normal 9V battery will do fine, but we do recommend

a real 9V DC power supply when the box is to be used indoors, since

normal batteries tend to run out quite fast, all these chips consume

quite a lot of current when the box is being used a lot. Check

the "power in" pins at the main board and check if there are 5V

comming out on the power out pins. Check if the clock jumper

shows  a 666666 Hz signal on a meter or a scope based to ground.

If this is ok the power, processor, quarz is working.  Now check

if one of the keyboard  pins  (p2-p7) based to ground shows a

frequenz (far  under  666666  Hz). Congratulation: You did the

hardest part if it works. Now connect the keyboard with p2-p7

for lines and t0-t1 for the rows with the ribbon cable. See

KEYBOARD.IFF how the keys are connected to the boards. You can

check it, when the sample-board is done.

-----------------------------------------------------------------

Building the Sample Board

'8049BB2P.PCX' shows where the components should be put, and '8049BB2.PCX',

and '8049BB2M.PCX' shows the back of the PCB in normal and inverted mode.

Put on all jumpers and sockets first. Then put on all bigger parts and

at least the ICs! Take the '27010SPL.ROM' file and program it into an

27010 eprom.  Press the eprom into it's socket too.

Use a ribbon cable and connect both boards with S0-S5, B0-B1, +5v, GND, +9V

and then the keyboard on T0, T1 and K0-K5.... Look at the pictures for

details on how to modify either an 12-to-1 common ground pad or an X-Y matrix

keypad, any of these types will work for use with this box if you just know

how to modify them!!! For my box I used a Digikey 12 key X-Y matrix keypad

which I opened up and rewired to a 6-to-2 keypad. To easily describe what a

6-to-2 keypad is, look at it like this: The CPU will ground one of six KEY

lines at a time, for example K0 will ground keys 1 and 7. If K0 is grounded

and the key "1" is pressed, input T0 will read zero, and the CPU will register

that "1" was pressed. K0 will also send key "7" grounded, but here is the

difference: If "7" is pressed the CPU will detect this by checking if T1 is

grounded, keys 1,2,3,4,5,6 will ground T0 is pressed, and 7,8,9,*,0,# will

ground T1. Just look at the sourcecode for further details, as this is a

quite advanced hardware project indeed!!!

Connect the poty and speaker also. Note that on this stage you should use

the Jollybox TEST-ROM and not the v4.3 software. The test eprom will just

cycle through all samples in bank 0 of the sample rom, which is the 16

DTMF tones available in the box!!

Turn  the  poty  to 1/4 of its resistance or less!  Listen to the

telephone speaker, when you turn on the power.

Now you can hear the box playing it's DTMF tone sample bank....

If all is well so far, program the eprom with v4.3 software and connect

the telephone keypad to the box as well. Now when you turn on the box, you

can press and hold any of the keys 1-4 to invoke the samplebank self-test

mode, which will cycle through all tones in the samplebabnk corresponding

to the key you held down when you powered up the box. This mode is good for

adjusting the volume with the 100 kOhm variable resistor on the sampleboard.

Set the volume as loud as you can get it without getting too much distortion

as that is the best setting. Too high volume can result in difficulties when

you want to use the DTMF dialer for instance, distorted tones are no good!!!

Also, you will notice that when you use a 9V battery, you will get a kind of

background noise, this is because this design really needs a bit more current

than a 9V battery can supply. If you use the box from home, I'd suggest you

go and get a 9V DC power adaptor for it, approx 750 mA - 1A current. I use

one with 750 mA for my own box, works great! Version 4.3 of the software

also supports a new feature which I am planning to implement to the hard-

ware of the box, namley battery backed up ram. The 8039 CPU can save the

contents of the 128 byte ram memory if you supply it with 5V DC between

pins 26 and 20. This has not yet been implemented, but as the CPU in normal

operation would supply the power for pin 26 from the battery/DC adaptor, I

would suggest that you get a small 5.5 V battery backup capacitor or some

kind of battery. The CPU only consumes about 2 uA for the RAM battery

backup so a normal Lithium cell would last for a long time. However, do not

forget that if you are to do this before I have made the plans for it, you

better know what you are doing, as this is not a simple task if you're not

very experienced in building electronic projects. But now ya have the

software support for battery backed up operation at least.

-----------------------------------------------------------------

Operating the BB

Manual for the ROM 4.3

If you turn on the power, all you will hear is silence. To activate the box

you must enter your 4 digit security code which you defined in the code before

you assembled it. The code is defined in label "AU_CODE:" in the source, and

you get it by taking the digits you want to have as the code and INVERT it, and

write it down in binary in the source. Look at this example: The default code

is "1234", lets first convert it to binary:

If we look it up in the table below, you'd soon realize that "1" in binary-inv

format would correspond to "11111110", so you'd write that down for code digit

number one. Do the same for the other 3 code digits. A special case is if you

want to include the digit 0 in the code, then you should have the JZ opcode

that jumps to the wrong-code routine jump to the next digit instead. If you

do not understand what I am talking about, then just don't use digit "0" in

your access code for the box!!! If somebody should enter an incorrect code,

the box will sound two beeps and enter the dumb-whitebox mode, and will just

act as a regular DTMF tone-dialer with digits 0-9 and *, #. When a correct

code is entered the box will sound three beeps and proceed to the bluebox

main-menu, with CCITT #5 R1 tones selected as default, and 40 ms duration

and 20 ms delay.

Dec:  Binary:    Inverted:

----  --------   ---------

"1" - 00000001 - 11111110

"2" - 00000010 - 11111101

"3" - 00000011 - 11111100

"4" - 00000100 - 11111011

"5" - 00000101 - 11111010

"6" - 00000110 - 11111001

"7" - 00000111 - 11111000

"8" - 00001000 - 11110111

"9" - 00001001 - 11110110

"0" - 00000000 - 11111111

After you have successfully powered up the box and entered the access code,

you now have a variety of options for different calls:

Whitebox:

Press the number (#) symbol to switch to whitebox.  Now press any

key and the DTMF tone for that key will be hold until you release

it again.  To exit the whitebox, press # again. The keys 0-9 will

generate the normal DTMF tones like a touch tone dialer.

*1=DTMF "*"

*2=DTMF "#"

*3=DTMF "A"

*4=DTMF "B"

*5=DTMF "C"

*6=DTMF "D"

Bluebox dial:

Press 0 to dial ram number #1, 1 to dial ram number #2. Press 20-27 to dial

numbers stored in phonebook #1. 30-37 is for dialing from fonebook #2...

Ram number #1 & #2 can be each up to 31 digits long, and the prefix can be up

to 15 digits. Do not enter any longer strings as I have no way of cehcking how

many digits ya enter to the poor little box!!!

Trunk and Seize:

To trunk locally in countries that use inband signalling use key 7 to send

an 1.5 second 2600 Hz tone. Key(s) 8 and 9 will generate international trunk

tones for boxing off other countries. 8 will send universal trunk #1, 9

will send universal trunk #2. If you should call by sattelite, press 4 to send

a DHLS clipping disable signal. This will give you better line quality....

Universal Trunk #1 can be altered using the *8 menu, look further in the

manual for details on how to do this. Note that your changes will be lost when

you turn the box off, as I have no battery backed up ram on it so far!!!!

Keys 5 and 6 generate Redbox tones for Dime and Quarter, works in US and most

likely in some other countries too. Press **0 to enter a new dialing prefix, the

default prefix is 2460 (KP2+460), end input with # key. The first digit should

be 1 or 2, standing for KP1 or KP2!! If you use 3 as the first digit, the number

will be outpulsed i DTMF instead of Mf or any other kind of bluebox tones.

Universal trunk #1 is 2400/2600 Hz for 130 ms, pause 1.75 sec, 2400 Hz for 300 ms.

Universal trunk #2 is 2400/2600 Hz for 130 ms, pause 2.00 sec, 2400 Hz for 130 ms.

The trunk tones may be reprogrammed at your convenience by alterin the labels named

TRUNK1N, TRUNK2N and LTRUNKN in the 8049 assembler source code.

Trunking Timing sub-menu.

=========================

*80X = Change Trunk Tone #1 Duration, X=20 ms steps.

*81X = Change Trunk Tone #2 Duration, X=20 ms steps.

*82X = Change Trunk Tone #3 Duration, X=20 ms steps.

*83X = Change Trunk Tone #4 Duration, X=20 ms steps.

*84X = Change Trunk Tone #1 Delay, X=20 ms steps.

*85X = Change Trunk Tone #2 Delay, X=20 ms steps.

*86X = Change Trunk Tone #3 Delay, X=20 ms steps.

*87X = Change Trunk Tone #4 Delay, X=20 ms steps.

*88X = Change Trunk Tone #1 Sample, X=0-15. (#=Skip, no tone!!!)

*89X = Change Trunk Tone #2 Sample, X=0-15. (#=Skip, no tone!!!)

*8*X = Change Trunk Tone #3 Sample, X=0-15. (#=Skip, no tone!!!)

*8#X = Change Trunk Tone #4 Sample, X=0-15. (#=Skip, no tone!!!)

Dialing Timing sub-menu.

========================

*90X = Change M-f Tone Duration, X=20 ms steps.

*91X = Change M-f Tone Delay, X=20 ms steps.

Timetable for all tone duration/delay keys:

===========================================

0 = 000 ms      1 = 020 ms   2 = 040 ms

3 = 060 ms      4 = 080 ms   5 = 100 ms

6 = 120 ms      7 = 140 ms   8 = 160 ms

9 = 180 ms      * = 250 ms   # = 500 ms

Custom Trunktone Frequence/Key Table:

=====================================

0 = $00 - 2400/2600 Hz

1 = $01 - 2400/2400 Hz

2 = $02 - 2040/2040 Hz

3 = $03 - 2040/2400 Hz

4 = $04 - 2280/2280 Hz

5 = $05 - 2600/2600 Hz

6 = $06 - 2100/2100 Hz

7 = $07 - Spare 2

8 = $08 - Spare 3

9 = $09 - 480/620 Hz (US Ringing Tone)

* = $0a - 350/440 Hz (US Dialtone)

X = $0b - 440/480 Hz (US Busy)

X = $0c - 440/440 Hz (Sw. Dialt & Busy).

X = $0d - 460/460 Hz (German Tone).

X = $0e - 1700/2200 Hz (US/DK Redbox)

# = $0f - Silence (Empty!!!).

Tones marked with "X" are not in use for trunking, and cannot be used.

The custom trunking freq consists of 2 tones, a predefined pause of

1.5 second, and then another two tones. If you do not wish to use a

total of four tones, you should enter "#" at the tones you do not want

to use, and enter a duration of 0 ms for them. Just use your knowledge

of phreaking and you'll understand what I am talking about. After all,

this box is only intended to be used by people who already knows alot

about phreaking and telecommunications.

The default settings for the user-costomizable trunk is the following:

Tone1: SPL: $00, Length: 130, Delay: 250

Tone2: SPL: $0f, Length: 000, Delay: 000

Tone3: SPL: $0f, Length: 000, Delay: 000

Tone4: SPL: $01, Length: 300, Delay: 000

Test-Mode for testing sample rom and volume:

If  you  press  1-4 while  turning  on, you get all the tones of the

sample bank you choosed...

Enter number:

Press **0 to enter a new dialling prefix.

Press **1 to enter ram resident number #1.

Press **2 to enter ram resident number #2.

While entering a number, use * to enter special tones like the DTMF *,

# and ABCD tones. For DTMF, *1=*, *2=#, *3-*6=ABCD. The same goes for bluebox

mode. The first digit entered is very important. A 0 means dial prefix, then

outpulse the rest of the number. A 1 means outpulse KP1 tone and then dial in

the bluebox tone mode currently in use. A 2 means the same as above, but

oupulse KP2 instead of KP1. To finish entering the number/prefix, press #.

In bluebox mode, the * key together with keys 1-6 also have important uses.

In mode 0:		In mode 1 & 2:

==========		==============

*1 = DTMF "*"		*1 = Code 11

*2 = DTMF "#"		*2 = Code 12

*3 = DTMF "A"		*3 = KP1

*4 = DTMF "B"		*4 = KP2

*5 = DTMF "C"		*5 = ST

*6 = DTMF "D"		*6 = 1850 Hz DHLS

Of course these keys also have special uses in CCITT #4, but as we do not

know very much about CCITT #4 you'll have to figure them out by yourself.

Anyway, according to the CCITT #4 documentation that I got the keys should be

defined as:

*1 = Code 11

*2 = Code 12

*3 = Space Code I

*4 = Incoming Half Echo Supressor

*5 = Space Code II

*6 = End of Pulsing (ST).

The KP1 and KP2 signals are not part of the binary coded digit signals in

CCITT #4, and can only be sent at the start of dialing.

Prefix:

Press **0 to enter a prefix.  Press 1 for KP1 and 2 for KP2, 3 for DTMF mode.

Enter the  full  prefix  and stop with #. If you have a phonebook with

numbers  of  austria,  you  have to dial b430....So just enter 0-XXXXXX-ST

in your phonebook.   When  you  dial a number with 0 at the begining, the

prefix  will  be added.  This is an additional function, which is

not necessary for normal calls. The prefix default is B460.

The  entered  numbers  including prefix will be erased and set to

default when turn off and on the BB.

Special Functions:

*0 - Select Sample Bank 0. (DTMF, All 16 Tones).

*1 - Select Sample Bank 1. (CCITT #5-R1).

*2 - Select Sample Bank 2. (CCITT #5-R2).

*3 - Select Sample Bank 3. (Break/Simulation Tones).

*4 - Select CCITT #4 Mode.

*# - Enter Special Tone Simulation Mode

In CCITT #4 mode, you can only dial the numbers stored in RAM position(s)

one and two, and the trunk keys 8 and 9 will send CCITT #4 clear forward and

xfer forward signals. These signals are coded according to the CCITT's own

specifications, buit has not yet been tested as of todays date. Hopefully

they'll work fine, but we haven't had any possibility of testing them yet.

-------------------------------------------------------------------------

Special Tone Simulation Mode:

1 - Swedish Dialtone

2 - Swedish Busy Tone

3 - Swedish Ring-Tone

4 - US Dialtone

5 - US Busy Tone

6 - US Ring-Tone

7 - Swedish Reorder Tone

8 - US Reorder Tone

9 - Greenbox Ringback Tone, 1000 ms

0 - Greenbox Coin Return Tone, 1000 ms

* - 2600 Hz Wink, 90 ms

# - Return to Main-Menu

-----------------------------------------------------------------

How to create a phonebook

=========================

Take the "8049vXX.rom".  this will be the file, which you have to

program  into the eprom for the CPU. Insert Phonebook 1 and 2 at

$300-$3ff in the binary rom file for the 8049 program eprom, and

after that, burn the program onto the 2764 eprom. Also, if you want

to have CCITT #4 capabilities in your box, do not forget to program

the 64 byte file "CCITT4.BIN" at $5C0 on the program rom. This file

contains the 16 binary coded CCITT #4 signals...... Run the program

BIN_INS.GFA to automatically insert the CCITT #4 data and the phone-

book at the proper places in the .ROM file for you.....

If you like modified this plan, I will give you an idea, how you

can create a phonebook by your own. The Rom for the CPU goes

from  $0000-$07ff. The Phonebook goes from $0300-$03ff for 10-17,

20-27.

An example is shown here:

$0300: 02 04 09 00 01 02 03 04 05 06 07 08 20 00 00 00 : b49012345678c... 10

$0310: 01 00 09 00 03 01 04 01 20 00 00 00 00 00 00 00 : a0903141c....... 11

$0320: 00 04 00 01 02 03 04 05 06 20 20 20 20 20 20 20 : 040123456c...... 12

$0330: 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 : ................ 13

$0340: 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 : ................ 14

$0350: 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 : ................ 15

$0360: 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 : ................ 16

$0370: 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 : ................ 17

$0380: 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 : ................ 20

$0390: 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 : ................ 21

$03a0: 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 : ................ 22

$03b0: 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 : ................ 23

$03c0: 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 : ................ 24

$03d0: 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 : ................ 25

$03e0: 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 : ................ 26

$03f0: 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 : ................ 27

Each of the 16 Bytes are one phonenumber. On the Right are the

Digits to press, when you want to dial them. For the number at

$0300  press 1 folowed by 0! Enter the number like you would do,

when you  enter it directly. Use $01 for 'kp1' or $02 for 'kp2',

$00 for prefix and $03 for DTMF dialing.

Then enter the digits: $00 for '0'...$09 for '9'.  The last

digit has to be a $20. The bluebox will add the 'st'-code.

In this example the first number is 'B49012345678C'. It will call

Germany: 0123/45678. The second will call rout operator.

The third will call (prefix40123456).  Use as prefix b490 for

this example if this is a german number.

The maximum is 15 Digits and $20 for st. If you like numbers

with more than 16 digits, just go on and use the next line of the

next number. The compuer will dial until it gets a $20 code. If

this isn't still enough, use more and more.

-----------------------------------------------------------------

ASS80XX

Written by Jolly Roger for the Jolly-Box

There is a short  descripion of the 8049 assembler for the Jolly-Box. It is

necessary because you need to modify the source to your own developing.

It includes all opcodes.

The format must be as followed:

-No case sensity

-Only one opcode in one line and one label

label:	clr	a

label2:

jmp	label

-You can use Decimal, Hex or Binary for values

mov	a,=128

mov	a,=$80

mov	a,=%10000000

-Characters after a semikolon will be ignored until the next line

clr	a	;set accu to zero

-Use  space  as well as tab to seperate.  Seperations between label and opcode,

opcode and semikolon are not necessary.

-Jumps must always jump to a label, numeric is not allowed

If you like to skip to a memory page:

page	2

will skip to byte 512 of memory. If it is already passed an error will occure.

Remember  that  only  JMP and CALL can jump to an adress within a page of 256!

How the opcodes work refer to the 8049 manual.

------------------------------------------------------------------------------

published by a member of

___                             ____     _

/_ _/_  _ _  ___/._  _  __ /   /_  //_/_ _  _

(_// )/(_'/ / )(_///(_)/ )(_//   /(_)// // (_'(_'

___

/_ /_  _  _   _  _

/  /(_ (_ (_'_) _)

- - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Downloaded from NIGHT-BOX

----------------------------

__  __   ___   _______   ___  __  ________

/ /\/ /\ / //\ / ___  /\ / //\/ /\/__   __/\

/  \/ / // // // //_/_/ // //_/ / /\_/ //\_\/         The ACF-BBS

/     / // // // //__\_\//  __  / /  / // /           -------------

/ /\  / // // // //_\ /\ / // / / /  / // /               SysOps

/_/ /_/ //__/ //______/ //__/ /_/ /  /__/ /             Pooky & Jacky

\_\/\_\/ \__\/ \______\/ \__\/\_\/   \__\/

05861 8795 (new number!)

_______   _______   __   ___

-- 8N1 --                     / __   /\ / ___  /\ /  \_/  /\

17-9h online Sa,So 24h             / /_/ _/ // // / / / \    __/ /

14.4K/12K/9600/7200/4800/2400/1200      / ___ \_\// // / / /__/  // _\/

-=====================================-   / //_/ /\ / //_/ / //  _   \ \

V.32/V.32bis HST V.42/V.42bis MNP 1-5   /______/ //______/ //__/ \__/ /

\______\/ \______\/ \__\/ \_\/

ATARI/ AMIGA/ IBM comptbl./ SNES/ M.Drive

.---.        .---.

/     \  __  /     \

Coming soon....                           / /     \(  )/     \ \

//////   ' \/ `   \\\\\\

JACKTRO                        //// / // :    : \\ \ \\\\

// /   /  /`    '\  \   \ \\

by the ACF Design Team!  //          //..\\          \\

====UU====UU====

'//||\\`

''``

__        __

________ ______/ /_______/ /_________ _________ __   __ ________

/ ______// ____/ /_  ____/ /_  ____  //  ____  // /\ / // ______/\

__/_/_____ / /\__/ / / /\__/ / / /\__/ / / /___/_// /_// // /\_____\/

/_______  // / / / / / / / / / / / / / / / //________  __// //__/\

\/ /___/ // /_/_/ / / /_/_/ /_/ /_/_/ /_/ /___/ /\__/ /\_/ /_\__\/

/_______//_______/ /_______//________//________/ / /_/ //_______/\

\_______\\_______\_\_______\\________\\________\/  \_\/ \_______\/

Thanx for calling this fine ACF BBS

Click...

-----------------------------------------------------------------------------

.                                                +             .

_______ ____   ________    ___    _______ _____/|

+      /  __  //    \ /  ______\  /  /   /  __  //  ____!

/  /_ \//  /\  \\  \  ___  /  /   /  /_ \//  /___

/  __/  /  /  \  \\  \ \  \/  /   /  __/   \____  \

\  \__/\\  \  /  / \  \/  /\__\___\  \__/\.____/  /    '

*         \_____/ \__\/  /   \____/    \__/ \_____/| _____/     |

/__/  .     /    _  /         |/       - --+- -

USR HST Dual Standard 14400  /    / \/ 285 Megs :            |

_____________________   /     \   ______________________ .

.      \__ __ _ _           \_/       \_/            _ _ __ __/

:        \___ __ _ _  Sysop: Zaphod Beeblebrox   _ _ __ ___/

|           \___ __ _ _   Control Team EHQ    _ _ __ ___/

---+--- -         \___________    ICS SHQ    ___________/

|                          \___   :   ___/

:            +46-18-262804     \  |  /     24 Hours/Day.           '

.      *                        \ | /    .

.______    _____/ | \___/|___________

|   _  \  /  _/ __|__ \__!  ______  /

'           |  | \  \/  /_\| /  /|/_  \/ |  | \/

|  |  \  \  _/ ' \___'  \    |  |              +

|  |  /  /  \__/\____/  /    |  |

!__! /__/\______| _____/     !__!

|/

:

-----------------------------------------------------------------------------