DATAPAC OUT-DIAL PORT ADDRESSES (E.G. PC TO PC)

Service     - DATAPAC 3101 (ASYNC/ITI) OUT-DIAL PROCEDURES (E.G. PC TO PC)

DATAPAC 3101 OUTDIAL PROCEDURES

===============================

1.  SERVICE CHARACTERISTICS

-----------------------

AVAILABILITY - EIGHTEEN MAJOR CENTRES (DPSA'S) ARE BEING SERVED BY

OUTDIAL.  THEY ARE:  VANCOUVER, CALGARY, EDMONTON, REGINA,

SASKATOON, WINNIPEG, TORONTO, CLARKSON, LONDON, WINDSOR,

KITCHENER, HAMILTON, OTTAWA, MONTREAL, QUEBEC, HALIFAX,

SAINT JOHN (N.B.) AND ST. JOHN'S (NFLD).

SPEED -        OUTDIAL IS AVAILABLE AT 300 OR 1200 BPS.

RESTRICTIONS - OUTIAL SERVICE WILL ONLY ACCEPT SENT PAID CALLS.  THE

ORIGINATING END MUST USE A DEDICATED ACCESS OR USE

A NETWORK USER IDENTIFIER (NUI).

- CALLING AT THE TERMINATING END IS RESTRICTED TO THE

LOCAL CALLING AREA FOR THE DPSA REACHED.

CONFIGURATION- THE OUTDIAL PORT USES PROFILE 6, EXCEPT THAT THE USER OF

THE OUTDIAL PORT IS ALLOWED TO ESCAPE TO COMMAND MODE BY USING

"CONTROL P".

- THE DESTINATION TERMINAL MUST BE SET AT EVEN PARITY

IN ORDER TO RECEIVE THE OUTDIAL CALL.

- ONCE CONNECTED, DATAPAC 3000 USERS CAN SET AND READ

THE REMOTE ITI PARAMETERS BY SENDING LEVEL 1

PACKETS (X.29).

2.  OUTDIAL PROCEDURES

------------------

1)  ESTABLISH A CALL TO DATAPAC VIA A DEDICATED OR DIAL-IN ACCESS.

NOTE:  IF USING A DIAL-IN ACCESS, A NETWORK USER IDENTIFIER

(NUI) MUST BE ACTIVATED BEFORE ESTABLISHING THE CALL.

2)  ENTER THE ADDRESS OF THE OUTDIAL PORT.

3)  DATAPAC WILL RESPOND WITH:

DATAPAC:  CALL CONNECTED

ENTER DESTINATION TELEPHONE NUMBER/ENTRER LE

NUMERO DE TELEPHONE DU DESTINAIRE

4)  ENTER THE 7-DIGIT TELEPHONE NUMBER (LOCAL) OF THE DESTINATION

TERMINAL.

5)  DATAPAC WILL RESPOND WITH:

DIALING/COMPOSITION DU NUMERO (XXX-XXXX)

PRINTING THE DESTINATION TELEPHONE NUMBER AS IT IS DIALED.

6)  DATAPAC WILL THEN INDICATE:

RINGING/SONNERIE

AS THE MODEM DETECTS RINGBACK TONE.

7)  WHEN THE DESTINATION MODEM ANSWERS THE CALL, DATAPAC WILL SEND

THE FOLLOWING MESSAGE TO THE ORIGINATING END:

CALL CONNECTED/COMMUNICATION ETABLIE

8)  THEN PROCEED WITH YOUR CALL.

9)  TO CLEAR A CALL UPON COMPLETION, ENTER THE CLEAR COMMAND:

(CONTROL P) CLEAR (CR)

DATAPAC WILL RESPOND WITH:

DATAPAC:  CALL CLEARED - REMOTE DIRECTIVE

DATAPAC:  TERMINAL ADDRESS

NOTE:  IF YOU HAVE USED A NUI TO PLACE THE CALL, TURN THE NUI OFF

BEFORE LEAVING THE NETWORK WITH THE COMMAND:

NUI OFF (CR)

DATAPAC WILL RESPOND WITH:

DATAPAC:  NETWORK USER IDENTIFIER NOT ACTIVE

DATAPAC OUT-DIAL PORT ADDRESSES (E.G. PC TO PC)

Service     - DATAPAC 3101 (ASYNC/ITI) OUT-DIAL PORT ADDRESSES

OUTDIAL PORT ADDRESSES

----------------------

OUTDIAL PORT

CITY (PROVINCE)      SPEED      ADDRESS

---------------      -----      -------

Calgary (ALTA)        300       63300900

1200       63300901

Clarkson (ONT)        300       91900900

1200       91900901

Edmonton (ALTA)       300       58700900

1200       58700901

Halifax (NS)          300       76101900

1200       76101901

Hamilton (ONT)        300       38500900

1200       38500901

Kitchener (ONT)       300       33400900

1200       33400901

London (ONT)          300       35600900

1200       35600901

Montreal (QUE)        300       82700902

1200       82700903

Ottawa (ONT)          300       85700901

1200       85700902

Quebec City (QUE)     300       48400900

1200       48400901

Regina (SASK)         300       72100900

1200       72100901

St-John's (NB)        300       74600900

1200       74600901

Saskatoon (SASK)      300       71100900

1200       71100901

St. John (NFLD)       300       78100900

1200       78100901

Toronto (ONT)         300       91600901

1200       91600902

Vancouver (BC)        300       67100900

1200       67100901

Windsor (ONT)         300       29500900

1200       29500901

Winnipeg (MAN)        300       69200902

1200       69200901

.