[][][][][][][][][][][][][][][][][][][][][][][][][][][][][][]

[]                                                        []

[]              Control Office Administration             []

[]              Of Enhanced 911 Services For              []

[]       Special Services And Major Account Centers       []

[]                                                        []

[]                   By The Eavesdropper                  []

[]                                                        []

[]                       March, 1988                      []

[]                                                        []

[][][][][][][][][][][][][][][][][][][][][][][][][][][][][][]

Description of Service

~~~~~~~~~~~~~~~~~~~~~~

The control office for Emergency 911 service is assigned in accordance with

the existing standard guidelines to one of the following centers:

o  Special Services Center (SSC)      o  Major Accounts Center (MAC)

o  Serving Test Center (STC)          o  Toll Control Center (TCC)

The SSC/MAC designation is used in this document interchangeably for any of

these four centers.  The Special Services Centers (SSCs) or Major Account

Centers (MACs) have been designated as the trouble reporting contact for all

E911 customer (PSAP) reported troubles.  Subscribers who have trouble on an

E911 call will continue to contact local repair service (CRSAB) who will refer

the trouble to the SSC/MAC, when appropriate.

Due to the critical nature of E911 service, the control and timely repair of

troubles is demanded.  As the primary E911 customer contact, the SSC/MAC is in

the unique position to monitor the status of the trouble and insure its

resolution.

System Overview

~~~~~~~~~~~~~~~

The number 911 is intended as a nationwide universal telephone number which

provides the public with direct access to a Public Safety Answering Point

(PSAP).  A PSAP is also referred to as an Emergency Service Bureau (ESB).  A

PSAP is an agency or facility which is authorized by a municipality to receive

and respond to police, fire and/or ambulance services.  One or more attendants

are located at the PSAP facilities to receive and handle calls of an emergency

nature in accordance with the local municipal requirements.

An important advantage of E911 emergency service is improved (reduced) response

times for emergency services.  Also close coordination among agencies providing

various emergency services is a valuable capability provided by E911 service.

1A ESS is used as the tandem office for the E911 network to route all 911 calls

to the correct (primary) PSAP designated to serve the calling station.  The

E911 feature was developed primarily to provide routing to the correct PSAP for

all 911 calls.  Selective routing allows a 911 call originated from a

particular station located in a particular district, zone, or town, to be

routed to the primary PSAP designated to serve that customer station regardless

of wire center boundaries.  Thus, selective routing eliminates the problem of

wire center boundaries not coinciding with district or other political

boundaries.

The services available with the E911 feature include:

Forced Disconnect         Default Routing

Alternative Routing       Night Service

Selective Routing         Automatic Number Id entification (ANI)

Selective Transfer        Automatic Location Identification (ALI)

Preservice/Installation Guidelines

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

When a contract for an E911 system has been signed, it is the responsibility

of Network Marketing to establish an implementation/cutover committee which

should include a representative from the SSC/MAC.  Duties of the E911

Implementation Team include coordination of all phases of the E911 system

deployment and the formation of an on-going E911 maintenance subcommittee.

Marketing is responsible for providing the following customer specific

information to the SSC/MAC prior to the start of call through testing:

o  All PSAP's (name, address, local contact)

o  All PSAP circuit ID's

o  1004 911 service request including PSAP details on each PSAP

(1004 Section  K, L, M)

o  Network configuration

o  Any vendor information (name, telephone number, equipment)

The SSC/MAC needs to know if the equipment and sets at the PSAP are maintained

by the BOCs, an independent company, or an outside vendor, or any combination.

This information is then entered on the PSAP profile sheets and reviewed

quarterly for changes, additions and deletions.

Marketing will secure the Major Account Number (MAN) and provide this number to

Corporate Communications so that the initial issue of the service orders carry

the MAN and can be tracked by the SSC/MAC via CORDNET.  PSAP circuits are

official services by definition.

All service orders required for the installation of the E911 system should

include the MAN assigned to the city/county which has purchased the system.

In accordance with the basic SSC/MAC strategy for provisioning, the SSC/MAC

will be Overall Control Office (OCO) for all Node to PSAP circuits (official

services) and any other services for this customer.  Training must be scheduled

for all SSC/MAC involved personnel during the pre-service stage of the project.

The E911 Implementation Team will form the on-going maintenance subcommittee

prior to the initial implementation of the E911 system.  This sub-committee

will establish post implementation quality assurance procedures to ensure that

the E911 system continues to provide quality service to the customer.

Customer/Company training, trouble reporting interfaces for the customer,

telephone company and any involved independent telephone companies needs to be

addressed and implemented prior to E911 cutover.  These functions can be best

addressed by the formation of a sub-committee of the E911 Implementation Team

to set up guidelines for and to secure service commitments of interfacing

organizations.  A SSC/MAC supervisor should chair this subcommittee and include

the following organizations:

1) Switching Control Center

- E911 translations

- Trunking

- End office and Tandem office hardware/software

2) Recent Change Memory Administration Center

- Daily RC update activity for TN/ESN translations

- Processes validity errors and rejects 3) Line and Number Administration

- Verification of TN/ESN translations 4) Special Service Center/Major

Account Center

- Single point of contact for all PSAP and Node to host troubles

- Logs, tracks & statusing of all trouble reports

- Trouble referral, follow up, and escalation

- Customer notification of status and restoration

- Analyzation of "chronic" troubles

- Testing, installation and maintenance of E911 circuits 5) Installation

and Maintenance (SSIM/I&M)

- Repair and maintenance of PSAP equipment and Telco owned sets 6)

Minicomputer Maintenance Operations Center

- E911 circuit maintenance (where applicable) 7) Area Maintenance Engineer

- Technical assistance on voice (CO-PSAP) network related E911 troubles

Maintenance Guidelines

~~~~~~~~~~~~~~~~~~~~~~

The CCNC will test the Node circuit from the 202T at the Host site to the

202T at the Node site.  Since Host to Node (CCNC to MMOC) circuits are official

company services, the CCNC will refer all Node circuit troubles to the SSC/MAC.

The SSC/MAC is responsible for the testing and follow up to restoration of

these circuit troubles.

Although Node to PSAP circuit are official services, the MMOC will refer PSAP

circuit troubles to the appropriate SSC/MAC.  The SSC/MAC is responsible for

testing and follow up to restoration of PSAP circuit troubles.

The SSC/MAC will also receive reports from CRSAB/IMC(s) on subscriber 911

troubles when they are not line troubles.  The SSC/MAC is responsible for

testing and restoration of these troubles.

Maintenance responsibilities are as follows:

SCC*            Voice Network (ANI to PSAP)                 *SCC responsible

for tandem switch SSIM/I&M        PSAP Equipment (Modems, CIU's, sets)

Vendor          PSAP Equipment (when CPE) SSC/MAC         PSAP to Node

circuits, and tandem to PSAP voice circuits (EMNT) MMOC            Node site

(Modems, cables, etc)

Note:  All above work groups are required to resolve troubles by

interfacing        with appropriate work groups for resolution.

The Switching Control Center (SCC) is responsible for E911/1AESS translations

in tandem central offices.  These translations route E911 calls, selective

transfer, default routing, speed calling, etc., for each PSAP.  The SCC is also

responsible for troubleshooting on the voice network (call originating to end

office tandem equipment).

For example, ANI failures in the originating offices would be a responsibility

of the SCC.

Recent Change Memory Administration Center (RCMAC) performs the daily tandem

translation updates (recent change) for routing of individual telephone

numbers.

Recent changes are generated from service order activity (new service, address

changes, etc.) and compiled into a daily file by the E911 Center (ALI/DMS E911

Computer).

SSIM/I&M is responsible for the installation and repair of PSAP equipment. PSAP

equipment includes ANI Controller, ALI Controller, data sets, cables, sets, and

other peripheral equipment that is not vendor owned.  SSIM/I&M is responsible

for establishing maintenance test kits, complete with spare parts for PSAP

maintenance.  This includes test gear, data sets, and ANI/ALI Controller parts.

Special Services Center (SSC) or Major Account Center (MAC) serves as the

trouble reporting contact for all (PSAP) troubles reported by customer.  The

SSC/MAC refers troubles to proper organizations for handling and tracks status

of troubles, escalating when necessary.  The SSC/MAC will close out troubles

with customer.  The SSC/MAC will analyze all troubles and tracks "chronic" PSAP

troubles.

Corporate Communications Network Center (CCNC) will test and refer troubles on

all node to host circuits.  All E911 circuits are classified as official

company property.

The Minicomputer Maintenance Operations Center (MMOC) maintains the E911

(ALI/DMS) computer hardware at the Host site.  This MMOC is also responsible

for monitoring the system and reporting certain PSAP and system problems to the

local MMOC's, SCC's or SSC/MAC's.  The MMOC personnel also operate software

programs that maintain the TN data base under the direction of the E911 Center.

The maintenance of the NODE computer (the interface between the PSAP and the

ALI/DMS computer) is a function of the MMOC at the NODE site.  The MMOC's at

the NODE sites may also be involved in the testing of NODE to Host circuits.

The MMOC will also assist on Host to PSAP and data network related troubles not

resolved through standard trouble clearing procedures.

Installation And Maintenance Center (IMC) is responsible for referral of E911

subscriber troubles that are not subscriber line problems.

E911 Center - Performs the role of System Administration and is responsible for

overall operation of the E911 computer software.  The E911 Center does A-Z

trouble analysis and provides statistical information on the performance of the

system.

This analysis includes processing PSAP inquiries (trouble reports) and referral

of network troubles.  The E911 Center also performs daily processing of tandem

recent change and provides information to the RCMAC for tandem input.  The E911

Center is responsible for daily processing of the ALI/DMS computer data base

and provides error files, etc. to the Customer Services department for

investigation and correction.  The E911 Center participates in all system

implementations and on-going maintenance effort and assists in the development

of procedures, training and education of information to all groups.

Any group receiving a 911 trouble from the SSC/MAC should close out the trouble

with the SSC/MAC or provide a status if the trouble has been referred to

another group.  This will allow the SSC/MAC to provide a status back to the

customer or escalate as appropriate.

Any group receiving a trouble from the Host site (MMOC or CCNC) should close

the trouble back to that group.

The MMOC should notify the appropriate SSC/MAC when the Host, Node, or all Node

circuits are down so that the SSC/MAC can reply to customer reports that may be

called in by the PSAPs.  This will eliminate duplicate reporting of troubles.

On complete outages the MMOC will follow escalation procedures for a Node after

two (2) hours and for a PSAP after four (4) hours.  Additionally the MMOC will

notify the appropriate SSC/MAC when the Host, Node, or all Node circuits are

down.

The PSAP will call the SSC/MAC to report E911 troubles.  The person reporting

the E911 trouble may not have a circuit I.D. and will therefore report the PSAP

name and address.  Many PSAP troubles are not circuit specific.  In those

instances where the caller cannot provide a circuit I.D., the SSC/MAC will be

required to determine the circuit I.D. using the PSAP profile.  Under no

circumstances will the SSC/MAC Center refuse to take the trouble.  The E911

trouble should be handled as quickly as possible, with the SSC/MAC providing as

much assistance as possible while taking the trouble report from the caller.

The SSC/MAC will screen/test the trouble to determine the appropriate handoff

organization based on the following criteria:

PSAP equipment problem:  SSIM/I&M     Circuit problem:  SSC/MAC     Voice

network problem:  SCC (report trunk group number)     Problem affecting

multiple PSAPs (No ALI report from all PSAPs):

Contact                                      the MMOC to check for NODE or

Host                                      computer problems before further

testing.

The SSC/MAC will track the status of reported troubles and escalate as

appropriate.  The SSC/MAC will close out customer/company reports with the

initiating contact.  Groups with specific maintenance responsibilities, defined

above, will investigate "chronic" troubles upon request from the SSC/MAC and

the ongoing maintenance subcommittee.

All "out of service" E911 troubles are priority one type reports.  One link

down to a PSAP is considered a priority one trouble and should be handled as if

the PSAP was isolated.

The PSAP will report troubles with the ANI controller, ALI controller or set

equipment to the SSC/MAC.

NO ANI:  Where the PSAP reports NO ANI (digital display screen is blank) ask if

this condition exists on all screens and on all calls.  It is important to

differentiate between blank screens and screens displaying 911-00XX, or all

zeroes.

When the PSAP reports all screens on all calls, ask if there is any voice

contact with callers.  If there is no voice contact the trouble should be

referred to the SCC immediately since 911 calls are not getting through which

may require alternate routing of calls to another PSAP.

When the PSAP reports this condition on all screens but not all calls and has

voice contact with callers, the report should be referred to SSIM/I&M for

dispatch.  The SSC/MAC should verify with the SCC that ANI is pulsing before

dispatching SSIM.

When the PSAP reports this condition on one screen for all calls (others work

fine) the trouble should be referred to SSIM/I&M for dispatch, because the

trouble is isolated to one piece of equipment at the customer premise.

An ANI failure (i.e. all zeroes) indicates that the ANI has not been received

by the PSAP from the tandem office or was lost by the PSAP ANI controller.  The

PSAP may receive "02" alarms which can be caused by the ANI controller logging

more than three all zero failures on the same trunk.  The PSAP has been

instructed to report this condition to the SSC/MAC since it could indicate an

equipment trouble at the PSAP which might be affecting all subscribers calling

into the PSAP.  When all zeroes are being received on all calls or "02" alarms

continue, a tester should analyze the condition to determine the appropriate

action to be taken.  The tester must perform cooperative testing with the SCC

when there appears to be a problem on the Tandem-PSAP trunks before requesting

dispatch.

When an occasional all zero condition is reported, the SSC/MAC should dispatch

SSIM/I&M to routine equipment on a "chronic" troublesweep.

The PSAPs are instructed to report incidental ANI failures to the BOC on a PSAP

inquiry trouble ticket (paper) that is sent to the Customer Services E911 group

and forwarded to E911 center when required.  This usually involves only a

particular telephone number and is not a condition that would require a report

to the SSC/MAC.  Multiple ANI failures which our from the same end office (XX

denotes end office), indicate a hard trouble condition may exist in the end

office or end office tandem trunks.  The PSAP will report this type of

condition to the SSC/MAC and the SSC/MAC should refer the report to the SCC

responsible for the tandem office.  NOTE: XX is the ESCO (Emergency Service

Number) associated with the incoming 911 trunks into the tandem.  It is

important that the C/MAC tell the SCC what is displayed at the PSAP (i.e.

911-0011) which indicates to the SCC which end office is in trouble.

Note:  It is essential that the PSAP fill out inquiry form on every ANI

failure.

The PSAP will report a trouble any time an address is not received on an

address display (screen blank) E911 call.  (If a record is not in the 911 data

base or an ANI failure is encountered, the screen will provide a display

noticing such condition).  The SSC/MAC should verify with the PSAP whether the

NO ALI condition is on one screen or all screens.

When the condition is on one screen (other screens receive ALI information) the

SSC/MAC will request SSIM/I&M to dispatch.

If no screens are receiving ALI information, there is usually a circuit trouble

between the PSAP and the Host computer.  The SSC/MAC should test the trouble

and refer for restoral.

Note:  If the SSC/MAC receives calls from multiple PSAP's, all of which

are receiving NO ALI, there is a problem with the Node or Node to Host

circuits or the Host computer itself.  Before referring the trouble the

SSC/MAC should call the MMOC to inquire if the Node or Host is in

trouble.

Alarm conditions on the ANI controller digital display at the PSAP are to be

reported by the PSAP's.  These alarms can indicate various trouble conditions

o so the SSC/MAC should ask the PSAP if any portion of the E911 system is not

functioning properly.

The SSC/MAC should verify with the PSAP attendant that the equipment's primary

function is answering E911 calls.  If it is, the SSC/MAC should request a

dispatch SSIM/I&M.  If the equipment is not primarily used for E911, then the

SSC/MAC should advise PSAP to contact their CPE vendor.

Note:  These troubles can be quite confusing when the PSAP has vendor

equipment mixed in with equipment that the BOC maintains.  The

Marketing representative should provide the SSC/MAC information

concerning any unusual or exception items where the PSAP should contact

their vendor. This information should be included in the PSAP profile

sheets.

ANI or ALI controller down:  When the host computer sees the PSAP equipment

down and it does not come back up, the MMOC will report the trouble to the

SSC/MAC; the equipment is down at the PSAP, a dispatch will be required.

PSAP link (circuit) down:  The MMOC will provide the SSC/MAC with the circuit

ID that the Host computer indicates in trouble.  Although each PSAP has two

circuits, when either circuit is down the condition must be treated as an

emergency since failure of the second circuit will cause the PSAP to be

isolated.

Any problems that the MMOC identifies from the Node location to the Host

computer will be handled directly with the appropriate MMOC(s)/CCNC.

Note:  The customer will call only when a problem is apparent to the

PSAP. When only one circuit is down to the PSAP, the customer may not be

aware there is a trouble, even though there is one link down,

notification should appear on the PSAP screen.  Troubles called into the

SSC/MAC from the MMOC or other company employee should not be closed out

by calling the PSAP since it may result in the customer responding that

they do not have a trouble.  These reports can only be closed out by

receiving information that the trouble was fixed and by checking with

the company employee that reported the trouble.  The MMOC personnel will

be able to verify that the trouble has cleared by reviewing a printout

from the host.

When the CRSAB receives a subscriber complaint (i.e., cannot dial 911) the RSA

should obtain as much information as possible while the customer is on the

line.

For example, what happened when the subscriber dialed 911?  The report is

automatically directed to the IMC for subscriber line testing.  When no line

trouble is found, the IMC will refer the trouble condition to the SSC/MAC.  The

SSC/MAC will contact Customer Services E911 Group and verify that the

subscriber should be able to call 911 and obtain the ESN.  The SSC/MAC will

verify the ESN via 2SCCS.  When both verifications match, the SSC/MAC will

refer the report to the SCC responsible for the 911 tandem office for

investigation and resolution.  The MAC is responsible for tracking the trouble

and informing the IMC when it is resolved.

For more information, please refer to E911 Glossary of Terms.

_______________________________________________________________________________

==Phrack Inc.==

Volume Two, Issue 24, File 6 of 13

[][][][][][][][][][][][][][][][][][][][][][][][][][][][][][]

[]                                                        []

[]                  Glossary Terminology                  []

[]                For Enhanced 911 Services               []

[]                                                        []

[]                   By The Eavesdropper                  []

[]                                                        []

[]                       March, 1988                      []

[]                                                        []

[][][][][][][][][][][][][][][][][][][][][][][][][][][][][][]

E911 - Enhanced 911:  Features available include selective routing,

selective transfer, fixed transfer, alternate routing, default routing,

Automatic Number Display, Automatic Location Identification, night

service, default routing, call detail record.

End Office - Telephone central office which provides dial tone to the

subscriber calling 911. The "end office" provides ANI

(Automatic Number Identification) to the tandem office.

Tandem Office

- Telephone central office which serves as a tandem (or hub) for all

911 calls.  Must be a 1AESS type of central office. The tandem office

translations contain the TN/ESN relationships which route the 911 call

to the proper SAP.  The tandem office looks up the ANI (TN) that it

receives from the end office and finds the ESN (routing information)

which corresponds to a seven digit number ringing in at a PSAP.

PSAP - Public Safety Answering Point, usually the police, fire and/or

rescue groups as determined by the local municipalities.  A "ringin"

will not have ANI or ALI capabilities, but just receives calls or

transferred calls from another PSAP.

ESN - Emergency Service Number (XXX) that is assigned to the

subscriber's telephone number in the tandem office translations The ESN

represents a seven digit number by which the tandem office routes the

call to the proper PSAP.  PSAPs with ALI capabilities also receive a

display of the ESN information which shows which police, fire and rescue

agency serves the telephone number calling 911.  An ESN is a unique

combination of police, fire, and rescue service for purposes of routing

the E911 call.

ANI - Automatic Number Identification corresponds to the subscriber's

seven digit telephone number.  The ANI displays at the PSAP on the

digital ANI display console.

ALI - Automatic Location Identification provides for an address display

of the subscriber calling 911.  With ALI, the PSAP receives the ANI

display and an ALI display on a screen.  The ALI display includes the

subscriber's address, community, state, type of service and if a

business, the name of the business.  The PSAP will also get a display of

the associated ESN information (police, fire, rescue).

Selective Routing - The capability to route a call to the particular

PSAP serving the address associated with the TN making the 911 call.

Selective routing is achieved by building TN/ESN translations in the

tandem central office. These translations are driven by the E911 data

base which assigns the ESN to each telephone number based on the

customer's address.  Service order activity keeps the E911 data base

updated.  The E911 data base, in turn, generates recent change to the

tandem office (through the SCC or RCMAC) to update the TN/ESN

translations in the tandem data base.

Selective Transfer - Provides the PSAP with the ability to transfer the

incoming 911 call to a fire or rescue service for the particular number

calling 911 by pushing one button for fire or rescue.  For example, if

an incoming 911 call was reporting a fire, the PSAP operator would push

the fire button on the ANI console; the call would go back to the tandem

office, do a lookup for the seven digit number associated with fire

department, for the ESN assigned to the calling TN, and automatically

route the call to that fire department.  This differs from "fixed"

transfer which routes every call to the same fire or rescue number

whenever the fire or rescue button is pushed.  The PSAP equipment is

optioned to provide either fixed or selective transfer capabilities.

Alternate Routing - Alternate routing provides for a predetermined

routing for 911 calls when the tandem office is unable to route the

calls over the 911 trunks for a particular PSAP due to troubles or all

trunks busy.

Default Routing - Provides for routing of 911 calls when there is an ANI

failure.  The call will be routed to the "default" ESN associated with

the he NNX the caller is calling from. Default ESNs are preassigned in

translations and are usually the predominant ESN for a given wire

center.

Night Service - Night service works the same as alternate routing in

that the calls coming into a given PSAP will automatically be routed to

another preset PSAP when all trunks are made busy due to the PSAP

closing down for the night.

Call Detail Record - When the 911 call is terminated by the PSAP

operator, the ANI will automatically print-out on the teletypewriter

located at the PSAP.  The printout will contain the time the call came

into the PSAP, the time it was picked up by an operator, the operator

number, the time the call was transferred, if applicable, the time the

call was terminated and the trunk group number associated with the call.

Printouts of the ALI display are now also available, if the PSAP has

purchased the required equipment.

ANI Failure - Failure of the end office to identify the call and provide

the ANI (telephone number) to the tandem office; or, an ANI failure

between the tandem office and the PSAP.

Misroute - Any condition that results in the 911 call going to the wrong

PSAP. A call can be misrouted if the ESN and associated routing

information are incorrect in the E911 data base and/or tandem data base.

A call can also be misrouted if the call is an ANI failure, which

automatically default routes.

Anonymous Call - If a subscriber misdials and dials the seven digit

number associated with the PSAP position, they will come in direct and

ANI display as 911-0000 which will ALI as an anonymous call.  The seven

digit numbers associated with the PSAP positions are not published even

to the PSAPs.

Spurious 911 Call - Occasionally, the PSAP will get a call that is not

associated with a subscriber dialing 911 for an emergency. It could be a

subscriber who has not dialed 911, but is dialing another number, or has

just picked up their phone and was connected with the PSAP.  These

problems are equipment related, particularly when the calls originate

from electromechanical or step by step offices, and are reported by the

E911 Center to Network Operations upon receipt of the PSAP inquiry

reporting the trouble. The PSAP may get a call and no one is there; if

they call the number back, the number may be disconnected or no one

home. Again these are network troubles and must be investigated.

Cordless telephones can also generate "spurious" calls in to the PSAPs.

Generally, the PSAP will hear conversation on the line, but the

subscribers are not calling 911.  The PSAP may report spurious calls to

to repair if they become bothersome, for example, the same number

ringing in continually.

No Displays - A condition where the PSAP ALI display screen is blank.

This type of trouble should be reported immediately to the SSC/MAC. If

all screens at the PSAP are blank, it is an indication that the problem

is in the circuits from the PSAP to the E911 computer.  If more than one

PSAP is experiencing no display, it may be a problem with the Node

computer or the E911 computer. The SSC/MAC should contact the MMOC to

determine the health of the HOST computer.

Record Not Found - If the host computer is unable to do a look up on a

given ANI request from the PSAP, it will forward a Record Not Found

message to the PSA ALI screen.  This is caused by service order activity

for a given subscriber not being processed into the E911 data base, or

HOST computer system problems whereby the record cannot be accessed at

that point in time.

No ANI - This condition means the PSAP received a call, but no telephone

number displayed on the ANI console.  The PSAP should report this

condition immediately to the SSC/MAC.

PSAP Not Receiving Calls - If a PSAP cannot receive calls or request

retrievals from the E911 host computer, i.e., cable cut, the calls into

that PSAP must be rerouted to another PSAP.  The Switching Control

Center must be notified to reroute the calls in the tandem office E911

translations.

MSAG - Master Street Address Guide.  The MSAG ledgers are controlled by

the municipality which has purchased the E911 ALI service, in that they

assign which police, fire or rescue agency will serve a given street and

number range.  They do this by assigning an ESN to each street range,

odd, even, community that is populated in the county or municipality

served.  These MSAGs are then used as a filter for service order

activity into the E911 computer data base to assign ESNs to individual

TN records.  This insures that each customer will be routed to the

correct agency for their particular address.  In a non-ALI County, TAR

codes are used by the Telephone company to assign ESNs to service

conductivity and the County does not control the ESN assignment.  TAR

codes represent the taxing authority for the given subscriber which

should correspond to their police, fire and rescue agencies.  The MG

method, of course, is more accurate because it is using the actual

service address of the customer to route the call and provides the

county with more flexibility in assigning fire and rescue district, etc.

The Customer Services E911 Group maintains the E911 computer data base

and interfaces with the County (customer) on all MSAG or data base

activity.

_______________________________________________________________________________