BeepMe 1.1

Documentation File

(C)1993 Bret Lanius

This Program and documentation is shareware and is copyrighted by Bret Lanius.

Please feel free to distribute this program and associated files so long as

none of the files are left out or modified in any way.

You are hereby granted permission to use this program for a period of no more

than 30 days to evaluate whether or not it meets your needs, at the end of

the 30 day period you must either register the program with me or delete it

from your system and discontinue use of the program.

Fill out the register.frm and send $12 to:

Bret Lanius

1493 Walnut Ridge Way

Stone Mountain, GA 30083

You may include comments and request for improvements along with your

registration.

What is it?

----------

BeepMe monitors your phone line by using a modem on either COM1 or COM2, then

calls your pager when your phone rings. I wrote this so that I'd know when

someone had left a message on my answering machine.

Use the Install Program!!!

--------------------------

Beepme requires a file named Pager.dat that tells it how to run. The

Install program creates this file and will copy it and the beepme

program and docs to the drive and directory you specify. If you just

can't stand to use an install program or you have trouble with it, do

the following:

Copy Beepme.exe to the desired directory, using an ASCII editor create

the pager.dat file as follows:

COM1

1200

999-9999,,,,,,

54325425

Pager.log

120

atz

Line one is the Com Port

Line two the baud rate 300-9600

Line three Pager Phone Number and commas to pause.

Line four numbers to page with

Line five Logfile name (Calls are logged to this file)

Line six How long in seconds to wait after phone rings before calling

Line seven Modem Init String (must at least have AT as the program waits

for OK before preceeding)

Waiting for a call

------------------

Once you've configured BeepMe you can then press "W" to start waiting

for the phone to ring. Press ALT-Q to exit the wait mode at any time.

While Waiting you can test your set up by pressing the up arrow key. The

Program will act as if it had received two rings and proceed to delay and

then dial. This is recommended to make sure you have enough commas in

your pager number.

Command Line Switches

--------------------

/W

If you run BeepMe with this switch BeepMe will go straight into the wait

mode.

/Rn

Where n is the number of rings to wait before calling out.

e.q. /R4 will wait for the phone to ring 4 times. This number is limited

to one digit and the default is 2 rings.

To Wait for 3 rings and to automatically start waiting you can call

BeepMe with

Beepme /W/R3 [ENTER]

=======================================================================

If you have Questions or Comments

Bret Lanius

1493 Walnut Ridge Way

Stone Mountain, GA 30083

Compuserve 72667,2365

INTERNET bret.lanius@ehbbs.com