��������                ��

��������              ���

��������            ���

��������          ���

��������        ���

��������      ���

��������    ���

��������  ���

������������     ���������

�����������       4  4  0

�������� ���     ���������

��������  ���

��������   ���

��������    ����

��������     �����

��������      ������

��������       �������

��������        ��������

��������  ���������������������

��������   S  p  o  i  l  e  r

��������  ���������������������

THE 440 SPOILER: VERSION 1.01

PRELIMINARY TECHNICAL-REFERENCE MANUAL

The 440  Spoiler lets  you use  and enjoy the full operating power of the

Kenwood TS-440S  amateur radio  transceiver.   The Spoiler is fully functional

software that  makes your transceiver superior to any other transceiver on the

market in speed, convenience, and versatility of operating.

The 440  Spoiler software is provided to you through shareware marketing,

a method  which gives  you the  finest software  available at a minimum price.

You are  invited to  use the  440 Spoiler for a period of up to one month.  If

you decide  to use  the 440  Spoiler thereafter, you must obtain a license for

the software  by registering  it with  the QSO2000  Project, 5518 12th Street,

Lubbock, TX,  79416 for  a nominal fee of $39.  A convenient registration form

is included at on the last page of this manual.

Your registering  your software  provides you  with significant benefits.

You receive  a diskette copy of the most-recent revision of the Spoiler.  This

version of  the software  will contain latest enhancements, latest maintenance

revisions,  and   no  shareware   notices.     In  addition,   you  receive  a

professionally written,  printed, and  bound copy of the 440 Spoiler Technical

Reference Manual.   You  also receive  technical support and assistance as you

require it in your use of the 440 Spoiler.

Your registration  is also  your way  of ensuring  continued improvement,

development, and  maintenance of  the 440  Spoiler throughout  the long useful

lifetime of you Kenwood TS-440S transceiver.

The last section of this documentation file provides detailed information

about registration  of the  440 Spoiler  as well  as  form  for  your  use  in

registering your copy.

INTRODUCTION

The Kenwood  TS-440S is  one of  the  most  sophisticated  high-frequency

transceivers available  for amateur-radio  use.  It contains 100 memories (110

counting the  separate transmit  and receive  memories for the upper 10 memory

channels). It  contains a  general-coverage receiver  and  transmitter  and  a

unique receive  and transmit increment-tuning system.  The operating potential

of the transceiver is astounding.

Because of  the operating  power of  the transceiver,  however, operation

from the  front panel  is of necessity, complex.  Storing, altering, and using

all of  the information  into TS-440S  memories is  time-consuming  and  often

confusing.

The TS-440S keypads present operating problems.  It is easy to forget the

sequence of keystrokes required by the keypads on the front panel if you don't

use them very often.  Also, infrequent use of the keypads leads to key bounce;

you can  press them  many times  before you condition them to switch only once

for each  time you press them.  Additionally, all storage and control from the

front panel  is performed  on the  current memory  or VFO: you can't prepare a

memory or  a vfo for switching to it while you are operating on another memory

or VFO.

The TS-440S  display also presents operating problems.  You can't see the

frequency and mode of a memory or VFO unless you select it and therefore cease

to receive  or transmit  on your  current frequency  and mode.   Moreover, you

can't see  your transmit frequency at the same time your receiving, no can you

see your  receive frequency  while you  are transmitting.   If  you are  using

increment tuning  on either  transmit or  receive, the  display shows  you the

result of  your using  it but  it doesn't show you the actual frequency of the

vfo or memory that is currently selected.

The 440  Spoiler overcomes  the problems  presented by  the  front  panel

switches and  the display.  Your MSDOS computer keyboard provides considerably

more convenient and legible control of the transceiver.  With a minimal number

of keystrokes,  you can  select any  of the  operating parameters  (frequency,

mode, etc.)  stored in  the  TS-440S,  and  you  can  conveniently  alter  the

operating parameters stored in the transceiver.

Additionally, the  Spoiler displays  all  operating  frequencies,  modes,

frequency splits,  tuning increments,  and other  operating information on the

screen of your computer.  Half of the parameter memories stored in the TS-440S

are displayed  on the  screen at  all times;  you can toggle between the lower

half and  the upper  half at  any time  with a  single keystroke.  The Spoiler

makes information  entry and display extremely simple, convenient, and fast as

prompts on the screen direct you through a minimum of key strokes.

The display always shows the current operating conditions of the TS-440S,

regardless of  whether you  have used the front panel or the computer keyboard

to change  the conditions.   If  you change  frequency, mode, split, increment

tuning, or  memories from  the front panel, the Spoiler updates its display to

show the remotely entered data.

The 440 Spoiler contains a complete memory-management system that enables

you to  store up  to tens  of thousands  of frequency  and  mode  memories  on

computer disk.   Stored  in banks  of 100 memories, the memories may be loaded

into your TS-440S with a few key strokes.  You may customize individual memory

banks for  different modes  of operation,  for  different  bands,  for  DX  or

traffic-net operation, for monitoring broadcasts, citizens band transmissions,

or whatever your specialized interests include.  Moreover, the Spoiler manages

VFOs and  memories in a way that does not disturb your current operation.  You

can enter  new frequencies  and modes into unselected memories or VFOs without

missing a word on a net or a calling frequency you are currently monitoring.

Simplicity of  operation is  the most important design feature of the 440

spoiler.   From your screen, you see prompts to lead you through any operation

you care  to perform.   If you want to change the operating mode of a vfo or a

memory, you  press  the  MODE  function  key;  you  press  SOURCE,  FREQUENCY,

INCREMENT  TUNING,   and  MEMORY   MANAGEMENT  keys   with  similar   ease  to

instantaneously control every feature of your transceiver.

The 440  Spoiler also contains provisions for accessing a log-maintenance

program and  a digital-controller  program while  you maintain the 440 Spoiler

resident in memory for quick recall.

HARDWARE REQUIREMENTS

The 440 Spoiler requires an MSDOS or PCDOS computer with at least 256K of

memory, one  diskette drive,  and one  serial (COM)  port.  Performance of the

program is  improved, of  course, with  your running  it from hard disk rather

than floppy.  The faster your CPU speed the more responsive the program.

Your TS-440S  must be  equipped for serial (RS-232C) communications.  You

must have  the two  interface chips  such as  the ones  Kenwood sells as a kit

designated as  the IC-10  as well  as a  TTL/RS-232C level  converter such  as

Kenwood sells  as the IF-232C adapter.  It is not necessary for you to use the

parts that  Kenwood markets.   The  necessary interface chips are common ones,

and if  you are experienced in digital-circuit design you might build your own

level converter.   Kenwood prices are high, but Kenwood is a convenient source

for ready-made components that are guaranteed to work with your transceiver.

OPERATION

The opening  screen of  unregistered copies  of the  Spoiler  contains  a

reminder that  the software  is a  shareware  package  that  depends  on  user

registration to provide for its initial and continued development.  The screen

is displayed  for 5  seconds: after  the initial  period, you can bring up the

operating screen  by pressing  any key.  Registered copies, of course, contain

no waiting period.

Registered copies  of the  Spoiler display an opening screen for only the

length of  time it takes you to press a key.  The opening screen also contains

no shareware notice.

The operating  screen shows  every parameter that enters into transceiver

operation, regardless  of whether  changes in  operation occur  from the front

panel or  from the  computer keyboard.   It  also shows  commands  which  make

computer-keyboard control of the TS-440S incredibly easy and fast.

THE OPERATING SCREEN

The operating  screen consists  of four  display areas  for each  of  the

following:  (1)  current  operating  sources,  (2)  command  options,  (3)  55

memories, and  (4) commands.  Figure 1 shows a typical operating screen (which

has  been   shortened  horizontally   to  fit   within  the  margins  of  this

documentation file.   Current  operating sources  are shown  in the box in the

upper left.   Command  options are shown in the box which contains the program

logo when  no command  has been issued.  Transceiver memory contents are shown

in the  large, J-shaped  box to  the right and bottom of the screen.  Commands

are shown on the bottom line.

����������������������������������������ͻ ���������͸����������������������ͻ

�      RECEIVE       �TR 21.005.00 CW  61� �11f  7.00�� CW �33f  2.500.00 AM �

�   21.005.00 CW     �������������������ĺ �12f  7.01�� CW �34f  5.000.00 AM �

�         61         �TR 27.285.00 AM  VA� �13f  7.01�� CW �35f 10.000.00 AM �

��������������������͹TR  3.921.44 LSB VB� �14f  7.02�� CW �36f 15.000.00 AM �

�      TRANSMIT      �������������������ĺ �15f  7.02�� CW �37f 20.000.00 AM �

�   21.005.00 CW     �No Increment..-0.05� �16f  7.03�� CW �38f 14.005.00 CW �

�         61         �                   � �17f  7.04�� CW �39f 14.010.00 CW �

����������������������������������������͹ �18f  7.06�� CW �40f 14.015.00 CW �

�Time:14:30:11�ZEROS1.MRY  �Date:01-01-90� �19f  7.07�� CW �41f 14.020.00 CW �

����������������������������������������ͼ �20f  7.15Z� LSB�42f 14.030.00 CW �

�����������������������ͻ�����������������ͼ21f  7.16�� LSB�43f 14.050.00 CW �

�     ����       �      ��00f  3.505.00 CW �22f  7.18�Z LSB�44f 14.070.00 CW �

�     ����     ��       ��01f  3.515.00 CW �23f  7.19�� LSB�45f 14.151.00 USB�

�     ����   ��         ��02f  3.525.00 CW �24f  7.20�� LSB�46f 14.155.00 USB�

�     ���� �� �����     ��03f  3.780.00 LSB�25f  7.21�� LSB�47f 14.160.00 USB�

�     ������  TS440     ��04f  3.790.00 LSB�26f  7.22�� LSB�48f 14.175.00 USB�

�     ���� �� �����     ��05f  3.905.00 LSB�27f  7.23�� LSB�49f 14.190.00 USB�

�     ����  ��          ��06f  3.910.00 LSB�28f  7.23�� LSB�50f 14.205.00 USB�

�     ����   ���        ��07f  3.928.00 LSB�29f  7.24�� LSB�51f 14.220.00 USB�

�     ���� ���������    ��08f  3.933.00 LSB�30f  7.24�� LSB�52f 14.240.00 USB�

�     ����  Spoiler     ��09f  3.935.00 LSB�31f  7.24�� LSB�53f 14.250.00 USB�

�     ���� ���������    ��10f  3.940.00 LSB�32f  7.29�� LSB�54f 14.270.00 USB�

�����������������������ͼ���������������������������;����������������������ͼ

1SLCT S 2FREQ S 3MODE S 4INCR T 5MOVE S 6FILE M 7EDIT M 8CTRL P 9LOG   10QUIT

Figure 1.- Operating screen

CURRENT-SOURCES DISPLAY BOX

The display  box in  the upper  left of  the  operating  screen  displays

current sources:  the two  VFOs, the  memory selected  by the TS-440S, and the

increment tuning.   This box shows every detail about the current operation of

the transceiver.  An example of the current-sources box is shown in figure 2.

����������������������������������������ͻ

�      RECEIVE       �TR 21.005.00 CW  61�

�   21.005.00 CW     �������������������ĺ

�         61         �TR 27.285.00 AM  VA�

��������������������͹TR  3.921.44 LSB VB�

�      TRANSMIT      �������������������ĺ

�   21.005.00 CW     �No Increment..-0.05�

�         61         �                   �

����������������������������������������͹

�Time:14:30:11�ZEROS1.MRY  �Date:01-01-90�

����������������������������������������ͼ

Figure 2.- Current-sources display

The box  contains areas  for receiving  and transmitting  frequency,  for

detailed source information, and for current Spoiler operation.

Receive and Transmit Areas

Overall transceiver  operation is  summarized in the Receive and Transmit

areas to  the left  of the  box.   These areas show, irrespective of operating

components that  go into  transceiver operation,  the  current  receiving  and

transmitting frequencies,  and modes.   The  frequency and  mode that  you are

hearing on  the transceiver,  as well  as  the  frequency  and  mode  you  are

transmitting on when your press your microphone button, tap a key, or activate

your digital  controller, are  displayed to an accuracy of 10 KHz in these two

areas.

Receive  and Transmit  areas show  the combined effects of VFO or memory

frequency selection,  split-frequency sources,  and currently active increment

tuning.   The displayed frequencies are the actual frequencies of transmission

and reception.   The  receiving frequency  accounts for  any receive increment

tuning (RIT),  and the  transmit frequency accounts for any transmit increment

tuning (XIT).

Source-Detail Area

To the  right of  the Receive  and Transmit  areas is  an area containing

detailed information  about the components of the current sources: frequencies

and modes  of the  two VFOs  and  the  selected  memory,  the  split-frequency

operating status, and the status of increment tuning.

In the source-detail box to the right, the upper partition identifies the

currently active source(s) and shows frequency and mode.  The middle partition

shows the  sources "on reserve" for your use at the press of the TS-440S VFO/M

and  A/B  switches.    The  lower  partition  shows  current  increment-tuning

frequency and status

Unlike the receive and transmit areas to the left or the TS-440S display,

the detail-area  source display  does not  reflect increment  turning.  If you

turn on increment tuning for transmit or receive, for example, the area to the

left will  show the  result of  the increment tuning, but the detail area will

continue to  show the  exact frequency  of the  source without  the effect  by

increment tuning.   The  last two  characters of  the line identify the source

with VA  for VFO  A, VB  for VFO  B, and  a number  between 00  and 99 for the

selected memory.   The  first two  characters of  the line identifies the role

that the source contributes to split-frequency operation.

The upper  partition of the detail-source display may contain one line to

indicate a  non-split source operating on both transmit and receive (therefore

prefaced by  TR), as  it does  in figure 2, or it may contain two lines, as in

the display of split-VFO operation shown in figure 3.

����������������������������������������ͻ

�      RECEIVE       �RX  7.209.04 CW  VA�

�    7.209.04 CW     �TX  3.959.23 LSB VB�

�         VA         �������������������ĺ

��������������������͹TR  3.525.00 CW  02�

�      TRANSMIT      �������������������ĺ

�    3.959.23 LSB    �No Increment..-0.03�

�         VB         �                   �

����������������������������������������͹

�Time:14:43:54�GENPUR1.MRY �Date:01-07-90�

����������������������������������������ͼ

Figure 3.- Split VFO display

During split-frequency operation, there are always two lines in the upper

partition.   The receiving-frequency  source is  always identified  in the top

line with  the designation RX, and the transmitting-frequency source is always

identified in the second line with the designation TX.  Split-frequency memory

operation is displayed , as shown in figure 4

����������������������������������������ͻ

�      RECEIVE       �RX  7.154.00 LSB 99�

�    7.154.00 LSB    �TX  7.131.02 LSB 99�

�         99         �������������������ĺ

��������������������͹RX  7.209.04 CW  VA�

�      TRANSMIT      �TX  3.959.23 LSB VB�

�    7.131.02 LSB    �������������������ĺ

�         99         �No Increment..-0.03�

����������������������������������������͹

�Time:14:47:11�DEFAULT.MRY �Date:01-07-90�

����������������������������������������ͼ

Figure 4.- Split memory display

When the  upper area  contains two lines, the source is split so that the

top line  shows the  receiving-source frequency  and mode  and the bottom line

shows the transmitting frequency and mode.  With a VFO selected, the frequency

and mode  of either  VFO A  or VFO B appears in the top line prefaced by an RX

designation indicating  a receiving  source.   The frequency  and mode  of the

transmitting VFO appears in the lower line prefaced by TX.

The middle partition of the detailed display area shows alternate sources

that are  currently selected:  these are  the sources  that  you  can  use  by

toggling between VFO A and VFO B or between VFO and memory.

The lower  partition shows  the frequency and status of increment tuning.

The frequency  is shown  to the  right of the line, and the status is shown to

the left.   Status  may be  No Increment,  RX  Increment  (receive  only),  TX

Increment (transmit only), or TR Increment (both transmit and receive).

You should notice very carefully that if you toggle between VFO A and VFO

B any  increment tuning that is in effect remains in effect, but if you toggle

between either  VFO and  memory any  increment turning loses its effect.  This

peculiarity is simply a design feature of the Kenwood TS-440S.

Current Spoiler Operation

The current  time, the  active file  of memories stored in your computer,

and the  current date  are displayed  in the  area across  the bottom  of  the

current-sources box.

MEMORY DISPLAY

The contents  of TS-440S  memory sources are displayed in the largest box

on the screen.  Figure 5 shows a full-width memory-display box.

�����������������������������������ͻ

�11f  7.006.12  CW�33f  7.000.00 LSB�

�12f  2.500.00  AM�34f  7.154.00 LSB�

�13f  0.000.00    �35f  7.131.02 LSB�

�14f  0.000.00    �36f  0.000.00    �

�15f  0.000.00    �37f  0.000.00    �

�16f  0.000.00    �38f  0.000.00    �

�17f  7.220.00 LSB�39f  0.000.00    �

�18f  7.213.00 LSB�40f  1.590.00 LSB�

�19f  7.249.00 USB�41f  1.320.00 LSB�

�20f  3.933.00 USB�42f  0.000.00    �

�����������������ͼ21f  0.000.00    �43f 28.000.00 LSB�

�00f  3.933.00 LSB�22f  7.290.00 LSB�44f  3.395.00 LSB�

�01f  3.935.00 LSB�23f  1.590.50 AM �45f  0.000.00    �

�02f  7.290.00 USB�24f 14.305.00 USB�46f  0.000.00    �

�03f 14.334.00 USB�25f  1.590.50 AM �47f  0.000.00    �

�04f 28.454.00 USB�26f  1.590.50 AM �48f  0.000.00    �

�05f  7.100.00  CW�27f  1.590.50 AM �49f  0.000.00    �

�06f 28.000.00 USB�28f  0.000.00    �50f 28.000.00 LSB�

�07f  7.250.00 LSB�29f  0.000.00    �51f  0.000.00    �

�08f 21.000.00 LSB�30f  3.900.00 LSB�52f  0.000.00    �

�09f 28.454.00 USB�31f  0.000.00    �53f  0.000.00    �

�10f  7.100.00  AM�32f  0.000.00    �54f  0.000.00    �

�����������������������������������������������������ͼ

Figure 5.- Memory display box

The box,  at any  particular time, shows the frequencies and modes stored

in half  of the  transceiver's 100  memories.   When the  Spoiler is initially

loaded, the  box shows  the contents of memories 0 through 54.  When you press

F9 (function key F9), the upper 55 memories are displayed.

Within the  memory box,  memories 0  through 89  are  identified  by  the

designations 00f through 89f.  The lower 90 memory channels of the TS-440S are

capable of  operating on  only a  single, non-split  frequency and  mode.  The

upper 10  memory channels  consist of  two memories  apiece:  one  memory  for

receive and  one memory  for transmit.    Beginning  with  memory  90,  memory

channels are,  therefore, identified  by a  channel number  with either an "r"

(receive) or  a "t" (transmit) suffix.  Channel 97 is therefore represented in

the memory display box by both 97r and 97t.

SPOILER COMMAND LINE

The bottom  line of  the Spoiler  display is  a prompt for command entry.

Figure 6 shows this line.

------------------------------------------------------------------------------

1SELECT 2FREQ   3MODE   4INCR   5COPY   6FILES  7LOG    8CNTL   9M TOG  10QUIT

------------------------------------------------------------------------------

Figure 6.- Spoiler command line

SPOILER COMMANDS

The Kenwood  TS-440S is  a completely self-contained transceiver, and the

front panel,  although sometimes  cumbersomely, provides  ways to  control its

operation.   Your MSDOS computer provides you with an alternative to the front

panel.   The Spoiler  display provides  current information  about transceiver

operating parameters,  regardless of  whether you change the parameters at the

front panel or at the keyboard.

Spoiler commands  are represented  by numbers  from 1  through 10.    The

numbers correspond  to function  keys on  MSDOS computers.  Command number 10,

labeled QUIT  on the  command line,  causes the Spoiler to cease operation and

control of the computer to return to the disk operating system (DOS).

The commands  are listed  on the  command  line  by  abbreviation.    The

abbreviations correspond to the brief descriptions shown in table 1.

Source selection.........................Select a VFO, VFO split, or a memory

Frequency selection.................Change the frequency of a VFO or a memory

Mode selection...........................Change the mode of a VFO or a memory

Increment Tuning........Add frequency increment to transmit of receive source

Copy sources.......................Copy or exchange between VFOs and memories

Memory management.......................Use computer files for memory storage

Logging program..........Run a logging program while the Spoiler is in memory

Controller program....Run a controller program while the Spoiler is in memory

Memory display toggle.........Toggle display between memories 0-54 and 55-99t

Quit................................Quite Spoiler operation and return to DOS

TABLE 1.- Brief descriptions of Spoiler commands

When you  press a  function key  to invoke  a spoiler command, you open a

window behind  the Spoiler  logo in the box at the lower left of the operating

screen.   The window  prompts you  for information that you need to provide to

complete any control operation for the transceiver.

The following  sections of  this documentation  file describe each of the

Spoiler commands in detail.  Each command option is also described.

SOURCE SELECTION

The source-select  command invoked by F1 determines the primary frequency

and mode  source for  TS-440S reception  and transmission.   The frequency and

mode of  your reception and transmission are always from a certain source, the

VFOs or  the memories,  as you  determine by  issuing a source-select command.

Figure 7  shows the  options you are presented with in the command-options box

for the source-select command.

�����������������������ͻ

�   Source Selection    �

� VFO A...............A �

� VFO B...............B �

� Memory..............M �

� Split...............S �

� Mem Num............## �

� Quit................Q �

�                       �

�                       �

�                       �

�                       �

�����������������������ͼ

Figure 7.- Source-select command options

As figure  7 shows, you may choose VFO A, VFO B, or the current memory to

be  your   current  reception   and  transmission  source.    You  may  choose

transmission and reception to be split, with one VFO being used on receive and

the other being used on transmit.  Additionally, you may also choose any other

memory channel  in the  transceiver to  be  your  reception  and  transmission

source, with  channels 0 through 89 providing for only non-split operation and

channels 90 through 99 providing for split transmit/receive operation.

The option  box is a simple reflection of this powerful command.  You may

press A, B, or M to select the VFO A, VFO B, or current memory.  You may enter

the number  of any  other memory to cause it to become both the current memory

and the  selected source.   And you may toggle the VFOs between split and non-

split operation.

Your pressing  a single  key (A,  B, or  M) causes  VFO A,  VFO B, or the

current memory  to become  the current source.  If the source is a VFO and the

VFOs were  last used  in split-source  operation, your  reselecting them  will

bring them  up for split operation (regardless of which of them you select for

your primary  (receive) VFO.  If the source is a memory, memories 0 through 89

always provide  you with  a non-split source, while memories 90 through 99 can

provide you  with either  split frequencies and modes or non-split frequencies

and modes, depending on what you have placed in them before.

Your pressing  the S  key while  either VFO  is selected  toggles between

split and  non-split operation.  Pressing the S key while a memory is selected

has no effect.

For the  VFOs, the  status of  split-frequency operation remains constant

until your  change it.  If you switch from a VFO to a memory source, while the

VFOs are  linked for  split-frequency operation,  you'll find  the VFOs  still

linked when you switch back from memory to VFO operation.

You do  have an  alternative to  pressing alphabetic  keys to  change the

current sources  for your  transceiver.   While the  source-selection  box  is

displayed, the  command letters  are displayed  across the  last line  of  the

screen.   Pressing a  function key accomplishes the same result as pressing an

alphabetic key.   Pressing  F2, for  example, selects  VFO B  as your  current

source.

Figure 8 shows the last-line display of source-selection command options.

------------------------------------------------------------------------------

1 A     2 B     3 M     4 S     5       6       7       8       9       10 Q

------------------------------------------------------------------------------

Figure 8.- Source-selection command options

Response to  your pressing the A, B, M, Q, or function keys is immediate.

The Spoiler  does not wait for you to press enter.  If you mistakenly enter an

unrecognized command  character, the  Spoiler will prompt you again for a good

command option.   If  you want to abort the source-selection operation, simply

press either Q or the enter key at the prompt.

FREQUENCY SELECTION

You have  two methods  of frequency  selection available  to you.    Your

primary method  is using a frequency-selection command to change the frequency

of any  source.  An alternate method permits you to tune your primary VFO from

the keyboard.

Frequency-Selection Command

The frequency-selection  command invoked by F2 provides you with the same

basic options  that you have with current-source selection.  You can choose to

enter a  frequency for  VFO A, VFO B, the current memory, or any memory stored

in the transceiver.

Figure 9  shows the options you are presented with in the command-options

box for the frequency-selection command.

�����������������������ͻ

�  Frequency Selection  �

� VFO A...............A �

� VFO B...............B �

� Memory..............M �

� Mem Num............## �

� Quit................Q �

�           _           �

�                       �

�                       �

�                       �

�                       �

�����������������������ͼ

Figure 9.- Frequency-selection source options

A powerful  feature of the Spoiler is that your entry of information into

an unselected  source does  not require  you to  select the  source before you

enter it.   If  you choose  to enter a new frequency into an unselected VFO or

memory, you do so without interrupting your listening to and transmitting from

your currently selected source.  If you are operating with VFO A or VFO B, for

example, you  can change  the contents  of all  memories and the alternate VFO

without missing  a single  word of  a net  or a  single baud  of  your  CW  DX

monitoring.

You may  select the source sources with the function keys as well as with

the alphabetic  keys.   Pressing F3,  as figure  9 shows,  selects the current

memory for frequency entry, just as pressing M does.

After you  have selected the source that you want to change the frequency

of, the  command-option box  prompts you  for the  new frequency,  as shown in

figure 10.   The  prompt verifies  the source  you have  selected (VFO  A, for

example, in  figure 10).   For  frequencies below  10 MHz,  you may  enter the

frequency in  either KiloHertz  or MegaHertz.   If you want to enter 7,001,010

Hz, you may enter either 700101 or 7.00101.  In addition, numeric precision is

assumed, so that your entering ????????

�����������������������ͻ

�  Frequency Selection  �

� VFO A...............A �

� VFO B...............B �

� Memory..............M �

� Mem Num............## �

� Quit................Q �

�           A           �

�  TR A: _              �

�                       �

�                       �

�                       �

�����������������������ͼ

Figure 10.- Frequency-selection prompt

If you  want to  abort the  frequency-selection command  at  the  prompt,

simply press enter.

If you  select a  memory channel  capable of split-source operation (from

channels 90  through 99),  you will  be prompted to enter frequencies for both

sources, as shown in figure 11.

�����������������������ͻ

�  Frequency Selection  �

� VFO A...............A �

� VFO B...............B �

� Memory..............M �

� Mem Num............## �

� Quit................Q �

�           A           �

�  RX 93: _             �

�  TX 93: _             �

�                       �

�                       �

�����������������������ͼ

Figure 11.- Frequency-selection prompt for split source

The prompt  for frequency of the receive memory appears first.  If you do

not want  to change the receive memory, simply press the enter key to bring up

the prompt  for frequency  of the  transmit memory.   Similarly,  if you  have

entered a frequency in receive memory and do not wish to enter a new frequency

in transmit memory, simply press the enter key at the TX prompt.  Pressing the

enter key at both prompts aborts the frequency-selection command.

A special  circumstance occurs  when a TS-440S memory is completely empty

(with no previous frequency or mode stored in it).

One of  the characteristic   of  the TS-440S  memory is that it can store

into an  empty memory  neither frequency  nor mode  without storing  the other

parameter.   Because  the  Spoiler  stores  frequency  and  mode  in  separate

operations, the program must store a "boilerplate" mode into a memory whenever

you store  a frequency.   This  mode is LSB.  If you enter a frequency into an

empty memory,  the LSB mode stored simultaneously in memory.  If you want some

other mode  for the  memory, you  can change  it quite  conveniently from  the

keyboard with the mode-selection command invoked by F3.

Tuning from the Keyboard

You may  tune the  frequency of  your currently  selected VFO by pressing

your UP  and DOWN  arrow keys.   Tuning  is inhibited  while you  are entering

command options

When you  use the  arrow keys  to tune  your currently  selected VFO, the

resulting change  in frequency  is always  applied  to  your  current  receive

frequency.   If you  have not selected split-frequency operation, it will also

alter your  transmit frequency.  If split-frequency operation is selected, the

transmit frequency remains unchanged.

If you  want to use keyboard tuning as extended receive increment tuning,

you should  enter the  same frequency  into both  VFOs and select split-source

operation.   Your transmit frequency will remain unchanged while you vary your

receive frequency.

MODE SELECTION

The mode-selection command invoked by F3 provides convenient selection of

one of  the six  operating modes  for either  VFO, the  current memory, or any

unselected memory.   All  of these  sources can  receive and  transmit in  the

following modes.

Lower sideband (LSB)

Upper sideband (USB)

Continuous wave (CW)

Frequency modulation (FM)

Amplitude modulation (AM)

Frequency-shift keying (for RTTY)

To select  a mode  for a  source, you  first select the source within the

command options box, as shown in figure 12.

�����������������������ͻ

�      Select Mode      �

� VFO A...............A �

� VFO B...............B �

� Memory..............M �

� Mem Num............## �

� Quit................Q �

�                       �

�                       �

�                       �

�                       �

�                       �

�����������������������ͼ

Figure 12.- Mode-selection source options

Your selection  is verified  in the mode-selection prompt that appears in

the box, as shown in figure 13.

�����������������������ͻ

�      Select Mode      �

� VFO A...............A �

� VFO B...............B �

� Memory..............M �

� Mem Num............## �

� Quit................Q �

�           86          �

�  TR 86 (LUCFAR): _    �

�                       �

�                       �

�                       �

�����������������������ͼ

Figure 13.- Mode-selection source options

The prompt  for mode  selection shown in figure 12 indicates that you are

selecting the mode for memory channel 86.  If you want to abort the selection,

simply press enter at the prompt.

If you select from memory channels 90 through 99 as your source, you will

be prompted  for the  mode of  both channels, as shown in figure 14.  Pressing

enter at  the receive-source  prompt leaves  the mode  for the  receive memory

unchanged, and  pressing  enter  at  the  transmit-source  prompt  leaves  the

transmit memory unchanged.  Pressing enter at both prompts aborts the command.

�����������������������ͻ

�      Select Mode      �

� VFO A...............A �

� VFO B...............B �

� Memory..............M �

� Mem Num............## �

� Quit................Q �

�           86          �

�  RX 91 (LUCFAR): _    �

�  TX 91 (LUCFAR): _    �

�                       �

�                       �

�����������������������ͼ

Figure 14.- Mode-selection source options for split source

A special  circumstance occurs  when a TS-440S memory is completely empty

(with no previous mode or frequency stored in it).

Because the Spoiler stores mode and frequency in separate operations, the

program must store a "boilerplate" frequency into an empty memory whenever you

store a  frequency.   This frequency 3.9956 MHz.  You may change the frequency

conveniently with the Frequency Selection command invoked by the F2 key.

INCREMENT TUNING

You may  use TS-440S  increment tuning  to alter  the receive or transmit

frequency of  your selected  VFOs or  memories.   Increment tuning, because it

affects frequencies  with memories  as a primary source, is more powerful than

keyboard tuning, but it has a more limited range.  You may tune above or below

your primary source frequency by only 1270 Hz (1.27 KHz).

You may  invoke increment  tuning with  the F4  key, or you may alter the

increment tuning frequency from the keyboard.

The Increment-Tuning Command

The increment-tuning command, invoked by F4, provides you with options to

apply  a   tuning  increment  to  your  current  receive  frequency,  transmit

frequency, or  both frequencies.   You  may set the increment frequency to any

value between  -1.27 KHz  and 1.27 KHz, or you may clear the increment (set it

to 0).  Figure 15 shows the options box for the increment-tuning command.

�����������������������ͻ

�    Increment Tuning   �

� RX Toggle...........R �

� TX.Toggle...........T �

� Clear...............C �

� Freq............�N.NN �

� Quit................Q �

�                       �

�                       �

�                       �

�                       �

�                       �

�����������������������ͼ

Figure 15.- Increment-tuning command options

The  results  of  toggling  receive  and  transmit  increment  tuning  is

displayed in  the lowest  partition of  the source-detail  area of the screen.

The possibilities  are No  Increment,  RX  increment,  TX  increment,  and  TR

Increment.

The source-detail  area of  the screen  shows the  frequency of increment

tuning even  when it  is not applied to receive and transmit frequencies.  You

may alter  this frequency  from the  keyboard or  the front panel at any time,

regardless of  whether it  is being  applied to a current frequency.  You will

notice that when you alter increment frequency from the keyboard, the position

of the  RIT control  on the  TS-440S no  longer corresponds  to the  increment

frequency; when  you next  alter increment  tuning from  the front  panel, the

position of the control will once more correspond to the increment frequency.

Whenever you  enter an increment frequency at the command-options prompt,

you will notice that the response to your entry is not immediate.  The Spoiler

must, in  fact, send  discrete commands  to the  TS-440S for each 10 Hz of the

increment, so full-range excursions may take up to 5 seconds (depending on the

speed of  your computer).   Clearing  the increment  (setting it to 0) occurs,

however, instantaneously.

Keyboard Increment Tuning

You may  use the RIGHT and LEFT arrow keys to alter the frequency of your

increment tuning.   Pressing  the keys, however, while a command option box is

displayed, has no effect

COPYING SOURCES

You may,  by using  the source-copy  command, copy  the contents  of  any

source into  any other  source.  You may also exchange the contents of any two

sources.   This command,  invoked by F5, provides you with a convenient way to

reorganize the frequencies and modes stored in your VFOs and memories.

The option box for the source-copy command is shown in figure 16.

�����������������������ͻ

�     Copy Sources      �

� Copy................C �

� Exchange............E �

� Quit................Q �

�           _           �

�                       �

�                       �

�                       �

�                       �

�                       �

�                       �

�����������������������ͼ

Figure 16.- Source-copy command options

If  you  select  the  copy  option,  you  are  prompted  for  origin  and

destination sources, as shown in figure 17.

�����������������������ͻ

�     Copy Sources      �

� Copy................C �

� Exchange............E �

� Quit................Q �

�           C           �

� From (A,B,M,##): A    �

�   To (A,B,M,##): _    �

�                       �

�                       �

�                       �

�                       �

�����������������������ͼ

Figure 17.- Copy option

You abort the copy operation if you press enter at either prompt.  If you

select the  Exchange option,  you are  provide similar  prompts, as  shown  in

figure 18.

�����������������������ͻ

�     Copy Sources      �

� Copy................C �

� Exchange............E �

� Quit................Q �

�           E           �

� Src 1 (A,B,M,##): 32  �

� Src 2 (A,B,M,##): _   �

�                       �

�                       �

�                       �

�                       �

�����������������������ͼ

Figure 18.- Exchange option

Both the  copy and  exchange options  handle the contents of single (non-

split) sources.   Your  entering memory number 99 as an origin or destination,

therefore, indicates  the receive  memory of  channel 99.    If  you  want  to

indicate the  transmit memory  of channel 99 you must append a T to the memory

number, as in figure 19.

�����������������������ͻ

�     Copy Sources      �

� Copy................C �

� Exchange............E �

� Quit................Q �

�           C           �

� From (A,B,M,##): 99T  �

�   To (A,B,M,##): _    �

�                       �

�                       �

�                       �

�                       �

�����������������������ͼ

Figure 19.- Transmit memory entry

MEMORY FILE MANAGEMENT

The 100  memory channels  in the  TS-440S provide  for a  great  deal  of

flexibility for most applications, but they may be insufficient for the varied

uses that  a single  transceiver may  be put to.  They are probably sufficient

for  most  people's  use  in  traffic-net  participation,  DX  monitoring,  SW

broadcast monitoring,  CB monitoring,  and  the  like,  but  they  are  likely

insufficient for  combinations of these activities.  Reorganizing and altering

100 memory channels is a tedious task that no one wants to do unnecessarily.

To help  you with  memory management,  the Spoiler lets you store TS-440S

memories in  computer disk  files.   With the file-management command, invoked

with the  F6 key,  you can  store all 100 memory channels to disk, restore all

100 channels from disk, and clear all 100 channels (setting them off).

Figure 20 shows the memory-management command option box.

�����������������������ͻ

�   Memory Management   �

� List................L �

� Open................O �

� New.................A �

� Clear XCVR..........C �

� Quit................Q �

�                       �

�                       �

�                       �

�                       �

�                       �

�����������������������ͼ

Figure 20.- Memory management options

The LIST  option displays the names of all TS-440S memory files stored on

disk (in  the current  drive and directory).  On disk, these files always have

an MRY  extension;  the  list,  however  shows  the  file  names  without  the

extension.   The list is displayed in the command-option box.  When the number

of files on disk exceeds the number of lines in the box, the display is paused

for your  reading a partial list until you press any key.  Figure 21 shows the

beginning and end of a list that is too long to fit into the option box.

�����������������������ͻ  �����������������������ͻ

�     List of Files     �  �     List of Files     �

�        CBRADIO        �  �        DEFAULT        �

�        TRAFFIC        �  �        SKEDS          �

�        CWNETS         �  � End of MRY file list  �

�        DXFONE         �  � Press a key . . .     �

�        SWL            �  �                       �

�        JACKS          �  �                       �

�        40CONTIN       �  �                       �

�        75CONTIN       �  �                       �

�        10SCAN         �  �                       �

� Press a key . . .     �  �                       �

�����������������������ͼ  �����������������������ͼ

Figure 21.- Initial and terminal displays of a file list

The OPEN  option reads  a file from disk into both Spoiler memory and the

memories of  the TS-440S.   The  option box  prompts you  for the name of your

file.  Note that when you initialize the spoiler for your TS-440S it reads the

contents of  the transceiver  memories into  a file called DEFAULT in order to

preserve the  memories for  you.   This prevents you from losing the data that

you already  have stored in the transceiver when you download other files into

the transceiver.

The file  that is  open is  continuously updated  by the  Spoiler as  you

change the memories of your transceiver.  New memory sources are automatically

saved as  you enter  them.   Also, when you leave the Spoiler to run any other

program, you  will find  when you  again run the spoiler that it automatically

opens the file you last opened before quitting.

The NEW  option writes  the currently opened file to disk, closes it, and

initializes a  new file with the same contents.  This option provides you with

a convenient  way to  edit the  contents of  an old file without automatically

storing the changes in the file.  The changes are stored only in the new file.

The CLEAR XCVR option lets you erase all of the memories in your TS-440S.

You may  want to  use this option whenever you wish to begin a new memory file

from scratch or you wish to maintain security about your operating frequencies

while other  operators are using your transceiver.  The option requires you to

enter a new file name; after the transceiver is cleared, new file by that name

containing zeroed  sources is  opened and  maintained on  disk.    Of  course,

thereafter you  may by  using the  OPEN command  load  your  transceiver  with

sources from any other file you have saved.

RUNNING A LOGGING PROGRAM

The F7  key provides you with a convenient way to run a program that logs

your QSOs  or SWL  observations.   The Spoiler  remains resident in memory, so

that returning to it takes a minimum of time.

Before you use F7, you should alter the batch file LOG.BAT to include the

name of your logging program.

When your  logging program  returns you to DOS, enter the EXIT command to

return to the Spoiler.

RUNNING A DIGITAL CONTROLLER PROGRAM

The F8 key provides you with a convenient way to run a program to control

RTTY, CW,  Packet, and  other such transmission.  The Spoiler remains resident

in memory, so that returning to it takes a minimum of time.

Before you  use F8,  you should  alter the batch file CLTR.BAT to include

the name of your logging program.

When your  controller program  returns you to DOS, enter the EXIT command

to return to the Spoiler.

TOGGLING THE MEMORY DISPLAY

The memories  currently displayed  by the  spoiler  are  toggled  between

numbers 0  through 54  and 55  through 99t  by your  pressing F9.    When  you

initially run  the spoiler,  the lower  numbered memories appear in the memory

display box.  Your pressing F9 displays the higher numbered memories, and your

pressing F9 again displays the lower numbers once more, and so on.

QUITTING THE SPOILER

Pressing F10  causes you  to exit  the spoiler  program, unload  it  from

memory, and return to DOS control of your computer.

USING THE 440 SPOILER

There is  only one task that you have to undertake to get the 440 Spoiler

working for  your radio-operating  pleasure.   You  must  connect  your  MSDOS

computer to  your TS-440S.  After your transceiver and computer are connected,

you are at your leisure to learn how to use the Spoiler by trial and error, by

reading, or by a most-efficient combination of both activities.

COMPUTER/TRANSCEIVER CONNECTION

Your simplest  way  to  physically  connect  your  transceiver  and  your

computer is  for you  to obtain an RS-232 interface box and integrated-circuit

chip set marketed by Kenwood.  You will also need a serial cable to connect an

RS-232 output  of your computer to the interface box.  The interface box, chip

set, and cabling will cost you approximately $100 retail.

Optionally, you  might want  to obtain  a switch so that your RS-232 port

for devices  other than  control of  your TS-440S  at the  flip of  a  switch.

Connectors and  people can  stand only  so much plugging and unplugging, but a

simple switch solves both problems.

A more-complex,  but much  cheaper, way  is for you to make up the cables

and build  an interface  box.  It is beyond the scope of this manual to detail

how the  equipment should  be constructed, but construction is a simple matter

if you  are familiar with digital technology.  In the do-it-yourself approach,

you can  expect to  save approximately $65 frugal dollars, but you'll probably

spend about 10 hours on construction and testing.

When you have the box and the cables in hand, plug them up.  Turn on your

transceiver, and run the Spoiler.

If something  is amiss in the hardware (the transceiver, the cabling, the

interface box, or the computer), the spoiler will tell you upon installation.

INSTALLING THE SPOILER

The Spoiler performs its installation procedure whenever it does not find

the file SPOILER.DEF in the current directory.

During installation,  the Spoiler  asks you  for the number of the serial

port that  you have connected your TS-440S interface box to.  The Spoiler then

proceeds  to   run  by  checking  your  communications  between  computer  and

transceiver and  reading all  of the  information stored  in your  transceiver

memories.   The Spoiler  stores the  information in a file called DEFAULT.MRY,

and it opens the file.

You will  quite likely  alter the  contents of  your  TS-440S  memory  by

exploring the  operation of  the Spoiler.  If you want to save the contents of

your transceiver  memories so  that you can restore them to be exactly as they

were before  you used  the Spoiler,  you'll want  to go  through the following

procedure that follows.

Issue a  Memory Management command by pressing F6.  At the command-option

prompt, enter  the NEW option by pressing the N key or pressing F3.  You'll be

asked to  enter a  file name  (2 through  8 alphanumeric  charcters  with  the

extension ".MRY"  assumed by  the program) for a new disk file for you to work

with.   The new file that the Spoiler produces will initially have exactly the

same contents  as DEFAULT.MRY,  and your can proceed with your installation in

the security  of knowing  that all  changes you  make to  TS-440S  memory  are

affecting only the transceiver and your new file.

If you  want to  restore the memories in the TS-440S to exactly what they

were before your exploration, you should perform the following procedure.

Open the  DEFAULT file  by pressing F6 and choosing the O (or F1) option.

Enter the file name DEFAULT at the prompt.  The Spoiler will load the original

memory values  into your  TS-440.   If you  don't want  to alter  the original

memory contents, you should then immediately quit the Spoiler by pressing F10.

Your transceiver  memories will  be restored to the same as they were when you

first ran the 440 Spoiler.

You'll want  to follow this procedure each time you work with the Spoiler

therefore if  you want  to save  your  original  memory  information  in  your

transceiver.   When you  feel comfortable  with your operation of the Spoiler,

you will  want to  alter the  memories extremely often, as the Spoiler permits

you to do at will.

COURSING THROUGH THE SPOILER

You should  initially consider  the Spoiler  a display  window  into  the

operation of  your TS-440S.  All operating parameters are displayed in the box

to the  upper-left of  your screen, and all memory information is presented in

the J-shaped  (largest) box  to the right and bottom of your screen.  When you

change any  operating parameter  (frequency, mode, split, RIT, XIT, source, or

memory) from  the TS-440S  front panel,  the Spoiler  updates its  display  to

reflect the change.

Later, you  should work  with the  Spoiler commands that are initiated by

your pressing  the function keys listed at the bottom of the Spoiler operating

screen.  Using the Spoiler, you'll find that you have little use for the front

panel of  your TS-440S  except for such "analog" functions as your controlling

volume, passband  tuning, antenna  tuning (if  your transceiver has an antenna

tuner installed, and the like.

The 440  Spoiler, combined  with the  operating  power  of  the  TS-440S,

provides you  with a  quantum leap in radio communications ability.  You'll be

able to operate many traffic nets simultaneously, and you'll quickly find your

DX tallies  among only  those possessed  by other  operators who are using the

Spoiler.   If you  are looking  for world  news from  international  shortwave

broadcasts during whatever international crisis the future brings, you'll find

it much  faster with  the Spoiler.   If  you're looking  for  music  broadcast

throughout the  world, you'll  find your  selection to be both wide and quick.

If you  want to  know what the CB citizenry has evolved into, the Spoiler will

help you find out with convenience that rewards your curiosity.

REGISTERING THE 440 SPOILER

As with all shareware, the 440 Spoiler is supported largely for those who

obtain registered  copies of  the software  and documentation.    The  QSO2000

project is  not Microsoft  Corporation, and  nobody in the project wants it to

become more than a cooperative endeavor among the avant garde communicators of

the world  so that  the world of amateur-radio communications can more rapidly

become better.

Several hundreds  of hours  of research,  trial-and-error,  and  software

development have  gone into  development of  the 440  Spoiler, and  before the

useful life  of the program is over several hundreds of hours will go into its

maintenance and  enhancement.   We urge you to consider the value that the 440

Spoiler represents  to you and to therefore register your copy of the software

by your  printing the  form provided below and mailing it with your remittance

to The QSO2000 Project.

Your registration of the 440 Spoiler brings you by return mail the latest

version of  the 440  Spoiler on  diskette and  a copy  of  the  professionally

written,  printed,  and  bound  440  Spoiler  Technical  Reference  Manual  (a

significant enhancement  and update  of the  material  in  this  documentation

file).   It also entitles you to free upgrades of the 440 Spoiler software and

updated documentation on disk for a year after your registration.  You will be

notified  of  the  upgrades  by  mail  for  the  entire  year  following  your

registration.

Shareware versions  of the  440 Spoiler  will be  maintained and enhanced

throughout the  useful like  of the package to a lesser extent than registered

versions are.   Rewards  for sharing  in the  resources  expended  by  further

development of the package will be significant.

Please distribute  copies of  the latest  shareware version  of  the  440

Spoiler to  other operators  as widely  as you care to distribute them so that

they are invited to share in our progress.  Please include all of the files in

the 440  Spoiler package.   You  may not, of course, distribute copies of your

registered version  of the  440  Spoiler  in  accordance  with  the  licensing

agreement on the form printed below.

Licensees for  a registered  version of  the Spoiler are welcome to write

the QSO2000  Project with  comments and questions.  Licensees are also welcome

to communicate  directly with  the authors  of the 440 Spoiler on the bulletin

board listed below,

Bulletin Board ECHO (Education)

806-741-1570

1200-2400 baud, 19200 baud USR-HST

Licensees are  automatically registered  on the  board, and  upon calling

they will be routed into the Wireless Conference, the home of the 440 Spoiler.

------------------------------------------------------------------------------

440 SPOILER REGISTRATION FORM

-----------------------------

Bill Brewer. K5KNC, Coordinator

The QSO2000 Project

5518 12th Street

Lubbock, Texas 79416

I hereby  apply for  licensed use of the 440 Spoiler software package and

enclose a  payment of  $39.00 for each licensed copy of the 440 Spoiler that I

am ordering.

I understand  that I  will receive  by return mail a diskette copy of the

most current  registered version  of  the  440  Spoiler  and  a  copy  of  the

professionally written,  printed, and  bound 440  Spoiler Technical  Reference

Manual.   I also  understand that I will receive software support by both mail

and through Bulletin Board ECHO (Education) for a period of one year following

registration and  that I may obtain free upgrades of the registered version of

the 440 Spoiler within a year of the date printed below.

I further  understand that  I will  be notified  of upgrades  as they are

available and  that I  may obtain  them by submitting the cost of shipping and

diskette to the QSO2000 project.

I will  not distribute any copy of my licensed, registered version of the

440 Spoiler  that I  receive to  persons who  do not  reside in  my  immediate

household.

---------------------------------------

Signature

---------------------------------------

Name (please print)

---------------------------------------

Street, P.O Box

---------------------------------------

City, State  ZIP

REMITTANCE: For _____ copies of registered versions at $39.00 each

A total of ________

Please make checks and money orders payable to BILL BREWER, K5KNC

------------------------------------------------------------------------------

JOINING THE QSO2000 PROJECT

You are  welcome to  join The  QSO2000 Project  in more  ways  than  your

registering for  your copy  of the  440 Spoiler.   We  will be happy to review

software that  you have  written that  is related to Amateur Radio and to work

with you in assessing it its role in the project.