440 SPOILER USER'S MANUAL

VERSION 1.01

There is  only one task that you have to undertake to get the 440 Spoiler

working for  your radio-operating  pleasure.   You  must  connect  your  MSDOS

computer to  your TS-440S.  After your transceiver and computer are connected,

you are at your leisure to learn how to use the Spoiler by trial and error, by

reading, or by a most-efficient combination of both activities.

COMPUTER/TRANSCEIVER CONNECTION

Your simplest  way  to  physically  connect  your  transceiver  and  your

computer is  for you  to obtain an RS-232 interface box and integrated-circuit

chip set marketed by Kenwood.  You will also need a serial cable to connect an

RS-232 output  of your computer to the interface box.  The interface box, chip

set, and cabling will cost you approximately $100 retail.

Optionally, you  might want  to obtain  a switch so that your RS-232 port

for devices  other than  control of  your TS-440S  at the  flip of  a  switch.

Connectors and  people can  stand only  so much plugging and unplugging, but a

simple switch solves both problems.

A more-complex,  but much  cheaper, way  is for you to make up the cables

and build  an interface  box.  It is beyond the scope of this manual to detail

how the  equipment should  be constructed, but construction is a simple matter

if you  are familiar with digital technology.  In the do-it-yourself approach,

you can  expect to  save approximately $65 frugal dollars, but you'll probably

spend about 10 hours on construction and testing.

When you have the box and the cables in hand, plug them up.  Turn on your

transceiver, and run the Spoiler.

If something  is amiss in the hardware (the transceiver, the cabling, the

interface box, or the computer), the spoiler will tell you upon installation.

INSTALLING THE SPOILER

The Spoiler provides for installation when you first run it.  It asks you

for the  number of  the serial port (1 or 2) your TS-440S is connected to, and

then then  it reads  all 110  memories in the transceiver.  The Spoiler writes

these memories  to a  disk file  named DEFAULT.MRY  and makes DEFAULT.MRY your

currently open file.

During installation,  the Spoiler  writes an  initialization  file  named

SPOILER.DEF.   This file  stores your  serial port  number and the name of the

disk file  that is  currently opened.   If  you ever  want  to  reinstall  the

Spoiler, simply delete SPOILER.DEF from your current directory (or diskette).

You will  quite likely  alter the  contents of  your  TS-440S  memory  by

exploring the  operation of  the Spoiler.  If you want to save the contents of

your transceiver  memories so  that you can restore them to be exactly as they

were before  you used  the Spoiler,  you'll want  to go  through the following

procedure that follows.

Issue a  Memory Management command by pressing F6.  At the command-option

prompt, enter  the NEW option by pressing the N key or pressing F3.  You'll be

asked to  enter a  file name  (2 through  8 alphanumeric  charcters  with  the

extension ".MRY"  assumed by  the program) for a new disk file for you to work

with.   The new file that the Spoiler produces will initially have exactly the

same contents  as DEFAULT.MRY,  and your can proceed with your installation in

the security  of knowing  that all  changes you  make to  TS-440S  memory  are

affecting only  the transceiver and your new file.  If you want to restore the

memories in the TS-440S to exactly what they were before your exploration, you

should perform the following procedure.

Open the  DEFAULT file  by pressing F6 and choosing the O (or F1) option.

Enter the file name DEFAULT at the prompt.  The Spoiler will load the original

memory values  into your  TS-440.   If you  don't want  to alter  the original

memory contents, you should then immediately quit the Spoiler by pressing F10.

Your transceiver  memories will  be restored to the same as they were when you

first ran the 440 Spoiler.

You'll want  to follow this procedure each time you work with the Spoiler

therefore if  you want  to save  your  original  memory  information  in  your

transceiver.   When you  feel comfortable  with your operation of the Spoiler,

you will want to alter the memories often, as the Spoiler permits you to do at

will.

COURSING THROUGH THE SPOILER

You should  initially consider  the Spoiler  a display  window  into  the

operation of  your TS-440S.  All operating parameters are displayed in the box

to the  upper-left of  your screen, and all memory information is presented in

the J-shaped  (largest) box  to the right and bottom of your screen.  When you

change any  operating parameter  (frequency, mode, split, RIT, XIT, source, or

memory) from  the TS-440S  front panel,  the Spoiler  updates its  display  to

reflect the change.

Later, you  should work  with the  Spoiler commands that are initiated by

your pressing  the function keys listed at the bottom of the Spoiler operating

screen.  Using the Spoiler, you'll find that you have little use for the front

panel of  your TS-440S  except for such "analog" functions as your controlling

volume, passband  tuning, antenna  tuning (if  your transceiver has an antenna

tuner installed, and the like.

The 440  Spoiler, combined  with the  operating  power  of  the  TS-440S,

provides you  with a  quantum leap in radio communications ability.  You'll be

able to operate many traffic nets simultaneously, and you'll quickly find your

DX tallies  among only  those possessed  by other  operators who are using the

Spoiler.   If you  are looking  for world  news from  international  shortwave

broadcasts during whatever international crisis the future brings, you'll find

it much  faster with  the Spoiler.   If  you're looking  for  music  broadcast

throughout the  world, you'll  find your  selection to be both wide and quick.

If you  want to  know what the CB citizenry has evolved into, the Spoiler will

help you find out with convenience that rewards your curiosity.

DETAILED OPERATION

The Spoiler includes features to tremendously aid your operating work and

play with  the TS-440S,  and it  would be  very boring  for you  to read about

features that are of use to someone else.  The 440 Spoiler Technical Reference

Manual provides a description of the features for you to reference as you need

to use  them.  The technical reference manual, though not suitable for bedside

reading for  anyone without  insomnia, is  your informational  friend during a

time that you have needs.

SUPPORT

As with all shareware, the 440 Spoiler is supported largely for those who

obtain registered  copies of  the software  and documantation.    The  QSO2000

project is  not Microsoft  Corporation, and  nobody in the project wants it to

become more than a cooperative endeavor among the avant garde communicators of

the world so that the world can become better, faster.

You are  welcome to  write with  comments and  questions to the following

address.

Bill Brewer. K5KNC, Coordinator

The QSO2000 Project

5518 12th Street

Lubbock, Texas 79416

If you are a registered user of the Spoiler, you may also telecommunicate

your comments  and questions  to the project at the home bulletin board of the

Spoiler.  To access the bulletin board, use the following information.

Bulletin Board ECHO (Education)

Wireless Conference (conference number 4)

806-741-1570

24 hours, 7 days a week

1200-2400 baud and USR/HST 19200 baud

The latest  shareware release of the 440 Spoiler is always available from

the Wireless Conference of ECHO (Education).