SB-829

TR-7800 Loss of Memory                       7-31-80

Intermittent loss of memory may be corrected by adding one resistor at the

microprocessor on the control unit.

Procedure:

1.   Remove bottom cover, 4 screws.

2.   From the Control unit X53-1180-xx unplug the rear mounted connectors

8-11.

3.   Remove 6 screws from the board.

4.   Swing the unit forward.

5.   Install a 470K ohm 1/8 watt or larger resistor from connector 11, "UP"

pin, to ground.  (microprocessor pin 35, PA2 port).  Space is limited:  Be

certain not to create a solder bridge when installing the resistor.

6.   Reverse steps 4-1 for reassembly.

JEB/yn

(c) 73180TKC