DOCUMENTATION FOR "INTMOD,Vers.6.0"

This program had it origins in a file put up on GEnie in 1986 by

David Kozinn.  It was titled "INTERMOD.BAS" and was file no. 719.

Since then several of us in the SouthEast who have some technical

responsibility for 2 meter Amateur Radio repeaters, have found that

original program to be functional, but far too tedious for much

practical utilization.

We began modifying it, with the goal of making it more useful for

"real world" applications.  Specifically, when your local repeater

is overwhelmed with interference that seems to be of the intermod-

ulation type, your first need is to identify the other signals that

are mixing to cause your problems.  Instead of taking shots blind-

folded, as it were, guessing who or what other signals MIGHT be

involved - and then examining the results to find those calculated

results CLOSE to your repeater input, then do a series of "cut and

try" changes -  there HAD TO BE a better way!!!!

The first changes were made by WD4HLZ, J. G. Walker, to allow the

user to limit the portion of spectrum from which any calculated

results would be reported to the user.  This saves printer paper

among other things.  But the user is still having to make shrewd

(hopefully) guesses as to the possible mixing stations.  This isn't

too hard to do for PART of the input - at least one signal is LIKELY

to be your own repeater output.  But one's imagination (and the sky)

is the limit for the remaining inputs.

Through collaboration between NG4S, Tommy Walker and myself, W4PFZ,

Olin McDaniel - we added what we feel is the much needed feature.

The program will "back calculate" FROM the Intermod product (which

is our interfering signal) and one or two known or PROBABLE inputs,

to give you the other possible signals.  Among these you may find

the culprit you seek.  Only then can you sensibly determine the

proper cure; filters and where to install them, etc.

In order to minimize the useless results, our revision suppresses

all negative numbers - I've never encountered any rf signals on a

negative frequency!  But we did not attempt to clean up the original

section which still reports negative values.  (Left enough for more

revisions.)

To access the new "back calculation" feature, choose option 2 from

the opening menu. (With either the original approach, option 1, or

this new option, you may choose either to direct the answers to the

screen or to the printer.  Incidentally the original option 1 allows

many, many guesses to be inputted on any run.  But it still only

examines a maximum of 3 at a time, for either 3rd order or 5th order

solutions.)  Option 2 also only considers a maximum of 3 input

signals, but it utilizes a total of 80 different possibilities for

combining and forming the offending interfering signal.  As far as

we can determine, these are FINALLY all the possibilities and hope-

fully are all valid.  (Admittedly many of these 5th order combin-

ations require some massive signal levels to reasonably expect any

troublesome mixing, but perhaps they do exist some places out

there.)

We hope that by providing this upgraded program, it will assist

cleaning up some of the pollution within the r.f. spectrum.  Any

feedback will be welcomed.  Send to GEnie address: O.MCDANIEL

#############################################################################