HAPNDRSI Device Driver for the DRSI PC-Packet Adapter

by Jack Botner, VE3LNY             August 15, 1990

(C) Copyright HAPN Hamilton and Area Packet Network 1990

Jack Botner                        H. A. P. N.

222 Divadale Drive                 Box 4466, Station D

Toronto, Ontario                   Hamilton, Ontario

Canada   M4G 2P7                   Canada   L8V 4S7

Compuserve: 73327,176 (private mail only, please)

Introduction

------------

This device driver allows the DRSI PC-Packet Adapter to operate with

the HAPN-1 software. The software developed to support the HAPN-1

adapter is very user-friendly and provides a high level of function.

In the HAPN implementation, all Level-1 (hardware) support functions

are done by the DOS device driver, making it possible to support

different adapters by changing only the device driver. Higher-level

functions (AX.25, KISS, etc.) are provided by separate modules that

are independent of the hardware that is being used.

This driver is installed as a DOS device driver by placing an entry in

CONFIG.SYS. It takes over all the hardware functions related to the

adapter and provides an application program interface for accessing

the packet queues and other adapter functions.

To use this device driver with MSYS, a module named HAPNKISS.COM is

required to provide the KISS interface between MSYS and the device

driver. Alternatively, the HAPN AX.25 Program Library (available from

HAPN) may be used to provide the usual AX.25 communications support.

Installing HAPNDRSI.SYS

-----------------------

HAPNDRSI.SYS is installed by placing a device entry in your boot disk

CONFIG.SYS file. For floppy disk only systems, a copy of HAPNDRSI.SYS must

be placed on the boot diskette. The CONFIG.SYS entry appears as follows.

All command line parameters appear within square brackets, which means

that they are optional. The default values are satisfactory for most

installations. If the parameters are coded, they must not contain any

blanks between the parameters.

device=[d:\path\]hapndrsi.sys [base=bbb][,irq=i][,ttsiz=z][,cts0=xx]

[,duplex0=yyy][,cts1=xx][,duplex1=yyy]

[,rxbuffers=rr][,txbuffers=tt]

[,baud0=nnnn][,baud1=nnnn][,pri=y]

[,tim=alt]

where bbb is the adapter 8530 base address (in hex). The default

value is 300. This value must match the setting of sw1-1.

i is the IRQ (interrupt) level. Values may be 0 through 7, and

the default value is 2. This value must match the setting

of sw1-3 through 8.

rr is the number of Rx buffers to allocate. Minimum

number is 5, maximum number is 25, default number is 10.

tt is the number of Tx buffers to allocate. Minimum

number is 5, maximum number is 30, default number is 15.

xx is the clear-to-send delay for each port in timer ticks (55 ms).

Values are 0 to 99, with default value 0.

yyy is hdx or fdx (only 1st letter scanned). Default is hdx.

z is the size of the trace table in kbytes. May be between

1 and 9. The default value is 4. Set it smaller to conserve

memory and larger to do datastream debugging.

nnnn to specify the baud rate when the driver is used with an

application that is unable to set up the baudrate. The

default value is 1200.

pri=y tells the driver to change the priority on the 8259 interrupt

controller so that the irq level specified for this adapter

has the highest priority. Don't use this unless you understand

what you're doing. Mainly used when interrupt overruns and

underruns are a problem.

tim=alt tells the driver to use int 1c (hex) for the timer tick

interrupt instead of int 8. Int 8 is generally more reliable

when programs are run that use the timer tick interrupt but

do not share it properly; however in some cases it may be

desired to use int 1c instead.

In its default configuration, HAPNDRSI.SYS uses 25K of memory. The amount

of memory depends on the number of Rx and Tx buffers allocated, and the

size of the trace table. For example, if you specify 8 Rx and 10 Tx

buffers, and a 1K trace table, 20K of memory will be taken. If you're

short of memory try using the minimum buffers but performance may suffer

badly at high packet volumes.

Example of CONFIG.SYS entry with no command line parameters:

device=hapndrsi.sys

or

device=c:\msys\hapndrsi.sys

Example of CONFIG.SYS entry with command line parameters to minimize

memory requirements and use different irq interrupt:

device=hapndrsi.sys irq=7,rxbuf=5,txbuf=5,ttsiz=1

Caution

-------

This software is currently at BETA test level. It probably contains

some bugs. If you experience a problem with it, try to describe the

environment and symptoms as thoroughly as possible and let me know about

it. I'll try to fix it if I can.