Using Packet-GOLD with

with Multi-Mode controllers.

INTRODUCTION

Operating the PK-232 or HK-232 in modes other than Packet requires

some special attention to parameters.  We recommend careful reading of

the AEA manual prior to using any operating mode (including packet) to

reduce the possibility of misunderstanding and frustration.

Considerable improvements to non-packet operation have been added

with Packet-GOLD 1.21. For example, we now have a split screen, to show

text that has been released to the TNC. This appears at the top of the

screen. You can Zoom In/Out using the [Alt-Z] key.  The size of this

window is set in the Monitor Settings area of the setup menu.

AMTOR operation has been completely revised. You now have status

information about whether your station is the Sending or Receiving

station, whether RQ has been sent (for retransmission), and if your

TNC is in the IDLE state, or is handling traffic.  Also, you can end

your AMTOR session using [Alt-F7] and selecting a method. Check out

AMTOR (if you have the PK232 or equivalent), it's a great mode for

low power, reliable qsos.

Switching Modes:

To change to a non-packet mode, all that is absolutely required is to

type the mode name on the command/text entry line, and hit the CMD key [F10].

This will indeed shift the TNC into another operating mode.

Alternatively, use [F10] without a parameter name on the command line.

This will bring up a "pick list" of .TNC files. Highlight the TNC filename

corresponding to the desired mode and hit enter. You may use the TNC file

editor (in the ALT-S menu area) or any ASCII text editor to modify or

create TNC files with the parameters needed for each mode.

Parameters should be checked in the other modes to assure that the TNC

will operate as expected.  Some of these parameters will be suggested in

the text that follows.

The Packet-GOLD software works well in these other modes, but it can be

crippled by improperly set parameters.  If you are having difficulty with

non-packet modes, you should check the relevant parameters.  As an

alternative, if all else fails, RESET the TNC as follows:

1.    Enter Direct Terminal mode using [Alt-F3].

2.    At the "cmd:" prompt, type "RESET" followed by the [Enter]

key. This will cause the TNC to clear all buffers, and

select default parameters. If this doesn't work, turn off

the TNC and turn it on again, and go to step 3.

3.    After the reset command, the tnc is waiting for you to

depress one or more asterisks (*) in order to "autobaud"

and establish a communications link with the computer.

Autobaud success is evidenced by the opening ROM

message naming the TNC, and ultimately presenting you

with another "cmd:" prompt.

4.    Leave the Direct Terminal Mode using [Alt-F3].

At this point, your callsign is entered as MYCALL, the

TNC is placed into HOST mode, and the Network View

screen will be presented.

5.    Hit the command key [F10] and select STARTUP to load

the startup.tnc parameters. This will include the

EAS ON (echo as sent) so that you can see what is

being transmitted.

Basic Operation:

First, set Echo as Sent to ON. This is imperative!   EAS ON [F10]

or put the EAS ON command in your startup.tnc file. The program echoes

characters to the screen as they are sent (in Non-packet modes) so without

the EAS (Echo As Sent) command ON, you won't see what's being sent.

All of the non-packet modes supported by Packet-GOLD use essentially

the same keystrokes. This makes it easy to remember what to do when

using these other modes.  All modes use WORDOUT which means you can

edit a word that is misspelled as long as you do not hit a [space] key.

After the [space] key, the text is either sent immediately, or queued

up for transmission, and is no longer available for editing.

The [ESC] key is the transmit/receive TOGGLE.  This is an important

key to remember in non-packet modes. It operates by placing a toggle

in the outgoing text buffer to unkey (or in ARQ mode, to send the

changeover characters).

You type ahead, and what you type will go into a transmit buffer

and get sent when possible. If your station is not keyed down and

transmitting, the text is just queued up for sending.  If you have

already keyed your transmitter (hit the [ESC] key to transmit) the

text is sent and echoed to your screen as it is being transmitted.

AMTOR:    ARQ: In this mode, the [ESC] key is used as the changeover

character. The program inserts the changeover (+?) sequence

into the outgoing text buffer at the next position in

the outgoing text buffer.  In this way, you can "queue" up

a changeover by typing (or sending a file of) text and then

depressing the [ESC] key.

To END an ARQ session cleanly, type just "R" (for Receive)

on the text entry line, and hit [F10]. You can also use the

[Alt-F7] key to initiate a "disconnect" similar to packet.

FEC:      The [ESC] key is used to Key down/up in this mode. It is

not a changeover sequence but rather, an indication that

the transmitter should key or unkey.  When AMTOR is showing

as the mode, and you type a few words or lines, then hit

[ESC] to key down, your text will be transmitted in FEC

mode.

Others:   In BAUDOT, ASCII, MORSE the meaning of the [ESC] toggle

is shown on the screen as a <<K>> (key down) or <<U>> (key

up pending) in the status line.  In AMTOR, the meaning

is similar, but there is no status indication. You will

have to use the [ESC] key carefully. When you hit it,

the changeover (or unkey in the case of FEC) is inserted

but not shown explicitly.

We have found that DIDDLE ON causes problems in ASCII mode

on some version of the AEA ROM. If you experience problems

in ASCII, set DIDDLE OFF.

Making contacts:

BAUDOT:   Enter the Baudot mode using the BAUDOT.TNC file or

by just typing BAUDOT [F10].

Set DIDDLE ON if you want to make it easy for other

stations to tune in your signal. DIDDLE will send

mark/space tones without actually affecting the remote

receiver/printer. The PK232 sets DIDDLE OFF by default,

meaning your station simply transmits a steady SPACE

tone when no characters are being transmitted.

Note: DIDDLE is not available on early AEA ROM releases.

Set the baud rate and shift using the "Fn" keys

according to the suggestions in the AEA manual.

Use (1) the "Brag" file facility or (2) the File transfer

facility for TEXT file transmission or (3) the Alt-N key

assignment, or (4) the cut/paste buffer editor as a

source for your CQ message or simply type your CQ

"on the fly" as you're keyed down.

Key down using [ESC], send your message or file, then

hit [ESC] again. The "unkey" command is queued up and

will take your system off the air when the buffer

is empty.

When another station sends text, you will see it on

your screen. You can "type ahead" any responses as you

think of them. They will not be sent until you hit the

[ESC] key to toggle the transmitter to the ON state.

When you have finished typing your transmission, hit the

[ESC] key. The text you are sending will finish transmitting

and your station will then unkey.

Usage of the [Alt-n] messages is a great idea for BAUDOT

and other modes as a place to store short lines, like

THANKS FOR NICE QSO, 73 73 73   SK DE WA4EGT WA4EGT KKKK

AMTOR:    You can send a CQ using FEC. When the AMTOR mode is shown,

and you are not "connected" to another station (not in ARQ)

you can type a CQ message, hit the [ESC] key to transmit

and your station will automaticaly go into FEC mode to send

your CQ.  Hit [ESC] a second time to unkey (you will see no

special indication that an "unkey" is in progress, but

it will occur after your text is transmitted).

If you include your SELCAL in your CQ, another station can

use ARQ to "connect" to your station.

To get to another station, type ARQ followed by the SELCAL

and hit [F10] key.

Use the [ESC] key to switch from Transmit to Receive.

To "disconnect" and turn off your transmitter, type

R [F10] or AMTOR [F10] (the former is preferred).

MORSE:    Set the morse speed and send a CQ. Hit [ESC] and your

station will key down, and start sending the CQ. Hit [ESC]

again and your station will unkey after sending the text.

SIAM:     If you hear a signal and want to "lock" on to it, use

the SIAM key [Alt-F5].

The SIAM key toggles to an "OK" key while the TNC analyzes

signals. If you agree with the analysis, a depression of

the SIAM key (now the OK key) will "lock into" that mode,

baud rate, etc.  If there is no analysis yet, hitting the

OK key will produce an error response, trying to tell you

that no mode has been guessed, so OK is premature.

Be sure to hit the CTRL key by itself when in NON-Packet modes. It will

display a line of commands that can be useful in your non-packet operation.

For examle, the ARQ key can be used if you want to type in just the SELCAL

and then hit the ARQ function key combination. Or, you can type ARQ SELCAL

and hit [F10].  The function keys are "short cuts" for your use.

Using the Callsign/Name log in NON-Packet modes:

Some other keys of interest: The [Alt-C] key in all modes will allow

you to enter a callsign for the session.  For non-packet modes, since

callsigns are not necessarily "known" to the TNC, you have to enter them

manually.  Using [Alt-C] you can "force" a callsign to anything. At that

time, the program will lookup information it may have about that callsign

including the name of the remote operator.

If a callsign is showing, you may use the [Alt-N] key to add/edit the

name of the remote operator. This is helpful for future reference. You

can also edit the "QSL card" information using [Alt-E] once the callsign

is shown.

The [Alt-P] key shows the results of "previous modes" such as packet.

This can be useful if you wish to "cut/paste" text that you might have

received from a packet BBS into a RTTY contact.  When you leave a

non-packet mode (or the direct terminal mode) all text is sent to the

previous connects screen.  In this way, you can change modes and have

access to text received/sent in other modes.  You could receive an

ARRL bulletin, or a NAVTEX bulletin, switch to another mode, select

the text and send it using the new mode.

Summary:

To change modes, we suggest that you use TNC files rather than

just typing in the mode name and hitting [F10].  This will assure

that relevant parameters are properly set.

Differences exist between HF and VHF operation. You might create

a VHFAMTOR.TNC and HFAMTOR.TNC file, or VHFBAUDO.TNC and HFBAUDOT.TNC

files, etc., depending you what you do routinely.  This way, all that

you do is hit [F10], and pick the right TNC file from the "pick list"

presented to you. Look over the TNC files we've provided.

For CQ messages, you can edit the [Alt-N] key assignments and/or

setup .BRG files in the packet directory with your CQ message. You can

also use the TextOps function (F5) to edit a CQ message in the "cut/paste"

editor buffer. This message can be sent numerous times by simply

hitting [F5] and selecting "copy to current session."  Then hit [ESC]

to start transmitting, and when the text begins going out, hit [ESC]

again to queue up the unkey command.

A note about FAX

The only mode NOT supported by Packet-PLUS and Packet-GOLD is FAX,

due in part to the "SWL" nature of FAX, and in part to the availability

of more appropriate hardware and software for this mode.  The Black/White

discrimination offered in the AEA product does not support the multi-level

resolution inherent in many FAX transmissions (e.g. photographs).  We have

tested other programs that offer 16-level resolution, using both dithering

and color substitution for the various levels, and have found the resulting

"pictures" to be excellent.  The PK-FAX program is adequate for black and

white drawings (weather maps).