APPENDIX D

MODIFICATIONS TO TNC-2 DATA CARRIER DETECTOR (DCD) CIRCUIT

These  modifications are to allow TNC-2 modem operation with

a narrow IF filter on HF.

NOTE! For 1200 baud HF work on 10 meters,  the normal 2.4

KHz  SSB filter constitutes a "narrow"  filter  for

the demodulator.

There are three objectives to these modifications:

1.  Provide  threshold  control for the DCD  circuit.   This

allows  the  operator  to  adjust  the  demodulator   to

compensate for the DCD threshold shift which accompanies

bandwidth limiting the audio fed to the demodulator when

a narrow filter is used in the radio.   This also allows

the  operator to adjust the DCD threshold for  operation

at  1200 baud on VHF FM to prevent excessive DCD falsing

when using unsquelched audio.

2.  Add  hysteresis  to the  Data  Carrier  Detector.   This

reduces  the DCD's sensitivity to noise.   It does  this

WITHOUT  DEGRADING DCD ACQUISITION TIME for a valid data

carrier.  It also prevents short glitches from appearing

at the DCD output on attack and release.

3.  Add DCD hang time on release.  This is to keep DCD valid

through a short multipath hit in the middle of a packet.

Without  this,  a TNC watching the DCD line  waiting  to

transmit  will  key  up  before  the  other  station  is

finished  sending its packet.   Currently,  this happens

very  frequently on HF  packet  channels.   Again,  this

benefit is gained WITHOUT DEGRADING DCD ACQUISITION TIME

for a valid packet.

The  MFJ-1278 is currently the only  commercially  available

TNC  I  know of for HF packet use which has a modem that  is

truly  optimized  for  the  mode  in  this   respect.    The

modifications given here will upgrade the modem in any TNC-2

or  clone (including MFJ-1274) which has a 2211  demodulator

to the level of DCD performance of the 1278 modem.

Part  numbers  referred  to are for the  original  TNC-2  as

produced (briefly) by TAPR.   I have not reviewed schematics

of  clones produced by all manufacturers so I cannot be sure

that  these numbers will be correct in all cases.   Since  I

have seen information on the MFJ series,  I can say that the

numbers  are ok for a 1270 and 1274.   If you are  in  doubt

about part number correspondence,  obtain a schematic of the

original TNC-2 as produced by TAPR.   Comparison between the

TAPR  schematic and the schematic of your TNC should resolve

any differences.  The TNC I used to verify this modification

was  a 1274 which I use primarily on HF packet.   Since  the

physical  layout of the modem area of the 1274 is  different

that  the layout of the original TNC-2 or  "pure"  unaltered

clones,  I  have  avoided giving specific physical  location

information in the modification procedure.

Any  manufacturer of the TNC-2 can feel free to  incorporate

this  change  into  their  hardware  if  they  wish  without

incurring any obligation to myself or TAPR.   I am available

to answer any quetions in this area.

NOTE! Do NOT use TNC-2 firmware earlier than version  1.6

with this modification.

Firmware   prior  to  V1.6  has  no  facility   for

detection  of  a DCD fault condition and  therefore

cannot warn you when the threshold control has been

improperly set.

STEP BY STEP MODIFICATION

1.  If  you  have  a  TNC-2 or clone  (except  MFJ-1274  and

1270B), and have not  already  removed the MF-10  filter

and  associated header parts,  do so at this time.   The

reason  for  this is that the operation of  this  filter

circuit is marginal in the  TNC-2.   This has the effect

of drastically reducing the modem dynamic range.

Removing  this  filter  is a  very   easy  modification.

Simply remove both the  MF-10 and  the header associated

with the MF-10.   Then  under the  board solder a jumper

between pins 1 and 8  of  the header  socket.   Removing

the  MF-10 also unloads the -5 volt  supply,   improving

its regulation and reducing  the noise  generated by the

charge pump circuit.

There   is  ABSOLUTELY   NO   PERFORMANCE   PENALTY  for

removing  this filter.

2.  Replace  C35 and C42 with 0.01  microfarad  caps.   This

step  is  not  necessary or possible in a  MFJ-1270B  or

1274.

3.  Remove  the 470 K resistor at R73.   Be careful  not  to

damage  the circuit board pads or traces as they will be

needed later in the modification.

4.  Remove CR13.

5.  Replace R70 with a 47 K resistor.

6.  Lift  the  cathode end of CR15 from the  circuit  board.

Install a 47 K resistor in series with CR15.  Solder one

end  of  this  resistor to the hole  vacated  by  CR15's

cathode end.   Solder the other end of this resistor  to

the cathode end of CR15 above the circuit board.

7.  Replace R74 with a 4.7 K resistor.

8.  Form  a parallel network consisting of a 180 K  resistor

and a 0.01 microfarad cap.  Make this network as compact

as  possible  as  it  will have to  fit  underneath  the

circuit board.   Solder this network in place under  the

board.   One end goes to pin 3 and the other to pin 6 of

the 2211 socket (U20).

9.  Replace R38 with a 100 K resistor.

10. Above  the  circuit  board,  using  leads  as  short  as

possible,  install  a  470  microfarad,  10  volt  rated

electrolytic  capacitor  between the -5 volt pad on  the

tuning  indicator  connector  (J3,  pin 1)  and  ground.

Connect  the positive  lead to ground.    Remember  that

this  cap  will have  to  clear the cabinet so  position

it as  near  the board as possible.

11. Under  the circuit board,  solder a 2.2  microfarad,  16

volt  rated electrolytic capacitor from the junction  of

R38 and R42 to the -5 volt supply.   The negative end of

this cap goes to the -5 volt supply.

12. Under  the  circuit board,  solder one end of a  22  ohm

resistor to the junction of R38 and R42.   The other end

goes to pin 5 of the 2211 (U20).

13. On  the front panel,  Mount a miniature  100  K,  linear

taper  potentiometer  (yes,  there  really IS  room  for

this).   This will be the DCD threshold control. It will

be  used  to  set the DCD trigger point  to  the  proper

value.

14. Solder a wire from the wiper of the 100 K pot to the pad

vacated  by R73 which connects to C45 and pin 3  of  the

2211 (U20).

15. Solder  one end of a 27 K resistor into the pad  vacated

by  R73 which connects to the -5 volt supply.   Solder a

wire from the other end of this resistor to one of the 2

remaining leads from the 100 K pot.   Use the lead  that

is  set  to  zero  resistance  when  the  shaft  of  the

potentiometer is turned fully CCW.

When  operating a TNC with a DCD threshold control,  set the

control  so  that  the DCD LED on the  front  panel  flashes

occasionally  when there is no signal present.   The  "false

DCD" duty cycle should be approximately 20 percent.

When operating VHF FM with the radio squelched, the DCD will

not  false.   If you MUST operate with the  radio  squelched

(thus  incurring the penalty of the additional delay time of

the squelch circuit),  set the threshold fully clockwise  as

described below.

The  audio  bandwidth of some VHF FM radios is so wide  that

the  DCD will not false regardless of the threshold  control

setting.   This will almost always be true when the audio is

obtained ahead of the radio's squelch controled stage before

deemphasis.   For these radios simply turn the control fully

clockwise.   This sets the DCD to maximum sensitivity.   DCD

operation will not be impaired.

This completes the TNC-2 modem modification.

73 Eric Gustafson, N7CL

EOF...