********************************************************************************

SET208.EXE

A Utility for setting up your TNPLUS version 2.08

software prior to programming

By William A. Beech, NJ7P

NJ7P@NJ7P.AZ.USA.NA

*************************************

This utility is specifically designed to assist NodeOps in setting up TNPlus

version 2.08 prior to programming.  It is menu driven and simple to use.  Be-

fore invoking SET208.EXE, it is assumed you have the TN208.HEX or TN208.ROM

files you will be working with in your current directory.    The commands for

bringing up SET208.EXE are:

(For monochrome monitors)  SET208 -b

(For CGA color monitors)   SET208

SET208.EXE will then load and show the main menu:

IMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMM

;    SETPLUS 2.08 (C) by Bill Beech, NJ7P   ;

;                                           ;

;         (1)   Load Binary ROM image       ;

;         (2)   Load HEX ROM image          ;

;         (3)   Edit Parameters             ;

;         (4)   Save Binary ROM Image       ;

;         (5)   Save HEX ROM Image          ;

;                                           ;

; Enter Desired Operation  (0 to Quit):     ;

HMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMM<

Ordinarily you will have available either a TN208.HEX file in the Intel Hex

format to treat, or a TN208.ROM file which is the binary format.  Use the

format which is preferable to you.  Following your selection of either number 1

or 2 from the window, SET208 will ask you to list the name of the file you

want loaded.   SET208 is designed to only accept the file name WITHOUT the

extension.  When you make your choice of either number 1 or number 2, it KNOWS

the file to be loaded will either be a .HEX or .ROM extension.  (If you try to

fool it, it'll let you know!)

After it feeds on your file choice, it will burp and display the main menu

again.  Here, your obvious choice is to select number 3, so you can set up the

specifics you want for YOUR node.  Other than installing the callsign/SSID,

node alias and password, there is not much else to do unless you see where the

parameters listed are way out of line.  Normally the recommended parameters

are close enough for you to get your node up and running.  Remember, if one of

the first 16 should not be suitable, you always have the option to change it

later by remotely SYSOPing it.  The Parameters following number 16 cannot be

changed after the node chip is programmed.

It should be noted  the values listed on the parameter menu are the same as

associated with the particular TN208.ROM or .HEX file you are working with.

Since this is subject to variation (for instance, the file passed on to you may

have had the parameters changed somewhere along the line), we have included the

recommended node default parameters in the following menu.  Suggest these be

printed out and compared with the ROM image you are working with.

PARAMETER MENU

with

Recommended node default parameters

IMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMM;

; 1.  Minimum Quality for Update 86       2.       HDLC Channel Quality 192   ;

; 3.      RS-232 Channel Quality 255      4.    Obs Count Initial Value 6     ;

; 5.  Obs Count Minimum for Bcst 5        6.  Nodes Bcst Interval (sec) 1800 ;

; 7.                 FRACK (sec) 6        8.                   MAXFRAME 7     ;

; 9.          Link Maximum Tries 10      10.                Digipeating Off   ;

;11.          Validate Callsigns On      12.         Host Mode Connects Off   ;

;13.              TxDELAY (10ms) 30      14.         Broadcast via Port Both  ;

;15.                 # Propagate Off     16.        Conn Command Enable On    ;

;17.     Destination List Length 100     18.   Time-to-live Initializer 32    ;

;19.     Transport Timeout (sec) 180     20.    Transport Maximum Tries 3     ;

;21.   Transport ACK Delay (sec) 3       22. Transport Busy Delay (sec) 180   ;

;23.       Transport Window Size 4       24.  Congestion Control Thresh 4     ;

;25.   No-Activity Timeout (sec) 900     26.       P-Persistance Thresh 64    ;

;27.            Slot Time (10ms) 10      28.     Link T2 Timeout (10ms) 100   ;

;29       Link T3 Timeout (10ms) 0       30.         Station ID Beacons Used  ;

;31.               CQ Broadcasts On      32.          Heard list Length 20    ;

;33.                 Full Duplex Off     34.        Node Callsign   NJ7P  -4  ;

;35.            Node Alias      SVATST   36.              Node Password       ;

;                                                                             ;

;Enter Number of Parameter to Change (0 to exit):                             ;

HMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMM<

The Password string requires a minimum of 5 characters and no more than 40

characters.

When finished with the parameter menu, select 0 and you will be returned back

to the main menu.  Here choices 4 or 5 are the ones to zero in on. This choice

should be based on whether your EPROM programmer likes a binary or a Hex file

to work with.  Again, SET208 will assign the appropriate extension, so it is

only necessary to select a suitable file name, such as your node alias when

the filename prompt appears.

If any questions or problems develop, drop me a msg via my BBS address.

73 and happy NodeOping!

de Bill, NJ7P

February 27, 1991