\*****************************************************************************/

;*                                                                           */

;*                                                                           */

;*                                                                           */

;*     *****                      *****                                      */

;*       *****                  *****                                        */

;*         *****              *****                                          */

;*           *****          *****                                            */

;*  ***************       ***************                                    */

;*  *****************   *****************                                    */

;*  ***************       ***************                                    */

;*           *****          *****          TheNet and TheNet Plus            */

;*         *****              *****        Portable. Compatible.             */

;*       *****                 *****       Public Domain                     */

;*     *****                     *****     NORD><LINK                        */

;*                                                                           */

;* This software is public domain ONLY for non-commercial use                */

;*                                                                           */

;*****************************************************************************/

;*****************************************************************************/

;*                                                                           */

;*                                                                           */

;*                                                                           */

;*                       THENET PLUS NODEOP MANUAL                           */

;*                             Version 2.08                                  */

;*                                                                           */

;*                              March  1991                                  */

;*                                                                           */

;*           **************************************************              */

;*                                                                           */

;*                                                                           */

;*                                                                           */

;*                                                                           */

;*                                                                           */

;*                                                                           */

;*                                                                           */

;*                                                                           */

;*                                                                           */

;*                 ***************************************                   */

;*                    *********************************                      */

;*                                                                           */

;*                                                                           */

;*                    Programming: William A. Beech, NJ7P                    */

:*                     Documentation: Jack Taylor, N7OO                      */

;*                      Editing: Neil Petersen, WD9DQH                       */

;*                                                                           */

;*                              *************                                */

;*                                *********                                  */

;*                                                                           */

;*                                                                           */

;*                                                                           */

;*                                                                           */

;*                                                                           */

;*****************************************************************************/

TNP2.08 UPDATE

(A supplement to TNPLUS206.DOC)

TheNet Plus version 2.08 incorporates several additional features over those

found in version 2.06.  For a detailed understanding of NodeOp considerations,

the NodeOp manual as presented in the TN206.DOC file should be consulted.  This

file was included with the version 2.06 distribution.

TNP2.08 has the following features over those found in TheNet version 1.0:

*1.  Aliases have replaced callsigns in the node ROUTES list.

2.  "Not found <aliascall>" response to an unknown node (instead of a

nodes dump).

3.  (B)ye command added.

4.  (H)eard command added (lists up to a max of 20 users over a 15 minute

period).

5.  INFO section completely SYSOP settable.

6.  TXD is remotely SYSOP settable.

7.  STA LED lights when a user connects to the node.

8.  ON - OFF remote control feature added.

9.  SYSOP KEY commands allow keying node radio with mark, space, or

diddle alignment tones.

10. Both L2 and L3 network management capabilities are available.

11. The lengthy Parameter command response has been shortened to those

parameters most commonly used during remote situations.

12. The SYSOP Password routine has been modified (in conjunction with

the new SET208.EXE utility) and is more versatile than that found in

version 2.06

13. A Parameter has been added to allow/disallow node broadcasts (beacons)

on either the RS-232 or HDLC (radio) ports.

14. A Parameter has been added to allow/disallow the propagation of the

# (hidden) nodes.

15. A Parameter has been added to allow/disallow CONNECT commands.

Features 10 through 15 above are new with version 2.08.  The emphasis of these

new features is aimed at giving NodeOps and network managers the tools

necessary for them to configure their systems to conform with local policy.

*  Included with this distribution is a varient ROM image which has BOTH call-

signs and  aliases in the ROUTES list.  This feature was requested by a group

which uses the same #node alias throughout their backbone trunking system.  The

208B.ROM file ("B" for backbone) otherwise has all the features of version

2.08.

NETWORK MANAGEMENT - BACKGROUND

To a large extent, most of todays amateur packet node network has just

"happened".  From local region to local region, public spirited individuals and

groups have patched together the network.  This has been primarily driven by a

desire to facilitate BBS activities.

Unfortunately the user applications (increase in BBS bulletin activity, TCP/IP)

have grown at a much greater rate than the network's capability to handle the

additional traffic.  This situation leaves the network builders with the

following options:

1.  Ignore the problem.

2.  Upgrade the system.

3.  Apply network management techniques.

Up to now, most of our effort has been concentrated on option #1.  Many of us

would like to upgrade the system, but are hindered by a lack of general

knowledge and by the expense of doing this.  Due to both a lack of knowledge

and lack of node capability, option #3 has not been generally pursued.  How-

ever if the future growth of our network is to be healthy, it is important

we learn and apply, network management.  Otherwise our upgrade attempts are apt

to just end up with a larger congested system.  A system which in turn will

require additional upgrades sooner than necessary.

While network management techniques will not stop the desirability of adding a

system upgrade, they will allow us to use our present resources more wisely.

Here are a couple examples of network management:

The Southern Ontario Packet Radio Association (SOPRA) is blessed by having

professional network types within their organization.  They surveyed the

coverage area in the Toronto area to determine what kind of system would be

necessary to serve their membership.  Today they run a relatively closed

system.  It consists of a 9600 baud backbone trunk tying together 5 or 6 1200

baud LAN nodes.  They have ONE multiport BBS which adequately serves all of

the members.  They disallow all TCP/IP nodes.  This doesn't mean they are

anti-TCP/IP per se, but recognized such activity could overwhelm their system.

By the same token, they recognized several BBSes within the same area is not

very efficient due to the multiple forwarding (and hence, higher network

usage) that needs to take place between them.

Today the SOPRA system is performing quite nicely.  A user can connect through-

out the network reliably with little fear of being prematurely disconnected.

Stepping into the outside world from their system, this situation dramatically

changes!  One finds the usual simplex hodge-podge overwhelmed with too many

BBSes and IP nodes.

The SOPRA group obviously defined the purpose of their system was to serve

their member users, as efficiently as possible, with a BBS and ragchew

capability.  They shaped their network to support this goal.  The main tool

allowing them to prevent outside encroachment from diluting their system was a

specially designed node chip.  This chip has the capability to ignore

uncoordinated nodes.

In the densely populated Los Angeles area, portions of the network are designed

to support BBS forwarding.  Most of the NodeOps are also BBS SysOps and use the

same specially designed node chip to promote this activity.  Uncoordinated

nodes, as a result, rapidly become history.  Without this tool, the forwarding

networks would soon saturate with multi-user activity.

Version 2.08 now has this network management capability.  This is a powerful

tool and like anything else, is capable of being misused.  NodeOps and net-

work managers should carefully use it ONLY to support the purpose and integrity

of their local system.

L2/L3 CALLSIGN DISALLOWANCE

Here is how it works:  TheNet has two logical ports, port 0 (the radio port)

and port 1 (the RS-232 port).  In version 2.08 two additional ports have been

added, ports 2 and 3.  These ports don't go anywhere.  If calls are assigned

to these ports, they don't go anywhere either.  Port 2 is designed to be SSID

specific and port 3 is effective with any SSID.

Assume we have a "rogue BBS" from an outside area who insists on forwarding

through our network.  He has been contacted and advised it would be a much more

efficient use of the network to instead forward from one of our local BBSes.

He refuses to honor our request.  It is then agreed this station should be

disallowed from our portion of the network.  At the version 2.08 node nearest

to the "rogue" we would go into the SYSOP mode and perform the following

command:

R 2 <callsign with ssid> + 0   (To disallow a specific callsign/ssid)

or,

R 3 <callsign> + 0   (to disallow this callsign with ANY ssid)

If this is a non-node callsign, it will be prevented entry to the node

immediately unless it is currently connected.  If the callsign is connected

when the entry is made, it will be prevented access upon a subsequent connect

attempt.  Following this entry, non-node callsigns will also be disallowed

from being listed on the Heard list.  Callsigns associated with nodes will

similarly be treated with the exception that the aliases will decrement out

of the node and/or routes tables in a normal manner following posting.

It is recommended port 2 be used for node and port 3 for non-node callsigns.

The entries of the disallowed calls will show in the routes list during SYSOP

control, but otherwise will not be seen.  If after due consideration, the

"rogue BBS" decides he wants to make efficient use of the network and forward

with a local BBS, he can be removed from the port 2/3 list by the SYSOP

command:

R 2 <callsign with ssid> - 0

or,

R 3 <callsign> - 0

PARAMETER STRING

In viewing hundreds of node responses to the Parameter command from around the

country, it was noted that from node to node, many of those in the normal 26

parameter string are identical.  As additional features were added to TheNet

Plus, it required additional parameters be added.  As a result, version 2.06

has a rather formidable looking parameter string composed of 30 groups.  With

version 2.08, this string has been reduced to 16 groups.  This doesn't mean the

"missing" parameters have gone away.  They are still available to be custom set

with the SET208.EXE utility prior to programming the node chip, if the NodeOp

desires to change the existing default values.  But under normal circumstances,

these parameters are rarely changed.

The parameters that appear in response to a node "P" command have been re-

arranged to appear in order at the beginning of the SET208.EXE utility as

indicated:

1.  Minimum Quality for Update

2.  HDLC (Radio port) Channel Quality

3.  RS-232 port Channel Quality

4.  Obsolescence Count Initial Value

5.  Obsolescence Minimum Count to be Broadcast

6.  Nodes Broadcast Interval

7.  FRACK

8.  MAXFRAME

9.  Link Maximum Tries

10. Digipeating

11. Validate Callsigns

12. Host Mode Connects

13. TxDelay

14. Broadcast via Port

15. Propagate # Nodes

16. Connect Commands

PASSWORD STRING

The Password string in version 2.08 requires a minimum of 5 and up to a maximum

of 40 characters.  These can be a combination of printable characters and

spaces.  SET208.EXE will automatically force spaces into that portion of the

string that may be unused thus preventing previously loaded material from

interfering with the new password.  (This procedure is different than in

version 2.06)

NODE BROADCASTS

As part of the network management process, there may be situations where

propagating node broadcasts through either the HDLC radio or the RS-232 port

may not be wanted.  On a backbone trunked LAN system for instance, it might be

decided no useful purpose is served by a LAN node sending out broadcasts on the

LAN frequency (assuming no other radio nodes are on the channel).  By turning

off the broadcasts on port 0, an improvement in LAN throughput will be realized

since a periodic source of unnecessary node transmissions has been eliminated.

There also may be a situation where an occasional "propagation" node is on the

LAN frequency.  This node may be located at a point within the network where

using the port 2 feature might not be appropriate.  By turning off the port 0

broadcasts on both nodes, confusing routing paths will not be introduced into

the system.  Another way to ignore ALL incoming node broadcasts on the radio

port would be to set Parameter 1 (Min quality for updates) to a value higher

than the setting of Parameter 2 (HDLC channel quality).

A possible application of shutting off the broadcasts from a LAN node to the

backbone node might exist on a network with duplicate gateways.  Here the

network planners may desire that the traffic be routed through a specific

gateway.  In case the primary gateway failed, then the "backup gateway" could

be SYSOPed into service.

Users of this feature are cautioned once the broadcasts are halted on any

port, nodes off that port will soon lose any memory of your node unless the

N perming command is appropriately used.

The SYSOP commands for this (parameter 14) feature are:

0 = Broadcasts disabled on all ports.

1 = Broadcasts enabled on port 0 (radio) only.

2 = Broadcasts enabled on port 1 (RS-232) only.

3 = Broadcasts enabled on both ports 0 and 1 (this is the normal, or

default setting).

HIDDEN NODES

The "Node star" (N *) command is of primary interest to NodeOps that operate

hidden nodes.  As a matter of routine they use this command to make sure their

# nodes are functioning.  The majority of other users have little interest in

the status of the hidden nodes.  Even so, the hidden nodes propagate throughout

the network the same as regular nodes.  In this process they create additional

(and oftentimes unnecessary) network overhead up and down the line.

New with version 2.08 is a routine that prevents most (but not all) hidden

node propagation.  The exception to this are hidden nodes which are either

permed or RS-232 (port 1) connected.  This feature is found at Parameter 15.

The SYSOP # Propagate commands are:

0 = Off (hidden nodes do NOT propagate; the normal or default setting).

1 = On (hidden nodes DO propagate).

NodeOp usage of this feature will depend on the configuration of his nodes.

The vast majority of NodeOps will find that the default setting will adequately

allow him to check out his portion of the network without creating unneeded

congestion for his neighbors.  In addition, the default setting does not affect

the propagation of the regular network nodes.

Note: While permed or RS-232'd hidden nodes are broadcast by version 2.08,

this only applies to those nodes that are DIRECTLY permed or RS-232'd.  Another

version 2.08 node downstream with Parameter 15 set to 0 will ignore the

existence of these hidden neighbor nodes.

CONNECT COMMAND

In the quest for the perfect system, some networks employ the "Protected

Backbone Trunking Concept".  This is where only one transmitter per backbone

frequency is allowed.  The concept requires multiple backbone trunking

frequencies on the system and is used in order to avoid "hidden transmitters"

from slowing system throughput.  Up to a five-fold improvement has been

reported, even at 1200 baud.  For a protected trunk to be successful, it is

necessary to discourage users from directly accessing the backbone.  High

volume users typified by uplinking BBS or TCP activities are the ones of

principle concern here.

Ideally all users would agree and cooperate by not directly uplinking to the

backbone trunk.  In circumstances where this is not possible, version 2.08

is capable of managing the situation.  One approach would be to use the

Heard and Users features to determine if activity is taking place.  If such

activity is noted, then the L2/L3 callsign disallowance feature can be

employed with specific offenders.

NodeOps can alternatively use the node disable CONNECT command feature.  A user

uplinking or otherwise connecting to a protected node will be allowed the use

of all node commands except the CONNECT command.  Thus by not being able to

connect FROM the node, such usage is discouraged and efficient network through-

put is maintained.

While throughput is enhanced by preventing users from directly UPLINKING to a

protected (or, for that matter, any) backbone trunk, there is no penalty in

allowing users to connect to protected nodes via the network.  Indeed, for

NodeOp diagnostic and out-of-area user mapping expeditions, it's desirable to

normally allow user connects (through the network) to these nodes.  It is

therefore cautioned that the CONNECT disable be used as a last resort.  Even

then, perhaps only on an intermittent as needed basis.

The SYSOP commands for this feature (Parameter 16) are:

0 = Off (Connects disallowed, user connect request will be quietly ignored)

1 = On (Connects allowed; the default setting)

Another application where Parameter 16 would be useful is that of a dedicated

point-to-point HF/VHF node-gateway circuit.  Here, HF user activity can

seriously degrade network throughput between the two node-gateways.  By

disallowing connects from the HF node, on-channel activity is discouraged and

circuit throughput is greatly improved.  (Network connects are unaffected as

long as they are initiated from "CONNECT enabled" nodes.)

Several NodeOps have reported difficulty with node initialization.  Although

addressed in the version 2.06 NodeOp manual, it bears mentioning again:  If the

new node fails to sign on, pull the TNC battery jumper for about 1 minute.

This procedure neutralizes conflicting memory instructions remaining in the RAM from previous firmware.

The new features found in version 2.08 have been requested by NodeOps.  We hope

you find these features useful as well.  Our goal remains to make packet net-

working as pleasurable as possible.  We appreciate the comments and suggestions

received to-date.  Please address comments on the software to NJ7P and on the

documentation to N7OO, both via packet @NJ7P.AZ.USA.NA.

73 de NJ7P and N7OO